3GPP TSG-RAN WG4 Meeting #117	R4-2520107

# Dallas, US, 17th – 21st Nov, 2025

Title: 	Discussion on RRM core maintenance for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	4.25.3

Document for:	Discussion

- Introduction

In the last meeting, there were still some remaining issues have not been solved for NR_NTN_Ph3 core part. This document will further discuss the RRM impact for UE capable of coverage enhancement and give our understanding and proposals.

- Discussion

- SMTC enhancement for UE capable of coverage enhancement

- More than 4 SMTCs per frequency

In RAN4#116 meeting, RAN4 discussed the RRM impact for more than 4 SMTCs per frequency, and the wayforward is copied as follows [1]:

## [Online # 2] Issue 0-A: More than 4 SMTCs per frequency (i.e. 6) RAN4 to further discuss the following in RAN4#116-bis: - For RRC Idle/Inactive mode RRM requirements, no update is needed except for the following addition to the specification where relevant: - The SMTCs referenced in the subsequent requirements refer to the SMTCs selected by the UE. - Note: The SMTC selection rule may be further discussed and specified or left to UE implementation.

Firstly, we confirm the first bullet. In the last meeting, RAN4 discussed whether to capture the related SMTC selection rule for RRC-IDLE/Inactive mode in RAN4 spec and how to capture it, as RAN2 had some related agreements as follows [2], but there was still no consensus.

- We introduce a capability to support STMC enhancements to FR1 (FFS on FR2 depending on RAN4 progress). - The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs). - RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements.

And the related capability defined in RAN2 spec (TS 38.306) is as follows [3]:

Location based SMTC selection for RRC_IDLE/RRC_INACTIVE It is optional for the UE in RRC_ILDE/RRC_INACTIVE to support SMTC selection based on UE position and associated location for each SMTC in NTN, as specified in TS 38.331 [9].

Therefore, we support to use the same description as TS 38.306 to support SMTC selection based on UE position and associated location for each SMTC in NTN (as specified in TS 38.331).

Proposal 1: If the NW configures more than 4 STMCs, RAN4 to use the same description as TS 38.306 to support SMTC selection based on UE position and associated location for each SMTC in NTN (as specified in TS 38.331) in RRC_ILDE/RRC_INACTIVE mode.

- Two different SMTC periodicities (with different offsets) in one frequency layer

In RAN4#116 meeting, RAN4 discussed the related issues about multiple SMTCs with different periodicities for downlink coverage enhancement, and the following agreements were reached [1]:

## [Online # 3] Issue 0-B: Two different SMTC periodicities (with different offsets) in one frequency layer Agreement (online): - When two different SMTC periodicities (with different offsets) in one frequency layer are configured, - No requirements if the SMTC periodicity is smaller than the actual SSB periodicity of the cells associated with the SMTC - For Trs in Handover requirement (resolved in RAN4#116-bis): - Trs is the SMTC corresponding to the physical cell ID of the target cell - Further discuss the proposal “or the longer SMTCconfigured in the measObjectNR having the same SSB frequency and subcarrier spacing.” - For Trs in RRC Connection Release with Redirection requirement (resolved in RAN4#116-bis): - If physical cell ID is provided in RAN2 signalling design, Trs is the SMTC corresponding to the physical cell ID of the target cell - Otherwise, Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing. - Kp and Kgap in RRC Connected mode intra/inter-frequency measurement requirement - The SMTC period in the requirement is the one used by the cell being identified Agreement (ad-hoc): - When two different SMTC periodicities (with different offsets) in one frequency layer are configured, - For Ttarget_cell_SMTC_period in paging interruption requirements: - Ttarget_cell_SMTC_period is the SMTC periodicity corresponding to the physical cell ID of the target cell - For Kmulti_SMTC, K_satellite, Kp, Klayer1_measurement, and Kgap in RRC Connected mode intra/inter-frequency measurement requirement: - To be confirmed in maintenance phase: - Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i. - Klayer1_measurement: Calculate scaling factor Klayer1_measurement based on the union of the SMTCs

When two different SMTC periodicities (with different offsets) in one frequency layer are configured, there were the issue of Kmulti_SMTC  calculation left. The related RAN2 agreements on SMTC enhancements of different SMTC periodicities is copied as following [2]:

- RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only.

According to RAN2 agreement, two different SMTC periodicities are applicable for idle/inactive/connected mode not only connected mode, so RAN4 need to consider the impact of two different SMTC periodicities for idle/inactive/connected mode.

Proposal 2: RAN4 need to consider the impact of two different SMTC periodicities (with different offsets) in one frequency layer for idle/inactive/connected mode.

For configuring two different SMTC periodicities (with different offsets) in one frequency layer, after RAN4#116-bis meeting, since the existing calculation for Kmulti_SMTC/K_satellite didn’t consider the multi SMTC periodicities scenario, so the calculation for Kmulti_SMTC/K_satellite for idle/inactive/connected mode neighbor cell measurements need to be discussed.

Firstly, for the case of SMTCs partially overlapping with each other, as 2 SMTC with different periodicity configuration may not always be collided for every SMTC occasion, so we believe that the definition of overlapping need to be further clarified. A simple method is to reuse the same method in RRC_IDLE mode for deriving Kmulti_SMTC/K_satellite as following [4]:

Note: for deriving Kmulti_SMTC for Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tdetect,NR_Intra, Tmeasure,NR_Intra or Tevaluate,NR_Intra.

Observation 1: For the case of SMTCs partially overlapping with each other, as 2 SMTC with different periodicity configuration may not always be collided for every SMTC occasion.

Proposal 3: RAN4 to clarify that the definition of overlapping when configuring two different SMTC periodicities (with different offsets) in one frequency layer, such as: two SMTCs are considered as overlapping if they overlap in one or more occasions.

Then, the scenario becomes very complicated, for the calculation of Kmulti_SMTC/K_satellite, different SMTC occasions in one SMTC may have different overlap cases, to define the minimum requirements, we support to use the maximum K, that is, for each SMTC#i, Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i.

Proposal 4: For each SMTC#i, Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i.

- Conclusion

This document discussed the issues about RRM core maintenance and the following observation and proposals are presented:

Proposal 1: If the NW configures more than 4 STMCs,  RAN4 to use the same description as TS 38.306 to support SMTC selection based on UE position and associated location for each SMTC in NTN (as specified in TS 38.331) in RRC_ILDE/RRC_INACTIVE mode.

Observation 1: For the case of SMTCs partially overlapping with each other, as 2 SMTC with different periodicity configuration may not always be collided for every SMTC occasion.

Proposal 2: RAN4 need to consider the impact of two different SMTC periodicities (with different offsets) in one frequency layer for idle/inactive/connected mode.

Proposal 3: RAN4 to clarify that the definition of overlapping when configuring two different SMTC periodicities (with different offsets) in one frequency layer, such as: two SMTCs are considered as overlapping if they overlap in one or more occasions.

Proposal 4: For each SMTC#i, Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i.

- Reference

- R4-2512337, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)

- R2-2506298, LS on additional agreements on SMTC enhancements, RAN2

- 3GPP TS 38.306, NR User Equipment (UE) radio access capabilities

- 3GPP TS 38.133, NR Requirements for support of radio resource management