3GPP TSG-RAN WG4 Meeting # 117                                                             R4-2520125

Dallas, USA, Nov. 17-21, 2025

Title: 	Discussion on BS conformance requirements for potentially new requirements for SBFD operation in FR1 and FR2-1

Source: 	CATT

Agenda item:	6.13.2.1

Document for:	Discussion

- Background

According to the WF[1] of RAN4#116bis, the RF conformance testing of new requirements for SBFD operation requires further study, this paper provides additional analysis on these aspects.

- General of conformance testing for SBFD

## 2.1 Requirement set applicability

Table 1: Requirement set applicability

|  | Requirement Set for non-SBFD symbols/slots |  | Requirement Set for SBFD symbols/slots |
| --- | --- | --- | --- |
| Requirement | Requirement set |  | Requirement set |
|  | 1-C | 1-H | 1-H |
| BS output power | 6.2 | 6.2 | [6.2] |
| Output power dynamics | 6.3 | 6.3 | [6.3] |
| Transmit ON/OFF power | 6.4 | 6.4 | [6.4] |
| Transmitted signal quality | 6.5 | 6.5 | [6.5] |
| Occupied bandwidth | 6.6.2 | 6.6.2 | [6.6.2] |
| ACLR | 6.6.3.5.3 | 6.6.3.5.4 | [6.6.3.5.4] |
| Operating band unwanted emissions | 6.6.4.5.7 | 6.6.4.5.8 | [6.6.4.5.8] |
| Transmitter spurious emissions | 6.6.5.5.3 | 6.6.5.5.4 | [6.6.5.5.4] |
| Transmitter intermodulation | 6.7.5.1 | 6.7.5.2 | [6.7.5.2] |
| Transient period between SBFD and non-SBFD | NA | NA | TBD |
| In-channel adjacent subband leakage | NA | NA | TBD |
| Reference sensitivity level | 7.2 | 7.2 | [7.2] |
| Dynamic range | 7.3 | 7.3 | [7.3] |
| In-band selectivity and blocking | 7.4 | 7.4 | [7.4] |
| Out-of-band blocking | 7.5 | 7.5 | [7.5] |
| Receiver spurious emissions | 7.6.5.2 | 7.6.5.3 | [7.6.5.3] |
| Receiver intermodulation | 7.7 | 7.7 | NA |
| In-channel selectivity | 7.8 | 7.8 | [7.8] |
| In-channel adjacent subband blocking | NA | NA | TBD |
| Performance requirements | 8 | 8 | [8] |

The requirement set applicability of SBFD need to be added to TS 38.141-1, as shown in Table 1, the specific clauses corresponding to each requirement can be supplemented after determination.

Proposal 1: The requirement set applicability of SBFD need to be added to TS 38.141-1, as shown in Table 1.

Table 2: Requirement set applicability

|  | Requirement Set for non-SBFD symbols/slots |  |  | Requirement Set for SBFD symbols/slots |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Requirement | Requirement set |  |  | Requirement set |  |  |
|  | BS type 1-H | BS type 1-O | BS type 2-O | BS type 1-H | BS type 1-O | BS type 2-O |
| Radiated transmit power | 6.2 | 6.2 | 6.2 | [6.2] | [6.2] | [6.2] |
| OTA base station output power |  | 6.3 | 6.3 | NA | [6.3] | [6.3] |
| OTA output power dynamics |  | 6.4 | 6.4 |  | [6.4] | [6.4] |
| OTA transmit ON/OFF power |  | 6.5 | 6.5 |  | [6.5] | [6.5] |
| OTA transmitted signal quality |  | 6.6 | 6.6 |  | [6.6] | [6.6] |
| OTA occupied bandwidth | NA | 6.7.2 | 6.7.2 |  | [6.7.2] | [6.7.2] |
| OTA ACLR |  | 6.7.3 | 6.7.3 |  | [6.7.3] | [6.7.3] |
| OTA out-of-band emission |  | 6.7.4 | 6.7.4 |  | [6.7.4] | [6.7.4] |
| OTA transmitter spurious emission |  | 6.7.5 | 6.7.5 |  | [6.7.5] | [6.7.5] |
| OTA transmitter intermodulation |  | 6.8 | NA |  | [6.8] | NA |
| OTA spatial emission | 6.9 | 6.9 | NA | NA | [6.9] | NA |
| OTA transient period between SBFD and non-SBFD | NA | NA | NA | NA | TBD | TBD |
| OTA in-channel adjacent subband leakage | NA | NA | NA | NA | TBD | TBD |
| OTA sensitivity | 7.2 | 7.2 | NA | [7.2] | [7.2] | NA |
| OTA reference sensitivity level |  | 7.3 | 7.3 | NA | [7.3] | [7.3] |
| OTA dynamic range |  | 7.4 | NA |  | [7.4] | NA |
| OTA in-band selectivity and blocking |  | 7.5 | 7.5 |  | [7.5] | [7.5] |
| OTA out-of-band blocking | NA | 7.6 | 7.6 |  | [7.6] | [7.6] |
| OTA receiver spurious emission |  | 7.7 | 7.7 |  | [7.7] | [7.7] |
| OTA receiver intermodulation |  | 7.8 | 7.8 |  | NA | NA |
| OTA in-channel selectivity |  | 7.9 | 7.9 |  | [7.9] | [7.9] |
| OTA in-channel adjacent subband blocking | NA | NA | NA | NA | TBD | TBD |
| Radiated performance requirements | NA | 8 | 8 | NA | [8] | [8] |

The requirement set applicability of SBFD need to be added to Table 4.8.1-1 in TS 38.141-2, as shown in Table 2, the specific clauses corresponding to each requirement can be supplemented after determination.

Proposal 2: The requirement set applicability of SBFD need to be added to TS 38.141-2, as shown in Table 2.

## 2.2 RF channels to be tested for SBFD operation

Table 3: RF channels to be tested for single carrier for conducted conformance testing

|  | RF channels to be tested for single carrier (SC) |  |
| --- | --- | --- |
| Requirement | For non-SBFD symbols/slots | For SBFD symbols/slots |
| BS output power | B,M,T | B,M,T |
| Output power dynamics | M | M |
| Transmit ON/OFF power | M | M |
| Transmitted signal quality | B,M,T | B,M,T |
| Occupied bandwidth | M | M |
| ACLR | B,M,T | B,M,T |
| Operating band unwanted emissions | B,M,T | B,M,T |
| Transmitter spurious emissions | B,T | B,T |
| Transmitter intermodulation | M | M |
| Transient period between SBFD and non-SBFD | NA | M |
| In-channel adjacent subband leakage | NA | B,M,T |
| Reference sensitivity level | B,M,T | B,M,T |
| Dynamic range | M | M |
| In-band selectivity and blocking | M | M |
| Out-of-band blocking | M | M |
| Receiver spurious emissions | M | M |
| Receiver intermodulation | M | NA |
| In-channel selectivity | M | M |
| In-channel adjacent subband blocking | NA | M |

Table 4: RF channels to be tested for single carrier for radiated conformance testing

|  | RF channels to be tested for single carrier (SC) |  |
| --- | --- | --- |
| Requirement | For non-SBFD symbols/slots | For SBFD symbols/slots |
| Radiated transmit power | B,M,T | B,M,T |
| OTA base station output power | B,M,T | B,M,T |
| OTA output power dynamics | M | M |
| OTA transmit ON/OFF power | M | M |
| OTA frequency ERROR | B,T | B,T |
| OTA transmitted signal quality | B,T | B,T |
| OTA occupied bandwidth | M | M |
| OTA ACLR | B,T | B,T |
| OTA out-of-band emission | B,M,T | B,M,T |
| OTA transmitter spurious emission | B,T | B,T |
| OTA transmitter intermodulation | M | M |
| OTA spatial emission | M | M |
| OTA transient period between SBFD and non-SBFD | NA | M |
| OTA in-channel adjacent subband leakage | NA | B,T |
| OTA sensitivity | M | M |
| OTA reference sensitivity level | B,M,T | B,M,T |
| OTA dynamic range | M | M |
| OTA in-band selectivity and blocking | M | M |
| OTA out-of-band blocking | M | M |
| OTA receiver spurious emission | B,T | B,T |
| OTA receiver intermodulation | M | NA |
| OTA in-channel selectivity | M | M |
| OTA in-channel adjacent subband blocking | NA | M |

For the SBFD BS conducted and radiated conformance testing, the configuration of the RF channel to be tested for each requirement can reuse the relevant configuration of the NR BS single carrier, as shown in table 3 for conducted conformance testing and table 4 for TS 38.141-2 for radiated conformance testing.

Proposal 3: For the SBFD BS conducted conformance testing, the configuration of the RF channel to be tested for each requirement can reuse the relevant configuration of the NR BS single carrier, as shown in table 3.

Proposal 4: For the SBFD BS radiated conformance test, the configuration of the RF channel to be tested for each requirement can reuse the relevant configuration of the NR BS single carrier, as shown in table 4.

- Conformance testing for new RF requirements

3.1 Additional manufacture declarations and updates

Following requirements are newly introduced, and need to be added to both TS 38.141-1 and TS 38.141-2.

•Transient period between SBFD and non-SBFD

•	Absolute limit for in-channel adjacent subband leakage

•	In-channel adjacent subband blocking

Proposal 5: Following new requirements for SBFD capable BS need to be added to both TS 38.141-1 and TS 38.141-2.

•	Transient period between SBFD and non-SBFD

•	Absolute limit for in-channel adjacent subband leakage

•	In-channel adjacent subband blocking

3.2 Additional test models for SBFD capable BS

Transmitter output power in UL subband/DL subband(s)

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 1: Transient period between SBFD and non-SBFD

For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD, as shown in figure 1.

Proposal 6: For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD.

For SBFD capable BS, the desired test model and frequency location of wanted signal for in-channel adjacent subband leakage requirement testing are the same as those used for BS ACLR absolute basic limit test, but with different test frequency ranges. These contents need to be added to TS 38.141-1 and TS 38.141- 2

Proposal 7: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are the same as those used for BS ACLR absolute basic limit testing, but with different test frequency ranges.

Proposal 8: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are necessary to added to both TS 38.141-1 and TS 38.141-2.

Proposal 9: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the test frequency range is necessary to added to both TS 38.141-1 and TS 38.141-2.

For SBFD capable BS, the desired Rx input signal and its frequency location for in-channel adjacent subband blocking testing are the same as those used for in-band blocking requirement. When test this requirement, the transmitters must be turned on and the test frequency range is different from in-band blocking. These contents need to be added to TS 38.141-1 and TS 38.141- 2

Proposal 10: For in-channel adjacent subband blocking testing of SBFD capable BS, the transmitters must be turned on.

Proposal 11: For in-channel adjacent subband blocking testing of SBFD capable BS, the desired Rx input signal and its frequency location are the same as those used for in-band blocking requirement.

Proposal 12: For SBFD capable BS, the test frequency range of in-channel adjacent subband blocking is necessary to added to both TS 38.141-1 and TS 38.141-2.

- Summary

In this contribution, we have the following proposals.

Proposal 1: The requirement set applicability of SBFD need to be added to TS 38.141-1, as shown in Table 1.

Proposal 2: The requirement set applicability of SBFD need to be added to TS 38.141-2, as shown in Table 2.

Proposal 3: For the SBFD BS conducted conformance testing, the configuration of the RF channel to be tested for each requirement can reuse the relevant configuration of the NR BS single carrier, as shown in table 3.

Proposal 4: For the SBFD BS radiated conformance test, the configuration of the RF channel to be tested for each requirement can reuse the relevant configuration of the NR BS single carrier, as shown in table 4.

Proposal 5: Following new requirements for SBFD capable BS need to be added to both TS 38.141-1 and TS 38.141-2.

•	Transient period between SBFD and non-SBFD

•	Absolute limit for in-channel adjacent subband leakage

•	In-channel adjacent subband blocking

Proposal 6: For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD.

Proposal 7: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are the same as those used for BS ACLR absolute basic limit testing, but with different test frequency ranges.

Proposal 8: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are necessary to added to both TS 38.141-1 and TS 38.141-2.

Proposal 9: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the test frequency range is necessary to added to both TS 38.141-1 and TS 38.141-2.

Proposal 7: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are the same as those used for BS ACLR absolute basic limit testing, but with different test frequency ranges.

Proposal 8: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are necessary to added to both TS 38.141-1 and TS 38.141-2.

Proposal 9: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the test frequency range is necessary to added to both TS 38.141-1 and TS 38.141-2.

Reference

[1] R4-, “Way Forward for [116][307] NR_duplex_evo_BSRF”, Huawei, RAN4#116bis