3GPP TSG-RAN WG4 Meeting # 117	R4-2520140

# Dallas, USA, 17th November – 21st November

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR Test cases requirements with FBS for RRC re-establishment and RRC connection release with redirection to NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-10-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UE RRC re-establishment delay requirements and RRC connection release with redirection to NR requirements miss test cases in the RRM spec for UE capable of reduced beam sweeping factor. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - UE RRC re-establishment delay requirements are tested for intra-frequency FR2-1 target cell when serving cell timing is available to UE. - UE RRC re-establishment delay requirements are tested for inter-freuqency FR2-1 target cell when serving cell timing is not available to UE. - UE RRC connection release with redirection to NR delay requirements are tested for inter-frequency target cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases requirement is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses: A.7.3.2.1.7, A.7.3.2.1.8, A.7.3.2.3.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | N/A. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | N/A. |  |  |  |  |  |  |  |  |

==============First change==============

A.7.3.2.1.7	Intra-frequency RRC Re-establishment in FR2 with UE capable of reduced beam sweeping factor

A.7.3.2.1.7.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR2 without known target cell is within the specified limits, where the UE is capable of reduced beam sweeping factor. These tests will verify the requirements in clause 6.2.1. While inter-frequency RRC re-establishment delay for UE capable of reduced beam sweeping factor is tested under the condition that the serving cell timing is not available to the UE in clause A.7.3.2.1.8, intra-frequency RRC re-establishment delay test case for UE capable of reduced beam sweeping factor is tested under the condition that the serving cell timing is still available to UE in this clause.

The test parameters are given in table A.7.3.2.1.7.1-1, table A.7.3.2.1.7.1-2 and table A.7.3.2.1.7.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure.

Table A.7.3.2.1.7.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.1.7.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case in FR2

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell 1 |  |
|  | Neighbour cells |  | 1 | Cell 2 |  |
| Final condition | Active cell |  | 1 | Cell 2 |  |
| RF Channel Number |  |  | 1 | 1 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | FR2 PRACH configuration 1 | Table A.3.8.3.1-1 |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 4.84 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 ) |
| T3 |  | s | 1 | 3 |  |

Table A.7.3.2.1.7.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| BWchannel | MHz | 1 | 100: NPRB,c = 66 |  |  | 100: NPRB,c = 66 |  |  |
| Data PRBs allocated |  | 1 | 24 |  |  | 24 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| AoA setup |  | 1 | Setup 1 defined in A.3.15.1 |  |  | Setup 1 defined in A.3.15.1 |  |  |
|  | dB | 1 | -0.12 | -infinity | -infinity | -3.46 | 2 | 2 |
| Note2 | dBm/15 kHz | 1 | -104.7 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -95.7 |  |  |  |  |  |
|  | dB | 1 | 4 | -infinity | -infinity | 2 | 2 | 2 |
| SS-RSRP Note3 | dBm/SCS | 1 | -91.7 | -infinity | -infinity | -93.7 | -93.7 | -93.7 |
| Io | dBm/95.04 MHz | 1 | -59.64 | -62.59 | -62.59 | -59.94 | -62.59 | -62.59 |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

A.7.3.2.1.7.2	Test Requirements

The RRC re-establishment delay is defined as the time from the moment UE declares RLF, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR intra frequency cell shall be less than 3 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90 %.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1.

Tidentify_intra_NR = N1*440 + Tproc ms, where N1 = reduced N and Tproc = 2 ms.

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of:

- 1747 ms for N1=2, which allows 7.84 s (4.84 s + 3 s) from the beginning of T2 in the test case, or

- 2147 ms for N1=4, which allows 7.84 s (4.84 s + 3 s) from the beginning of T2 in the test case, or

- 2547 ms for N1=6, which allows 7.84 s (4.84 s + 3 s) from the beginning of T2 in the test case.

==============Next change==============

##### A.7.3.2.1.8	Inter-frequency RRC Re-establishment in FR2 without serving cell timing with UE capable of reduced beam sweeping factor

A.7.3.2.1.8.1	Test Purpose and Environment

The purpose is to verify that the NR inter-frequency RRC re-establishment delay in FR2 without serving cell timing is within the specified limits, when the UE is capable of reduced beam sweeping factor. These tests will verify the requirements in clause 6.2.1. While intra-frequency RRC re-establishment delay for UE capable of reduced beam sweeping factor is tested under the condition that the serving cell timing is still available to the UE in clause A.7.3.2.1.7, inter-frequency RRC re-establishment delay test case for UE capable of reduced beam sweeping factor is tested under the condition that the serving cell timing is not available to UE in this clause.

The test parameters are given in table A.7.3.2.1.8.1-1, table A.7.3.2.1.8.1-2 and table A.7.3.2.1.8.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the UE has the context of the carrier frequency of cell 2 by the end of T1. During T1 and T2, cell 2 is not detectable to the UE.

Table A.7.3.2.1.8.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.1.8.1-2: General test parameters for NR inter-frequency RRC Re-establishment test case in FR2

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell 1 |  |
|  | Neighbour cells |  | 1 | Cell 2 |  |
| Final condition | Active cell |  | 1 | Cell 2 |  |
| RF Channel Number |  |  | 1 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | FR2 PRACH configuration 1 | Table A.3.8.3.1-1 |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 4.84 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 ) |
| T3 |  | s | 1 | 5 |  |

Table A.7.3.2.1.8.1-3: Cell specific test parameters for NR inter-frequency RRC Re-establishment test case in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| AoA setup |  | 1 | Setup 3 as specified in clause A.3.15 |  |  |  |  |  |
|  |  |  | AoA1 |  |  | AoA2 |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| BWchannel | MHz | 1 | 100: NPRB,c = 66 |  |  | 100: NPRB,c = 66 |  |  |
| Data PRBs allocated |  | 1 | 24 |  |  | 24 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| Note2 | dBm/15 kHz | 1 | -92.1 |  |  | -92.1 |  |  |
| Note2 | dBm/SCS | 1 | -83.1 |  |  | -83.1 |  |  |
|  | dB | 1 | 0 | -infinity | -infinity | -infinity | -infinity | 0 |
| BB Note 5 | dB | 1 | -1.01 | -infinity | -infinity | -infinity | -infinity | -1.01 |
| SSB_RP Note3 | dBm/SCS | 1 | -83.1 | -infinity | -infinity | -infinity | -infinity | -83.1 |
| Io | dBm/95.04 MHz | 1 | -55.5 | -58.51 | -58.51 | -58.51 | -58.51 | -55.5 |
| Propagation Condition |  | 1 | AWGN |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that a constant total transmitted power is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |

A.7.3.2.1.8.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell shall be less than the delay specified below for Tre-establish_delay.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90 %.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 2.

Tidentify_intra_NR = MAX (1000, (N1 x 10 x TSMTC + Tproc)) ms, where N1 = reduced N and Tproc = 2 ms.

Tidentify_inter_NR = MAX (1000, (N1 x 13 x TSMTC, i + Tproc)) ms, where N1 = reduced N and Tproc = 2 ms.

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target inter-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of:

- 3345 ms for N1=2, which allows 5 s in the test case, or

- 3385 ms for N1=4, which allows 5 s in the test case, or

- 4107 ms for N1=6, which allows 5 s in the test case.

==============Next change==============

##### A.7.3.2.3.2	Redirection from NR in FR2 to NR in FR2 with UE capable of reduced beam sweeping factor

A.7.3.2.3.2.1	Test Purpose and Environment

This test is to verify RRC connection release with redirection from NR to NR requirements when the UE is capable of reduced beam sweeping factor, which are specified in clause 6.2.3.2.1.

A.7.3.2.3.2.2	Test Parameters

Supported test configurations are shown in table A.7.3.2.3.2.2-1. The time delay is tested by using the parameters in table A.7.3.2.3.2.2-2, and A.7.3.2.3.2.2-3.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The RRCRelease message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2.

Table A.7.3.2.3.2.2-1: Redirection from NR to NR test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.3.2.2-2: General test parameters for Redirection from NR to NR test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 3 |  |

Table A.7.3.2.3.2.2-3: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  | Rough |  |
| AoA setup |  |  |  | Setup 1as defined in A.3.15 |  |  |  |
| NR RF Channel Number |  |  |  | 1 |  | 2 |  |
| Duplex mode |  |  |  | TDD |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  |  | MHz | 100: NPRB,c = 66 |  |  |  |
| BWP BW |  |  | MHz | 100: NPRB,c = 66 |  |  |  |
| Data PRBs allocated |  |  |  | 66 |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP. 1 |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  |  |  | SSB.3 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  |  |  | TCI.State.2 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15 kHz | -104.7 |  | -104.7 |  |
| Note2 |  |  | dBm/SCS | -95.7 |  | -95.7 |  |
|  |  |  | dB | 5 | 5 | -Infinity | 5 |
|  |  |  | dB | 5 | 5 | -Infinity | 5 |
| IoNote3 |  |  | dBm/95.04 MHz | -60.5 | -60.5 | -66.7 | -60.5 |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 5: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 6: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

A.7.3.2.3.2.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than the below specified delay Tconnection_release_redirect_NR from the beginning of time period T2.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90 %.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH,

where:

TRRC_procedure_delay = 110 ms in the test.

Tidentify-NR = MAX (880, N x 11 x Trs + Tproc) ms, where N = reduced beam sweeping factor, Trs = 20 ms, Tproc = 2 ms.

TSI-NR = 1280 ms, it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target NR cell.

TRACH = 10 ms in the test.

This gives a total of:

-	2280 ms for N=2, allow 3 s in the test case for T2, or

-    2282 ms for N=4, allow 3 s in the test case for T2, or

-    2722 ms for N=6, allow 3 s in the test case for T2.

==============End of change==============