3GPP TSG-RAN WG4 Meeting #117	                                                                                            R4-2520165

Dallas, US, 13th –17th Oct, 2025

Source: 	CATT

Title: 	Discussion on remaining issue for A-IoT Device

Agenda item:	4.20.1.1

Document for:	Approval

- Introduction

In RAN4 #116-bis meeting, a maintenance CR [1] for ambient IoT Device spec TS 38.191 has been approved. However, there remain some issues that require further clean up. This paper shares some views on maintenance for ambient IoT Device spec.

- Discussion

For A-IoT Device spec TS 38.191, the clause numbering from Clause 6 onwards appears somewhat disorganized. Given that the previous meeting was a bis-meeting, the maintenance CR submitted for TS 38.191 at last meeting has not been deployed. However, in the last endorsed CR for TS 38.191, the clause number for Output RF spectrum emissions requirement has changed and not aligned with the latest version of TS 38.191. This may be due to adjustments made in certain CR to the clause numbers of all subclauses within clause 6, which were not correctly reflected in the endorsed draft CR. Therefore, we believed that for A-IoT Device spec TS 38.191, the clauses numbering from Clause 6 onwards may require reordering.

Observation 1: For A-IoT Device spec TS 38.191, the clauses numbering from Clause 6 onwards may require reordering.

Meanwhile, certain issues may still exist with regard to the current requirements for Output RF spectrum emissions, as shown below:

## 6.2 Output RF spectrum emissions ### 6.2.1 Out of band emissions #### 6.2.1.1 General The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. #### 6.2.1.2 Spectrum emission mask The spectrum emission mask of the device applies to frequencies (ΔfOOB) starting from the centre of the assigned D2R channel bandwidth. For frequencies offset greater than ΔfOOB as specified in Table 6.2.1.2-1 the spurious requirements in clause 6.2.2 are applicable. The emission within ΔfOOB of a device shall be lower than the level specified in Table 6.2.1.2-1 compared to the D2R backscatter power under same incident CW power level. The requirement is verified with the test metric of EIRP and the incident CW power level at the device set to -5 dBm. The test direction is the same as the backscatter output power as specified in clause 6.1. Table 6.2.1.2-1 Spectrum emission mask for device ΔfOOB (MHz); Spectrum emission limit (dBc); Measurement bandwidth. ± 0.5*D2R CBW - FOOB; 10; 0.5* Nominal D2R transmission Bandwidth without SFO. NOTE 1: FOOB is the OOB boundary frequency specified in Table 6.2.1.2-1..

For ΔfOOB, the current spec indicates the values as ± 0.5*D2R CBW - FOOB. However, as D2R CBW is not a fixed value, the applicability of different CBW for Output RF spectrum emissions requirements is not clear. In accordance with the discussions on Output RF spectrum emissions requirements in the past, the requirements should apply to all D2R CBWs.

Observation 2: For A-IoT Device spec TS 38.191, the applicable D2R CBW of the Output RF spectrum emissions requirements is not clear.

- Conclusion

This contribution provides our views on maintenance for ambient IoT Device spec. The following proposals are concluded as follows:

Observation 1: For A-IoT Device spec TS 38.191, the clauses numbering from Clause 6 onwards may require reordering.

Observation 2: For A-IoT Device spec TS 38.191, the applicable D2R CBW of the Output RF spectrum emissions requirements is not clear.

- References

- R4-2515142, Corrections for device RF requirements, RAN4 #116-bis