3GPP TSG-RAN WG4 Meeting #117						         	       	R4-2520193

Dallas, USA, Nov. 17-21, 2025

Title: 	Proposal on 1UL band 2CC and 2UL band 3CC HPUE MSD

Source: 	Skyworks Solutions, Inc.

Agenda Item:	4.6.1.1	High power UE (HPUE) for CA in terrestrial network (TN)	[NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN#117, we submitted a contribution [1] that proposed HPUE MSDs for 1UL band 2CC and 2UL band 3CC configurations. Based on the agreed way forward [2], we re-submit this discussion paper with corrected IMD indexes in support of the related CRs [3, 4].

# Discussion

## 2.1 Existing intra-band ULCA FDD and TDD HPUE cases

There are only a few intra-band ULCA cases in the September version of TS 38.101-1, as shown in Table 1.

Table 1 provides:

- The band type: TDD or FDD. FDD bands are not applicable to this simplification proposal

- Whether a 2Tx UL MIMO case is specified

- Whether this UL configuration is applicable to:

  - 1UL2CC HPUE MSD simplification proposed in this contribution

  - 2UL3CC HPUE MSD simplification proposed in this contribution

- Existing power classes.

Table 1: current 1UL2CC cases

| NR intra-band ULCA configurations | Band type | UL MIMO | Applicable to 1UL2CC simplification | Applicable to 2UL3CC simplification | Power class |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3B | FDD |  | No FDD not applicable to HPUE MSD simplification |  |  |  | 3 |
| CA_n5B | FDD |  |  |  |  |  | 3 |
| CA_n7B | FDD |  |  |  |  |  | 3 |
| CA_n26(2A) | FDD |  |  |  |  |  | 3 |
| CA_n40B | TDD |  | yes | yes |  | 2 | 3 |
| CA_n40(2A) |  |  | yes | No 3 UL clusters |  |  |  |
| CA_n41B | TDD |  | yes | yes |  | 2 | 3 |
| CA_n41C |  | X |  |  | 1.5 | 2 | 3 |
| CA_n41(2A) |  |  | yes | No 3 UL clusters | 1.5 | 2 | 3 |
| CA_n48B | TDD |  | yes | yes |  |  | 3 |
| CA_n77C | TDD | X | yes | yes | 1.5 | 2 | 3 |
| CA_n77(2A) |  |  | yes | No 3 UL clusters | 1.5 | 2 | 3 |
| CA_n78C | TDD | X | yes | yes | 1.5 | 2 | 3 |
| CA_n78(2A) |  |  | yes | No 3 UL clusters | 1.5 | 2 | 3 |
| CA_n79C | TDD | X | yes | yes | 1.5 | 2 | 3 |
| CA_n79(2A) |  |  | yes | No 3 UL clusters |  |  | 3 |
| CA_n104C | TDD |  | yes | yes |  |  | 3 |

Observations:

- There are only a few FDD intra-band UL CA cases, all are PC3.

  - The only related inter-band case is CA_n2A-n5B which is PC3 only.

- Non-contiguous ULCA is not applicable to 2UL3CC cases.

- HPUE cases are only for n41, n77, n78 and n79 bands which are large BW and thus are critical for IMDs of the 2CC into other bands and triple beat into FDD DL or third band DL.

As a result of the band BW being small for FDD bands:

- The ULCA IMDs are only an issue for close proximity bands within the same band group

- Triple beat issues only happen for small duplex distance in the 1UL1CC band for 2DL and close proximity bands within the same band group for a third band in a 3DL case.

Additionally, in these FDD cases, it is not assumed that MPR is applied as the duplex filter performance is critical to the IMD level. Furthermore, higher IMD order up to IMD13 are considered.

When checking into TS 38.101-3 for the intra-band NR ULCA related configurations, we observed the following aspects:

- There are no inter-band ENDC cases with an UL configuration of NR 1UL2CC band as two UL are needed for ENDC. However, if it is part of a 2UL band configuration the MSDs related to the intra-band UL band IMDs should still be analysed.

- In clause 6, the followings 2UL 3CC cases with 2CC in NR UL band are found:

  - DC_3A_n7B, DC_8A_n79C, DC_28A_n7B, DC_41A_n79C

  - All are PC3 cases.

- Additionally, in clause 5 there is one UL configuration that is not specified in clause 6: DC_1A_n7B

  - It should be removed from clause 5.5B.4 or added in clause 6.

Proposal for HPUE 1UL2CC and 2UL3CC MSD simplification:

- When the intra-band ULCA band of an inter-band 1UL2CC or 2UL3CC UL configuration of a DL NRCA or ENDC combination is a FDD band, the HPUE MSD simplification does not apply:

  - This HPUE MSD analysis is rare but will still be needed in further releases

  - This must be accounted for in the block approval process in Release-20 and must be triggered when a new HPUE configuration with FDD intra-band ULCA is added in clause 6 and/or clause 5.

- When the intra-band ULCA band of an inter-band 1UL2CC or 2UL3CC UL configuration of a DL NRCA or ENDC combination is a TDD band:

  - HPUE MSD simplification for 1UL2CC and 2UL3CC is introduced in 38.101-1 for inter-band NRCA

  - HPUE MSD simplification for 1UL2CC and 2UL3CC is introduced in 38.101-3 for inter-band ENDC.

  - HPUE MSD analysis will not be needed in further releases except for the rare case where a PC3 MSD is negligible but an HPUE MSD level is not.

## 2.2 Proposal for one TDD UL band 2CC case

The MSD for the one UL band 2CC cases (UL CA_nXX class B/C or 2A) is related to:

- The odd order IMDs of the 1RB+1RB allocation in each CC for victim bands in the proximity of the UL band with:

  - For TDD bands: IMD3 to IMD9 for class B or C and IMD3 to IMD7 for class 2A considering MPR

  - For FDD bands: Higher order odd IMDs may be considered, especially when the two bands are in the same band group.

- The IMD4 and IMD6 for victim bands in proximity of the 2nd harmonic ranges of the UL band

- The IMD5 for victim bands in proximity of the 3rd harmonic ranges of the UL band

- IMD4 of class 2A falling into low-band DL for bands with >300MHz BW (NR bands n77/n78/n79)

Since the IMD3 and IMD5 depend on applying MPR to meet SEM mask for TDD bands, their level is independent of the power class. This is also the case for any IMDs with an order higher than 3 since they decay in a similar or faster way compared to PC3.

For TDD bands it is reasonable to assume that HPUE MSDs caused by odd-orders IMDs from a single UL band with 2CC have the same value as PC3.

For even orders however, their power is proportional to the transmitted power, but accounting for MPR needed for IMD3 and/or IMD5 to meet -30dBm/MHz. For the 1RB+1RB outer2 allocations used for the single-band 2CC test points, compared to PC3:

- PC2 1Tx sees a 0.5dB higher MPR and PC2 2Tx sees a 1.5dB higher MPR

- PC1.5 2Tx sees a 3.5dB higher MPR.

- To simplify, we can assume that PC2 and PC1.5, 2CC band power increases by 2dB and 3dB versus PC3 respectively:

- IMD4 increases by 8dB for PC2 and 12dB for PC1.5. These can be mapped to ΔMSDmax= 9dB and 12dB from the 2UL MSD LUT, respectively

- IMD6 increases by 12dB for PC2 and 18dB for PC1.5. These can be mapped to ΔMSDmax= 12dB and 18dB from the 2UL MSD LUT, respectively.

- Note that in non-contiguous intra-band ULCA cases, the MPR is larger than what is considered above for contiguous intra-band ULCA which is the only case where the BB IMD4 overlaps with low-bands.

Proposal on HPUE one UL band 2CC MSD:

- For TDD UL bands:

- For odd IMDs, the PC2 and PC1.5 HPUE MSD is equal to PC3 MSD: ΔMSD(dB)=0

- For even IMDs:

  - IMD4 MSD is mapped to ΔMSDmax= 9dB and 12dB 2UL ΔMSD LUT for PC2 and PC1.5 respectively

  - IMD6 MSD is mapped to ΔMSDmax= 12dB and 18dB 2UL ΔMSD LUT for PC2 and PC1.5 respectively

- For FDD UL bands, the HPUE MSD is studied case-by-case.

- The associated CR to 38.101-1 [3] and 38.101-3 [3] are agreed upon.

## 2.3 Proposal for one UL band 1CC plus one TDD UL band 2CC case

The MSD for the two UL band 3CC cases (UL CA_nXXA-nYYB/C) is related to the triple beat (IMD3) between the 1CC band and the 2CC band allocations.

For PC2 inter-band HPUE, the configuration goes from 20dBm+20dBm for PC3 to 23dBm+23dBm. Thus, one could assume that the interference follows the IMD3 9dB ΔMSDmax from the 2UL MSD simplification LUT. However, to meet emissions for IMD3 and IMD5, MPR is allowed, thus reducing the power for the band with 2UL CCs. The same happens for the PC1.5 26+26dBm case.

It is unlikely that HPUE intra-band contiguous UL CA, which is the only type considered in triple beat cases, will meet SEM without some backoff applied (but not necessarily the full amount allowed by the specification). The amount of backoff increase between PC3 and PC2 or PC3 and PC1.5 is likely to follow what is defined for MPR.

For the 1RB+1RB outer2 allocations used for the triple beat test points, compared to PC3:

- PC2 1Tx sees a 0.5dB higher MPR and PC2 2Tx sees a 1.5dB higher MPR

- PC1.5 2Tx sees a 3.5dB higher MPR.

To target the same simplification for the 2Tx and 3Tx cases as for the two UL band inter-band 2CC case, we propose to use 1dB MPR for PC2 cases and 3dB for the PC1.5 case, meaning that:

- For PC2 (assuming a PC3 inter-band based on PC3+PC3 (20dBm+20dBm sharing) case to a PC2 case in the same configuration):

  - 2CC band power increases by 2dB versus PC3

  - 1CC band power increases by 3dB versus PC3

  - Thus, triple beat interference power increases by 2*2+1*3 =7dB => PC2 triple beat ΔMSDmax is 7dB and can be simplified to 6dB to map on an existing LUT.

- For PC1.5 (assuming a PC3 inter-band based on PC3+PC3 (20dBm+20dBm sharing) case to a PC2 case based on PC2+PC2 configuration):

  - 2CC band power increases by 3dB versus PC3

  - 1CC band power increases by 6dB versus PC3

  - Thus, triple beat interference power increases by 2*3+1*6 =12dB => PC2 triple beat ΔMSDmax is 12dB.

Proposal on HPUE two UL band 3CC MSD:

- PC2 2UL band 3CC triple beat (IMD3) is mapped to ΔMSDmax= 6dB 2UL ΔMSD LUT

- PC1.5 2UL band 3CC triple beat (IMD3) is mapped to ΔMSDmax= 12dB 2UL ΔMSD LUT

- The associated CR to 38.101-1 [3] and 38.101-3 [3] are agreed upon.

- For HPUE, the PC3 band-group rules for 2DL and 3DL triple beat MSD applies

# Conclusion

In this contribution, we analysed the HPUE delta MSDs for the one UL band 2CC IMDs and two UL band 3CC triple beat PC2 and PC1.5 cases and make the following proposals.

Proposal for HPUE 1UL2CC and 2UL3CC MSD simplification:

- When the intra-band ULCA band of an inter-band 1UL2CC or 2UL3CC UL configuration of a DL NRCA or ENDC combination is a FDD band, the HPUE MSD simplification does not apply:

  - This HPUE MSD analysis is rare but will still be needed in further releases

  - This must be accounted for in the block approval process in Release-20 and must be triggered when a new HPUE configuration with FDD intra-band ULCA is added in clause 6 and/or clause 5.

- When the intra-band ULCA band of an inter-band 1UL2CC or 2UL3CC UL configuration of a DL NRCA or ENDC combination is a TDD band:

  - HPUE MSD simplification for 1UL2CC and 2UL3CC is introduced in 38.101-1 for inter-band NRCA

  - HPUE MSD simplification for 1UL2CC and 2UL3CC is introduced in 38.101-3 for inter-band ENDC.

  - HPUE MSD analysis will not be needed in further releases except for the rare case where a PC3 MSD is negligible but an HPUE MSD level is not.

Proposal on HPUE one UL band 2CC MSD:

- For TDD UL bands:

- For odd IMDs, the PC2 and PC1.5 HPUE MSD is equal to PC3 MSD: ΔMSD(dB)=0

- For even IMDs:

  - IMD4 MSD is mapped to ΔMSDmax= 9dB and 12dB 2UL ΔMSD LUT for PC2 and PC1.5 respectively

  - IMD6 MSD is mapped to ΔMSDmax= 12dB and 18dB 2UL ΔMSD LUT for PC2 and PC1.5 respectively

- For FDD UL bands, the HPUE MSD is studied case-by-case.

- The associated CR to 38.101-1 [3] and 38.101-3 [3] are agreed upon.

Proposal on HPUE two UL band 3CC MSD:

- PC2 2UL band 3CC triple beat (IMD3) is mapped to ΔMSDmax= 6dB 2UL ΔMSD LUT

- PC1.5 2UL band 3CC triple beat (IMD3) is mapped to ΔMSDmax= 12dB 2UL ΔMSD LUT

- The associated CR to 38.101-1 [3] and 38.101-3 [3] are agreed upon.

- For HPUE, the PC3 band-group rules for 2DL and 3DL triple beat MSD applies

# References

[1] R4-2514408 Proposal on 1UL band 2CC and 2UL band 3CC HPUE MSD, Skyworks Solutions Inc., RAN4#116b

[2] R4-2514628 Way Forward on HPUE notes in 38.101-1 Clause 5 tables, Apple, Skyworks, Nokia, Samsung, RAN4#116b

[3] R4-2520194 CR to R19 38.101-1 to introduce HPUE MSD simplification for UL configurations including intra-band ULCA, Skyworks Solutions Inc., RAN4#117

[4] R4-2520195 CR to 38.101-3 to introduce HPUE MSD simplification for UL configurations including intra-band ULCA, Skyworks Solutions Inc., RAN4#117