3GPP TSG-RAN WG4 Meeting #117			                        R4-2520438

Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.4.2

Source: 	CMCC

Title: 	Discussion on 6G NTN related BS RF

Document for:	Discussion

# 1. Introduction

As an important component in network deployment, NTN has been widely applied in 5G network deployment. Since R17, research on NTN requirements has been carried out in multiple versions. Therefore, further evolving NTN in 6GR meets the needs of operators and users.

This contribution will focus on 6G NTN related BS RF

- Discussion

### 2.1 General consideration on 6GR BS

In the last meeting, not much discussion was conducted on 6G NTN. In order to clarify the direction of discussion in the future meetings, the following WF were reached at the last meeting.

Way Forward: Additional input from satellite companies is expected for RAN4#117: - List of requirements to be further studied (using the same format proposed in 1.1.2) - List of coexistence studies to be considered (using the format proposed in 1.3.1).

The coexistence of NTN and TN in 6G research is a very important point. After several releases of research, studies on the coexistence of NTN and TN are still ongoing in R20. NTN networks have advantages in many scenarios, especially in areas where TN cannot cover. Against the backdrop of continuously increasing transmission power, it is of great significance to conduct research on the coexistence of TN and NTN in 6G

Proposal 1: Research on the coexistence between TN and NTN needs to continue under the new device/BS architecture and scenarios of 6G.

In addition, compared with 5G NTN, 6G NTN needs to be re-evaluated in terms of deployment scenarios, system parameters, etc. For instance, in terms of network deployment, since R17, considering that the UE will prioritize connecting to the TN network when there is TN network deployment, a 1.5km isolation distance has been set between the NTN network and the TN network, as shown in Figure 1.

Figure 1. Isolation distance between NTN and TN network

This 1.5km distance setting mainly takes into account the actual usage of the NTN network. However, considering the actual usage of NTN, we cannot continue to assume that NTN HPUE will be used with a 1.5 km isolation from the edge of the TN network in 6G, especially when considering the coexistence research between NTN and TN networks. The change in the simulation scene also means that the RF indicators need to be reconsidered and evaluated. Therefore, from our perspective, the setting of ISD should be 0.

Observation 1: The scenario where a 1.5km isolation distance between NTN and the TN network, assuming that terminals within the coverage area of the TN network will not connect to NTN, is not reasonable in actual usage scenarios.

Proposal 2: Re-evaluate the isolation distance assumptions between NTN and TN networks in 6G for coexistence research.

# 3. Conclusions

Proposal 1: Research on the coexistence between TN and NTN needs to continue under the new device architecture and scenarios of 6G.

Observation 1: The scenario where a 1.5km isolation distance between NTN and the TN network, assuming that terminals within the coverage area of the TN network will not connect to NTN, is not reasonable in actual usage scenarios.

Proposal 2: Re-evaluate the isolation distance assumptions between NTN and TN networks in 6G for coexistence research.

# 4. Reference

- R4-2514638, WF on 6G BS RF and co-existence, Feature Lead (Ericsson)