# 3GPP TSG-RAN WG4 Meeting #117	R4-2520665

# Dallas, USA, Nov 17 – 21, 2025

Agenda item:	7.8.4

Source:	Apple

Title:	On RRM requirement for R20 LB CA enhancement

Document for:	Discussion

# Introduction

The new WI on enhancement of low NR band carrier aggregation via switching was agreed in RAN #109 meeting[1]. The working scope is as follows:

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

The WF on RRM requirements for NR_LBCA_Sw_enh [2] have been agreed in RAN4#116bis, and the open issues have been summarized in the topic summary for [116-bis][224] NR_LBCA_Sw_enh_RRM [3].

In this contribution, we discuss the RRM requirement for R20 LB CA via switching.

# RRM requirements impact of R20 LB CA

In R20 LB CA, the two cases for switching are:

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

  - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

Different from R19 LB CA case 2, the case 3 can have simultaneous DL on both FDD PCC and SDL SCC, however, case 3 has no UL. The impact of case 3 could be a subset of case 2, since the DL on PCC won’t be impacted during the SDL SCC duration in case 3. In R19 LB CA, we agreed the follow requirement summary when the switching between case 1 and case 2 happens.

Table 1. Summary of RRM requirements in R19 LB CA

| Index | Requirement |
| --- | --- |
| 1 | Activated SDL SCell L3 measurement |
|  | Deactivated SDL SCell measurement |
| 2 | Interruption for deactivated SDL SCell measurement |
| 3 | SDL SCell activation/deactivation and corresponding interruption |
| 4 | BFD/CBD for SDL SCell and PCell |
| 5 | TCI state switch delay for SDL SCell and PCell (if needed) |
| 6 | PL-RS switch delay for PCell (if needed) |
| 7 | L1-RSRP/SINR for SDL SCell and PCell (if needed) |
| 8 | PCC L3 measurement delay requirement (if needed) |
|  | RLM requirement for PCell (if needed) |

In the WID, it highlighted that,

  - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible

  - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching

  - Reuse Rel-19 LB-CA RAN4 requirements as much as possible

In our view, the R20 LB CA framework is quite similar as R19 LB CA WI, and therefore we think the R19 requirement scope for LB CA can be the baseline for R20 LB CA.

### Sub-topic 1-2 Carrier switching between CC/Switching pattern applicability

Thursday online Agreement: - Switching pattern applicability for RRM requirement in Rel-20 LB-CA via switching: - For the SCell which has been activated (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is applied. - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied.

### Issue 1-2-2: Interruption or scheduling restriction requirement for CC switching

In addition, there were some switching pattern related key consensus in R19, and they shall also be applied for R20 LB CA according to the WID. For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier.

However, in the last meeting, companies mentioned that RAN1 has not yet thoroughly discussed whether the switching pattern and collision rules defined for R19 LB-CA can be reused for R20 LB-CA. Therefore, we are also fine with sending an LS to RAN1 to clarify this understanding before concluding on the necessity of interruption and scheduling restriction requirements in RAN4.

Proposal 1: For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier.

## Sub-topic 1-3 Applicability of Rel-19 LB-CA RRM requirements

Thursday online Agreement: - For L3 measurement and L1 measurement on activated SDL SCell, reuse the Rel-19 requirement as starting point. - For TCI state switch delay requirements, reuse the Rel-19 requirement as starting point. - For SDL SCell activation/deactivation delay requirements, reuse the Rel-19 requirement as starting point. - No new RRM requirement is expected for deactivated SCell measurement period requirements

### Issue 1-3-1: Requirement design for gap-less based L3 measurement and L1 measurement of activated SDL SCC

Regarding the RRM requirement applicability for R20 LB CA via switching, we think the same logic from R19 can still be used. The only difference is: the DL measurement on PCell will still be available even though UE RF can cover the SDL SCell DL. However, the SDL SCell DL measurement shall still follow the R19 LB CA principle.

Proposal 2: In R20 LB CA, for gap-less based L3 measurement and L1 measurement of activated SDL SCC:

- a.   If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, no delay extension is allowed.

- b.   If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

- c.   If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Issue 1-4-1: SDL SCell related RRM requirement

In R19 LB CA RAN4 agreed that,

- For R19 LB CA via switching, interruption length on PCC due to measurements on deactivated SDL is (tuning time in slot level + TSMTC_duation + retuning time in slot level), - Tuning time from PCell to SCell is equal to the RRC configured switching gap from PCell to SCell LBCA-SwitchingGap-Duration-PCelltoSCell-r19 - Returning time from SCell to PCell is equal to the RRC configured switching gap from SCell to PCell, i.e., LBCA-SwitchingGap- SCelltoPCell-r19 - Missed ACK/NACK rate For R19 LB CA via switching, missed ACK/NACK rate due to measurements on deactivated SDL are allowed with up to X= YmeasCycleSCell, where Y=2* interruption length.

However, different from R19 LB CA case, UE can support Rx on both FDD carrier 1 and SDL carrier 2, and therefore the TSMTC_duation is not needed for the interruption length. We still think it would be simple for UE implementation if the switching time is still located at the end of the slot. Thus, we propose:

Proposal 3: RAN4 to further discuss the interruption requirement for deactivated SDL SCell measurement.

Proposal 4: For R20 LB CA via switching, interruption length on PCC due to measurements on deactivated SDL SCC is:

- (1) interruption of switching FDD PCC to SDL SCC before SMTC is tuning time in slot level, and tuning time from PCell to SCell is equal to the RRC configured switching gap from PCell to SCell LBCA-SwitchingGap-Duration-PCelltoSCell-r19

- (2) interruption of switching SDL SCC to FDD PCC after SMTC is returning time in slot level, and retuning time from SCell to PCell is equal to the RRC configured switching gap from SCell to PCell, i.e., LBCA-SwitchingGap- SCelltoPCell-r19

BFD/CBD for SDL SCell and PCell

Same as R19 LB CA, the switching pattern shall be applied. The BFD/CBD requirement on PCell could be almost same as legacy BFD/CBD requirement, and only some clarification is needed if the FDD-SDL carrier switching will cause interruption to the BFD/CBD measurement on PCell.

Regarding the SDL SCell BFD/CBD, the requirement shall be defined based on the switching pattern as what RAN4 specified in R19 LB-CA., i.e., SDL SCell BFD/CBD for R19 LB-CA can be reused for R20 LB-CA.

TCI state switch delay for SDL SCell and PCell

Same situation as BFD/CBD.

L1-RSRP/SINR for SDL SCell and PCell

Same situation as BFD/CBD.

Proposal 5: In R20 LB-CA, the BFD/CBD, L1-RSRP/SINR, and TCI state switch delay requirements on FDD PCell could be same as legacy requirement, and only clarification is needed if the FDD-SDL carrier switching will cause interruption to the BFD/CBD, L1-RSRP/SINR measurement, and TCI state switch on FDD PCell.

Proposal 6: SDL SCell BFD/CBD , L1-RSRP/SINR, and TCI state switch delay requirements in R19 LB-CA can be reused for corresponding SDL SCell requirements in R20 LB-CA.

### Issue 1-4-2: FDD PCell related RRM requirement

PCC L3 measurement delay requirement

The L3 measurement requirement on PCell could be almost same as legacy requirement in LB-CA, and only some clarification is needed if the FDD-SDL carrier switching will cause interruption to the L3 measurement on PCell.

RLM requirement for PCell

Same as PCell L3 measurement.

Proposal 7: In R20 LB-CA, the RLM and L3 measurement requirements on FDD PCell could be same as legacy requirement, and only clarification is needed if the FDD-SDL carrier switching will cause interruption to the RLM and L3 measurement on FDD PCell.

### Issue 1-4-3: Active BWP switching for PCell and SCell

Active BWP switching for PCell and SCell

Since the active BWP switching of R19 LB-CA is still under discussing, we propose to discuss R20 case after we conclude the BWP switching requirement for R19 LB-CA.

Proposal 8: RAN4 to further discuss active BWP switching requirement for R20 LB-CA after completely concluding on active BWP switching requirement for R19 LB-CA.

# Conclusion

In this contribution, we discuss the RRM requirement for R20 LB CA via switching.

Proposal 1: For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier.

Proposal 2: In R20 LB CA, for gap-less based L3 measurement and L1 measurement of activated SDL SCC:

- a.   If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, no delay extension is allowed.

- b.   If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

- c.   If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Proposal 3: RAN4 to further discuss the interruption requirement for deactivated SDL SCell measurement.

Proposal 4: For R20 LB CA via switching, interruption length on PCC due to measurements on deactivated SDL SCC is:

- (1) interruption of switching FDD PCC to SDL SCC before SMTC is tuning time in slot level, and tuning time from PCell to SCell is equal to the RRC configured switching gap from PCell to SCell LBCA-SwitchingGap-Duration-PCelltoSCell-r19

- (2) interruption of switching SDL SCC to FDD PCC after SMTC is returning time in slot level, and retuning time from SCell to PCell is equal to the RRC configured switching gap from SCell to PCell, i.e., LBCA-SwitchingGap- SCelltoPCell-r19

Proposal 5: In R20 LB-CA, the BFD/CBD, L1-RSRP/SINR, and TCI state switch delay requirements on FDD PCell could be same as legacy requirement, and only clarification is needed if the FDD-SDL carrier switching will cause interruption to the BFD/CBD, L1-RSRP/SINR measurement, and TCI state switch on FDD PCell.

Proposal 6: SDL SCell BFD/CBD , L1-RSRP/SINR, and TCI state switch delay requirements in R19 LB-CA can be reused for corresponding SDL SCell requirements in R20 LB-CA.

Proposal 7: In R20 LB-CA, the RLM and L3 measurement requirements on FDD PCell could be same as legacy requirement, and only clarification is needed if the FDD-SDL carrier switching will cause interruption to the RLM and L3 measurement on FDD PCell.

Proposal 8: RAN4 to further discuss active BWP switching requirement for R20 LB-CA after completely concluding on active BWP switching requirement for R19 LB-CA.

# References

[1] RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Moderator (Huawei), RANP#109

[2] R4-2514824, WF on NR_LBCA_Sw_enh_RRM, Samsung, RAN4#116bis

[3] R4-2513530, Topic summary for [116bis][224] NR_LBCA_Sw_enh_RRM, Samsung, RAN4#116bis