3GPP TSG-RAN WG4 Meeting # 117	R4-2520972

# Dallas, USA, 17th – 21st Nov, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 3097 | rev |  | Current version: | 19.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 38.101-1 Maintenance for time mask of Tx switching for 3Tx UEs [TxSwitch_R19] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the agreed RAN2 CR R2-2506611, the final IE name for the switch period capability is uplink3TxSwitchingPeriodUpTo2TPerBandDualUL. Therefore, the RAN4 spec needs to be revised correspondingly |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the current IE name from uplink3TxSwitchingPeriodUpTo2TperBand-r19 to uplink3TxSwitchingPeriodUpTo2TperBandDualUL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 38.331 and TS 38.101-1 are not aligned |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===========First change==============

##### 6.3A.3.3.8	Time mask for switching between two uplink bands with three transmit antenna connectors and maximum two transmit antenna connectors for each band

In addition to the requirements in 6.3A.3.3.1 and the maximum output power requirement specified in Table 6.2H.3.1-1 with uplink configured to two NR bands, the switching time mask specified in this clause is applicable for an uplink band pair of an inter-band UL CA configuration when the capability Uplink3TxSwitchingPeriodUpTo2TPerBandDualUL is present, and is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where NR UL carriers in bands A and B are capable of a maximum of two transmit antenna connectors, but simultaneously only a maximum of three transmit antenna connectors are supported across both bands. Band A and band B are different bands with different carrier frequencies. The UE shall support the switch between the following two transmit configurations 1 and 2:

- Configuration 1: Two antenna ports on band A and one antenna port on band B

- Configuration 2: One antenna port on band A and two antenna ports on band B

In the above configurations 1 and 2, the UE shall follow the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports and single layer transmission with 1 antenna port shall be supported on NR UL carriers of band A and band B.

The switching periods described in Figure 6.3A.3.3.8-1a and Figure 6.3A.3.3.8-1b are located in either band A or band B as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability Uplink3TxSwitchingPeriodUpTo2TPerBandDualUL.

When switching from one configuration to another, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. In addition, the RRC signalling uplinkTxSwitchingPeriodLocation is ignored by the UE and does not take effect in this case.

Figure 6.3A.3.3.8-1a: Time mask for switching between UL Configuration 1 and UL Configuration 2, where the switching period is located in Configuration 1

Figure 6.3A.3.3.8-1b: Time mask for switching between UL Configuration 1 and UL Configuration 2, where the switching period is located in Configuration 2

The following applies for the uplink switching cases specified in clause 6.1.6.2 of [10] with uplinkTxSwitchingOption set to dualUL when the configuration of the location of the switching period by uplinkTxSwitchingPeriodLocation is ignored by the UE:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the uplink switching gap indicated by Uplink3TxSwitchingPeriodUpTo2TPerBandDualUL on any of the carriers before T0, transient periods of 10 ms are located at the end of the last symbol(s) configured or scheduled on the carriers before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switched-to carrier.

The requirements apply for the case of both non-co-located and co-located and synchronized network deployment for the two uplink carriers.

The time mask is applicable to uplink transmissions when configured with dualUL.

==============End of change==============