3GPP TSG-RAN4 Meeting #117	R4-2520989

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | Draft | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft bigCR to TS38.101-4 Introduce the UE demodulation requirement for Rel-19 ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-11-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The Big draftCR contains the following draftCR for Rel-19 ATG RRM perf: - R4-2522831 - CMCC; - draftCR to TS 38.101-4 Introduce the requirement applicability and FRC for Rel-19 ATG; - 5.1.1.7.8 (new), 5.1.1.12, A.3.2.1.1, A.3.2.2.2. - R4-2522752 - Ericsson; - draft CR on minimum requirement for DL MIMO for ATG; - 5.2.2.1.22, 5.2.2.2.23, 5.2.3.1.21, 5.2.3.2.22. - R4-2522753 - ZTE Corporation, Sanechips; - Draft CR on 38.101-4 for ATG CA demodulation requirements; - 5.2A, 5.2A.2.7, 5.2A.3.6. |
| Summary of change: |  | Introduce UE demodulation requirement for NR ATG enhancement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel-19 ATG spec will not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.7.8 (new), 5.1.1.12, 5.2.2.1.22, 5.2.2.2.23, 5.2.3.1.21, 5.2.3.2.22, 5.2A.2.7 (new), 5.2A.3.6 (new), A.3.2.1.1, A.3.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ==============First change==============

5.1.1.7.8	Applicability and test rules for ATG

The performance requirement for CA ATG UE demodulation tests in Clause 5.2A are defined independent of CA configurations and bandwidth combination sets specified in Clause 5.5J of TS 38.101-1. For ATG UEs supporting different CA configurations and bandwidth combination sets, the applicability and test rules are defined in Table 5.1.1.7.8-1, Table 5.1.1.7.8-2 and Table 5.1.1.7.8-3. For simplicity, CA configuration below refers to combination of CA configuration and bandwidth combination set.

The performance requirements in Table 5.1.1.7.8-1 shall apply for ATG UEs which support optional feature airToGroundNetwork-r18.

Table 5.1.1.7.8-1: Applicability and test rules for CA ATG UE demodulation tests

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x1 and Test x2 in Clause 5.2A.2.x and 5.2A.3.x | CA_C, CA_AX | Table 5.1.1.7.8-2 | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x3 in Clause 5.2A.2.x and 5.2A.3.x | CA_AX | Table 5.1.1.7.8-2 | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |
| Test x4 in Clause 5.2A.2.x and 5.2A.3.x | CA_AX | Table 5.1.1.7.8-2 | Largest aggregated CA bandwidth combination | FDD CC |

Table 5.1.1.7.8-2: Selection of CA configurations

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |
| NOTE 1: For CA_AX capability, if CA configuration from step 2 is CA configuration with the largest number of bands then Step 3 and Step 4 are skipped. Otherwise, the two CA configurations selected from Step 2 and Step 4 are used for testing. NOTE 2: Maximum supported data rate for Step 2 and Step 4 is calculated based clause 4.1.2 of TS 38.306 [14]. NOTE 3: Tested data rate for Step 2 and Step 4 is calculated based on the equation DataRate=10−3j=1JTBSj2μj and FRCs used in the test. |  |  |  |  |

Table 5.1.1.7.8-3: Requirements applicability for ATG UEs

| Tests x1, x3 in Clause 5.2A.2.x and 5.2A.3.x | Only if ATG UE does not supports optional features k1-RangeExtensionATG-r18, or maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |
| --- | --- |
| Tests x2, x4 in Clause 5.2A.2.x and 5.2A.3.x | Only if ATG UE supports the optional features k1-RangeExtensionATG-r18, and maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |

# ==============Next change==============

#### 5.1.1.12	Applicability of requirements for ATG

The performance requirements in Table 5.1.1.12-1 shall apply for ATG UEs which support optional feature airToGroundNetwork-r18.

Other performance requirements mandatory for UE supporting NR operation defined in Section 5 but not included in table 5.1.1.12-1 should not be considered applicable to ATG UEs.

The antenna type is up to UE declaration.

Table 5.1.1.12-1: Requirements applicability for ATG UEs

| UE capability | Test type |  | Test list | Applicability notes |  |
| --- | --- | --- | --- | --- | --- |
| ATG UE with 2RX | FR1 FDD | PDSCH | Tests 1-1, 1-2, 1-3 in Clause 5.2.2.1.22 |  |  |
|  |  |  | Tests 1-4, 1-5 in Clause 5.2.2.1.22 | Only if ATG UE supports Cross Polarized Antennas |  |
|  |  | PDCCH | Test number 1-1, 1-3 and 1-5 in Table 5.3.2.1.1-1 Test number 1-3 in Table 5.3.2.1.2-1 |  |  |
|  |  | PBCH | Clause 5.4.2.1 |  |  |
|  |  | SDR | Clause 5.5.1 |  |  |
|  | FR1 TDD | PDSCH | Tests 1-1, 1-2, 1-3 in Clause 5.2.2.2.23 | Only if ATG UE does not supports optional features k1-RangeExtensionATG-r18, or maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |  |
|  |  |  | Tests 1-10, 1-11 in Clause 5.2.2.2.23 | Only if ATG UE does not supports optional features k1-RangeExtensionATG-r18, or maxHARQ-ProcessNumberATG-r18 with 32 DL processes, and supports Cross Polarized Antennas |  |
|  |  |  | Tests 1-4, 1-5, 1-6 in Clause 5.2.2.2.23 | Only if ATG UE supports the optional features k1-RangeExtensionATG-r18, and maxHARQ-ProcessNumberATG-r18 with 32 DL processes. | If UE supports outOfOrderOperationDL-r16. |
|  |  |  | Tests 1-7, 1-8, 1-9 in Clause 5.2.2.2.23 |  | If UE does not support outOfOrderOperationDL-r16. |
|  |  |  | Tests 1-12, 1-13 in Clause 5.2.2.2.23 | Only if ATG UE supports optional features k1-RangeExtensionATG-r18, and maxHARQ-ProcessNumberATG-r18 with 32 DL processes, and supports Cross Polarized Antennas |  |
|  |  | PDCCH | All test cases in Table 5.3.2.2.1 All test cases in Table 5.3.2.2.2 |  |  |
|  |  | PBCH | Clause 5.4.2.2 |  |  |
|  |  | SDR | Clause 5.5.1 |  |  |
| ATG UE with 4RX | FR1 FDD | PDSCH | Tests 1-1, 1-2, 1-3 in Clause 5.2.3.1.21 |  |  |
|  |  |  | Tests 1-4, 1-5 in Clause 5.2.3.1.21 | Only if ATG UE supports Cross Polarized Antennas |  |
|  |  | PDCCH | Test number 1-1, 1-3 and 1-5 in Table 5.3.3.1.1-1 Test number 1-3 in Table 5.3.3.1.2-1 |  |  |
|  |  | PBCH | Clause 5.4.3.1 |  |  |
|  |  | SDR | Clause 5.5.1 |  |  |
|  | FR1 TDD | PDSCH | Tests 1-1, 1-2, 1-3 in Clause 5.2.3.2.22 | Only if ATG UE does not supports optional features k1-RangeExtensionATG-r18, or maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |  |
|  |  |  | Tests 1-10, 1-11 in Clause 5.2.3.2.22 | Only if ATG UE does not supports optional features k1-RangeExtensionATG-r18, or maxHARQ-ProcessNumberATG-r18 with 32 DL processes, and supports Cross Polarized Antennas |  |
|  |  |  | Tests 1-4, 1-5, 1-6 in Clause 5.2.3.2.22 | Only if ATG UE supports the optional features k1-RangeExtensionATG-r18, or maxHARQ-ProcessNumberATG-r18 with 32 DL processes. | If UE supports outOfOrderOperationDL-r16. |
|  |  |  | Tests 1-7, 1-8, 1-9 in Clause 5.2.3.2.22 |  | If UE does not support outOfOrderOperationDL-r16. |
|  |  |  | Tests 1-12, 1-13 in Clause 5.2.3.2.22 | Only if ATG UE supports optional features k1-RangeExtensionATG-r18, and maxHARQ-ProcessNumberATG-r18 with 32 DL processes, and supports Cross Polarized Antennas |  |
|  |  | PDCCH | All test cases in Table 5.3.3.2.1 All test cases in Table 5.3.3.2.2 |  |  |
|  |  | PBCH | Clause 5.4.3.2 |  |  |
|  |  | SDR | Clause 5.5.1 |  |  |

# ==============Next change==============

##### 5.2.2.1.22	Minimum requirements for ATG

The performance requirements are specified in Table 5.2.2.1.22-3, with the addition of test parameters in Table 5.2.2.1.22-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.22-1.

Table 5.2.2.1.22-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE PDSCH performance under the ATG scenario | 1-1, 1-2, 1-3. 2-1, 2-2 |

Table 5.2.2.1.22-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.2.1.22-3: Minimum performance for ATG with rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | AWGN | 220Hz | 2x2 | 70 | 6.0 |
| 1-2 | R.PDSCH.1-3.6 FDD | 10 / 15 | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | 13.6 |
| 1-3 | R.PDSCH.1-4.1 FDD | 10 / 15 | 256QAM, 0.82 | AWGN | 220Hz | 2x2 | 70 | 22.5 |

Table 5.2.2.1.22-4: Minimum performance for ATG with rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-3.9 FDD | 10 / 15 | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | [16.8] |
| 2-2 | R.PDSCH.1-4.3 FDD | 10 / 15 | 256QAM, 0.82 | AWGN | 220Hz | 2x2 | 70 | [25.7] |

==============Next change==============

##### 5.2.2.2.23	Minimum requirements for ATG

The performance requirements are specified in Table 5.2.2.2.23-3, with the addition of test parameters in Table 5.2.2.2.23-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.23-1.

Table 5.2.2.2.23-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE PDSCH performance under the ATG scenario | 1-1, 1-2, 1-3, 1-4, 1-5, 1-6, 1-7, 1-8, 1-9, 2-1, 2-2, 2-3, 2-4 |

Table 5.2.2.2.23-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | Test 1-1, 1-2, 1-3, 2-1, 2-2: 8 Test 1-4, 1-5, 1-6, 1-7, 1-8, 1-9, 2-3, 2-4: 32 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.2.2.23-3: Minimum performance for ATG with rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | AWGN | 500Hz | 2x2 | 70 | 6.0 |
| 1-2 | R.PDSCH.2-3.6 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-1 | AWGN | 500Hz | 2x2 | 70 | 13.5 |
| 1-3 | R.PDSCH.2-4.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-1 | AWGN | 500Hz | 2x2 | 70 | 22.6 |
| 1-4 | R.PDSCH.2-31.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | 6.0 |
| 1-5 | R.PDSCH.2-32.1 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | 13.5 |
| 1-6 | R.PDSCH.2-33.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | 22.6 |
| 1-7 | R.PDSCH.2-40.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | 6.0 |
| 1-8 | R.PDSCH.2-41.1 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | 13.5 |
| 1-9 | R.PDSCH.2-42.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | 22.6 |

Table 5.2.2.2.23-4: Minimum performance for ATG with rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) |  | SNR (dB) |  |
| 2-1 | R.PDSCH.2-3.9 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-1 | AWGN | 500Hz | 2x2 | 70 | [16.8] |
| 2-2 | R.PDSCH.2-4.5 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-1 | AWGN | 500Hz | 2x2 | 70 | [25.4] |
| 2-3 | R.PDSCH.2-41.2 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | [16.8] |
| 2-4 | R.PDSCH.2-42.2 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | [25.4] |

==============Next change==============

##### 5.2.3.1.21	Minimum requirements for ATG

The performance requirements are specified in Table 5.2.3.1.21-3, with the addition of test parameters in Table 5.2.3.1.21-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.21-1.

Table 5.2.3.1.21-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE PDSCH performance under the ATG scenario | 1-1, 1-2, 1-3, 2-1, 2-2 |

Table 5.2.3.1.21-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.3.1.21-3: Minimum performance for ATG with rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | AWGN | 220Hz | 2x4 | 70 | 3.1 |
| 1-2 | R.PDSCH.1-3.6 FDD | 10 / 15 | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | 10.6 |
| 1-3 | R.PDSCH.1-4.1 FDD | 10 / 15 | 256QAM, 0.82 | AWGN | 220Hz | 2x4 | 70 | 19.6 |

Table 5.2.3.1.21-4: Minimum performance for ATG with rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-3.9 FDD | 10 / 15 | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | [13.8] |
| 2-2 | R.PDSCH.1-4.3 FDD | 10 / 15 | 256QAM, 0.82 | AWGN | 220Hz | 2x4 | 70 | [22.7] |

==============Next change==============

##### 5.2.3.2.22	Minimum requirements for ATG

The performance requirements are specified in Table 5.2.3.2.22-3, with the addition of test parameters in Table 5.2.3.2.22-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.22-1.

Table 5.2.3.2.22-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE PDSCH performance under the ATG scenario | 1-1, 1-2, 1-3, 1-4, 1-5, 1-6, 1-7, 1-8, 1-9, 2-1, 2-2, 2-3, 2-4 |

Table 5.2.3.2.22-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | Test 1-1, 1-2, 1-3, 2-1, 2-2: 8 Test 1-4, 1-5, 1-6, 1-7, 1-8, 1-9, 2-3, 2-4: 32 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.3.2.22-3: Minimum performance for ATG with rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | AWGN | 500Hz | 2x4 | 70 | 3.1 |
| 1-2 | R.PDSCH.2-3.6 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-1 | AWGN | 500Hz | 2x4 | 70 | 10.6 |
| 1-3 | R.PDSCH.2-4.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-1 | AWGN | 500Hz | 2x4 | 70 | 19.6 |
| 1-4 | R.PDSCH.2-31.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | 3.1 |
| 1-5 | R.PDSCH.2-32.1 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | 10.6 |
| 1-6 | R.PDSCH.2-33.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | 19.6 |
| 1-7 | R.PDSCH.2-40.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | 3.1 |
| 1-8 | R.PDSCH.2-41.1 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | 10.6 |
| 1-9 | R.PDSCH.2-42.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | 19.6 |

Table 5.2.3.2.22-4: Minimum performance for ATG with rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.2-3.9 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-1 | AWGN | 500Hz | 2x4 | 70 | [13.7] |
| 2-2 | R.PDSCH.2-4.5 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-1 | AWGN | 500Hz | 2x4 | 70 | [22.4] |
| 2-3 | R.PDSCH.2-41.2 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | [13.7] |
| 2-4 | R.PDSCH.2-42.2 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | [22.4] |

# ==============Next change==============

#### 5.2A.2.7	Minimum requirements for ATG CA

The performance requirements are specified in Table 5.2A.2.7-8 and Table 5.2A.2.7-9 based on the single carrier requirements for different bandwidth specified in Table 5.2A.2.7-5, Table 5.2A.2.7-6, and Table 5.2A.2.7-7, with the addition of test parameters in Table 5.2A.2.7-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2A.2.7-1.

Table 5.2A.2.7-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 2 receive antenna conditions in the ATG scenario defined in B.1.1 with CA | 1-1, 1-2, 2-1, 2-2 |

Table 5.2A.2.7-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| TDD pattern |  |  | FR1.30-1 or FR1.30-8 |
| Propagation condition |  |  | Static propagation condition |
| Antenna configuration |  |  | 2x2 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | As defined in Table 5.2A-2 for FR1.30-1 TDD pattern, Table 5.2A.2.7-3 for FR1.30-8 TDD pattern |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3 for FR1.30-1 TDD pattern, Table 5.2A.2.7-4 for FR1.30-8 TDD pattern |

Table 5.2A.2.7-3: Test parameters for number of HARQ processes for ATG CA requirements

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 13 |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 13 | N/A |
| Note: The test parameters only applicable for FR1.30-8 TDD pattern. |  |  |  |

Table 5.2A.2.7-4: Test parameters for K1 values for ATG CA requirements

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | 2 if mod(i,40) = {1,4,5,9,10,14,15,19,20,24,25,28,29} |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 33 if mod(i,40) = 1 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 10 if mod(i,40) = 29 | N/A |
| Note: The test parameters only applicable for FR1.30-8 TDD pattern. |  |  |  |

Table 5.2A.2.7-5: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-26.1 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | [12.57] |
| 10 | R.PDSCH.1-3.6 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | [12.53] |
| 15 | R.PDSCH.1-26.2 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | [12.53] |
| 20 | R.PDSCH.1-26.3 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | [12.50] |
| 25 | R.PDSCH.1-26.4 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | [12.53] |
| 30 | R.PDSCH.1-27.1 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | [12.57] |
| 35 | R.PDSCH.1-27.2 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | [12.50] |
| 40 | R.PDSCH.1-27.3 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | [12.50] |
| 45 | R.PDSCH.1-27.4 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | [12.53] |
| 50 | R.PDSCH.1-27.5 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | [12.60] |

Table 5.2A.2.7-6: Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-43.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.60] |
| 10 | R.PDSCH.2-43.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.67] |
| 15 | R.PDSCH.2-43.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.73] |
| 20 | R.PDSCH.2-43.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.70] |
| 25 | R.PDSCH.2-43.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.60] |
| 30 | R.PDSCH.2-44.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.73] |
| 40 | R.PDSCH.2-3.6 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.50] |
| 50 | R.PDSCH.2-44.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.70] |
| 60 | R.PDSCH.2-44.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.67] |
| 80 | R.PDSCH.2-44.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.73] |
| 90 | R.PDSCH.2-44.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.57] |
| 100 | R.PDSCH.2-44.6 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.57] |

Table 5.2A.2.7-7: Single carrier performance for TDD (UL-DL pattern FR1.30-8) 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-45.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.60] |
| 10 | R.PDSCH.2-45.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.67] |
| 15 | R.PDSCH.2-45.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.73] |
| 20 | R.PDSCH.2-45.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.70] |
| 25 | R.PDSCH.2-45.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.60] |
| 30 | R.PDSCH.2-46.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.73] |
| 40 | R.PDSCH.2-32.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.50] |
| 50 | R.PDSCH.2-46.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.70] |
| 60 | R.PDSCH.2-46.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.67] |
| 80 | R.PDSCH.2-46.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.73] |
| 90 | R.PDSCH.2-46.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.57] |
| 100 | R.PDSCH.2-46.6 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | [12.57] |

Table 5.2A.2.7-8: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1-1 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.2.7-5 and Table 5.2A.2.7-6 per CC |
| 1-2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.2.7-6 |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7.8. Note 2: The minimum requirements only applicable for FR1.30-1 TDD pattern. |  |  |

Table 5.2A.2.7-9: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 2-1 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.2.7-5 and Table 5.2A.2.7-7 per CC |
| 2-2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.2.7-7 |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7.8. Note 2: The minimum requirements only applicable for FR1.30-8 TDD pattern. |  |  |

# ==============Next change==============

5.2A.3.6	Minimum requirements for ATG CA

The performance requirements are specified in Table 5.2A.3.6-6 and Table 5.2A.3.6-7 based on the single carrier requirements for different bandwidth specified in Table 5.2A.3.6-3, 5.2A.3.6-4, and Table 5.2A.3.6-5, with the addition of test parameters in Table 5.2A.3.6-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2A.3.6-1.

Table 5.2A.3.6-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions in the ATG scenario defined in B.1.1 with CA | 1-1, 1-2, 2-1, 2-2 |

Table 5.2A.3.6-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| TDD pattern |  |  | FR1.30-1 or FR1.30-8 |
| Propagation condition |  |  | Static propagation condition |
| Antenna configuration |  |  | 2x4 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | As defined in Table 5.2A-2 for FR1.30-1 TDD pattern, Table 5.2A.2.7-3 for FR1.30-8 TDD pattern |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3 for FR1.30-1 TDD pattern, Table 5.2A.2.7-4 for FR1.30-8 TDD pattern |

Table 5.2A.3.6-3: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-26.1 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | [9.8] |
| 10 | R.PDSCH.1-3.6 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | [9.73] |
| 15 | R.PDSCH.1-26.2 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | [9.73] |
| 20 | R.PDSCH.1-26.3 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | [9.63] |
| 25 | R.PDSCH.1-26.4 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | [9.67] |
| 30 | R.PDSCH.1-27.1 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | [9.73] |
| 35 | R.PDSCH.1-27.2 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | [9.60] |
| 40 | R.PDSCH.1-27.3 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | [9.67] |
| 45 | R.PDSCH.1-27.4 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | [9.83] |
| 50 | R.PDSCH.1-27.5 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | [9.8] |

Table 5.2A.3.6-4 Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-43.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.63] |
| 10 | R.PDSCH.2-43.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.67] |
| 15 | R.PDSCH.2-43.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.70] |
| 20 | R.PDSCH.2-43.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.67] |
| 25 | R.PDSCH.2-43.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.77] |
| 30 | R.PDSCH.2-44.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.70] |
| 40 | R.PDSCH.2-3.6 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.60] |
| 50 | R.PDSCH.2-44.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.67] |
| 60 | R.PDSCH.2-44.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.63] |
| 80 | R.PDSCH.2-44.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.73] |
| 90 | R.PDSCH.2-44.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.63] |
| 100 | R.PDSCH.2-44.6 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.63] |

Table 5.2A.3.6-5 Single carrier performance for TDD (UL-DL pattern FR1.30-8) 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-45.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.63] |
| 10 | R.PDSCH.2-45.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.67] |
| 15 | R.PDSCH.2-45.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.70] |
| 20 | R.PDSCH.2-45.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.67] |
| 25 | R.PDSCH.2-45.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.77] |
| 30 | R.PDSCH.2-46.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.70] |
| 40 | R.PDSCH.2-32.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.60] |
| 50 | R.PDSCH.2-46.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.67] |
| 60 | R.PDSCH.2-46.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.63] |
| 80 | R.PDSCH.2-46.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.73] |
| 90 | R.PDSCH.2-46.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.63] |
| 100 | R.PDSCH.2-46.6 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | [9.63] |

Table 5.2A.3.6-6: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1-1 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.6-3 and Table 5.2A.3.6-4 per CC |
| 1-2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.6-4 |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7.8. Note 2: The minimum requirements only applicable for FR1.30-1 TDD pattern. |  |  |

Table 5.2A.3.6-7: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 2-1 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.6-3 and Table 5.2A.3.6-5 per CC |
| 2-2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.6-5 |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7.8. Note 2: The minimum requirements only applicable for FR1.30-8 TDD pattern. |  |  |

# ==============Next change==============

A.3.2.1.1	Reference measurement channels for SCS 15 kHz FR1

# <unchanged part skipped>

Table A.3.2.1.1-3: PDSCH Reference Channel for FDD (64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-3.1 FDD | R.PDSCH.1-3.2 FDD | R.PDSCH.1-3.3 FDD | R.PDSCH.1-3.4 FDD | R.PDSCH.1-3.5 FDD | R.PDSCH.1-3.6 FDD | R.PDSCH.1-3.7 FDD | R.PDSCH.1-3.8 FDD (Note 5) | R.PDSCH.1-3.9 FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 52 | 52 | 26 (Note 3) | 26 (Note 4) | 52 | 52 | 52 | 52 | 52 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 19 | 19 | 19 | 19 | 19 | 22 | 17 | 17 | 22 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.51 | 0.51 | 0.51 | 0.51 | 0.51 | 0.65 | 0.43 | 0.43 | 0.65 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 1 | 1 | 4 | 4 | 2 |
| Number of DMRS REs |  | 12 | 24 | 24 | 24 | 12 | 12 | 24 | 48 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 42016 | 37896 | 18960 | 18960 | 21000 | 26632 | 64552 | 51216 | 53288 |
| Transport block CRC per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |  | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A | N/A |  | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 5 | 5 | 3 | 3 | 3 | 4 | 8 | 7 | 7 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |  | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits | 78624 | 67392 | 33696 | 33696 | 39312 | 39312 | 142272 | 112320 | 78624 |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 82368 | 74880 | 37440 | 37440 | 41184 | 41184 | 149760 | 119808 | 82368 |
| Max. Throughput averaged over 2 frames | Mbps | 39.915 | 36.001 | 18.012 | 18.012 | 19.950 | 25.300 | 61.32 | 48.66 | 50.624 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: PDSCH is scheduled in PRB numbers from 0 to 25. Note 4: PDSCH is scheduled in PRB numbers from 26 to 51. Note 5: Two codewords and given per codeword |  |  |  |  |  |  |  |  |  |  |

Table A.3.2.1.1-4: PDSCH Reference Channel for FDD (256QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-4.1 FDD | R.PDSCH.1-4.2 FDD | R.PDSCH.1-4.3 FDD |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Number of allocated resource blocks | PRBs | 52 | 52 | 52 |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 |  |  |
| MCS table |  | 256QAM | 256QAM | 256QAM |  |  |
| MCS index |  | 24 | 20 | 24 |  |  |
| Modulation |  | 256QAM | 256QAM | 256QAM |  |  |
| Target Coding Rate |  | 0.82 | 0.67 | 0.82 |  |  |
| Number of MIMO layers |  | 1 | 1 | 2 |  |  |
| Number of DMRS REs |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 1,…, 19 | Bits | 45096 | 36896 | 90176 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A |  |  |
| For Slots i = 1,…, 19 | CBs | 6 | 5 | 11 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 10, 11 | Bits | 52416 | 52416 | 104832 |  |  |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 54912 | 54912 | 109824 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 42.841 | 35.051 | 85.667 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

# <unchanged part skipped>

Table A.3.2.1.1-26: PDSCH Reference Channel for ATG FDD CC with CA scenario

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-26.1 FDD | R.PDSCH.1-26.2 FDD | R.PDSCH.1-26.3 FDD | R.PDSCH.1-26.4 FDD |
| Channel bandwidth | MHz | 5 | 15 | 20 | 25 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 25 | 79 | 106 | 133 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 22 | 22 | 22 | 22 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.65 | 0.65 | 0.65 | 0.65 |
| Number of MIMO layers |  | 1 | 1 | 1 | 1 |
| Number of DMRS REs |  | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 12808 | 40976 | 54296 | 67584 |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 2 | 5 | 7 | 9 |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits | 18900 | 59724 | 80136 | 100548 |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 19800 | 62568 | 83952 | 105336 |
| Max. Throughput averaged over 2 frames | Mbps | 12.168 | 38.927 | 51.581 | 64.205 |

Table A.3.2.1.1-27: PDSCH Reference Channel for ATG FDD CC with CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-27.1 FDD | R.PDSCH.1-27.2 FDD | R.PDSCH.1-27.3 FDD | R.PDSCH.1-27.4 FDD | R.PDSCH.1-27.5 FDD |
| Channel bandwidth | MHz | 30 | 35 | 40 | 45 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 160 | 188 | 216 | 242 | 270 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 22 | 22 | 22 | 22 | 22 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 |
| Number of MIMO layers |  | 1 | 1 | 1 | 1 | 1 |
| Number of DMRS REs |  | 12 | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  | 81976 | 96264 | 110632 | 125016 | 139376 |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits |  |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 10 | 12 | 14 | 15 | 17 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits | 120960 | 142128 | 163296 | 182952 | 204120 |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 126720 | 148896 | 171072 | 191664 | 213840 |
| Max. Throughput averaged over 2 frames | Mbps | 77.877 | 91.451 | 105.1 | 118.765 | 132.407 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

# ==============Next change==============

A.3.2.2.2	Reference measurement channels for SCS 30 kHz FR1

# <unchanged part skipped>

Table A.3.2.2.2-3: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-3.1 TDD | R.PDSCH.2-3.2 TDD | R.PDSCH.2-3.3 TDD | R.PDSCH.2-3.4 TDD | R.PDSCH.2-3.5 TDD | R.PDSCH.2-3.6 TDD | R.PDSCH.2-3.7 TDD | R.PDSCH.2-3.8 TDD (Note 5) | R.PDSCH.2-3.9 TDD |
| Channel bandwidth | MHz | 40 | 40 | 40 | 40 | 20 | 40 | 40 | 40 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 106 | 106 | 53 (Note 3) | 53 (Note 4) | 51 | 106 | 106 | 106 | 106 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | NA | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 | 31 | 31 | 31 | 27 | 31 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 19 | 19 | 19 | 19 | 19 | 22 | 17 | 17 | 22 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.51 | 0.51 | 0.51 | 0.51 | 0.51 | 0.65 | 0.43 | 0.43 | 0.65 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 1 | 1 | 4 | 4 | 2 |
| Number of DMRS REs |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 12 | 12 | 12 | 6 | 6 | 12 | N/A | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 24 | 24 | 24 | 12 | 12 | 24 | 48 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 27144 | 23040 | 11528 | 11528 | 6528 | 14856 | 38936 | N/A | 34816 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 83976 | 77896 | 38936 | 38936 | 20496 | 54296 | 131176 | 104496 | 108552 |
| Transport block CRC per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | N/A | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6}for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 4 | 3 | 2 | 2 | 1 | 2 | 5 | N/A | 5 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 10 | 10 | 5 | 5 | 3 | 7 | 16 | 13 | 13 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 20, 21 | Bits | 160272 | 145008 | 68688 | 72504 | 38556 | 83952 | 290016 | 228960 | 160272 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 53424 | 45792 | 22896 | 22896 | 12852 | 22896 | 91584 | NA | 53424 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 167904 | 152640 | 76320 | 76320 | 40392 | 80136 | 305280 | 244224 | 167904 |
| Max. Throughput averaged over 2 frames | Mbps | 118.796 | 109.768 | 54.869 | 54.869 | 28.975 | 76.271 | 184.88 | 141.07 | 153.508 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: PDSCH is scheduled in PRB numbers from 0 to 52. Note 4: PDSCH is scheduled in PRB numbers from 53 to 105. Note 5: Two codewords and given per codeword |  |  |  |  |  |  |  |  |  |  |

Table A.3.2.2.2-4: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (256QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-4.1 TDD | R.PDSCH.2-4.2 TDD | R.PDSCH.2-4.3 TDD | R.PDSCH.2-4.4 TDD | R.PDSCH.2-4.5 TDD |
| Channel bandwidth | MHz | 40 | 20 | 20 | 40 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 106 | 51 | 51 | 106 | 106 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 | 31 |
| MCS table |  | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| MCS index |  | 24 | 24 | 20 | 21 | 24 |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Target Coding Rate |  | 0.82 | 0.82 | 0.67 | 0.69 | 0.82 |
| Number of MIMO layers |  | 1 | 1 | 1 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | 6 | 6 | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 29192 | 14088 | 11528 | 49176 | 58384 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 92200 | 44040 | 35856 | 155776 | 184424 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 4 | 2 | 2 | 6 | 7 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 11 | 6 | 5 | 19 | 22 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 20, 21 | Bits | 106848 | 51408 | 51408 | 213696 | 213696 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 35616 | 17136 | 17136 | 71232 | 71232 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 111936 | 53856 | 53856 | 223872 | 223872 |
| Max. Throughput averaged over 2 frames | Mbps | 130.308 | 62.272 | 50.711 | 220.133 | 260.649 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

# <unchanged part skipped>

Table A.3.2.2.2-41: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-8 (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-41.1 TDD | R.PDSCH.2-41.2 TDD |  |  |  |
| Channel bandwidth | MHz | 40 | 40 |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 |  |  |  |
| Allocated resource blocks | PRBs | 106 | 106 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} |  | N/A | N/A |  |  |  |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames |  | 13 | 13 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 22 | 22 |  |  |  |
| Modulation |  | 64QAM | 64QAM |  |  |  |
| Target Coding Rate |  | 0.65 | 0.65 |  |  |  |
| Number of MIMO layers |  | 1 | 2 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} |  | N/A | N/A |  |  |  |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | Bits | N/A | N/A |  |  |  |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} | Bits | 54296 | 108552 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | Bits | N/A | N/A |  |  |  |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | CBs | N/A | N/A |  |  |  |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} | CBs | 7 | 13 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | Bits | N/A | N/A |  |  |  |
| For Slots i = 20 | Bits | 80136 | 160272 |  |  |  |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 24, 25, 28, 29} | Bits | 83952 | 167904 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 35.292 | 70.559 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-42: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-8 (256QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-42.1 TDD | R.PDSCH.2-42.2 TDD |  |  |  |
| Channel bandwidth | MHz | 40 | 40 |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 |  |  |  |
| Allocated resource blocks | PRBs | 106 | 106 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} |  | N/A | N/A |  |  |  |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames |  | 13 | 13 |  |  |  |
| MCS table |  | 64QAM | 256QAM |  |  |  |
| MCS index |  | 24 | 24 |  |  |  |
| Modulation |  | 256QAM | 256QAM |  |  |  |
| Target Coding Rate |  | 0.82 | 0.82 |  |  |  |
| Number of MIMO layers |  | 1 | 2 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} |  | N/A | N/A |  |  |  |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | Bits | N/A | N/A |  |  |  |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} | Bits | 63528 | 184424 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | Bits | N/A | N/A |  |  |  |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | CBs | N/A | N/A |  |  |  |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} | CBs | 8 | 22 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | Bits | N/A |  |  |  |  |
| For Slots i = 20 | Bits | 80136 | 213696 |  |  |  |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 24, 25, 28, 29} | Bits | 83952 | 223872 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 41.293 | 119.876 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-43: PDSCH Reference Channel for ATG TDD CC with UL-DL pattern FR1.30-1 and CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-43.1 TDD | R.PDSCH.2-43.2 TDD | R.PDSCH.2-43.3 TDD | R.PDSCH.2-43.4 TDD | R.PDSCH.2-43.5 TDD |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 11 | 24 | 38 | 51 | 65 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 | 31 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 22 | 22 | 22 | 22 | 22 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 |
| Number of MIMO layers |  | 1 | 1 | 1 | 1 | 1 |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | 6 | 6 | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 1800 | 3904 | 6144 | 8456 | 10760 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 5682 | 12296 | 19464 | 26120 | 33816 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 16 | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 1 | 1 | 1 | 2 | 2 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 1 | 2 | 3 | 4 | 5 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 20, 21 | Bits | 8316 | 18144 | 28728 | 38556 | 49140 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 2772 | 6048 | 9576 | 12852 | 16380 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 8712 | 19008 | 30096 | 40392 | 51480 |
| Max. Throughput averaged over 2 frames | Mbps | 7.963 | 17.38 | 27.505 | 36.953 | 47.804 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-44: PDSCH Reference Channel for ATG TDD CC with UL-DL pattern FR1.30-1 and CA scenario

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-44.1 TDD | R.PDSCH.2-44.2 TDD | R.PDSCH.2-44.3 TDD | R.PDSCH.2-44.4 TDD | R.PDSCH.2-44.5 TDD | R.PDSCH.2-44.6 TDD |
| Channel bandwidth | MHz | 30 | 50 | 60 | 80 | 90 | 100 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 78 | 133 | 162 | 217 | 245 | 273 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 | 4 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 | 31 | 31 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 22 | 22 | 22 | 22 | 22 | 22 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 |
| Number of MIMO layers |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of DMRS REs |  |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | 6 | 6 | 6 | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 12808 | 22032 | 26632 | 35856 | 39936 | 45096 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 39936 | 67584 | 83976 | 112648 | 127080 | 139376 |
| Transport block CRC per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 2 | 3 | 4 | 5 | 5 | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 5 | 9 | 10 | 14 | 16 | 17 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 20, 21 | Bits | 58968 | 100548 | 122472 | 164052 | 185220 | 206388 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 19656 | 33516 | 40824 | 54684 | 61740 | 68796 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 61776 | 105336 | 128304 | 171864 | 194040 | 216216 |
| Max. Throughput averaged over 2 frames | Mbps | 56.475 | 95.645 | 118.694 | 159.246 | 179.545 | 197.177 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |  |

Table A.3.2.2.2-45: PDSCH Reference Channel for ATG TDD CC with UL-DL pattern FR1.30-8 and CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-45.1 TDD | R.PDSCH.2-45.2 TDD | R.PDSCH.2-45.3 TDD | R.PDSCH.2-45.4 TDD | R.PDSCH.2-45.5 TDD |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 11 | 24 | 38 | 51 | 65 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 13 | 13 | 13 | 13 | 13 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 22 | 22 | 22 | 22 | 22 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 |
| Number of MIMO layers |  | 1 | 1 | 1 | 1 | 1 |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} |  | 12 | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} | Bits | 5632 | 12296 | 19464 | 26120 | 33816 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} | CBs | 1 | 2 | 3 | 4 | 5 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 20 | Bits | 8316 | 18144 | 28728 | 38556 | 49140 |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 24, 25, 28, 29} | Bits | 8712 | 19008 | 30096 | 40392 | 51480 |
| Max. Throughput averaged over 2 frames | Mbps | 3.661 | 7.992 | 12.652 | 16.978 | 21.98 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-46: PDSCH Reference Channel for ATG TDD CC with UL-DL pattern FR1.30-8 and CA scenario

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-46.1 TDD | R.PDSCH.2-46.2 TDD | R.PDSCH.2-46.3 TDD | R.PDSCH.2-46.4 TDD | R.PDSCH.2-46.5 TDD | R.PDSCH.2-46.6 TDD |
| Channel bandwidth | MHz | 30 | 50 | 60 | 80 | 90 | 100 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 78 | 133 | 162 | 217 | 245 | 273 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 13 | 13 | 13 | 13 | 13 | 13 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 22 | 22 | 22 | 22 | 22 | 22 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 | 0.65 |
| Number of MIMO layers |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of DMRS REs |  |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} | Bits | 39936 | 67584 | 83976 | 112648 | 127080 | 139376 |
| Transport block CRC per Slot |  |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | CBs | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 20, 24, 25, 28, 29} | CBs | 5 | 9 | 10 | 14 | 16 | 17 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |
| For Slots 0, 2, 3, 6, 7, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 and For Slots i = {30, …, 39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 20 | Bits | 58968 | 100548 | 122472 | 164052 | 185220 | 206388 |
| For Slot i = {1, 4, 5, 9, 10, 14, 15, 19, 24, 25, 28, 29} | Bits | 61776 | 105336 | 128304 | 171864 | 194040 | 216216 |
| Max. Throughput averaged over 2 frames | Mbps | 25.958 | 43.93 | 54.584 | 73.221 | 82.602 | 90.594 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |  |

# ==============End of change==============