# 3GPP TSG RAN WG4 Meeting#117	R4-2520995

# Dallas, USA, Nov 17 – 21, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for NTN testing for NGSO

Agenda item:	6.6.3.3

Document for:	Discussion

# 1. Introduction

During the last meeting, RAN4 discussed issues related to the demodulation performance for NTN testing for NGSO [1]. In this paper, we share our perspectives on the remaining open issues of test applicability.

2. Test Applicability

Applicability of Rel-19 tests with time-varying Doppler shift and propagation delay model

For UE declaring Rel-19 UE,

- Option 1

  - Applicable tests are 1-5, 1-6, 1-3 and 1-4.

- Option 2

  - Applicable tests are 1-5, 1-6.

For UE declaring Rel-17/18 UE,

- Option 1a:

  - If UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 1-5, 1-6, 1-3 and 1-4.

  - Otherwise, applicable tests are 1-1, 1-2, 1-3 and 1-4.

- Option 1b:

  - If UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 1-5, 1-6.

  - Otherwise, applicable tests are 1-1, 1-2, 1-3 and 1-4.

- Option 2:

  - Applicable tests are 1-1, 1-2, 1-3 and 1-4.

The way forward is outlined below.

As a reference point for this issue, we also conducted simulations for Rel-17 Tests 1-3 and 1-4 using the Rel-19 channel model, which incorporates time-varying Doppler shift and propagation delay. The observed performance degradation was comparable to that of Tests 1-1 and 1-2 under the same Rel-19 channel model. Specifically, an additional margin of 0.5 dB remains applicable to Tests 1-3 and 1-4 when compared to the Rel-17 channel model. Therefore, incorporating Tests 1-3 and 1-4 into the Rel-19 test suite is straightforward and should be pursued. This approach streamlines the testing process and eliminates the need to evaluate Rel-19 UEs using the artificial setup of Rel-17 channel model for these scenarios. The corresponding Rel-19 tests (under the Rel-19 channel model) for Tests 1-3 and 1-4 in Rel-17 can be designated as Tests 1-7 and 1-8, respectively.

Observation 1: The additional 0.5 dB margin applied to Tests 1-1 and 1-2 under the Rel-19 channel model is also adequate for Tests 1-3 and 1-4. The observed performance degradation for these tests remains within the same bounds as previously agreed for Tests 1-1 and 1-2.

Proposal 1: Include Rel-17 Tests 1-3 and 1-4 in the Rel-19 test suite, applying the same 0.5 dB additional margin. These Rel-17 tests, i.e., Tests 1-3 and 1-4  under the Rel-19 channel model can be designated as Tests 1-7 and 1-8, respectively.

Proposal 2: For Rel-19 UEs, Tests 1-5, 1-6, 1-7, and 1-8 shall apply.

Proposal 3: For Rel-17/18 UEs, the satellite motion-based time-varying channel model can be optionally applied to Tests 1-5, 1-6, 1-7, and 1-8, provided the channel model is available at the TE and RAN5 test framework permits the DUT to choose to do so.

3. Conclusions

This paper presents our perspectives on test applicability. The following proposals have been made.

Observation 1: The additional 0.5 dB margin applied to Tests 1-1 and 1-2 under the Rel-19 channel model is also adequate for Tests 1-3 and 1-4. The observed performance degradation for these tests remains within the same bounds as previously agreed for Tests 1-1 and 1-2.

Proposal 1: Include Rel-17 Tests 1-3 and 1-4 in the Rel-19 test suite, applying the same 0.5 dB additional margin. These Rel-17 tests, i.e., Tests 1-3 and 1-4  under the Rel-19 channel model can be designated as Tests 1-7 and 1-8, respectively.

Proposal 2: For Rel-19 UEs, Tests 1-5, 1-6, 1-7, and 1-8 shall apply.

Proposal 3: For Rel-17/18 UEs, the satellite motion-based time-varying channel model can be optionally applied to Tests 1-5, 1-6, 1-7, and 1-8, provided the channel model is available at the TE and RAN5 test framework permits the DUT to choose to do so.

4. References

[1] R4-2515053, “Way Forward for [116bis][328] Rel-19 demodulation part3,” 3GPP TSG RAN WG4 Meeting #116bis, Prague, Czech Republic, Oct 13 – 17, 2025.