3GPP TSG-RAN WG4 Meeting #117	R4-2521357

# Dallas, USA, 17 - 21 Nov, 2025

Title: 	Discussion on SBFD demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	6.13.4

Document for:	Discussion

Background

During RAN4#116bis meeting, Way Forward [1] for NR_duplex_evo_demod was approved. In this contribution, we share our view on SBFD demodulation performance requirements.

Discussion

General Scope

Agreement: If RAN4 to define Rel-19 SBFD PUSCH requirement with UL muting only considering inter-site gNB-gNB interference, self-interference and intra-site inter sector interference are not taken into account. - For inter-site gNB-gNB interference modelling - Considering the RF requirement, the following INR values derived based on link budget analysis can be considered for evaluation. Based on evaluation results, RAN4 can further discuss whether to introduce the PUSCH requirement with UL muting Table 2-1 Revised INR values for FR1 Conditions; FR1. Interf1 > Interf2; INR1 <= 9.2dB, INR2 <= -0.8dB Trial simulation could try following sets: [INR1, INR2] = {[5, -5], [9, -1]} dB. Interf1 = Interf2; INR1 = INR2 <=3.2dB Trial simulation could try following sets: [INR1, INR2] = {[-1, -1], [3, 3]} dB. Note: The interference wave form could use PUSCH with 1+1 DM-RS and CP-OFDM to simplify simulation setup.. - Not pursue to derive INR based system level simulation for inter-site gNB-gNB interference modelling when defining PUSCH requirement with UL muting for SBFD in Rel-19. Agreement: FFS on whether RAN4 introduce the PUSCH requirement with UL symbols muting

Here we provide our evaluation results based on the agreed WF.

| Case | Resource muting | Mapping Type | INR | MCS | Antenna | SNR@70% of Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Using muting | Not use |
| 1 | 1 symbol | A | (5, -5) | 16 | 1x2 | 12.6 | 13.3 |
| 2 | 1 symbol | A | (9, -1) | 16 | 1x2 | 13.7 | 14.5 |
| 3 | 1 symbol | A | (-1, -1) | 16 | 1x2 | 12.0 | 12.6 |
| 4 | 1 symbol | A | (3, 3) | 16 | 1x2 | 12.8 | 13.5 |
| 5 | 1 symbol | B | (5, -5) | 16 | 1x2 | 12.6 | 13.3 |
| 6 | 1 symbol | B | (9, -1) | 16 | 1x2 | 13.7 | 14.5 |
| 7 | 1 symbol | B | (-1, -1) | 16 | 1x2 | 12.0 | 12.6 |
| 8 | 1 symbol | B | (3, 3) | 16 | 1x2 | 12.8 | 13.5 |

We can observe that there is about 0.5~1.0dB performance gain when UL muting is enabled. To properly verify PUSCH performance under gNB-gNB interference scenario with UL symbol muting, we propose introduce the PUSCH requirement with UL symbols muting.

RAN4 introduces the PUSCH performance requirement with UL symbols muting.

Test setup for demodulation requirement if introduced

Issue 1-2-7 Number of symbol muting and symbol configuration Way forward Option 1: 1 - For Mapping type, A - For Mapping type B Option 2: 2 - For Mapping type A - For Mapping type B

For the number of symbol muting and symbol configuration, both 1 and 2 are feasible. From throughput point of view, 1 UL symbol is preferred.

Select Option 1 for the number of symbol muting and symbol configuration.

Proposals

In this contribution, we share our view on SBFD demodulation performance requirements. Our observations and proposals are:

- RAN4 introduces the PUSCH performance requirement with UL symbols muting.

Select Option 1 for the number of symbol muting and symbol configuration.

Reference

R4-2515038, Way Forward for [116bis][311] Rel-19 Demodulation_Part1, RAN4#116bis, Samsung