3GPP TSG-RAN WG4 Meeting #117	R4-2521394

Dallas, US, 17th – 21st November 2025

Agenda item:	8.6

Source: 		NTT DOCOMO, INC.

Title: 	Views on 6G RRM

Document for:	Discussion

- Introduction

Discussion on 6G RRM has started from RAN4#116bis. Opinions on RRM design for 6G were solicited from various companies, and the direction of the discussion was summarized in WF [1]. This contribution presents our view on RRM.

- Discussion

According to WF [1], observed issues are captured as below.

Measurement gap(MG) and interruption Issue 4-1: MG related scope Agreement in main session (Tue): Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. - Interruption enhancement due to gap-less measurement will be part of the MG discussion. [Way forward] - FFS followings in RAN4#117 (other sub-topics are not precluded) - based on the majority views from companies, RAN4 RRM to discuss which MG related sub-topics can be prioritized from the following candidate list, and then discuss the solutions for the selected topics in 6G SI: - MG pattern reduction from 5G (10 companies: Apple, MTK, QC, OPPO, Sony, Xiaomi, vivo, Ericsson, Samsung, Nokia) - Gap-less measurement and its side conditions (10 companies: MTK, Samsung, CMCC, ZTE, NTT DCM, HW, Ericsson, Nokia, QC, Xiaomi) - In scenarios with and without an available RF chain. - Unified MG (7 companies: Apple, OPPO, LGE, Xiaomi, ZTE, Ericsson, QC), e.g., - Unified MG configuration - Unified MG and scheduling restriction - Unified MG for different feature related measurements - Adapative MG operation, e.g., activation/deactivation/cancellation/skipping (7 companies: Apple, OPPO, CMCC, Xiaomi, ZTE, Ericsson, LGE) - Using which 5G MG enhancement features to 6G day 1 (OPPO, CMCC, Xiaomi, vivo) - E.g., needforGap, NCSG, concurrent MG, preconfigured MG and etc. - MG sharing(Apple, OPPO, vivo), e.g., - among intra-frequency, inter-frequency, and inter-RAT measurement (including L3 and L1 measurement) - UE assisted MG configuration (Apple, QC) - e.g., MG requesting by UE (Apple, QC) - Multi-CC measurements in MG (Apple, CMCC, ZTE(searcher related)) - Optimization on MGL and RF tuning/retuning (Apple, MTK, QC) - MG applicability for per-UE, per-FR, per-CC, or per-CC group (OPPO, Xiaomi, ZTE(granularity)) Mobility related RRM Issue 6: Mobility related RRM [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to start from mobility related RRM solutions with less RAN1/2-dependency. - RAN4 to identify which of the following topics can be starts directly in RAN4: - Latency and/or interruption reduction for mobility through RAN4-defined components (Apple, MTK(interruption only), QC, LGE, ZTE, HW, vivo, Ericsson, Nokia, CMCC, Samsung) - discuss RRM part reduction during mobility, e.g., L1/L3 measurement, beam sweeping and etc. - discuss the scenarios/conditions for such reduction (known, unknown, or other status) - discuss NW controlled and UE initiated L1/L3 measurement report - Solutions for longer SSB periodicity in mobility (MTK, Samsung, Ericsson) - Early RRC decoding, and/or, DL/UL sync, and/or, early T/F tracking for mobility (MTK, Nokia) - Virtual RRM UE group (Apple, ZTE) - Unified measurement and mobility framework (QC) - E.g., based on 5G LTM - End-to-end handover latency target (QC) - RAN4 to study the practically achievable end-to-end handover latency target, taking into account user-plane data forwarding latency, to better align handover requirements with practical effectiveness. - Unified UE measurement requirements across cell changes (Nokia) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other mobility related RRM topics if there are sufficient conclusions from other WGs - FFS: Check point timeline Option 2: RAN4 to check whether or not starting discussion on other mobility related RRM topics once there are sufficient conclusions from other WGs.

Measurement gap (MG) and interruption

Gap-less measurement allows the UE to perform inter-frequency measurements without interrupting ongoing data transmission, leveraging idle RF chains for parallel reception. In NR, solutions like NCSG and NeedForGap were introduced but faced limited deployment due to late specification and ambiguity in UE behaviour. For 6G, it is proposed that gap-less capability be specified from Day-1, including both signalling and RRM requirements. From an operator’s perspective, this feature is essential to reduce UE power consumption and improve network throughput, especially as multi-RF chain UEs become common. Defining clear conditions and capabilities for gap-less operation will enable more efficient scheduling and minimize service interruption during measurement procedures.

In order to enable efficient and interruption-free measurement procedures in 6G, it is proposed that RAN4 initiate a focused study on the conditions under which gap-less measurement can be supported, and the corresponding UE capabilities required. For 6G, it is essential to establish a clear and unified framework that defines the physical layer prerequisites (e.g., RF chain availability, baseband processing capacity, retuning time) and architectural assumptions (e.g., modular capability blocks) that determine whether a UE can support gap-less measurement. This includes aligning on baseline UE architecture across device types (e.g., TN, NTN, MBB, FWA) and ensuring that measurement capability definitions are modular and reusable across RRM procedures. By doing so, RAN4 can avoid fragmented specifications, reduce implementation complexity, and ensure predictable system behaviour from Day-1 of 6G deployment.

Observation 1: The implementation architecture of the UE's RF chain and baseband processing capabilities significantly impacts the applicability and interruption of measurement gaps. Therefore, ambiguity in behaviour arises when the UE's capabilities and conditions are not clearly defined.

Proposal 1: RAN4 should initiate studies to clarify the application conditions for gap-less measurement and UE capabilities (such as RF chain count, simultaneous reception capability, BWP configuration, etc.).

In NR, over 25 measurement gap patterns were defined, but only a few were implemented in practice, leading to unnecessary complexity in UE implementation, testing, and standardization. For 6G, it is proposed to reduce the number of mandatory gap patterns and focus only on those relevant to real deployment scenarios. Furthermore, a unified measurement gap concept should be introduced to consolidate NR MG and scheduling restrictions, and harmonize gap sharing across intra-frequency, inter-frequency, and inter-RAT measurements. From an operator’s viewpoint, this simplification will reduce overhead, improve spectrum efficiency, and streamline network configuration and UE behaviour.

Observation 2: While numerous MG patterns were specified for NR, only a small fraction is used in operation, and the differing MG designs for each function are causing complexity and overhead.

Proposal 2: For 6G, a unified MG design integrating NR MG and scheduling constraints should be introduced. This should explore reducing MG patterns and mechanisms for sharing MGs across intra-frequency, inter-frequency, and inter-RAT measurements.

Mobility related RRM

In NR, handover procedures often involve significant interruption due to delayed synchronization and rigid execution timing. While enhancements like DAPS and LTM have been proposed, their complexity has limited commercial adoption. For 6G, it is proposed to prioritize solutions that reduce handover interruption independently of delay, such as early RRC decoding and early time/frequency tracking. From an operator’s perspective, minimizing interruption is more critical than reducing delay, as it directly impacts user experience and service continuity. A Day-1 handover solution should be universally applicable and not rely on restrictive conditions, ensuring robust mobility performance across diverse deployment scenarios.

Observation 3: While advanced handover techniques such as LTM were introduced in NR, commercial deployment remains limited. Handover interruptions and throughput degradation are still being observed.

Proposal 3: In 6G, it is promising to introduce a generic handover design with low condition dependency as a Day-1 specification, leveraging the advantages of LTM. RAN4 should lead efforts to explore the potential for reducing interruptions.

- Conclusion

In this contribution, we offer our 6GR view as follows.

Observation 1: The implementation architecture of the UE's RF chain and baseband processing capabilities significantly impacts the applicability and interruption of measurement gaps. Therefore, ambiguity in behaviour arises when the UE's capabilities and conditions are not clearly defined.

Proposal 1: RAN4 should initiate studies to clarify the application conditions for gap-less measurement and UE capabilities (such as RF chain count, simultaneous reception capability, BWP configuration, etc.).

Observation 2: While numerous MG patterns were specified for NR, only a small fraction is used in operation, and the differing MG designs for each function are causing complexity and overhead.

Proposal 2: For 6G, a unified MG design integrating NR MG and scheduling constraints should be introduced. This should explore reducing MG patterns and mechanisms for sharing MGs across intra-frequency, inter-frequency, and inter-RAT measurements.

Observation 3: While advanced handover techniques such as LTM were introduced in NR, commercial deployment remains limited. Handover interruptions and throughput degradation are still being observed.

Proposal 3: In 6G, it is promising to introduce a generic handover design with low condition dependency as a Day-1 specification, leveraging the advantages of LTM. RAN4 should lead efforts to explore the potential for reducing interruptions.

References

[1] 3GPP, R4-2514644, “WF on 6G RRM”, Feature lead (Apple).