3GPP TSG-RAN WG4 Meeting #117		 			                        R4-2521565
Dallas, US, November 17th – 21st, 2025
 
Agenda Item: 7.3.3 Enhanced test methodologies to cover NTN Ka and Ku bands

Source:	SKY Perfect JSAT

Title: Discussion on VSAT Test Methodology

Document for:	Discussion

# Introduction

VSAT UE measurement method will be discussed in the following RAN4 meeting.  This document introduces the testing methodology for existing Ku-Band VSATs as a GSO operator.  In addition, introduces the sizes and weights of currently available VSAT antennas that are applicable as NTN VSAT Types.

# Analysis and Discussion

## Ku-Band VSAT Test Methodology

ETSI standards measurement methods for earth stations communicating with satellite systems for Ku-band as listed Table.1.

Table 1: ETSI standards for Ku-Band

| ETSI No. | Frequency Bands | Earth Station Type |
| --- | --- | --- |
| EN 303 980[1] | 11 GHz to 14 GHz | Fixed and in-motion Earth Stations |
| EN 303 981[2] | 11 GHz to 14 GHz | Fixed and in-motion Wide Band Earth Stations |
| EN 301 428[3] | 11 /12 / 14 GHz | Very Small Aperture Terminals (VSATs) |
| EN 302 186[4] | 11 /12 / 14 GHz | Satellite mobile Aircraft Earth Stations (AESs) |
| EN 302 340[5] | 11 /12 / 14 GHz | Earth Stations on board Vessels (ESVs) |

As a sample, as for fixed and in-motion Earth Stations communicating with non-geostationary satellite systems (NEST) in the 11 GHz to 14 GHz frequency bands, the two types of testing methodology are described[1]. One is radiated measurement method and the other is conducted measurement method. Other ETSI standards referenced above also specify both the radiated(OTA) measurement method and the conducted measurement method.

Observation 1： ETSI standards specify conducted and OTA test methods for various types of earth stations, such as AES, ESV, and fixed earth stations.

Refer to the ETSI documentation regarding the radiated measurement method and the conducted measurement method.

(1) Radiated measurement

The radiated measurement method is defined as follows in ETSI standard.

(2) Conducted measurement:

The conducted measurement method is defined as follows in ETSI standard.

The ETSI standards’ Conducted Method refers to a testing approach where measurement equipment such as a spectrum analyzer is connected to the antenna port. The antenna port can also be described as the output port of the VSAT’s HPA (High Power Amplifier). This conducted measurement method (measurement at the HPA output port) has long been adopted in the satellite communications industry and is characterized by its simplicity.

What is noteworthy is that ETSI standards only specify terminal requirements for OTA (Over-The-Air) testing, even though both OTA and conducted test methods are defined. In other words, ETSI does not define separate requirements for the conducted method; instead, the requirement values for the conducted method are derived from the OTA requirements by considering antenna gain information and measurement system losses.

For example, in the case of an EIRP (Equivalent Isotropically Radiated Power) test, as stated in the excerpt from the ETSI standards mentioned above, the EIRP is calculated based on the measured value, the known antenna gain relative to an isotropic antenna, and, if applicable, any losses in the measurement system due to cables and connectors.

Observation 2: There is no need to define new requirements for the conducted measurement method, as the requirement values for the conducted method are derived from the OTA requirements by taking into account antenna gain information and measurement system losses.

A VSAT with an antenna port (an HPA output port) can use the conducted measurement method, whereas a VSAT without an antenna port can only be measured using the OTA measurement method.

Observation 3: A VSAT with an antenna port (an HPA output port) can use the conducted measurement method, whereas a VSAT without an antenna port can only be measured using the OTA measurement method.

On the other hand, in TS 38.101-5(Rel 19, ver19.2.0)[6], there are 6 types defined as NTN VSAT types..

Table 2: TS38.101-5 Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52, 3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 64 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: GSO support is applicable to bands n512, n511, n510, n509, n508 and n248, n247; LEO support is only applicable to bands n509, n508 and n248, n247. NOTE 4: The NTN VSAT type 6 is only applicable to bands n509, n508 and n248, n247. |  |  |

It is difficult to determine whether an NTN VSAT type has an antenna port (an HPA output port) or not, and it is assumed that for each NTN VSAT type, both terminals with an antenna port and those without an antenna port may exist.

Observation 4: It is difficult to determine whether an NTN VSAT type has an antenna port (an HPA output port) or not, and it is assumed that for each NTN VSAT type, both terminals with an antenna port and those without an antenna port may exist.

Consequently, it is desirable to define measurement methods for both types to accommodate all NTN VSAT types.

Proposal 1: RAN4 to consider defining both radiated and conducted measurement methods so that they are applicable to various NTN VSAT types, and should also ensure that the measurement method can be selected for each terminal regardless of its type.

## VSAT Device Size and Weight

In the previous meeting, VSAT vendors/NTN operators to provide the maximum VSAT device size, antenna size and  weight limits for each NTN VSAT type. This section introduces the sizes and weights of currently available VSAT antennas that are applicable as NTN VSAT Types.

Table 3: VSAT Device Size and Weight

| Antenna Vendor/Model | Antenna Type | NTN VSAT Type | Antenna Size | Antenna Weight | BUC/LNB Weight* |
| --- | --- | --- | --- | --- | --- |
| Global Skyware Type960 ClassII | Parabolic Fixed VSAT antenna | 1 | 96cm | Approx. 17kg | Max. 5.4kg |
| Global Skyware Type988 ClassIII | Parabolic Fixed VSAT antenna | 1 | 98cm | Approx. 22kg | Max. 11kg |
| Intellian v60E | Parabolic Maritime VSAT antenna | 4 | 65cm | Approx. 45kg | (included) |
| Intellian v100NX | Parabolic Maritime VSAT antenna | 4 | 100cm | Approx. 113kg | (included) |
| Greenerwave Ku small GEO | Electronic steering antenna | 5 | ~ 59 cm x 48 cm x 12 cm | ~ 12 kg | (included) |
| Kymeta Hawk u8 | Electronic steering antenna | 5 | 89.5 cm × 89.5 cm × 14 cm | 30.4kg | (included) |

*Generally, the LNB and BUC are mounted on the antenna flange of a parabolic fixed VSAT antenna. The aggregate antenna weight should be included LNB and BUC weight.

Observation 5:  AS shown below, introduce several types of VSAT antennas currently deployed and show their sizes and weights for reference.

| Antenna Vendor/Model | Antenna Type | NTN VSAT Type | Antenna Size | Antenna Weight | BUC/LNB Weight* |
| --- | --- | --- | --- | --- | --- |
| Global Skyware Type960 ClassII | Parabolic Fixed VSAT antenna | 1 | 96cm | Approx. 17kg | Max. 5.4kg |
| Global Skyware Type988 ClassIII | Parabolic Fixed VSAT antenna | 1 | 98cm | Approx. 22kg | Max. 11kg |
| Intellian v60E | Parabolic Maritime VSAT antenna | 4 | 65cm | Approx. 45kg | (included) |
| Intellian v100NX | Parabolic Maritime VSAT antenna | 4 | 100cm | Approx. 113kg | (included) |
| Greenerwave Ku small GEO | Electronic steering antenna | 5 | ~ 59 cm x 48 cm x 12 cm | ~ 12 kg | (included) |
| Kymeta Hawk u8 | Electronic steering antenna | 5 | 89.5 cm × 89.5 cm × 14 cm | 30.4kg | (included) |

Proposal 2 : RAN4 to take into account the existing deployed antennas, as shown in the table below,  when considering the measurement environment.

| Antenna Vendor/Model | Antenna Type | NTN VSAT Type | Antenna Size | Antenna Weight | BUC/LNB Weight* |
| --- | --- | --- | --- | --- | --- |
| Global Skyware Type960 ClassII | Parabolic Fixed VSAT antenna | 1 | 96cm | Approx. 17kg | Max. 5.4kg |
| Global Skyware Type988 ClassIII | Parabolic Fixed VSAT antenna | 1 | 98cm | Approx. 22kg | Max. 11kg |
| Intellian v60E | Parabolic Maritime VSAT antenna | 4 | 65cm | Approx. 45kg | (included) |
| Intellian v100NX | Parabolic Maritime VSAT antenna | 4 | 100cm | Approx. 113kg | (included) |
| Greenerwave Ku small GEO | Electronic steering antenna | 5 | ~ 59 cm x 48 cm x 12 cm | ~ 12 kg | (included) |
| Kymeta Hawk u8 | Electronic steering antenna | 5 | 89.5 cm × 89.5 cm × 14 cm | 30.4kg | (included) |

# Conclusions

This contribution made the following observations and proposals:

Observation 1： ETSI standards specify conducted and OTA test methods for various types of earth stations, such as AES, ESV, and fixed earth stations.

Observation 2: There is no need to define new requirements for the conducted measurement method, as the requirement values for the conducted method are derived from the OTA requirements by taking into account antenna gain information and measurement system losses.

Observation 3: A VSAT with an antenna port (an HPA output port) can use the conducted measurement method, whereas a VSAT without an antenna port can only be measured using the OTA measurement method.

Observation 4: It is difficult to determine whether an NTN VSAT type has an antenna port (an HPA output port) or not, and it is assumed that for each NTN VSAT type, both terminals with an antenna port and those without an antenna port may exist.

Proposal 1: RAN4 to consider defining both radiated and conducted measurement methods so that they are applicable to various NTN VSAT types, and should also ensure that the measurement method can be selected for each terminal regardless of its type.

Observation 5:  AS shown below, introduce several types of VSAT antennas currently deployed and show their sizes and weights for reference.

| Antenna Vendor/Model | Antenna Type | NTN VSAT Type | Antenna Size | Antenna Weight | BUC/LNB Weight* |
| --- | --- | --- | --- | --- | --- |
| Global Skyware Type960 ClassII | Parabolic Fixed VSAT antenna | 1 | 96cm | Approx. 17kg | Max. 5.4kg |
| Global Skyware Type988 ClassIII | Parabolic Fixed VSAT antenna | 1 | 98cm | Approx. 22kg | Max. 11kg |
| Intellian v60E | Parabolic Maritime VSAT antenna | 4 | 65cm | Approx. 45kg | (included) |
| Intellian v100NX | Parabolic Maritime VSAT antenna | 4 | 100cm | Approx. 113kg | (included) |
| Greenerwave Ku small GEO | Electronic steering antenna | 5 | ~ 59 cm x 48 cm x 12 cm | ~ 12 kg | (included) |
| Kymeta Hawk u8 | Electronic steering antenna | 5 | 89.5 cm × 89.5 cm × 14 cm | 30.4kg | (included) |

Proposal 2 : RAN4 to take into account the existing deployed antennas, as shown in the table below,  when considering the measurement environment.

| Antenna Vendor/Model | Antenna Type | NTN VSAT Type | Antenna Size | Antenna Weight | BUC/LNB Weight* |
| --- | --- | --- | --- | --- | --- |
| Global Skyware Type960 ClassII | Parabolic Fixed VSAT antenna | 1 | 96cm | Approx. 17kg | Max. 5.4kg |
| Global Skyware Type988 ClassIII | Parabolic Fixed VSAT antenna | 1 | 98cm | Approx. 22kg | Max. 11kg |
| Intellian v60E | Parabolic Maritime VSAT antenna | 4 | 65cm | Approx. 45kg | (included) |
| Intellian v100NX | Parabolic Maritime VSAT antenna | 4 | 100cm | Approx. 113kg | (included) |
| Greenerwave Ku small GEO | Electronic steering antenna | 5 | ~ 59 cm x 48 cm x 12 cm | ~ 12 kg | (included) |
| Kymeta Hawk u8 | Electronic steering antenna | 5 | 89.5 cm × 89.5 cm × 14 cm | 30.4kg | (included) |

# References

[1] ETSI EN 303 980, Satellite Earth Stations and Systems (SES); Fixed and in-motion Earth Stations communicating with non-geostationary satellite systems (NEST) in the 11 GHz to 14 GHz frequency bands; Harmonised Standard for access to radio spectrum.

[2] ETSI EN 303 981, Satellite Earth Stations and Systems (SES); Fixed and in-motion Wide Band Earth Stations communicating with non-geostationary satellite systems (WBES) in the 11 GHz to 14 GHz frequency bands; Harmonised Standard for access to radio spectrum

[3] ETSI EN 301 428, Satellite Earth Stations and Systems (SES); Harmonised Standard for Very Small Aperture Terminal (VSAT); Transmit-only, transmit/receive or receive-only satellite earth stations operating in the 11/12/14 GHz frequency bands covering the essential requirements of article 3.2 of Directive 2014/53/EU

[4] ETSI EN 302 186, Satellite Earth Stations and Systems (SES); Satellite mobile Aircraft Earth Stations (AESs) operating in the 11/12/14 GHz frequency bands; Harmonised Standard for access to radio spectrum

[5] ETSI EN 302 340, Satellite Earth Stations and Systems (SES); Harmonised Standard for satellite Earth Stations on board Vessels (ESVs) operating in the 11/12/14 GHz frequency bands allocated to the Fixed Satellite Service (FSS) covering the essential requirements of article 3.2 of the Directive 2014/53/EU

[6] TS 38.101-5(V19.2.0), User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 19)