3GPP TSG RAN WG4 Meeting #117	R4-2521822

Dallas, USA 17th – 21st November 2025

Agenda Item:	6.7.3

Source: 	Ericsson

Title:	Discussions on open issues of ATG CA RRM Performance

Document for:	Discussion

# Introduction

During our previous meeting, we reviewed the ATG CA test cases and agreed on the work split. The approved workflow for RAN4#116 is available in [1]. This contribution primarily aims to address the open issues related to the detailed test configurations.

# Discussion

## Test Scope

In the last meeting, we agreed on the following test cases.

Issue 2-1-1: Test scope - separate tests for neighbour cell skipping/not skipping Agreement: - Introduce Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle (no signalling for neighbour cell skipping) - FFS whether and how to introduce Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms or 160ms SCell measurement cycle (with signalling for neighbour cell skipping) Issue 2-1-3: Detail test configuration and test procedure - Event triggered reporting on SCC with deactivated SCell Agreement: - For Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle (no signalling for neighbour cell skipping), the test in Clause A.6.6.1.8 TS38.133 can be taken as the reference, except that: - No highSpeedMeasCA-Scell-r17 indication configured - the DRX configuration shall be OFF - the Initial UL BWP configuration and Active UL BWP configuration for SCell shall be N/A - the propagation condition shall follow R18 ATG test cases. PCell shall be continuously scheduled in DL - The T2 and test requirements shall be derived based on Table 9.2D.5.1-2 and Table 9.2D.5.2-2, and include the interruption requirement - For Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle (with signalling for neighbour cell skipping), if the test will be introduced, the test in Clause A.6.6.1.8 TS38.133 can be taken as the reference, except that: - No highSpeedMeasCA-Scell-r17 indication configured - the DRX configuration shall be OFF - the Initial UL BWP configuration and Active UL BWP configuration for SCell shall be N/A - the measCycleSCell shall be configured as 160ms - the neighbour cell measurement skipping indication shall be configured - the propagation condition shall follow R18 ATG test cases - The T2 and test requirements shall be derived based on Table 9.2D.5.1-2 and Table 9.2D.5.2-2. - Measurement report triggering event: FFS - Note: whether this test will be introduced is depending on Issue 2-1-1.

When neighbour cell measurement skipping is configured via the neighSCellMeasSkipping IE, the UE skips neighbour-cell measurements on the SCell to reduce measurement overhead and improve throughput performance. This is only applicable to UE supporting antennaArrayType-r18 and CA in inter-band CA scenario.

In co-located inter-band CA scenarios, the UE may reuse neighbour-cell measurement results obtained on the PCell to represent equivalent SCell-neighbour conditions. Allowing this reuse enables the UE to continue evaluating Event A6 in a consistent manner, even though the SCell’s own neighbour measurements are skipped.

Currently, this reuse behaviour is not explicitly captured in RAN4 or RAN2 spec. Therefore, it is proposed to introduce a dedicated test case to verify UE behaviour when neighSCellMeasSkipping is enabled on the SCell.

RAN4 should agree to introduce separate Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160ms SCell measurement cycle with signalling for neighbour cell skipping.

Furthermore, when neighbour cell measurement skipping is configured for SCell via the neighSCellMeasSkipping IE, the UE reuses the neighbouring cell measurement obtained on the PCell to represent the SCell threshold measurement report triggering event A6, provided the PCell measurement is valid for SCell. This should be captured in the test procedure. The rest of the test configuration should be similar to baseline event-triggered reporting on SCC, with the SCell test deactivated under non-DRX for 160 ms SCell measurement cycle.

Measurement report triggering event A6, with the condition that PCell measurement is valid for SCell, should be captured in the test procedure.

The primary objective of this test should be to verify that when neighSCellMeasSkipping is enabled for the SCell, the UE correctly triggers event A6 by reusing PCell neighbouring cell measurements under co-located inter-band CA conditions.

During the test, the UE should continuously monitor the PCell in period T1. When the event condition (e.g., A6) is met, relying on reasonable PCell measurement, the UE sends a measurement report within T2, with a reporting delay of less than 1600 ms from the beginning of T2. This shorter cycle ensures timely event triggering and preserves reporting latency comparable to the non-skipping configuration.

Primary objective of this test should be to verify that when neighSCellMeasSkipping is enabled for the SCell, the UE correctly triggers an event A6 by reusing PCell neighbouring cell measurements under co-located inter-band CA conditions. The rate of correct events observed during repeated tests shall be at least 90 %.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 RRM performance aspects for ATG UE DL CA and made the following Proposals.

Proposal 1	RAN4 should agree to introduce separate Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160ms SCell measurement cycle with signalling for neighbour cell skipping.

Proposal 2	Measurement report triggering event A6, with the condition that PCell measurement is valid for SCell, should be captured in the test procedure.

Proposal 3	Primary objective of this test should be to verify that when neighSCellMeasSkipping is enabled for the SCell, the UE correctly triggers an event A6 by reusing PCell neighbouring cell measurements under co-located inter-band CA conditions. The rate of correct events observed during repeated tests shall be at least 90 %.

# References

- R4-2514795, WF on NR_ATG_enh_RRM, CMCC

- R4-2512122, WF on RRM requirements for NR_ATG_enh, ZTE, CMCC

- R4-2508263, WF on RRM requirements for NR_ATG_enh, CMCC