3GPP TSG-RAN WG4 Meeting #117				                                         R4-2522041

Dallas, USA, Nov. 17-21, 2025

Agenda item:	13

Source:	Sony, Thales, Iridium, Nordic Semiconductor, Qualcomm, Aalyria, [Apple], [Vodofone], [Cambridge Consultants LTD]

Title:	Motivation on Introducing the extended guard band for IoT and NR NTN system in Rel-20

Document for:	Discussion

# Background

At the WG RAN4 #116 an LS from ETSI was received [1] asking RAN4 to provide opinion on benefits of ETSI’s Nominated guard band concept. RAN4 replied in [2] the following:

3GPP RAN4 discussed the ETSI SES(25)000071/R4-2509023 task and would like to report the following feedback to TC SES: - The nominated channel bandwidth concept is considered to be useful to minimize NB-IoT/NR NTN UL power backoff (i.e., A-MPR) for meeting ETSI SEM requirements. The nominated channel bandwidth concept can be adopted to the 3GPP specifications with modifications, such as restriction on the possible extended guard-band size(s) and fixed symmetric extended guard-band values. - The utilization of the above concept would be under network’s control to use it depending on the deployment. - 3GPP RAN WG4 plans to work on the design of this concept aiming at a general framework that would be applicable to other satellite bands.

To develop such a general framework of extended guard band, a dedicated WI is preferred in Rel-20. In this contribution, we motivate on the reason why such a work is urgently needed for 3GPP NR and IoT NTN and also clarify the expected workload that would be required to develop such a general framework.

# Issues without extended guard band in NTN system

The main motivation to introduce extended guard bands are the very strict requirements of ETSI in MSS bands. The requirements for spectral emission mask defined in ETSI resulted in very large power backoff (A-MPR) values in 3GPP NTN devices. Taking NB-IoT in b254 as an example, up to 15 dB A-MPR has been specified for Power class 3 (PC3) UEs (see Table 6.2B.3.2-2 in 36.102), It is worth highlight that those high A-MPR values are also confirmed by measurement of commercial devices via multiple sources in RAN4, which further demonstrate the issues in real life.

Similar issues can also observe in NR NTN system, where up to 10 dB A-MPR has also been specified, for example in n254 to meet the ETSI emission mask.

A-MPR value for NB-IoT NTN in b254 to meet the emission mask defined in ETSI in 36.102

A-MPR value for NR NTN in n254 to meet the emission mask defined in ETSI in 38.101-5

For NTN system, even just a few dB power back can lead to a sever link budge issue and with such a high AMPR, it may lead to the 3GPP IoT NTN and NR NTN system to be infeasible to operate in practice. Moreover, the HPUE for NTN system that has been defined in Rel-19 and continuously evolved in Rel-20 likely to even face a even more sever situation in terms of required A-MPR and thus make it even more challenging to be used in the field.

Observation 1: Strict ETSI requirements in MSS bands resulted in very high A-MPRs (>10 dB) and make 3GPP NTN system deployment infeasible in such bands in practice.

# Benefit with extended guard band in NTN system

While strict requirements exist in ETSI, ETSI also allows a UE to declare a nominated bandwidth being as much as 5x larger than channel bandwidth defined in 3GPP, which move the corresponding emission mask defined in ETSI to be further away from the RB/tones that the UE is configured to transmit. The concept allows UEs to operate with minor or even zero A-MPRs while still meet the ETIS emission mask.

Observation 2: The concept of nominated BW defined in ETSI requirements allow UEs to operate with small or even zero A-MPRs while still meet the ETIS emission mask.

Figure. 1 illustration of nominated bandwidth and unwanted emissions mask in ETSI requirements (source: ETSI EN 301 441 V2.1.1)

Based on the discussion in RAN4#116, RAN4 concluded that the nominated channel bandwidth concept is useful to minimize NB-IoT/NR NTN UL power backoff (i.e., A-MPR) for meeting ETSI SEM requirements, and it can be adopted to the 3GPP specifications with modifications, such as restriction on the possible extended guard-band size(s) and fixed symmetric extended guard-band values [2].

During the further discussion, it has also been identified that the concept of extended guard-band can be useful to minimize the AMPR that is required from other regions/regulators that are beyond ETSI requirements (e.g., FCC, ITU-R M.1343, etc.). Therefore, this work targets to define the extended guard-band as a general feature where NTN UEs can use to fulfil certain emission limits with a minimal AMPR value.

Observation 3: RAN4 concluded the nominated channel bandwidth concept is useful to minimize NB-IoT/NR NTN UL power backoff (i.e., A-MPR) for meeting regulatory requirements, and it can be it can be adopted to the 3GPP specifications as extended guard band with modifications.

Although RAN4 plans to work on the design of extended guard band [], but no clear placeholder for such a work in Rel-20. Although initially a TEI agenda was proposed, considering the importance of the work and potential load of it may beyond 1 quarter, it was proposed to create a new RAN4 led WI in Rel-20 to accommodate such a work.

Observation 4: there is no clear placeholder or guidance where the planned work on designing of extended guard band can be carried out in the time frame of Rel-20

Proposal 1: It is proposed to have a new RAN4 led WI in Rel-20 to accommodate the work of designing of extended guard band for NR and IoT NTN to meet regulatory requirements with small or zero AMPR.

# Expected workload on defining extended guard band

RAN4#116 progressed already significantly with the design of extended guard bands, see agreements in [3][4]. In particularly, the general UE behaviour has been agreed, and some high-level design principles below have been agreed, which can be applied to both IoT and NR NTN, for example, as summarized below:

| UE extended guard band capability | Network configuration |  |
| --- | --- | --- |
|  | No extended guard band | Is extended guard band |
| UE capability reported | UE applies defined AMPR if needed | No AMPR is applied |
| No UE capability reported | UE cannot understand SAN guard-band configuration, and a UE applies A-MPR, if needed |  |

- extended guard bands would not be declared by UE, but under control of network.

- extended guard bands would be symmetric.

High level designed principle and UE behaviour agreed in RAN4#116

Observation 5: the general UE behaviour with respect to the network controlled extended guard band has been agreed, and some high-level design principles below have been agreed in RAN4#116, which can be applied to both IoT and NR NTN.

Therefore, the remaining work would be focus on determine the proper value of extended guard band with consideration of different NTN bands and power classes for NR and IoT NTN, respectively. Based on our assessments, two quarter of RAN4 meeting with 0.25 TU would be sufficient to carry out such a work.

Observation 6: Based on the high-level principle that RAN4 already agreed for designing the extended guard-band, the remaining work would be mainly about determine the proper value of extended guard band with consideration of NTN bands and power classes for NR and IoT NTN, respectively.

Observation 7: Two quarter of RAN4 meeting with 0.25 TU would be sufficient to carry out such a work.

With having said above, we propose RAN to consider approve a new RAN4-led WI in Rel-20 with the following objectives:

Proposal 2: It is proposed to consider the following objectives in the new RAN4 led Rel-20 WI on Extended guard band for NR and IoT NTN:

- Specify the extended guard-band feature for NR NTN and IoT NTN, as a to assist UE meeting regulatory requirements, with zero or small AMPR values in FR1-NTN bands. [RAN4]

  - Define one or two fixed value (s) of extended guard-band for NR NTN and IoT NTN, respectively.

    - The extended guard-band shall be symmetrical to the transmission channel bandwidth.

    - Define the necessary UE capability and network signalling.

    - The extended guard band will be applicable to all existing power classes.

  - Example bands are b254/n254 and b249, [while regulatory requirements of other bands (e.g. b255) can be considered as well in this work item].

Note 0: These objectives are needed to alignment with ongoing development of new harmonized standards for 3GPP-based IoT NTN and NR NTN capable UEs in ETSI TC SES as mentioned in the LSs R4-2509023 and R4-2512668.

Note 1: FFS on whether the extended guard-band will be release independent and from which release should it start being applicable.

Note 2: The extended guard-band is assumed to be configured within the spectrum of the operator.

Note 3: The feature should be designed to be general and can be applicable in other NTN bands than the example bands wherever regulation allows.

Note 4: The framework should allow for the possibility of other extended guard band values being specified for possible new power classes in future WI.

# Conclusions

In this contribution, we discussed introduction of extended guard band concept in 3GPP.  The following is observed References

Observation 1: Strict ETSI requirements in MSS bands resulted in very high A-MPRs (>10 dB) and make 3GPP NTN system deployment infeasible in such bands in practice.

Observation 2: The concept of nominated BW defined in ETSI requirements allow UEs to operate with small or even zero A-MPRs while still meet the ETIS emission mask.

Observation 3: RAN4 concluded the nominated channel bandwidth concept is useful to minimize NB-IoT/NR NTN UL power backoff (i.e., A-MPR) for meeting regulatory requirements, and it can be it can be adopted to the 3GPP specifications as extended guard band with modifications.

Observation 4: there is no clear placeholder or guidance where the planned work on designing of extended guard band can be carried out in the time frame of Rel-20

Observation 5: the general UE behaviour with respect to the network controlled extended guard band has been agreed, and some high-level design principles below have been agreed in RAN4#116, which can be applied to both IoT and NR NTN.

Observation 6: Based on the high-level principle that RAN4 already agreed for designing the extended guard-band, the remaining work would be mainly about determine the proper value of extended guard band with consideration of NTN bands and power classes for NR and IoT NTN, respectively.

Observation 7: Two quarter of RAN4 meeting with 0.25 TU would be sufficient to carry out such a work.

Proposal 1: It is proposed to have a new RAN4 led WI in Rel-20 to accommodate the work of designing of extended guard band for NR and IoT NTN to meet regulatory requirements with small or zero AMPR.

Proposal 2: It is proposed to consider the following objectives in the new RAN4 led Rel-20 WI on Extended guard band for NR and IoT NTN:

- Specify the extended guard-band feature for NR NTN and IoT NTN, as a to assist UE meeting regulatory requirements, with zero or small AMPR values in FR1-NTN bands. [RAN4]

  - Define one or two fixed value (s) of extended guard-band for NR NTN and IoT NTN, respectively.

    - The extended guard-band shall be symmetrical to the transmission channel bandwidth.

    - Define the necessary UE capability and network signalling.

    - The extended guard band will be applicable to all existing power classes.

  - Example bands are b254/n254 and b249, [while regulatory requirements of other bands (e.g. b255) can be considered as well in this work item].

Note 0: These objectives are needed to alignment with ongoing development of new harmonized standards for 3GPP-based IoT NTN and NR NTN capable UEs in ETSI TC SES as mentioned in the LSs R4-2509023 and R4-2512668.

Note 1: FFS on whether the extended guard-band will be release independent and from which release should it start being applicable.

Note 2: The extended guard-band is assumed to be configured within the spectrum of the operator.

Note 3: The feature should be designed to be general and can be applicable in other NTN bands than the example bands wherever regulation allows.

Note 4: The framework should allow for the possibility of other extended guard band values being specified for possible new power classes in future WI.

# Reference

- R4-2509023, LS on Harmonised Standard for NTN capable UE, ETSI

- R4-2512539, LS reply to ETSI TC SES, RAN4

- R4-2512688, Way Forward for [116][315] IoT_NTN_Ph3, Moderator (MediaTek)

- R4-2512666, Way Forward for [116][316] IoT_NTN_TDD_UE_SAN_RF, Moderator (Thales)