3GPP TSG-RAN WG4 Meeting #117 	R4-2522063

Dallas, US, Nov 17th – 21st, 2025

Agenda Item: 4.8.2

Source: MediaTek inc.

Title: Discussion on low-band CA switching

Document for: Discussion

# Discussion

The main objectives of the work item to introduce the physical procedures and requirements to enable low band CA via switching, detailed below:

### Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4]

This contribution provides discussion on remaining issues for the low-band CA with the new carrier switching feature, which are captured in WF [2].

Issue 1-1-4a: Application time of disabling switching pattern in SCell deactivation Agreement: - The switching pattern is NOT applied after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation - TLBCA =1 ms for MAC-CE based - Further discuss for timer triggered SCell deactivation Issue 1-1-4b: switching pattern based carrier switching in SCell activation Agreement: - If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation. - Following RAN1 logic, the carrier switching, if applicable, shall be placed at the end of a slot. - FFS RAN4 to capture this sub-bullet in the CR. - Further discuss for timer triggered SCell deactivation

RAN4 discussed on whether the switching pattern is not applied after how many slots from the triggered SCell deactivation, however, there has been no conclusion on this issue. To our understanding, the switching pattern is not applied after n+ (3ms/slot-length).

- RAN4 shall define requirements for timer triggered SCell deactivation with switching pattern is not applied after n+ (3ms/slot-length).

# Summary

In summary:

Proposal 1: RAN4 shall define requirements for timer triggered SCell deactivation with switching pattern is not applied after n+ (3ms/slot-length).

# References

- RP-243317, “New WID on low band carrier aggregation via switching”, Madrid, Spain, Dec 2024.

- R4-2514870, “WF for [116bis][202] NR_LBCA_Sw_RRM”, Apple, Prague, CZ, Oct 2025.