3GPP TSG-RAN4 Meeting #117	R4-2522845

# Dallas, USA, 17 – 21 Novermber, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big draftCR for SBFD RRM performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-11-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This big draftCR captures the following draftCRs endorsed in RAN4#117. Tdoc; Title; Source company. R4-2522802; Draft CR on test cases of L1-CLI-RSSI measurement in FR1/FR2; Samsung. R4-2522806; Draft CR for CSI-RS based BFR and LR Test for FR1 and FR2 in SBFD; LG Electronics Inc.. R4-2522805; DraftCR Test case for L1-SRS-RSRP measurement accuracy in FR1 and FR2 in SBFD operation; CATT. R4-2522807; draftCR on L1 CLI performance requirements; Nokia, Nokia Shanghai Bell. R4-2522804; DraftCR on TC for collision between CSI-RS based RLM and UL (TC11-12); CMCC. R4-2522812; Draft CR on test configuration for SBFD; ZTE Corporation, Sanechips. R4-2522810; draft CR on SBFD test case for FR2 CSI-RS based L1-RSRP measurement; Ericsson. R4-2522811; draft CR on SBFD test case for FR1 CSI-RS based L1-RSRP measurement; Ericsson. R4-2522803; Draft CR for TC17-18 L1-RSRP accuracy for SBFD; vivo. R4-2522809; draftCR on TC7-8 (L1-CLI-RSSI accuracy) for SBFD; Huawei, HiSilicon. |
| Summary of change: |  | Introduce RRM performance requirements for SBFD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM performance requirements for SBFD are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 10.1.x L1 CLI measurement accuracy requirements

#### 10.1.x.1	L1-SRS-RSRP

##### 10.1.x.1.1	L1-SRS-RSRP Accuracy

The L1-SRS-RSRP measurement reported by the UE shall fulfil the accuracy requirements defined in table 10.1.x.1.1-1 for FR1 and table 10.1.x.1.1-2 for FR2-1, provided that the following conditions are met. The accuracy requirements in this clause are derived based on AWGN radio propagation conditions.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SRS-RSRP measurements are fulfilled according to annex B.2.7 for a corresponding Band for each relevant SRS resource configured for measurement.

-	The time difference between UE’s DL reference timing in the serving cell and SRS arrival time is no larger than Terror_SRS_RSRP, where

-	Terror_SRS_RSRP = TC × NTA_offset + 4.67µs for 15 kHz SCS and 30 kHz SCS for FR1

-	Terror_SRS_RSRP = TC × NTA_offset + 1.67 µs for 60 kHz SCS for FR1.

-	Terror_SRS_RSRP = TC × NTA_offset + 0.67µs for FR2

-	NTA_offset is defined in table 7.1.2-2

-	TC is 0.509 ns

-	The number of SRS ports in the SRS resource configured for measurement is 1,

-	The number of symbols in the SRS resource configured for measurement is 1,

-	The number of repetitions in the SRS resource configured for measurement is 1,

-	Frequency hopping, sequence group hopping or sequence hopping is disabled in the SRS resource configured for measurement,

-	The bandwidth of the SRS resource is 24 PRBs.

-	One of the following conditions is met

-	There is no other SRS resource with the same root sequence and on the same symbol and with same comb as the relevant SRS resource.

-	If multiple SRS resources are on the same symbol and with same comb, the distance between cyclic shifts of any two resources is no less than 6 if transmissionComb = n4, and no less than 4 if transmissionComb = n2.

Table 10.1.x.1.1-1:  L1-SRS-RSRP absolute accuracy in FR1

| Accuracy |  |  |  |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  | Extreme condition |  |  | SRS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB |  |  |  |  |  | dB |  | dBm / SCSSRS |  |  | dBm/BW Channel | dBm/BW Channel |
| SCSSRS (kHz) |  |  | SCSSRS (kHz) |  |  |  |  |  |  |  |  |  |
| 15 | 30 | 60 | 15 | 30 | 60 |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| TBD | TBD | TBD | TBD | TBD | TBD | 1 | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| TBD | TBD | TBD | TBD | TBD | TBD | 1 | NR_TDD_FR1_A, NR_TDD_FR1_C, NR_TDD_FR1_D, NR_TDD_FR1_E | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 10.1.x.1.1-2: L1-SRS-RSRP absolute accuracy in FR2

| Accuracy |  |  |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  | Extreme condition |  | SRS Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  |  |  | Minimum Io |  |  | Maximum Io |
| dB |  |  |  | dB | dBm / SCSSRS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
| SCSSRS (kHz) |  | SCSSRS (kHz) |  |  | SCSSRS = | SCSSRS = |  |  |
| 60 | 120 | 60 | 120 |  | 60 kHz | 120 kHz |  |  |
| TBD | TBD | TBD | TBD | ≥1 | Same value as SRS_RP in Table B.2.7-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| TBD | TBD | TBD | TBD | ≥1 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on REFSENS and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |  |

##### 10.1.x.1.2	L1-SRS-RSRP report mapping

The reporting range of L1-SRS-RSRP is defined from -140 dBm to -44 dBm with 1 dB resolution. The mapping of measured quantity is defined in table 10.1.x.1.2-1. The range in the signalling may be larger than the guaranteed accuracy range.

The reporting range of differential SRS-RSRP for L1 reporting is defined from 0 dB to -30 dB with 2 dB resolution. The mapping of measured quantity is defined in table 10.1.x.1.2-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.x.1.2-1: L1-SRS-RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| L1-SRS-RSRP_0 | L1-SRS-RSRP<-140 | dBm |
| L1-SRS-RSRP_1 | -140≤ L1-SRS-RSRP<-139 | dBm |
| L1-SRS-RSRP_2 | -139≤ L1-SRS-RSRP<-138 | dBm |
| L1-SRS-RSRP_3 | -138≤ L1-SRS-RSRP<-137 | dBm |
| L1-SRS-RSRP_4 | -137≤ L1-SRS-RSRP<-136 | dBm |
| .. | .. | … |
| L1-SRS-RSRP_95 | -46≤ L1-SRS-RSRP<-45 | dBm |
| L1-SRS-RSRP_96 | -45≤ L1-SRS-RSRP<-44 | dBm |
| L1-SRS-RSRP_97 | -44≤ L1-SRS-RSRP | dBm |
| L1-SRS-RSRP_98 | Infinity |  |
| NOTE: ‘Infinity’ means that UE cannot detect SRS due to too strong signal to measure. |  |  |  |

Table 10.1.x.1.2-2: Differential SRS-RSRP measurement (for L1 reporting) report mapping

| Reported value | Measured quantity value (difference in measured RSRP from strongest RSRP) | Unit |
| --- | --- | --- |
| DIFF_SRS-RSRP_0 | 0≥ΔRSRP>-2 | dB |
| DIFF_SRS-RSRP_1 | -2≥ΔRSRP>-4 | dB |
| DIFF_SRS-RSRP_2 | -4≥ΔRSRP>-6 | dB |
| DIFF_SRS-RSRP_3 | -6≥ΔRSRP>-8 | dB |
| DIFF_SRS-RSRP_4 | -8≥ΔRSRP>-10 | dB |
| DIFF_SRS-RSRP_5 | -10≥ΔRSRP>-12 | dB |
| DIFF_SRS-RSRP_6 | -12≥ΔRSRP>-14 | dB |
| DIFF_SRS-RSRP_7 | -14≥ΔRSRP>-16 | dB |
| DIFF_SRS-RSRP_8 | -16≥ΔRSRP>-18 | dB |
| DIFF_SRS-RSRP_9 | -18≥ΔRSRP>-20 | dB |
| DIFF_SRS-RSRP_10 | -20≥ΔRSRP>-22 | dB |
| DIFF_SRS-RSRP_11 | -22≥ΔRSRP>-24 | dB |
| DIFF_SRS-RSRP_12 | -24≥ΔRSRP>-26 | dB |
| DIFF_SRS-RSRP_13 | -26≥ΔRSRP>-28 | dB |
| DIFF_SRS-RSRP_14 | -28≥ΔRSRP>-30 | dB |
| DIFF_SRS-RSRP_15 | -30≥ΔRSRP | dB |

#### 10.1.x.2	L1-CLI-RSSI

##### 10.1.x.2.1	L1-CLI-RSSI Accuracy

The L1-CLI-RSSI measurement reported by the UE shall fulfil the accuracy requirements defined in table 10.1.x.2.1-1 for FR1 and table 10.1.x.2.1-2 for FR2, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

Table 10.1.x.2.1-1: L1-CLI-RSSI absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSRS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| 3.5 | 6.5 | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| 5.5 | 8.5 | Note 3 | Note 3 | Note 3 | Note 3 | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |

Table 10.1.x.2.1-2: L1-CLI-RSSI absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |
|  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dBm / SCSSRS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  | SCSSRS = 60 kHz | SCSSRS = 120 kHz |  |  |
| 5 | 8 | Same value as SRS_RP in Table B.2.7-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 7 | 10 | Note 4 |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on REFSENS and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |

##### 10.1.x.2.2	L1-CLI-RSSI report mapping

The reporting range of L1-CLI-RSSI is defined from -100 dBm to -25 dBm with 1 dB resolution. The mapping of measured quantity is defined in table 10.1.x.2.2-1. The range in the signalling may be larger than the guaranteed accuracy range. UE shall scale the measured L1-CLI-RSSI to report a nominal RSSI equivalent to 6RB measurement with 15 kHz SCS.

The reporting range of differential L1-CLI-RSSI reporting is defined from 0 dB to -30 dB with 2 dB resolution. The mapping of measured quantity is defined in table 10.1.x.2.2-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.x.2.2-1: L1-CLI-RSSI measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| L1-CLI-RSSI_00 | L1-CLI-RSSI < -100 | dBm |
| L1-CLI-RSSI_01 | -100  L1-CLI-RSSI < -99 | dBm |
| L1-CLI-RSSI_02 | -99  L1-CLI-RSSI < -98 | dBm |
| … | … | … |
| L1-CLI-RSSI_74 | -27  L1-CLI-RSSI < -26 | dBm |
| L1-CLI-RSSI_75 | -26  L1-CLI-RSSI < -25 | dBm |
| L1-CLI-RSSI_76 | -25  L1-CLI-RSSI | dBm |

Table 10.1.x.2.2-2: Differential CLI-RSSI measurement (for L1 reporting) report mapping

| Reported value | Measured quantity value (difference in measured RSSI from strongest RSSI) | Unit |
| --- | --- | --- |
| DIFF_CLI-RSSI_0 | 0≥ΔCLI-RSSI>-2 | dB |
| DIFF_CLI-RSSI_1 | -2≥ΔCLI-RSSI>-4 | dB |
| DIFF_CLI-RSSI_2 | -4≥ΔCLI-RSSI>-6 | dB |
| DIFF_CLI-RSSI_3 | -6≥ΔCLI-RSSI>-8 | dB |
| DIFF_CLI-RSSI_4 | -8≥ΔCLI-RSSI>-10 | dB |
| DIFF_CLI-RSSI_5 | -10≥ΔCLI-RSSI>-12 | dB |
| DIFF_CLI-RSSI_6 | -12≥ΔCLI-RSSI>-14 | dB |
| DIFF_CLI-RSSI_7 | -14≥ΔCLI-RSSI>-16 | dB |
| DIFF_CLI-RSSI_8 | -16≥ΔCLI-RSSI>-18 | dB |
| DIFF_CLI-RSSI_9 | -18≥ΔCLI-RSSI>-20 | dB |
| DIFF_CLI-RSSI_10 | -20≥ΔCLI-RSSI>-22 | dB |
| DIFF_CLI-RSSI_11 | -22≥ΔCLI-RSSI>-24 | dB |
| DIFF_CLI-RSSI_12 | -24≥ΔCLI-RSSI>-26 | dB |
| DIFF_CLI-RSSI_13 | -26≥ΔCLI-RSSI>-28 | dB |
| DIFF_CLI-RSSI_14 | -28≥ΔCLI-RSSI>-30 | dB |
| DIFF_CLI-RSSI_15 | -30≥ΔCLI-RSSI | dB |

<End of Change 1>

<Start of Change 2>

### A.3.1.4	TDD UL/DL configuration

Table A.3.1.4-1: TDD UL/DL configuration for SCS=15 kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.1.1 | TDDConf.1.2 |  |
| referenceSubcarrierSpacing | kHz | 15 | 15 |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DSUU’ S=’10DL:2GP:2UL’ | ‘DSUU’ S=’ 6DL: 2GP: 6UL’ |  |
| dl-UL-TransmissionPeriodicity | ms | 4 | 4 |  |
| nrofDownlinkSlots |  | 1 | 1 |  |
| nrofDownlinkSymbols |  | 10 | 6 |  |
| nrofUplinkSlot |  | 2 | 2 |  |
| nrofUplinkSymbols |  | 2 | 6 |  |
| TDD UL/DL pattern 2 Note 2 |  | ‘D’ | ‘D’ |  |
| dl-UL-TransmissionPeriodicity | ms | 1 | 1 |  |
| nrofDownlinkSlots |  | 1 | 1 |  |
| nrofDownlinkSymbols |  | 0 | 0 |  |
| nrofUplinkSlot |  | 0 | 0 |  |
| nrofUplinkSymbols |  | 0 | 0 |  |
| NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information. |  |  |  |  |

Table A.3.1.4-2: TDD UL/DL configuration for SCS=30 kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.2.1 | TDDConf.2.2 | TDDConf.2.3 |
| referenceSubcarrierSpacing | kHz | 30 | 30 | 30 |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S4U’ S=’6DL:4GP:4UL’ | ‘1D1S2U’ S=’11DL: 1GP:2UL’ | ‘3D1S4U’ S=’4DL:4GP:6UL’ |
| dl-UL-TransmissionPeriodicity | ms | 4 | 2 | 4 |
| nrofDownlinkSlots |  | 3 | 1 | 3 |
| nrofDownlinkSymbols |  | 6 | 11 | 4 |
| nrofUplinkSlot |  | 4 | 2 | 4 |
| nrofUplinkSymbols |  | 4 | 2 | 6 |
| TDD UL/DL pattern 2 Note 2 |  | ‘DD’ | Not configured | ‘DD’ |
| dl-UL-TransmissionPeriodicity | ms | 1 | Not configured | 1 |
| nrofDownlinkSlots |  | 2 | Not configured | 2 |
| nrofDownlinkSymbols |  | 0 | Not configured | 0 |
| nrofUplinkSlot |  | 0 | Not configured | 0 |
| nrofUplinkSymbols |  | 0 | Not configured | 0 |
| NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information |  |  |  |  |

Table A.3.1.4-3: TDD UL/DL configuration for SCS=120 kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.3.1 | TDDConf.3.2 |  |
| referenceSubcarrierSpacing | kHz | 120 | 120 |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DDDSU’ S=’10DL:2GP:2UL’ | ‘DDDSU’ S=’ 6DL: 2GP: 6UL’ |  |
| dl-UL-TransmissionPeriodicity | ms | 0.625 | 0.625 |  |
| nrofDownlinkSlots |  | 3 | 3 |  |
| nrofDownlinkSymbols |  | 10 | 6 |  |
| nrofUplinkSlot |  | 1 | 1 |  |
| nrofUplinkSymbols |  | 2 | 6 |  |
| TDD UL/DL pattern 2 Note 2 |  | Not configured | Not configured |  |
| dl-UL-TransmissionPeriodicity | ms | Not configured | Not configured |  |
| nrofDownlinkSlots |  | Not configured | Not configured |  |
| nrofDownlinkSymbols |  | Not configured | Not configured |  |
| nrofUplinkSlot |  | Not configured | Not configured |  |
| nrofUplinkSymbols |  | Not configured | Not configured |  |
| NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information |  |  |  |  |

Table A.3.1.4-4: TDD UL/DL configuration of SBFD for SCS=30 kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TDDConf.2.4 |
| referenceSubcarrierSpacing | kHz | 30 |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S1U’ S=’10DL:2GP:2UL’ |
| dl-UL-TransmissionPeriodicity | ms | 2.5 |
| nrofDownlinkSlots |  | 3 |
| nrofDownlinkSymbols |  | 10 |
| nrofUplinkSlot |  | 1 |
| nrofUplinkSymbols |  | 2 |
| TDD UL/DL pattern 2 Note 2 |  | Not configured |
| dl-UL-TransmissionPeriodicity | ms | Not configured |
| NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information |  |  |

<End of Change 2>

<Start of Change 3>

### A.3.10.1	SSB Configurations for FR1

<Texts without change are ommited>

#### A.3.10.1.14	SSB pattern 14 in FR1: SSB allocation for SSB SCS=30 kHz in 100 MHz

Table A.3.10.1.14-1: SSB.14 FR1: SSB Pattern 14 for SSB SCS=30 kHz in 100 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10 ms)/10 ms) = 0 |
| PRB numbers containing SSB within channel BW | (RBJ, PRBJ+1,.…, PRBJ+19)Note 1 |
| NOTE 1: PRBs containing SSB can be configured in any frequency location within a single DL subband bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. NOTE 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

<End of Change 3>

<Start of Change 4>

### A.3.10.2	SSB Configurations for FR2

<Texts without change are ommited>

#### A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz

Table A.3.10.2.3-1: SSB.3 FR2: SSB Pattern 3 for SSB SCS = 120 kHz in 100 MHz channel with 1 SSB per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 120 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSBs Note 2 | 4-7 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10 ms)/10 ms) = 0 |
| PRB numbers containing SSBs within channel BW | (RBJ, PRBJ+1,.…, PRBJ+19)Note 1 |
| NOTE 1: PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. NOTE 3: When this SSB pattern is applied in SBFD carrier, PRBs containing SSB can be configured in any frequency location within a single DL subband bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. |  |

<Texts without change are ommited>

#### A.3.10.2.28	SSB pattern 28 in FR2: SSB allocation for SSB SCS=120 kHz in 200 MHz

Table A.3.10.2.28-1: SSB.28 FR2: SSB Pattern 28 for SSB SCS = 120 kHz in 200 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 200 MHz |
| SSB SCS | 120 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSBs Note 2 | 4-7 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10 ms)/10 ms) = 0 |
| PRB numbers containing SSBs within channel BW | (RBJ, PRBJ+1,.…, PRBJ+19)Note 1 |
| NOTE 1: PRBs containing SSB can be configured in any frequency location within a single DL subband bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

<End of Change 4>

<Start of Change 5>

## A.3.X	SBFD configurations

### A.3.X.1	SBFD configurations for FR1

This clause provides typical SBFD configurations used for SBFD RRM test cases defined in annex A. Both the time domain and frequency domain configurations for SBFD are provided for FR1.

The applicable TDD DL/UL configurations for SBFD RRM test cases are defined in Clause A.3.1.4. Based on the applicable TDD DL/UL configurations for SBFD, the time domain configuration of SBFD slots/symbols is set as ‘DXXXU’, where ‘X’ denotes the SBFD slot with all ‘D’ and ‘S’ symbols in the slots configured as SBFD symbol.

For the frequency domain configuration of SBFD subbands, DU and DUD cases are allowed. For the case of DU, 80MHz is in D subband, and 20MHz is in U subband. For the case of DUD, 40MHz is in the 1st D subband, 20MHz is in U subband, and 40MHz is in the 2nd D subband.

#### A.3.X.1.1	SBFD.1 FR1

Table A.3.X.1.1-1: SBFD.1 FR1

| SBFD Parameters | Values |
| --- | --- |
| SCS | 30 kHz |
| Frequency domain configuration | DU, 80MHz in D subband, 20MHz in U subband |
| SBFD starting slot index | 1 |
| SBFD starting symbol index | 0 |
| SBFD ending slot index | 3 |
| SBFD ending symbol index | 13 |

#### A.3.X.1.2	SBFD.2 FR1

Table A.3.X.1.1-2: SBFD.2 FR1

| SBFD Parameters | Values |
| --- | --- |
| SCS | 30 kHz |
| Frequency domain configuration | DUD, 40MHz in 1st D subband, 20MHz in U subband, 40MHz in 2nd D subband |
| SBFD starting slot index | 1 |
| SBFD starting symbol index | 0 |
| SBFD ending slot index | 3 |
| SBFD ending symbol index | 13 |

### A.3.X.2	SBFD configurations for FR2

This clause provides typical SBFD configurations used for SBFD RRM test cases defined in annex A. Both the time domain and frequency domain configurations for SBFD are provided for FR2.

The applicable TDD DL/UL configurations for SBFD RRM test cases are defined in Clause A.3.1.4. Based on the applicable TDD DL/UL configurations for SBFD, the time domain configuration of SBFD slots/symbols is set as ‘DXXXU’, where ‘X’ denotes the SBFD slot with all ‘D’ and ‘S’ symbols in the slots configured as SBFD symbol.

#### A.3.X.2.1	SBFD.1 FR2

Table A.3.X.2.1-1: SBFD.1 FR2

| SBFD Parameters | Values |
| --- | --- |
| SCS | 120kHz |
| Frequency domain configuration | DU, 50 MHz in D subband, 50 MHz in U subband |
| SBFD starting slot index | 1 |
| SBFD starting symbol index | 0 |
| SBFD ending slot index | 3 |
| SBFD ending symbol index | 13 |

#### A.3.X.2.2	SBFD.2 FR2

Table A.3.X.2.1-2: SBFD.2 FR2

| SBFD Parameters | Values |
| --- | --- |
| SCS | 120kHz |
| Frequency domain configuration | DUD, 75 MHz in 1st D subband, 50 MHz in U subband, 75 MHz in 2nd D subband |
| SBFD starting slot index | 1 |
| SBFD starting symbol index | 0 |
| SBFD ending slot index | 3 |
| SBFD ending symbol index | 13 |

<End of Change 5>

<Start of Change 6>

## A.3.Y	Collision configurations between CSI-RS and UL scheduling for SBFD

It is possible that the UL scheduling collides with the CSI-RS. In the test a single collision between UL scheduling and CSI-RS happens and the collision is randomly determined by the TE.

<End of Change 6>

<Start of Change 7>

#### A.6.5.1.y	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for a UE operating with SBFD

##### A.6.5.1.y.1	Test Purpose and Environment

The purpose of this test is to verify that when the UE supports supportSBFD and SBFD is configured by the network, the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when there are overlapping between occasions of the CSI-RS resource for RLM and dynamic UL transmission on SBFD symbols. This test will partly verify the FR1 PCell CSI-RS In-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in tables A.6.5.1.y.1-1, A.6.5.1.y.1-2, A.6.5.1.y.1-3 and A.6.5.1.y.1-4 below. There is one cells, Cell 1which is the PCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.1.x.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE is configured to perform inter-frequency measurements using GP ID #0 (40 ms) in test. In the test, SSB0 is configured as the BFD-RS.

CSI-RS resource for RLM are on SBFD symbols. During T5, there is overlapping between occasions of the CSI-RS resource for RLM and dynamic UL transmission on SBFD symbols, as specified in A.3.

Table A.6.5.1.y.1-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 30kHz SSB SCS,100 MHz bandwidth |

Table A.6.5.1.y.1-2: General test parameters for FR1 PCell for CSI-RS in-sync testing in DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | TDD |
| TDD Configuration | Config 1 |  | TDDConf.2.4 |
| SBFD configuration | Config 1 |  | SBFD.1 FR1 |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.2.1 TDD |
| SSB Configuration | Config 1 |  | SSB.14 FR1 |
| SMTC Configuration | Config 1 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 30 kHz |
| TRS configuration | Config 1 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State. 2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 4000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 1.24 |
| T4 |  | s | 0.2 |
| T5 |  | s | TBD |
| T6 |  | s | 3.88 |
| NOTE 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.6.5.1.y.1-3: Cell specific test parameters for FR1 for CSI-RS in-sync radio link monitoring in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSSPDCCH_beta |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRSPDCCH_DMRS_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSSPBCH_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRSPSS_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSSSSS_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS PDSCH_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 1 | dBm/15kHz | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300 ns 100 Hz |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. NOTE 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. NOTE 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 7: SNR levels correspond to the signal to noise ratio over the SSS REs. NOTE 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.6.5.1.8.1-1. NOTE 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is specified in clause A.3.6.1.1. |  |  |  |  |  |  |  |

Table A.6.5.1.y.1-4: Measurement gap configuration for FR1 CSI-RS in-sync radio link monitoring in DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| NOTE 1: Void |  |

Figure A.6.5.1.x.1-1: SNR variation for CSI-RS in-sync testing

##### A.6.5.1.y.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting on the PCell.

The rate of correct events observed during repeated tests shall be at least 90 %.

<End of Change 7>

<Start of Change 8>

A.6.5.5.x	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for a UE operating with SBFD

A.6.5.5.x.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting supportSBFD properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell operating on SBFD, and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the DL subband of UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

Supported test configurations are specified in Table A.6.5.5.x.1-1. General test parameters as specified in Table A.6.5.5.3.1-2 with config 3 apply except those specified in Table A.6.5.5.x.1-2. Cell specific test parameters as specified in Table A.6.5.5.3.1-3 apply except those specified in Table A.6.5.5.x.1-3.

The test procedure specified in clause A.6.5.5.3.1 applies to this test. In addition, during T3, there is overlapping between occasions of the CSI-RS resource for BFD (q0) and dynamic UL transmission on SBFD symbols, as specified in A.3.

Table A.6.5.5.x.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 30 kHz SSB SCS, 100 MHz bandwidth |

Table A.6.5.5.x.1-2: General test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
|  |  | Test 1 |  |
| TDD Configuration |  | TDDConf.2.4 | A.3.1.4 |
| SBFD configuration |  | SBFD.2 FR1 | A.3.X |
| RMSI CORESET Reference Channel |  | CR.2.1 TDD | A.3.1.2 |
| Dedicated CORESET Reference Channel |  | CCR.2.1 TDD | A.3.1.3 |
| SSB Configuration |  | SSB.14 FR1 | A.3.10.1 |
| CSI-RS configuration for q0 and q1 |  | CSI-RS.2.2 TDD | A.3.14 |
| CSI-RS configuration for CSI reporting |  | CSI-RS.2.1 TDD | A.3.14 |
| TRS configuration |  | TRS.1.2 TDD | A.3.17.1 |
| CSI-RS-Index assigned as RLM RS |  | CSI-RS.2.2 TDD | A.3.14 |
| T1 | s | [0.2] | During this time the the UE shall be fully synchronized to Cell 1 |
| T2 | s | TBD |  |
| T3 | s | TBD |  |
| T4 | s | 0 |  |
| T5 | s | TBD |  |
| D1 | s | TBD |  |
| NOTE 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.6.5.5.x.1-3: Cell specific test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| SNR_CSI-RS of set q0 | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | dB | -10 | -10 | 10 | 10 | 10 |
| CSI-RS_RP of set q1 | dBm/SCS kHz | -105 | -105 | -85 | -85 | -85 |
|  | dBm/15 kHz | -98 |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 4: Void NOTE 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. NOTE 6: The signal contains PDCCH for Ues other than the device under test as part of OCNG. NOTE 7: SNR levels correspond to the signal to noise ratio over the Res carrying CSI-RS. NOTE 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.6.5.5.3.1-1. NOTE 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |

A.6.5.5.x.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 30+20 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90 %.

<End of Change 8>

<Start of Change 9>

A.6.6.4.x	CSI-RS based L1-RSRP measurement when DRX is not used for SBFD aware UE with DU configuration

A.6.6.4. x.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.2, with the testing configurations for NR cells in table A.6.6.4.3.1-1.

Table A.6.6.4.x.1-1: Applicable NR configurations for FR1 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 30 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

A.6.6.4.x.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.6.4.x.2-1 and table A.6.6.4.x.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. UE is also configured to measure L1-RSRP based on SSB. After 80 ms from the beginning of the test, the DCI trigger comes in slot 0 of a frame and UE provides the report back based on the reporting configuration as defined in table A.6.6.4.x.2-1. In the test, UE is configured to report L1-RSRP for SBFD symbols.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.6.6.4.x.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.2.X |
| BWchannel | 1 | MHz | 100: NPRB,c = 273 |
| SBFD configuration | 1 |  | SBFD.1 FR1 |
| PDSCH Reference measurement channel | 1 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 2.1 TDD Note 2 |
| OCNG Patterns | 1 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1 |  | SMTC.1 |
| DRX configuration | 1 |  | Off |
| reportConfigType | 1 |  | aperiodic |
| reportQuantity | 1 |  | cri-RSRP |
| Number of reported RS | 1 |  | 1 |
| qcl-Info | 1 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1 | slots | 8 |
| T1 | 1 | s | 5 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: In the test the Period of 16 slots overrides the Period of 10 slots in CSI-RS 2.1 TDD. |  |  |  |

Table A.6.6.4.x.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | DL slots | SBFD slots |
| --- | --- | --- | --- | --- |
| Note1 | 1 | dBm/15 kHz | -94.65 |  |
| Note1 | 1 | dBm/SSB SCS | -91.65 |  |
|  | 1 | dB | 0 | 3 |
| CSI-RS RSRP Note2 | 1 | dBm/SSB SCS | -91.65 | -88.65 |
| Io Note2 | 1 | dBm/[95.04] MHz | [-63.97] | [-57.47] |
|  | 1 | dB | 0 | 3 |
| NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

A.6.6.4.x.3	Test Requirements

After 80ms from the beginning of the test, the UE shall send L1-RSRP report at the 8th slot from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for CSI-RS resource occasions on SBFD symbols only while meeting the absolute accuracy requirement in clause 10.1.20.1.1 and relative accuracy requirement in clause 10.1.20.1.2.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change 9>

<Start of Change 10>

A.6.6.x	L1 CLI measurements

#### A.6.6.x.2	L1-CLI-RSSI measurement with DRX with SBFD

##### A.6.6.x.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-CLI-RSSI measurement with UE configured with two DL subbands in SBFD DUD operation. The RSSI resource CLI-RSSI-MeasResource is configured across two DL subbands. This test will verify the CLI-RSSI measurement requirements in clause 9.18.3 with the testing configurations for NR cells in table A.6.6.x.2.1-1.

Table A.6.6.x.2.1-1: Applicable NR configurations for FR1 L1-CLI-RSSI test

| Configuration | Description |
| --- | --- |
| 1 | NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex mode |

##### A.6.6.x.2.2	Test Parameters

One cell is deployed in the test, which are FR1 PCell (Cell 1). The test parameters for PCell is given in table A.6.6.x.2.2-1 and A.6.6.x.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-CLI-RSSI measurement on CLI-RSSI-MeasResource and report aperiodically. The test consists of two successive time periods, with time duration of T1 and T2, respectively. The higher layer parameter timeRestrictionForChannelMeasurements is configured to UE in the test. At 640ms after the beginning of time period T2, the DCI triggering L1-CLI-RSSI reporting is sent to UE and UE provides the report back based on the reporting configuration.

There is no measurement gap configured in the test. The L1-CLI-RSSI measurement resource CLI-RSSI-MeasResource configuration is in table A.6.6.x.2.2-3.

Table A.6.6.x.2.2-1: General test parameters for L1-CLI-RSSI reporting for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | NR Cell 1 |  |
| RF Channel Number |  | 1 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.14 FR1 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| CLI-RSSI-MeasResource |  | 1 | A.6.6.x.2.2-3 |  |
| CP length |  | 1 | Normal |  |
| DRX |  | 1 | DRX.7 |  |
| Time offset between DL from serving cell and OCNG from test system | s | 1 | 17.67 |  |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 2 |  |

Table A.6.6.x.2.2-2: NR Cell specific test parameters for L1-CLI-RSSI reporting for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.2.4 |  |
| SBFD configuration |  | 1 | SBFD.2 FR1 |  |
| PDSCH RMC configuration |  | 1 | SR.2.1 TDD |  |
| PUSCH parameters |  | 1 | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.2.1 TDD |  |
| OCNG Patterns Note 3 |  | 1 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |
| on L1-CLI-RSSI measurement resource Note 2 | dBm/30 kHz | 1 | -113 | -105 |
| on L1-CLI-RSSI measurement resource Note 2 | dBm/SCS | 1 | -113 | -105 |
| Io on L1-CLI-RSSI measurement resource | dBm/[98.28] MHz | 1 | [-77.84] | [-69.84] |
| Io on L1-CLI-RSSI measurement resource | dBm/1.08 MHz | 1 | -97.44 | -89.44 |
| Propagation Condition |  | 1 | AWGN |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: OCNG is not transmitted in the CLI-RSSI measurement resources. |  |  |  |  |

Table A.6.6.x.2.2-3: CLI-RSSI-MeasResource measurement resource configuration for measurement reporting

| RSSI-Resource | cli-RSSI-MeasResourceId-r19 | 0 |
| --- | --- | --- |
|  | startPRB-r19 | 0 |
|  | nrofPRBs-r19 | 273 |
|  | startSymbol-r19 | 3 |
|  | nrofSymbols-r19 | 11 |
|  | cli-RSSI-PeriodicityAndOffset-r19 | sl640, 1 |

##### A.6.6.x.2.3	Test Requirements

Within 650 ms from the beginning of time period T2, the UE shall send L1-CLI-RSSI report. The nominal RSSI used to evaluate the requirement shall be based on Io. The UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change 10>

<Start of Change 11>

### A.6.7.X	L1 CLI measurements

#### A.6.7.X.1	SA L1-SRS-RSRP measurement accuracy with FR1 serving cell with SBFD

##### A.6.7.X.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SRS-RSRP measurement accuracy in SBFD operation is within the specified limits. This test will verify the requirements in clauses 10.1.X.1.1 with the testing configurations for NR cell in table A.6.7.X.1.1-1.

Table A.6.7.X.1.1-1: Applicable NR configurations for FR1 L1-SRS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | 30 kHz SRS SCS, 100 MHz bandwidth, TDD duplex mode |

##### A.6.7.X.1.2	Test Parameters

In this set of test cases there is one cell in the test, FR1 PCell (Cell 1) with SBFD operation. The test parameters for the Cell 1 are given in table A.6.7.X.1.2-1 below. The test parameters for the (virtual) aggressor UE transmitting SRS are given in table A.6.7.X.1.2-2.

Before the test UE is configured to perform L1-SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in UL subband in SBFD slots according to the SRS configuration in table A.6.7.X.1.2-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

The test consists of two successive time periods, with time duration of T1 and T2, respectively. At the beginning of T2, the test equipment sends an DCI to trigger UE to report L1-SRS-RSRP.

Table A.6.7.X.1.2-1: FR1 test parameters for L1-SRS-RSRP accuracy for PCell

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | TDD | TDD |
| TDD configuration |  | 1 |  | TDDConf.2.4 | TDDConf.2.4 |
| SBFD configuration |  | 1 |  | SBFD.1 FR1 | SBFD.1 FR1 |
| BWchannel_total |  | 1 | MHz | 100: NPRB,c = 273 | 100: NPRB,c = 273 |
| PDSCH Reference measurement channel |  | 1 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.14 FR1 | SSB.14 FR1 |
| OCNG Patterns |  | 1 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1 |  | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system |  | 1 | s | 17.67 | 17.67 |
| EPRE ratio of PSS to SSS |  | 1 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/15 kHz | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/SRS SCS | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  | -109 |
| NOTE 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

Table A.6.7.X.1.2-2: FR1 test parameters for L1-SRS-RSRP accuracy for aggressor UE

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/15 kHz | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  | -109 |
| on SRS |  | 1 | dB | 1 | 1 |
| SRS RSRP Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -87 | -110 |
|  | NR_TDD_FR1_C |  |  |  | -109 |
|  | NR_TDD_FR1_D |  |  |  | -108.5 |
|  | NR_TDD_FR1_E |  |  |  | -108 |
| Io Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/47.88 MHz | -52.43 | -75.43 |
|  | NR_TDD_FR1_C |  |  |  | -74.43 |
|  | NR_TDD_FR1_D |  |  |  | -73.93 |
|  | NR_TDD_FR1_E |  |  |  | -73.43 |
| on SRS |  | 1 | dB | 1 | 1 |
| Propagation condition |  | 1 |  | AWGN | AWGN |
| Antenna configuration |  | 1 |  | 1x2 | 1x2 |
| SRS configuration |  | 1 |  | SRSConf.1 | SRSConf.1 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of the test. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

Table A.6.7.X.1.2-3: SRS configuration parameters for FR1 L1-SRS-RSRP accuracy

|  | Field | SRSConf.1 |
| --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 |
|  | srs-ResourceIdList | 0 |
|  | resourceType | Periodic |
|  | Usage | Codebook |
| SRS-Resource | SRS-ResourceId | 0 |
|  | nrofSRS-Ports | Port1 |
|  | transmissionComb | n2 |
|  | combOffset-n2 | 0 |
|  | cyclicShift-n2 | 0 |
|  | resourceMapping startPosition | 0 |
|  | resourceMapping nrofSymbols | n1 |
|  | resourceMapping repetitionFactor | n1 |
|  | freqDomainPosition | 0 |
|  | freqDomainShift | 0 |
|  | freqHopping c-SRS | 12 |
|  | freqHopping b-SRS | 0 |
|  | freqHopping b-hop | 0 |
|  | groupOrSequenceHopping | Neither |
|  | resourceType | Periodic |
|  | periodicityAndOffset-p | sl40, 16 |
|  | sequenceId | 0 |

##### A.6.7.X.1.3	Test Requirements

The L1-SRS-RSRP measurement accuracy shall fulfil the requirements in clauses 10.1.X.1.1. The rate of correct L1-SRS-RSRP measurement accuracy observed during repeated tests shall be at least 90 %.

#### A.6.7.x.2	L1-CLI-RSSI measurement accuracy in FR1 with SBFD

##### A.6.7.X.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-CLI-RSSI measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.X.2.1 with the testing configurations for NR cells in table A.6.7.X.2.1-1.

Table A.6.7.X.2.1-1: Applicable NR configurations for FR1 L1-CLI-RSSI accuracy test

| Config | Description |
| --- | --- |
| 1 | NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex mode |

##### A.6.7.X.2.2	Test parameters

In this set of test cases there is one cell in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.7.X.2.2-1 below.

Before the test UE is configured to perform L1-CLI-RSSI measurement. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on symbols for L1-CLI-RSSI resource. The L1-CLI-RSSI measurement resource configuration is in table A.6.7.X.2.2-2.

The test consists of two successive time periods, with time duration of T1 and T2, respectively. At the beginning of T2, the test equipment sends an DCI to trigger UE to report L1-CLI-RSSI.

Table A.6.7.X.2.2-1: FR1 test parameters for L1-CLI-RSSI accuracy

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD configuration | 1 |  | TDDConf.2.4 |
| BWchannel | 1 | MHz | 100: NPRB,c = 273 |
| SBFD configuration | 1 |  | SBFD.2 FR1 |
| PDSCH Reference measurement channel | 1 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.14 FR1 |
| OCNG Patterns Note6 | 1 |  | OP.1 |
| TRS configuration | 1 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1 |  | SMTC.1 |
| Time offset between DL from serving cell and OCNG from test system | 1 | s | 0 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| on CLI-RSSI measurement resource Note2 | 1 | dBm/30 kHz | -103 |
| on CLI-RSSI measurement resource | 1 | dB | -Infinity |
| RSRP on CLI-RSSI measurement resource Note3 | 1 | dBm/30 kHz | -Infinity |
| Io on CLI-RSSI measurement resource Note3 | 1 | dBm/17.28 MHz | -75.40 |
| Io on CLI-RSSI measurement resource Note3 | 1 | dBm/1.08 MHz | -87.44 |
| on CLI-RSSI measurement resource | 1 | dB | -Infinity |
| Propagation condition | 1 |  | AWGN |
| Antenna configuration | 1 |  | 1x2 |
| T1 | 1 | ms | 200 |
| T2 | 1 | ms | 100 |
| NOTE 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification NOTE 6: OCNG is not transmitted in the CLI-RSSI measurement resources. |  |  |  |

Table A.6.7.X.2.2-2: L1-CLI-RSSI measurement resource configuration for FR1 L1-CLI-RSSI accuracy

|  | Field | Config | SRSConf.1 |
| --- | --- | --- | --- |
| CLI-RSSI measurement resource | rssi-ResourceId | 1 | 0 |
|  | startPRB | 1 | 110 |
|  | nrofPRBs | 1 | 48 |
|  | startPosition | 1 | 3 |
|  | nrofSymbols | 1 | 11 |
|  | rssi-PeriodicityAndOffset | 1 | sl40, 9 |
| NOTE 1: startPRB is set such that CLI-RSSI measurement resource is fully contained within UL subband. |  |  |  |

##### A.6.7.X.2.3	Test Requirements

The CLI-RSSI measurement accuracy shall fulfil the requirements in clauses 10.1.X.2.1.

<End of Change 11>

<Start of Change 12>

#### A.7.5.1.y	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode

##### A.7.5.1.y.1	Test Purpose and Environment

The purpose of this test is to verify that when the UE supports supportSBFD and SBFD is configured by the network, the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when there are overlapping between occasions of the CSI-RS resource for RLM and dynamic UL transmission on SBFD symbols. This test will partly verify the FR2 PCell CSI-RS In-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.7.5.1.y.1-1, A.7.5.1.y.1-2, A.7.5.1.y.1-3 and A.7.5.1.y.1-4 below. There is one cells, cell 1which is the PCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.1.y.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 10 ms. The UE is configured to perform inter-frequency measurements using GP ID #0 (40 ms) in test. In the test, SSB0 and SSB1 are configured as BFD-RS.

CSI-RS resource for RLM are on SBFD symbols. During T5, there is overlapping between occasions of the CSI-RS resource for RLM and dynamic UL transmission on SBFD symbols, as specified in A.3.

Table A.7.5.1.y.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, [100 MHz] bandwidth |

Table A.7.5.1.y.1-2: General test parameters for FR2 PCell for CSI-RS in-sync testing in DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | TDD |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |
| SBFD configuration | Config 1 |  | SBFD.1 FR2 |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.3.1 TDD |
| SSB Configuration | Config 1 |  | SSB.3 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| TRS configuration |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  | TCI.State.3 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | *gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 2000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |
| reportConfigType |  |  | periodic |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| CSI reporting periodicity |  | slot | 40 |
| CSI reporting offset |  | slot | 4 |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 1.64 |
| T4 |  | s | 0.2 |
| T5 |  | s | TBD |
| D1 |  | s | 1.84 |
| NOTE 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.7.5.1.y.1-3: Cell specific test parameters for FR2 for CSI-RS in-sync radio link monitoring in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  | dB | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS1 | Config 1 | dB | 2Note 11 | -6Note 11 | -15 | -4.5 | 2Note 11 |
| SNR on RLM-RS2 | Config 1 | dB | 2Note 11 | -15 | -15 | -15 | -15 |
|  | Config 1 | dBm/15KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. NOTE 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. NOTE 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 7: SNR levels correspond to the signal to noise ratio over the SSS REs. NOTE 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.5.1.8.1-1. NOTE 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. NOTE 10: Information about types of UE beam is given in clause B.2.1.3 and does not limit UE implementation or test system implementation. NOTE 11: This value allows up to 1 dB degradation from applied SNR to UE baseband. |  |  |  |  |  |  |  |

Table A.7.5.1.y.1-4: Measurement gap configuration for FR2 CSI-RS in-sync radio link monitoring in DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| NOTE 1: RLM RS is partially overlapped with measurement gap |  |

Figure A.7.5.1.y.1-1: SNR variation for CSI-RS in-sync testing

##### A.7.5.1.y.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting on the PCell.

The rate of correct events observed during repeated tests shall be at least 90 %.

<End of Change 12>

<Start of Change 13>

A.7.5.5.x	Beam Failure Detection and Link Recovery Test for FR2 Pcell configured with CSI-RS-based BFD and LR in non-DRX mode for a UE operating with SBFD

A.7.5.5.x.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting supportSBFD properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell operating on SBFD, and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the Ues active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in clause 8.5.

Supported test configurations are specified in Table A.7.5.5.x.1-1. General test parameters as specified in Table A.7.5.5.3.1-2 apply except those specified in Table A.7.5.5.x.1-2. Cell specific test parameters as specified in Table A.7.5.5.3.1-3 apply except those specified in table A.7.5.5.x.1-3.

The test procedure specified in clause A.7.5.5.3.1 applies to this test. In addition, during T3, there is overlapping between occasions of the CSI-RS resource for BFD (q0) and dynamic UL transmission on SBFD symbols, as specified in A.3.

Table A.7.5.5.x.1-1: Supported test configurations for FR2 Pcell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, [100] MHz bandwidth |

Table A.7.5.5.x.1-2: General test parameters for FR2 Pcell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
|  |  | Test 1 |  |
| TDD Configuration |  | TDDConf.3.1 |  |
| SBFD configuration |  | SBFD.2 FR2 | A.3.X |
| BWchannel |  | [100]: NPRB,c = [66] |  |
| Data PRBs allocated |  | [66] |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 |  |
| PDSCH Reference Channel |  | SR.3.2 TDD | A.3.1.1 |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | A.3.1.2 |
| Dedicated CORESET Reference Channel |  | CCR.3.1 TDD | A.3.1.3 |
| SSB Configuration |  | SSB.28 FR2 | A.3.10.2 |
| CSI-RS for tracking |  | TRS.2.1 TDD | A.3.17.2 |
| CSI-RS configuration for BFD/CBD/RLM |  | CSI-RS.3.2 TDD | A.3.14.2 |
| CSI-RS configuration for CSI reporting |  | CSI-RS.3.1 TDD | A.3.14.2 |
| CSI reporting periodicity | slot | 40 |  |
| CSI reporting offset | slot | 4 |  |
| T310 | ms | 1000 |  |
| N310 |  | 2 |  |
| T1 | s | [1] | The UE shall be fully synchronized to cell 1 during T1 |
| T2 | s | TBD |  |
| T3 | s | TBD |  |
| T4 | s | 0 |  |
| T5 | s | TBD |  |
| D1 | s | TBD |  |

Table A.7.5.5.x.1-3: Cell specific test parameters for FR2 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumptpion for UE beams Note 10 |  |  | Rough |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 Note 11 | -3 Note 11 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| CSI-RS_RP of set q1 | Config 1 | dBm/SCS | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
|  | Config 1 | dBm/120 KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30 ns 75 Hz |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 4: Void NOTE 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. NOTE 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. NOTE 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.5.5.3.1-1. NOTE 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. NOTE 10: Information about types of UE beam is given in clause B.2.1.3 and does not limit UE implementation or test system implementation. NOTE 11: This value allows up to 1 dB degradation from applied SNR to UE baseband |  |  |  |  |  |  |  |

A.7.5.5.x.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 260+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90 %.

<End of Change 13>

<Start of Change 14>

A.7.6.3.x	CSI-RS based L1-RSRP measurement when DRX is not used

A.7.6.3.x.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.2, with the testing configurations for NR cells in table A.7.6.3.3.1-1.

Table A.7.6.3.3.1-1: Applicable NR configurations for FR2 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz CSI-RS SCS, 200 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

A.7.6.3.x.2	Test parameters

There is one cells in the test, the FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.7.6.3.X.2-1 and table A.7.6.3.X.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. UE is also configured to measure L1-RSRP based on SSB. After 480 ms from the beginning of the test, the DCI trigger comes in slot 1  of a frame and UE provides the report back based on the reporting configuration as defined in table A.7.6.3.X.2-1. In the test, UE is configured to report for SBFD symbols.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.7.6.3.3.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 200: NPRB,c =132 |
| SBFD configuration | 1 |  | SBFD.1 FR2 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
| CSI-RS configuration | 1 |  | CSI-RS.3.3 TDD |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD |
| CSI-RS | 1 |  | CSI-RS.3.2 TDD Note 3 |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 |
| DRX configuration | 1 |  | Off |
| reportConfigType | 1 |  | aperiodic |
| reportQuantity | 1 |  | cri-RSRP |
| Number of reported RS | 1 |  | 2 |
| qcl-Info | 1 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1 |  | 8 |
| Propagation condition | 1 |  | AWGN |
| T1 | 1 | s | 5 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.6.3.3.2-1: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration | 1 |  | Setup 1 according to A.3.15.1 |  |
| Beam AssumptionNote 4 | 1 |  | Rough | Rough |
| Note1 | 1 | dBm/15 kHz | -105 |  |
| Note1 | 1 | dBm/SSB SCS | -95.97 |  |
|  | 1 | dB | 0 | 9 |
| CSI-RS RSRP Note2 | 1 | dBm/SSB SCS | -95.97 | -86.97 |
| Io Note2 | 1 | dBm/[395.04] MHz | [-63.97] | [-57.47] |
|  | 1 | dB | 0 | 9 |
| NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

A.7.6.3.x.3	Test Requirements

After 480 ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8  from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the accuracy requirements defined in clause 10.1.20.1. The reported L1-RSRP value shall include the Rx antenna gain in the range of [-10 ~ +20] dB.

For absolute accuracy of CSI-RS0 and absolute accuracy of CSI-RS1, the UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in table A.7.6.3.X.3-1.

For relative accuracy of CSI-RS0 compared with CSI-RS1, the UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements in table 10.1.20.2.2-1.

<End of Change 14>

<Start of Change 15>

A.7.6.x	L1 CLI measurements

#### A.7.6.x.2	L1-CLI-RSSI measurement with DRX with SBFD

##### A.7.6.x.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-CLI-RSSI measurement with UE configured with two DL subbands in SBFD DUD operation. The RSSI resource CLI-RSSI-MeasResource is configured across two DL subbands. This test will verify the CLI-RSSI measurement requirements in clause 9.18.3 with the testing configurations for NR cells in table A.7.6.x.2.1-1.

Table A.7.6.x.2.1-1: Applicable NR configurations for FR2 L1-CLI-RSSI test

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SCS, 200 MHz bandwidth, TDD duplex mode |

##### A.7.6.x.2.2	Test Parameters

One cell is deployed in the test, which are FR2 PCell (Cell 1). The test parameters for PCell is given in table A.7.6.x.2.2-1 and A.7.6.x.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-CLI-RSSI measurement on CLI-RSSI-MeasResource and report aperiodically. The test consists of two successive time periods, with time duration of T1 and T2, respectively. The higher layer parameter timeRestrictionForChannelMeasurements is configured to UE in the test. At 5ms after the beginning of time period T2, the DCI triggering L1-CLI-RSSI reporting is sent to UE and UE provides the report back based on the reporting configuration.

There is no measurement gap configured in the test. The L1-CLI-RSSI measurement resource CLI-RSSI-MeasResource configuration is in table A.7.6.x.2.2-3.

Table A.7.6.x.2.2-1: General test parameters for L1-CLI-RSSI reporting for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | NR Cell 1 |  |
| RF Channel Number |  | 1 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.28 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| CLI-RSSI-MeasResource |  | 1 | A.7.6.x.2.2-3 |  |
| CP length |  | 1 | Normal |  |
| DRX |  | 1 | OFF | Non-DRX |
| Time offset between DL from serving cell and OCNG from test system | s | 1 | 17.67 |  |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 1 |  |

Table A.7.6.x.2.2-2: NR Cell specific test parameters for L1-CLI-RSSI reporting for PCell in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| SBFD configuration |  | 1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |
| PUSCH parameters |  | 1 | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |
| OCNG Patterns Note 3 |  | 1 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |
| Propagation Condition |  | 1 | AWGN |  |
| NOTE 1: OCNG is not transmitted in the CLI-RSSI measurement resources. |  |  |  |  |

Table A.7.6.x.2.2-3: NR OTA Cell specific test parameters for CLI-RSSI event triggered reporting for PCell in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| AoA setup |  | 1 | Setup 1 defined in A.3.15.1 |  |
| Beam assumption Note 3 |  | 1 | Fine | Fine |
| on CLI-RSSI measurement resource Note 2 | dBm/15 kHz | 1 | -119 | -108 |
| on CLI-RSSI measurement resource Note 2 | dBm/SCS | 1 | -110 | -99 |
| Io on CLI-RSSI measurement resource | dBm/95.04 MHz | 1 | -81.01 | -70.01 |
| Io on CLI-RSSI measurement resource | dBm/1.08 MHz | 1 | -100.46 | -89.46 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |

Table A.7.6.x.2.2-4: CLI-RSSI-MeasResource measurement resource configuration for measurement reporting

| RSSI-Resource | cli-RSSI-MeasResourceId-r19 | 0 |
| --- | --- | --- |
|  | startPRB-r19 | 0 |
|  | nrofPRBs-r19 | 66 |
|  | startSymbol-r19 | 3 |
|  | nrofSymbols-r19 | 11 |
|  | cli-RSSI-PeriodicityAndOffset-r19 | sl40, 25 |

##### A.7.6.x.2.3	Test Requirements

Within 7ms from the beginning of time period T2, the UE shall send L1-CLI-RSSI report. The nominal RSSI used to evaluate the requirement shall be based on Io. The UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change 15>

<Start of Change 16>

#### A.7.7.4.X	CSI-RS based L1-RSRP measurement with SBFD

##### A.7.7.4.X.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits,  when UE is configured with 2 DL subbands in SBFD and the CSI-RS BW is each DL subband is 24 RB. This test will verify the requirements in clauses 9.5.3 and clause 10.1.20.2 for L1-RSRP measurements based on CSI-RS with the testing configurations for NR cells in table A.7.7.4.X.1-1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15.

Table A.7.7.4.X.1-1: Applicable NR configurations for FR2 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz CSI-RS SCS, 200 MHz bandwidth, TDD duplex mode |

##### A.7.7.4.X.2	Test parameters

In this set of test cases there are one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in table A.7.7.4.X.2-1 and table A.7.7.4.X.2-2 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in table A.7.7.4.X.2-1 and table A.7.7.4.X.2-2.

There is no measurement gap configured in the test. Before the test, UE is configured one CSI-RS resource set with two CSI-RS resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB.

Table A.7.7.4.X.2-1: FR2 CSI-RS based L1-RSRP general test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN | 1 |  | freq1 | freq1 |
| Duplex mode | 1 |  | TDD | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | 1 | MHz | 200: NPRB,c = 132 | 200: NPRB,c = 132 |
| SBFD configuration | 1 |  | SBFD.2 FR2 | SBFD.2 FR2 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.28 FR2 | SSB.28 FR2 |
| OCNG Patterns | 1 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 | TCI.State.2 |
| SMTC configuration | 1 |  | SMTC.1 | SMTC.1 |
| CSI-RS | 1 |  | CSI-RS.3.2 TDD Note 3 | CSI-RS.3.2 TDD Note 3 |
| reportConfigType | 1 |  | periodic | periodic |
| reportQuantity | 1 |  | cri-RSRP | cri-RSRP |
| Number of reported RS | 1 |  | 2 | 2 |
| L1-RSRP reporting period | 1 |  | slot80 | slot80 |
| Propagation condition | 1 |  | AWGN | AWGN |
| Antenna configuration | 1 |  | 1x2 | 1x2 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 6: In this test case, startingRB of CSI-RS configuration is 20, and nrofRBs is 92. |  |  |  |  |

Table A.7.7.4.X.2-2: FR2 CSI-RS based L1-RSRP OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 NOTE 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CSI-RS0 | CSI-RS1 | CSI-RS0 | CSI-RS1 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  |
| Assumption for UE beamsNote 4 |  |  | Rough |  | Rough |  |
|  | 1 | dBm/15 kHz | -103 |  | n.a. |  |
|  | 1 | dBm/SSB SCS | -94 |  | n.a. n.a. |  |
|  | 1 | dB | 10 | -2 | n.a. |  |
| CSI-RS-RSRPNote1 | 1 | dBm/SCS | -84 | -96 | As in table B.2.4-2 |  |
| IoNote1 | 1 | dBm/ [190.08] MHz | [-51.59] | [-59.88] | SS-RSRP+28.98 |  |
|  | 1 | dB | 10 | -2 | n.a. |  |
| NOTE 1: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 2: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 3: No additional noise is added by the test system in Test 2. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

##### A.7.7.4.X.3	Test Requirements

After 640 ms from the beginning of the test, the L1-RSRP measurement accuracy for CSI-RS#0 and CSI-RS#1 of Cell 1 shall fulfil the requirements in clause 10.1.20.2. The following requirements are to be verified:

For Test 1:

Absolute accuracy of CSI-RS0. The UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in table A.7.7.4.X.3-1.

Relative accuracy of CSI-RS0 compared with CSI-RS1. The UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements in table 10.1.20.2.2-1.

For Test 2:

Absolute accuracy of CSI-RS resource reported by UE in L1-RSRP report (CSI-RS0 or CSI-RS1). The UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in table A.7.7.4.X.3-1.

Relative accuracy of CSI-RS0 compared with CSI-RS1. The UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements in table 10.1.20.2.2-1.

Table A.7.7.4.X.3-1: L1-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| CSI-RS0 | CSI-RS _RP0 -δ + Gmin ≤ Reported RSRP(dBm) ≤CSI-RS _RP0 +δ + Gmax |
| CSI-RS1 | CSI-RS _RP1 -δ + Gmin ≤ Reported RSRP(dBm) ≤CSI-RS _RP1 +δ + Gmax |
| NOTE 1: CSI-RS_RPn is the equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone configured in the test for the CSI-RS n under consideration NOTE 2: δ is the RSRP absolute accuracy requirement from Table 10.1.20.2.1-1, selected according to the Io used in the test NOTE 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

<End of Change 16>

<Start of Change 17>

### A.7.7.X	L1 CLI measurements

#### A.7.7.X.1	SA L1-SRS-RSRP measurement accuracy with FR2 serving cell with SBFD

##### A.7.7.X.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SRS-RSRP measurement accuracy in SBFD operation is within the specified limits. This test will verify the requirements in clauses 10.1.X.1.1 with the testing configurations for NR cells in table A.7.7.X.1.1-1.

Table A.7.7.X.1.1-1: Applicable NR configurations for FR2 L1-SRS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | 120 kHz SRS SCS, [100] MHz bandwidth, TDD duplex mode |

##### A.7.7.X.1.2	Test parameters

In this set of test cases there is one cell in the test, FR2 PCell (Cell 1) with SBFD operation. The test parameters for the Cell 1 are given in table A.7.7.X.1.2-1 and A.7.7.X.1.2-2 below. The test parameters for the (virtual) aggressor UE transmitting SRS are given in table A.7.7.X.1.2-2.

Before the test UE is configured to perform L1-SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in the UL subband in SBFD slots according to the SRS configuration in table A.7.7.X.1.2-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 2 data symbols before SRS to be transmitted.

The test consists of two successive time periods, with time duration of T1 and T2, respectively. At the beginning of T2, the test equipment sends an DCI to trigger UE to report L1-SRS-RSRP.

Table A.7.7.X.1.2-1: FR2 test parameters for L1-SRS-RSRP accuracy

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN | 1 |  | freq1 | freq1 |
| Duplex mode | 1 |  | TDD | TDD |
| TDD configuration | 1 |  | TDDConf.3.1 | TDDConf.3.1 |
| SBFD configuration | 1 |  | SBFD.1 FR2 | SBFD.1 FR2 |
| BWchannel_total | 1 | MHz | 100: NPRB,c = 66 | 100: NPRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR2 | SSB.3 FR2 |
| OCNG Patterns | 1 |  | OP.1 | OP.1 |
| TRS configuration | 1 |  | TRS.2.1 TDD | TRS.2.1 TDD |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.3 ULBWP.1.3 | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1 |  | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system | 1 | s | 7.76 | 7.67 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation condition | 1 |  | AWGN | AWGN |
| Antenna configuration | 1 |  | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table A.7.7.X.1.2-2: L1-SRS-RSRP accuracy OTA related test parameters for PCell and aggressor UE in FR2

| Parameter | Unit | T1 | T2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined A.3.15.1 | Setup 1 defined A.3.15.1 |
| Beam assumption Note 5 |  | Fine | Fine |
| Note1 | dBm/15 kHzNote3 | -100 | N/A |
| Note1 | dBm/SCSNote3 | -91 | N/A |
|  | dB | 2 | N/A |
| Es | dBm/SCSNote3 |  | (Table B.2.7-2 Rx Beam Peak) |
| SRS_RPNote2 | dBm/SCS | -89 | (Table B.2.7-2 Rx Beam Peak) |
| BB Note4 | dB | >1 | 1 |
| IoNote2 | dBm/46.08 MHz Note3 | -61.03 | (Table B.2.7-2 Rx Beam Peak +[50.79] dB) |
| NOTE 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SRS_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 2 dB for UE multi-band relaxation factor ∑MBP from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 5: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |

Table A.7.7.X.1.2-3: SRS configuration parameters for FR2 L1-SRS-RSRP accuracy

|  | Field | SRSConf.1 |
| --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 |
|  | srs-ResourceIdList | 0 |
|  | resourceType | Periodic |
|  | Usage | Codebook |
| SRS-Resource | SRS-ResourceId | 0 |
|  | nrofSRS-Ports | Port1 |
|  | transmissionComb | n2 |
|  | combOffset-n2 | 0 |
|  | cyclicShift-n2 | 0 |
|  | resourceMapping startPosition | 0 |
|  | resourceMapping nrofSymbols | n1 |
|  | resourceMapping repetitionFactor | n1 |
|  | freqDomainPosition | 0 |
|  | freqDomainShift | 0 |
|  | freqHopping c-SRS | 12 |
|  | freqHopping b-SRS | 0 |
|  | freqHopping b-hop | 0 |
|  | groupOrSequenceHopping | Neither |
|  | resourceType | Periodic |
|  | periodicityAndOffset-p | sl160, 26 |
|  | sequenceId | 0 |

##### A.7.7.X.1.3	Test Requirements

The L1-SRS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in clauses 10.1.X.1.1 and the rate of correct L1-SRS-RSRP measurement accuracy observed during repeated tests shall be at least 90 %. The following requirements are to be verified:

During T1:

The UE is deemed to meet the requirement if the reported L1-SRS-RSRP is in the range shown in table A.7.7.X.1.3-1.

During T2:

The UE is deemed to meet the requirement if the reported L1-SRS-RSRP is in the range shown in table A.7.7.X.1.3-1.

Table A.7.7.X.1.3-1: L1-SRS-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| SRS | SRS_RP -δ +Gmin ≤ Reported SRS-RSRP(dBm) ≤SRS_RP +δ +Gmax |
| NOTE 1: SRS_RP is the equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone configured in the test NOTE 2: δ is the RSRP absolute accuracy requirement from Table 10.1.22.1.1-2, selected according to the Io used in the test NOTE 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

#### A.7.7.x.2	L1-CLI-RSSI measurement accuracy in FR2 with SBFD

##### A.7.7.X.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-CLI-RSSI measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.X.2.1 with the testing configurations for NR cells in table A.7.7.X.2.1-1.

Table A.7.7.X.2.1-1: Applicable NR configurations for FR2 CLI-RSSI accuracy test

| Config | Description |
| --- | --- |
| 1 | 120 kHz SRS SCS, 200 MHz bandwidth, TDD duplex mode |

##### A.7.7.X.2.2	Test parameters

In this set of test cases there is one cell in the test, FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.7.7.X.2.2-1 and A.7.7.X.2.2-2 below.

Before the test UE is configured to perform L1-CLI-RSSI measurement. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on symbols for L1-CLI-RSSI resource. The L1-CLI-RSSI measurement resource configuration is in table A.7.7.X.2.2-3.

The test consists of two successive time periods, with time duration of T1 and T2, respectively. At the beginning of T2, the test equipment sends an DCI to trigger UE to report L1-CLI-RSSI.

Table A.7.7.X.2.2-1: FR2 test parameters for CLI-RSSI accuracy

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 200: NPRB,c = 132 |
| SBFD configuration | 1 |  | SBFD.2 FR2 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.28 FR2 |
| OCNG Patterns Note2 | 1 |  | OP.1 |
| TRS configuration | 1 |  | TRS.2.1 TDD |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1 |  | SMTC.1 |
| Time offset between DL from serving cell and OCNG from test system | 1 | s | 0 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1 |  | AWGN |
| Antenna configuration | 1 |  | 1x2 |
| T1 | 1 | ms | 200 |
| T2 | 1 | ms | 100 |

Table A.7.7.X.2.2-2: CLI-RSSI accuracy OTA related test parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined A.3.15.1 |
| Beam assumption Note 5 |  | Fine |
| on CLI-RSSI measurement resource Note1 | dBm/15 kHzNote3 | -100 |
| on CLI-RSSI measurement resource Note1 | dBm/SCSNote3 | -91 |
| on CLI-RSSI measurement resource | dB | -Infinity |
| RSRP on CLI-RSSI measurement resource Note2 | dBm/SCS | -Infinity |
| BB on CLI-RSSI measurement resource Note4 | dB | -Infinity |
| Io on CLI-RSSI measurement resource Note2 | dBm/40.32 MHz Note3 | -65.44 |
| Io on CLI-RSSI measurement resource Note2 | dBm/1.08 MHz | -81.46 |
| NOTE 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SRS_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 2 dB for UE multi-band relaxation factor ∑MBP from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 5: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |

Table A.7.7.X.2.2-3: CLI-RSSI measurement resource configuration for FR2 CLI-RSSI accuracy

|  | Field | SRSConf.1 |
| --- | --- | --- |
| CLI-RSSI measurement resource | rssi-ResourceId | 0 |
|  | startPRB | 50 |
|  | nrofPRBs | 30 |
|  | startPosition | 3 |
|  | nrofSymbols | 11 |
|  | rssi-PeriodicityAndOffset | sl160, 9 |
| NOTE 1: startPRB is set such that CLI-RSSI measurement resource is fully contained within UL subband. |  |  |

##### A.7.7.X.2.3	Test Requirements

The CLI-RSSI measurement accuracy shall fulfil the absolute accuracy requirements in clauses 10.1.X.2.1. The UE is deemed to meet the requirement if the reported CLI-RSSI is in the range shown in table A.7.7.X.2.3-1.

Table A.7.7.X.2.3-1: L1-CLI-RSSI absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
|  | Io -δ +Gmin ≤ Reported CLI-RSSI(dBm) ≤Io +δ +Gmax |
| NOTE 1: Io is the equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone configured in the test for 1.08 MHz NOTE 2: δ is the L1-CLI-RSSI absolute accuracy requirement from Table 10.1.X.2.1-2, selected according to the Io used in the test NOTE 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

<End of Change 17>