3GPP TSG-RAN WG4 Meeting #117													R4-2522267

Dallas, Texas, United States, 17th – 21st November, 2025

Agenda item:			8.1

Source:	Feature lead (Qualcomm)

Title:	Topic Summary for [117][102] 6G general RF and UE RF

Document for:	Information

# Introduction

This summary gathers together Tdocs from agenda 8.4 General and UE RF. The chairman provided additional guidance on the scope in the agenda: The scope includes non-spectrum and non-AI UE RF, spectrum aggregation framework, UE RF related coverage and energy efficiency, and other joint UE-BS RF issues.

Overall, there were 21 contributions submitted.

In the SI a TR is designated for RAN4. No input to TR is expected from UE RF agenda in this meeting.

| Internal TR | 38.8XX | Study on 6G Radio RAN4 aspects | TSG#115 | TSG#116 | This TR is led by RAN4 TR editor: Schumacher, Joseph, AT&T, jq304t@att.com |
| --- | --- | --- | --- | --- | --- |

The intent of the study is not to provide all final requirement values for 6GR. In the early phase focus is on analysis on benefits/drawbacks, and then making conclusions based on the analysis on which solutions/approaches should be pursued/not pursued. As such, we are scoping the WI based on analysis and conclusions, while of course some specification values may be concluded during the process.

Target for the 2nd meeting

Main goal for the second meeting is to end up with a clear scope on what topics and what areas within each topic RAN4 should study. Which aspects should be studied, which should not. Therefore, the focus is on gathering ideas together and having guidelines which aspects future contributions should cover. First technical discussions beyond scoping are also expected to take place.

Recommendations from feature lead to all contribution authors:

The 6G UE RF study will be more successful if contributions, including observations and proposals, are made as easy to understand as possible. Typically, observations and proposals get more visibility, and it is beneficial if they are as self-contained as possible. Following recommendations are given

- Avoid proposals written as: “adopt alternative 1”, where definition of alternative 1 is nowhere near your proposal in the Tdoc: your proposal is at higher risk of being misrepresented in the summary if it is not clear and self-contained.

- Aim towards actionable proposals: “Discuss topic X” is an understandable proposal in the first meeting of the study but more clarity is always appreciated: what it is related to topic X you want to discuss? Could it be “discuss if requirement Y is needed for topic X” or “discuss if approach A is better suited than approach B for topic X”.

- Aim to spell out abbreviations at least once: we are working on new technology, and some abbreviations/acronyms may not be familiar to all 3GPP participants.

# Topic #1: Single carrier output power requirements

## Companies’ contributions summary

| T-doc number and Company | Proposals / Observations |
| --- | --- |
| R4-2520093 CATT | Proposal 1: For 410MHz ~ 7125MHz, RAN4 can decide that the output power requirements are specified at the antenna connector(s) of the UE with a single or multiple transmit antenna(s). FFS the following sentence is still valid or not for output power requirements as general UE antenna efficiency is less than 100%. Unless otherwise stated, a reference antenna with a gain of 0 dBi is assumed for UE. Proposal 3: For the study of the default power class, RAN4 should not only consider UE implementation, but also need to study the link budget considering different frequency range, e.g. comparison between UE UL MCL and Maximum Path Loss. Pcmax Proposal 2: For the improvements on configured maximum output power requirements and associated parameters, the following three points are proposed. P2-1: There is no need to consider ∆TC,c based on the state-of-the-art technology. P2-2: For ΔTIB,c, based on the state-of-the-art RFIC technology, there is no need to discuss it per band-combination specific and FFS whether ΔTIB,c still need to be kept or not. P2-3: For ∆TRxSRS, RAN4 need to explore how to reflect it in the RF requirements. Alternatively, RAN4 can consider to specify a separate Pcmax,SRS. |
| Skyworks R4-2520292 R4-2520545 | Proposal on single carrier output power: - Unless otherwise limited by regulation, the default maximum nominal output power is 26dBm - At least for smartphone form factor, smaller form factors may still use 23dBm as the default maximum nominal output power, while CPE/FWA may target higher power. - Maximum nominal output power is defined for a fully allocated DFT-s-OFDM QPSK waveform of: - 20MHz for ≤ ~1GHz UL bands - 50MHz for ≤ ~2GHz UL bands - 100MHz for < ~5GHz band - 200MHz for > ~5GHz and < ~52GHz bands - FFS if frequency thresholds should be defined differently (at least potentially for NTN bands) - The tolerance for all maximum nominal output power <29dBm regardless of #Tx, and frequency range is +2/-2dB. - DeltaT is not specified and considered within the post PA losses while considering multi-band support and triplexing frequency sub-ranges onto one antenna. - For any maximum nominal output power from 23dBm to 29dBm the UE is allowed to boost its output power by up to 3dB unless limited by an explicit maximum output power limit signalled by the network. - Within the maximum nominal output power range from 23dBm to 32dBm: - Up to 4Tx is supported up to 16GHz, FFS for 8Tx for larger form factors - 4x4 UL MIMO should be considered for smartphone form factor. - 2x1 to 8x8 antenna arrays are supported from >10GHz to cover all mobile/transportable UE form factors. FFS for larger arrays. 2x2 UL MIMO is supported. - Support of circular polarisation should be studied for NTN - For bands <10GHz, up to 200MHz UL CBW is considered to allow PA efficiency improvement techniques. - If APT and ET including CFR/pre-distortion aspects must be considered, it is unlikely that a common model and assumptions can be agreed in RAN4. - Also, if these techniques can improve PA linearity in-band and within the ACLR1 region at low backoff levels, it comes at the expense of lower linearity improvement at higher backoff and potentially worse performance at larger frequency offsets: the intrinsic PA linearity is still what counts in the most critical OOB emission cases. |
| KDDI R4-2520305 | Japanese regulation on HPUE operation for FDD bands In Japan, currently HPUE operation for any FDD bands is not allowed, and also this domestic regulatory situation will be same in 2030 era due to co-existence requirements with other domestic services. RAN4 should take into account that there will be countries not being allowed to operate HPUE for any FDD bands. Proposal 1: Take into account that there will be countries not being allowed to operate HPUE for any FDD bands. Proposal 2: Default power class for FDD band is 23dBm. |
| Huawei R4-2520325 | Proposal 1-1: For the case (#1-1) when the UE is configured with single-carrier UL and single-carrier DL on a band, further study how to efficiently support TxD and ULFPTx, including UE RF requirements as well as capability signaling design, by considering but not limited to the underlying PA configuration as well as insertion loss difference between different Tx paths. Proposal 1-2: For the case (#1-2a) when the UE is configured with DL CA but single-carrier UL, the power class per band could be considered if there are no strong concerns from UE vendors. Proposal 1-3: For the case (#1-2b) when a UE is configured with UL CA but with only single-carrier activated, the UE may support the same power class as case #1-2a. However, new signaling design might be needed to account for the activation status as well as its changes. Proposal 1-4: For the case (#1-2c) when a UE is configured with UL CA, both activated but only single-carrier scheduled, the UE may be able to support a higher power class than that for two scheduled carriers, provided that the UE is capable of Tx switching (for intra-band or inter-band). The RF requirements (including power class, Tx switching time, DL interruption, etc.) as well as capability signaling should be considered within the generic Tx switching framework, which is FFS. Proposal 1-5: The default power class for a band may be defined as the power class that a band shall support when the band is first introduced or re-farmed to the 6GR specifications. In this sense, PC3 for FDD bands and PC2 for TDD bands could be considered as the default power class. Other consequences need to be further studied, such as the baseline coverage for network deployment, the baseline REFSENS requirements, etc. Proposal 1-6: For FR2, different power classes for handheld UE to address identified issue(s) together with scenarios such as hot spot in 5G should be considered. Proposal 1-7: Regarding solutions for meeting SAR/MPE requirements, P-MPR could be the baseline. Other solutions can be studied, but they have to be implementable and verifiable, and also show notable advantages compared with P-MPR. |
| Mediatek R4-2520342 | Proposal 1: For eMBB service in 6GR, RAN4 can consider default power class with PC2 (26dBm) for 6G new candidate spectrum at around 7GHz. Observation1: The large MPR under a default power class of PC2: 26dBm and 200MHz channel bandwidth could shrink the uplink coverage compared to 5G NR in 3.5GHz. SAR/MPE Observation 1: The duty-cycle restriction was already considered in NW configuration for high power UE with less than 50% for TDD bands. Observation 2: The existing duty-cycle solution is based on UE capability, leading to scheduling inflexibility that hinders further improvement in UL performance. Proposal 1: RAN4 need to study how to define a flexible/efficient duty-cycle control mechanism for 6GR. Proposal 2: RAN4 can consider both P-MPR and duty-cycle control mechanism to satisfy the SAR/MPE requirements in 6GR. |
| Samsung R4-2520358 | Observation #2.2-1: Pros and cons for Power class concept. - Pros: - Easy for requirements building (i.e., all requirements are built upon PC concept) and conformance test - Cons: - Inefficient PA utilization: Some UE may not fully utilize their PA capacity. Futures like “Power boosting” and “increasing higher power limit”, introduced in 5G for higher power transmission, have led to fragmented UE implementation choices, reducing the likelihood for real implementation Observation #2.2-2: The first thing to address is whether to retain the power class concept, and if retained whether to maintain a 3dB between power classes. Proposal #2.2-1: It is advisable to keep the power class concept with a 3dB step and its reporting. Proposal #2.2-2: Further exploit the full usage of PA capacity in the meanwhile avoiding excessive fragmented implementation choices. Default power class Observation #2.2-3: For about 7GHz, to achieve comparable coverage as C-band with same assumption of inter-site deployment, BS can implement more antenna elements. Proposal #2.2-3: PC3 should still be the default power class. This allows for scalability and performance optimization without affecting low-end UEs. Pcmax Proposal #2.2-4: Remove ΔTIB from CA requirements, while retaining it in the single carrier PCMAX_L equation as it is. Observation #2.2-4: Simplification kind of discussion for ΔTIB should take place in Spectrum thread. Proposal #2.2-5: More justification should be provided for asymmetric TxD (e,g, 26 + 23 dBm) for single carrier, if it is still of interest. Proposal #2.2-6: Do not specify duty cycle solution for SAR issue. |
| CMCC R4-2520432 | Proposal 1: RAN4 is suggested to study the 6G power class framework from following aspects. - RAN4 is suggested to be involved early for RAN2 band combination related capability discussion and avoid the band combination fallback related issues - Further study the possibility to allow much dynamic power class capability report mechanism - Allow highpowerlimit feature from Day 1 Proposal 2: the default power class for higher frequency band, e.g. >6GHz should be PC1.5 |
| Xiaomi R4-2520512 | Observation 1: It’s necessary to use a more flexible MOP level grading methodology to fully use the PA capability. Proposal 1: 1 dB step size is used for the MOP level grading methodology. Proposal 2: Option 1 is kept as a candidate solution to solve the MOP level grading issue. Option 1: Remove the power class concept but define the nominal MOP level with 1 dB step size. Tolerance is still +2/-2(-3) dB. How to define MOP for different features ¦(TxD, MIMO…) Proposal 3: The two following options are kept to further discuss how to define MOP for different features. Option 1: MOP is defined according to the features/working modes. Option 2: MOP is defined according to power capability of RF chain. Pcmax Proposal 4: Remove ∆TRxSRS from the Pcmax equation. Default power class Proposal 5: Revisit the name of “default power class” after MOP methodology discussion is clear. Proposal 6: For FR1, the lower limit of MOP is 23 dBm for FDD band, 26 dBm for TDD band. Proposal 7: Remove duty cycle solution in Pcmax clause. SAR/MPE solution is left to implementation. |
| Nokia R4-2520554 | Proposal 1: For 6GR bands the default power class is PC2 (i.e. 26 dBm) Proposal 2: For 6GR bands PC2 is the first power class to be defined. Proposal 3: In 6G RAN4 do not define interband UL CA power classes. UL CA output power capabilities are determined by constituent single bands power classes. Approach is similar as NR higherPowerLimit feature. Pcmax Proposal 7: Investigate the simplification of PCMAX,f,c equations and the optimization of associated parameters |
| Apple R4-2520601 | Goals: • Uplink coverage improvement over 5G • Maximize UE’s hardware capability. • Minimize UE capability signaling such as transparent power boosting. • Merge ΔTIB into the UE maximum output power capability to reduce specifications complexity. • Address UL MIMO or Tx diversity versus 1Tx maximum output power capability. Proposal 2: RAN4 to consider the following approach for 6G UE maximum output power framework: • For single carrier - UE maximum output power capability is specified at the 0dB MPR waveform which can be the 6G MPR calibration waveform. - Define default maximum output power requirements for handheld UE under 1Tx. • [23 – 25] dBm for FDD bands • [24 – 26] dBm for TDD bands - Allow UE to indicate its maximum output power capability higher than the default value with 1dB increment up to [29] dBm. - +2/-2 dB tolerance - Define only one set of MPR requirements which applies for all UE indicated maximum output power capability. - ACLR = 31dB from default maximum output power up to 29dBm, FFS on ACLR requirement for output power higher than 29dBm. - For UL allocations and waveforms other than the 0dB MPR waveform, allow [3]dB more output power to accommodate power boosting. - For UL band supporting more than 1Tx (such as nTx), the total maximum output power from the sum of all antenna connectors can be up to 10log(n)dB higher than 1Tx maximum output power. • For DL CA with single carrier UL - The maximum output power capability is the same as single carrier. • For UL CA - For intra-band contiguous UL CA with 1Tx, the maximum output power capability is the same as single carrier. - For inter-band UL CA, the maximum output power capability is per-band based. There is no need to specify the total maximum output power capability from all constituent bands. The per-band capability under inter-band UL CA configuration can be different from single carrier capability for the same band. - For intra-band non-contiguous UL CA, only single switched UL is supported if specified. The per-carrier maximum output power capability is the same as single carrier. |
| vivo R4-2520724 | Observation 1: The LTE/NR power class framework is originally designed for 1Tx, NW cannot totally understand the power ability of each RF chain when UE equip with multiple RF chain. Observation 2: Power loss exist in NR due to fixed power class framework. Observation 3: Power boosting in NR is already used to utilize implementation margin. Proposal 1: RAN4 to study the power class framework with the clarification that the power class is the maximum power via 1Tx, FFS how to reflect the power ability due to UE equip with multiple Tx chain and implementation margin, e.g., using power boosting. Pcmax Observation 4: ΔPPowerClass in NR is too complicated which require a lot clarification in the spec especially for the requirement applicability when the power class is changed. Observation 5: ∆TRxSRS only apply to SRS transmission occasion but captured in common Pcmax formula, which from RAN5 perspective couldn’t be identified as dedicated RF requirement for verification. Proposal 2: For Pcmax simplification, the following two points can be further studied - Remove ΔPPowerClass, and FFS how to utilize duty-cycle in 6G - Remove ∆TRxSRS, and consider treat it as a dedicated RF requirement - Retain the TIB and RIB, and FFS the simplification method under the 6G spectrum SI. DL CA Observation 6: For intra-band CA, there is gap of MPR between single CC and CA, which may require adjustment of RF component, e.g., PA, which make it would be hard to perform fast change and the characteristic could be different for each UE. Proposal 3: The following table can be considered as starting point for discussion to clarify the UE power ability under different conditions, further study how to accommodate different UE implementations. CA with Scell is deactivated.; CA without scheduling on Scell. Inter-band CA; Follow single carrier power class; Follow single carrier power class. Intra-band CCA; Follow single carrier power class when TxD is not indicated; Follow CA power class. Intra-band NCCA; Follow single carrier power class when TxD is not indicated; Follow CA power class. Default power class Observation 7: The chip size and power consumption to support HPUE in low band (<1GHz) will increase and the system performance gain could be diluted due to the increased intra-system interference and duty-cycle constrain. Observation 8: To support 400MHz UL CBW at 7GHz, it could be challenge for UE design to transmit with 26 dBm due to the severer memory effect, higher post-PA loss, worse out of band emission. Proposal 4: For the default power class in 6G, it is suggested to further divide FR1 into 3 sub range for further discussion: - Range 1(f<1GHz)：Study whether the system performance gain is still valid, considering high RU, 50% duty cycle constrain and intra-system interference - Range 2(1GHz<f<6GHz): PC2 default power class as starting point. - Range 3(>6GHz): Study the characteristic and feasibility of PA to support PC2 as default power class, especially for supporting large UL CBW, e.g., 400MHz SAR/MPR Observation 9: The time averaged SAR is already universally used in SAR regulation. Proposal 5: RAN4 further study how to better utilize duty cycle concept to mitigate the SAR related issue in HPUE. |
| Spreadtrum, Unisoc R4-2520766 | Proposal 1: Tx diversity, full power mode and UL MIMO in 5G can be reused in 6GR. UE report power class of every Tx chain to NW can be considered. Proposal 2: The parameters in configured maximum output power including ΔPPowerClass, ΔTIB,c and ∆TRxSRS can be considered to be optimized/removed/simplified in priority. Proposal 3: For FDD bands, PC3 can still be the default power class. For around 7GHz, PC2 can be the default power class. For TDD bands in FR1, the default class can be further discussed. Proposal 4: It is not necessary to consider duty-cycle for SAR solution in 6GR. We can keep P-MPR as a SAR solution in 6GR. Proposal 5: SAR can be applied in below 6GHz, MPE (PD) can be applied in above 6GHz in 6GR. Proposal 6: Whether to keep 3dB step for power class or alter other values for higher power class can be further studied in 6GR. |
| Qualcomm R4-2520825 | Observation 2.1.1-1: Current specifications force UE to limit its power potential without known reason Proposal 2.1.1-1: Relax upper limit of the Pcmax H-L window to allow UE leverage lower IL of the front end Observation 2.1.1-2: Output power accuracy and predictability does not need to conflict with enabling UE to maximise it output power potential SAR & Default power class Observation 2.1.2-1: Benefit of using of default power class as it is in 5G-NR with SAR method is not clear Proposal 2.1.2-1: For 6G, there is no need to define a default power class. A self-consistent approach would be for all UEs to indicate power class. Proposal 2.1.2-2: RAN4 does not define a duty cycle limitation in 3GPP based on SAR or MPE compliance Proposal 2.1.2-3: RAN4 to study use cases and deployment scenarios under 6G SI to concluded what is feasible UL duty cycle requirement from network needs point of view to ensure optimal component cost and size. Proposal 2.1.3-1: For 6G, the default understanding is that the UE may use multiple antenna ports to transmit, and no explicit ‘TxD’ indication is required from the UE. |
| Ericsson R4-2521135 | TX power accuracy Proposal 3.1: RAN4 collaborates with RAN1 from the start of the 6G study on the design of any reference symbols to be used for reciprocity-based DL-MIMO, if specified, and for coherent UL-MIMO with due account of expected UE transmit performance. UE power class structure and the default power class Proposal 3.2: Reuse PC3 (23 dBm) as the default power class for all operating bands in 6GR. Proposal 3.3: Study how to design the UE capability framework on the supported UE power class for single carrier/CA, such that it is unambiguous for the NW and robust, so that it works for either single or multiple configured UL carriers. Pcmax Proposal 3.4: modification or removal of current maximum output power limits could be studied, but only if accompanied by the appropriate UE reporting. |
| ZTE R4-2521281 | Proposal 2: To allow UE to deliver more output power, enhancing PA capability in each Tx chain and/or adopting large array UL MIMO could be considered. Observation 1: For single band, the current power class definition in 5G may constrain the UE maximum output power, especially for UL MIMO. Observation 2: Introducing too many power classes would cause power class fragment and complexity for the requirements definition. Proposal 3: For 6GR, it is proposed to consider the real transmission output power per antenna connector or per Tx chain/PA for the single band power class framework. Pcmax Observation 10: In 5G NR, ∆TRxSRS requirements were not verified in reality, and the SRS insertion loss imbalance issues was discussed buts not solved. Observation 11: Duty cycle SAR solution was not implemented in 5G NR according to the previous discussion. Observation 12: Delta Tib/Rib requirements are time consuming work to be validated. Removing the delta Tib value can enable a bit higher configured transmitted power, and removing the delta Rib value can enable a bit lower receive sensitivity requirements. Proposal 13: To remove duty cycle SAR solution in 6GR. Proposal 14: To solve ∆TRxSRS issues in the configured transmitted power requirement. |
| Oppo R4-2521450 | Proposal 2: Power class definition should make sure UE can fully utilize the PA power capabilities while distinguish different levels of UEs in terms of Tx power. Observation 2: Less than 3dB power step makes the power classes are not well distinguished and has much overlapping power. Proposal 3: Keep 3dB power step among power classes. Observation 3: For UE with multiple Tx PAs, the gap between max UE power capability and power class can reach 1.8dB. Observation 4: The +/-2dB power tolerance was introduced to accommodate massive production variations. Proposal 4: Study how to handle the issue of power gap b/w UE max Tx power capability and power class reporting in the beginning of 6G Proposal 5: Consider reuse the power boosting scheme that allow UE to report the boosted power comparing to the reported power class. Observation 15: Pcmax,h has become the gating factor for UE to transmit higher power especially when UE has multi-Tx PAs. Observation 16: Power boosting or remove the Pcmax,h are two potential approaches to allow UE transmit higher power whenever possible. Proposal 16: Study how to enable UE to fully utilize its PA capabilities. And power boosting approach is preferred comparing to remove the Pcmax,h limits. Power class for different MIMO layers Observation 5: For UEs not supporting TxD, the power capability could be different for different MIMO layer configurations, like PC2 in single port, but PC1.5 for UL MIMO. This power difference cannot be differentiated with 5G NR per band power class reporting. Proposal 7: For single band power class reporting, allow UE to report different power class for different MIMO Layers. Default power class Observation 6: For TDD bands, PC2 has been widely supported on the market. Proposal 8: For TDD bands, PC2 is defined as baseline. FFS whether PC2 or PC3 is baseline for FDD bands. SAR Observation 17: Duty cycle-based SAR solutions in RAN4 spec is very complex, but it hasn’t been implemented in UE. Only PMPR is applied. Proposal 17: In 6G, only rely on PMPR to solve SAR issue and no other SAR solution is to be defined. |
| LG Electronixs R4-2521515 | Pcmax Proposal 1: A review of the 5G configured Tx power equation parameters is necessary for 6G ∆TC,c: Needs re-examination considering the 6G RF characteristics and the new Channel Bandwidth. ΔPPowerClass: The necessity of the parameter itself should be fundamentally re-evaluated for exclusion from the 6G standard or replacement with a clearer alternative. ΔPPowerBoost: Investigate through 6G studies whether there are more efficient and clear methods to support UL high power. ΔTIB,c: Discussions should be held to find and implement ways to either remove or simplify this parameter. SAR Observation 1. For frequencies below 10 GHz, exposure assessment is primarily based on absorbed power metrics such as SAR or APD, whereas for frequencies above 10 GHz, the assessment is primarily based on incident power metrics such as IPD, reflecting the dominant interaction mechanism at higher frequencies. Observation 2. At higher frequencies, uplink performance suffers due to greater propagation loss, prompting consideration of High Power UE configurations. However, higher transmit power increases also SAR and APD. This is attributed to the frequency-dependent increase in human tissue conductivity — 0.97 S/m @ 900 MHz, 1.5 S/m @ 2100 MHz, 2.91 S/m @ 3500 MHz, 6.07 S/m @ 6500 MHz, 8.46 S/m @ 8500 MHz, and 10.4 S/m @ 10000 MHz — which leads to greater RF energy absorption. Since conductivity increases by nearly a factor of 10 across this frequency range, SAR/APD risk can also increase by approximately 10× under the same electric field (E) conditions. Consequently, while higher power operation can improve communication performance, it also introduces additional challenges in meeting human RF exposure compliance requirements. Observation 3: Measurements of commercial devices show also a negative trend in average power with increasing frequency. Table 1- Average Power Settings for 10 recent 5G phones from two different companies n66 1710 - 1780; B25/n25 1850 - 1915; n7 or n41 2500-2570; n77 3300 - 4200. Avg Power Set of A company; 20.3 dBm; 20.4 dBm; 19.3 dBm; 18.4 dBm. Avg Power Set of B company; 19.5 dBm; 19.3 dBm; 18.3 dBm; 17.4 dBm. Proposal 2: Take observations 1, 2 and 3 into account when discussing the UE transmission and output power related aspects during the study item. |
| Sony R4-2522049 | Proposal 5: RAN4 may need to collaborate with RAN1 in 6GR to develop an improved power control framework, with clearer UE behavior defined in terms of actual output power. Proposal 7: RAN4 shall study whether it is feasible to define a unified requirement for each power class that is agnostic to the UE Tx implementation, thereby simplifying the HPUE requirement framework. Observation 7: Very limited output power level is observed around 7GHz and 15 GHz with 100% duty cycle due to the MPE (APD) limitation in ICNIRP. Proposal 6: RAN4 shall investigate the SAR/MPE solution around 7GHz and 15 GHz with consideration both power backoff and duty cycle based on approach, with target also to achieve a more clear UE behavior in terms of output power than 5G NR. |

## Open issues summary

### Sub-topic 1-1 Single carrier requirements

- Agreements

  - Study 6G output power requirements to enable more flexibility to different UE implementations to reflect the RF front-end capability for single band

    - Encourage inputs on how to allow UE to deliver more output power and how power class framework includes e.g. Tx diversity and UL-MIMO

  - Study improvements on configured maximum output power requirements and associated parameters

  - Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements

  - Study the default power class for FDD and TDD bands for 6G

  - Study how to handle SAR/MPE for 6GR

Agreement from previous meeting:

Issue 1-1-1: Power class framework

Issues 1-1-1 and 1-1-2 and 1-1-3 are related. It is beneficial to discuss them all, but it is likely that the resulting way forward needs to consider all three issues together.

Desired improvements for issues 1-1-1 to 1-1-3

- Make sure UE can fully utilize the PA power capabilities

- Avoid fragmentation and ambiguity

Key observations

- Smaller step between power classes could ease fully using PA capabilities

- Smaller step between power classes could also result in more fragmentation and there will be a lot of overlap between power classes

- 3 dB step size can avoid fragmentation and reduce requirement complexity

- Proposals

  - Proposal 1: Retain the current power class framework with 3 dB step size

    - FFS on more accurate reporting of real output power capability

  - Proposal 2: Define nominal power with more granularity, e.g. 1 dB

    - FFS on how many sets of e.g. MPR and A-MPR requirements to be defined

- Recommended WF

  - TBA

Issue 1-1-2: Power class for multi-Tx features (e.g. TxD, UL-MIMO)

Key observations

- Proposals

  - Proposal 1: Consider transmission power per antenna connector or per chain/PA

    - This includes asymmetric power capability per antenna connector or per chain/PA

  - Proposal 2: Use same framework independent of number of chains/Pas

    - UE may use multiple antenna ports to transmit, and no explicit ‘TxD’ indication is required from the UE.

    - FFS how to ensure this enables fully utilizing PA capabilities

- Recommended WF

  - TBA

Issue 1-1-3: Enabling fully utilizing PA capabilities

Key observations

- Output power accuracy and predictability does not need to conflict with enabling UE to maximise it output power potential

- Network has control over absolute maximum output power via Pmax

- Proposals

  - Proposal 1: Power boosting associated with signaling

    - Proposal 1a: Report nominal output power capability with more granularity

    - Proposal 1b: Explicit signaling for power boosting

  - Proposal 2: More flexible power class definition

    - Proposal 2a: Allow more flexibility to Pcmax_H while maintaining power accuracy

- Recommended WF

  - TBA

Issue 1-1-4: Default power class

Key observations

- In some countries regulation does not allow HPUE, e.g. in Japan FDD bands are only allowed to operating in PC3.

- Proposals

  - Proposal 1: Do not define default power class. UE always indicates its power class.

  - Proposal 2: 23 dBm for all operating bands

  - Proposal 3: [23..26] dBm for FDD bands

  - Proposal 4: [26..29 dBm] for TDD bands

When discussing proposals, aspects to consider include e.g. network UL coverage / UL link budget at different frequency sub-ranges, impact to refsens

- Recommended WF

  - TBA

Issue 1-1-5: SAR/MPE solution

Almost unanimously companies agree that duty cycle should not be used for SAR/MPE and P-MPR should be the baseline solution. Meanwhile, it has been observed by some companies that limited output power with increased frequencies, e.g., 7GHz and 15GHz, due to SAR/MPE limits. Further discussion is needed on separation of SAR and MPE and whether anything else than P-MPR is studies for SAR.

- Proposals

  - Proposal 1: Use P-MPR as baseline for SAR

  - Proposal 2: No duty cycle solution for SAR or MPE purpose

  - Proposal 3: SAR can be applied in below 6GHz, MPE (PD) can be applied in above 6GHz in 6GR.

  - Proposal 4: SAR/MPE solution with consideration of both power backoff and duty cycle based on approach, with target also to achieve a more clear UE behavior in terms of output power than 5G NR.

- Recommended WF

  - TBA

Issue 1-1-6: Duty cycle

- Proposals

  - Proposal 1: Study use cases and deployment scenarios under 6G SI to concluded what is feasible UL duty cycle requirement from network needs point of view to ensure optimal component cost and size.

  - Proposal 2: RAN4 need to study how to define a flexible/efficient duty-cycle control mechanism for 6GR.

- Recommended WF

  - TBA

Issue 1-1-7: Pcmax

- Proposals

  - Proposal 1: Review the 5G configured Tx power equation parameters considering at least the following

    - ∆TC,c : can it be removed, considering also new wider CBW

    - ∆TRxSRS: should it be separated from general Pcmax equating as it only addresses SRS transmission

    - ∆Tib: can the requirement be removed and accommodated to per band power capability

    - ΔPPowerClass: Is it still needed in some cases even if not associated with SAR

- Recommended WF

  - TBA

Issue 1-1-8: Output power accuracy

- Proposals

  - Proposal 1: RAN4 collaborates with RAN1 from the start of the 6G study on the design of any reference symbols to be used for reciprocity-based DL-MIMO, if specified, and for coherent UL-MIMO with due account of expected UE transmit performance

- Recommended WF

  - TBA

Issue 1-1-9: Antenna aspects

Key observations

- In FR1 conducted TS 38.101-1, “for UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s)”. It is worth discussing if the conducted requirement could be related to the OTA requirement by at least assuming a reasonable antenna efficiency, e.g. a typical FR1 antenna efficiency is -5.5 dB.

- Feature lead: Antenna efficiency assumption would impact also converting regulatory EIRP/TRP limits into conducted requirements

- Proposals

  - Proposal 1: Discuss a more realistic assumption on antenna efficiency for FR1 band tests, e.g. -5.5 dB

- Recommended WF

  - TBA

# Topic #2: Power class framework for CA

## Companies’ contributions summary

| T-doc number Company | Proposals / Observations |
| --- | --- |
| Skyworks R4-2520292 R4-2520545 | Proposal for UL CA output power and Spectrum aggregation: - Contiguous Intra-band ULCA power class is the same than the band power class - Non-contiguous allocation is not supported or specified - intra-band CA should not be specified for a band if its spectrum can be addressed by a single CC - it should support the same number of Tx chain than in single CC case. - Non-contiguous Intra-band ULCA is not supported - If supported, it should be based on UL switching - If RAN4 still studies a simultaneous 2CC NC ULCA, only 2LO(2PA) case is supported - In all cases the ULCA power class is the same than the band power class, but it can only be reached for balanced TxBW between the two CCs. - No power class is defined for inter-band ULCA - The power capability in inter-band ULCA is the power sum of the power capability in each band signalled by the UE and no inter-band power class is defined/signalled - The limitation of only up to two UL clusters is maintained but 2CC in each band can be studied (4CC total). However, it should not be a priority since wider CBW than in 5G should be supported in 6G thus the need for 2UL CC in each of the two bands must first be justified. - There should be no limitation on the #Tx per band but it may need a per band per BC signalling when PAs are shared. - DeltaT is not specified and considered within the post PA losses considering multi-band support and triplexing on one antenna. |
| Huawei R4-2520325 | Proposal 2-1: For inter-band/intra-band UL CA, the power class per band per BC and/or power class per CC should be considered. For Tx switching, the applicable power class for a band or a BC should be further studied. |
| Qualcomm R4-2520825 | Proposal 2.2-1: RAN4 should strive to define requirements for CA in such manner that the UE potential is fully utilised. Proposal 2.2-2: RAN4 to identify clear reasons for limiting power on a band based on conditions on the other band in a inter-band CA configuration If such reasons re not identified during the study, RAN4 should not define CA power class in WI phase. |
| Ericsson R4-2521135 | Observation 3.1: specification of a power class for CA depends on whether RAN1 specifies a total power limit at which the UE prioritizes UL transmissions. Proposal 6.1: for spectrum aggregation, study the feasibility of maintaining the ‘single carrier’ UE output power capability of a ‘primary’ or ‘anchor’ carrier at configuration/activation of the spectrum aggregation scheme (cf. increase of the allowed MPR for the Pcell at NR CA configuration). Proposal 6.2: for spectrum aggregation, the total configured power PCMAX, or equivalent power, should be related to the behaviour of the UE for any multi-carrier scheme specified. Changes of the total output power capability could be reported by (a)periodic dedicated signalling (cf. P-MPR reporting for NR). Proposal 6.3: transmission on a single active/configured UL carrier within a DL multi-carrier bandwidth should not increase allowed UE power reduction (cf. no power degradation for NR DL-only CA). |
| ZTE R4-2521281 | Observation 3. In NR, the IE higherPowerLimit-r17 can enable higher maximum output power for inter-band combination, but it is not applied to pure intra-band contiguous/non-contiguous CA. Observation 4: For UL CA configuration in NR, the transmission power is restricted by the band combination power class. Proposal 4: For CA power class framework in 6GR, the restriction of the band combination power class should be released in the power class framework discussion. Proposal 5: To remove the band combination power class, instead of using sum of power from each band to define the maximum output power. |
| Oppo R4-2521450 | Proposal 6: For band combination, not define the total power limitation. Rely on the power capability of each band in the BC (ue-PowerClassPerBandPerBC) to determine the max Tx power. |
| Apple R4-2520601 | - For DL CA with single carrier UL - The maximum output power capability is the same as single carrier. - For UL CA - For intra-band contiguous UL CA with 1Tx, the maximum output power capability is the same as single carrier. - For inter-band UL CA, the maximum output power capability is per-band based. There is no need to specify the total maximum output power capability from all constituent bands. The per-band capability under inter-band UL CA configuration can be different from single carrier capability for the same band. - For intra-band non-contiguous UL CA, only single switched UL is supported if specified. The per-carrier maximum output power capability is the same as single carrier. |

## Open issues summary

### Sub-topic 2-1 Power class framework for CA

- Agreements

  - Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements

  - Study how to maximize the UL transmission power capability for CA

  - Prioritize discussion for single carrier in RAN4#117 and RAN4#118

Agreement from previous meeting:

Issue 2-1-1: inter-band CA power class framework

Key observations:

- Specification of a power class for CA depends on whether RAN1 specifies a total power limit at which the UE prioritizes UL transmissions.

- Proposals for inter-band CA power class

  - Option 1: Maximum output power capability is set by per band capability, no power class is defined to limit the total power, i.e. similar to always using higherPowerLimit-r17

    - Power class reporting could be per band per BC

  - Option 2: CA power class it still needed pending on RAN1 applying a total power limit.

    - RAN4 to identify clear reasons for limiting power on a band based on conditions on the other band in a inter-band CA configuration

- Additional proposals for CA output power

  - Proposal 1: Study the feasibility of maintaining the ‘single carrier’ UE output power capability of a ‘primary’ or ‘anchor’ carrier at configuration/activation of the spectrum aggregation scheme

  - Proposal 2: Transmission on a single active/configured UL carrier within a DL multi-carrier bandwidth should not increase allowed UE power reduction (cf. no power degradation for NR DL-only CA).

- Recommended WF

  - Deprioritize discussion in this meeting

  - For next meeting, identify if any there is any motivation from RAN4 perspective to limit total output power.

    - Consider liaising with RAN1 to report RAN4 conclusions

  - Study the feasibility to maintain single carrier output power with no MPR increase at Pcell/anchor carrier

    - When only single UL carrier is configured/activated/scheduled

    - When additional carriers are added

  - Study power class for Tx switching: is it needed, how it should be defined

Issue 2-1-2: Intra-band CA power class framework

- Proposals for intra-band CA

  - Option 1: Intra-band UL CA power class is the same as single band/carrier power class

- Recommended WF

  - Deprioritize discussion in this meeting

  - Study the feasibility to maintain single carrier output power with no MPR increase at Pcell/anchor carrier

    - When only single UL carrier is configured/activated/scheduled

    - When additional carriers are added

# Topic #3: Tx requirements

## Companies’ contributions summary

| T-doc number Company | Proposals / Observations |
| --- | --- |
| R4-2520093 CATT | Proposal 4: For UE Tx impairments, the following parts are proposed. P4-1: In the 6GR phase, if it’s agreed to mandatorily support 256QAM, the carrier leakage and image leakage requirements above 10dBm output power should be adjusted to meet total 3.5% EVM budget of Tx chain. P4-2: RAN4 need to discuss whether 60dB CIM3 and 70dB CIM5 for MPR simulation are still valid or necessary for MPR simulation in around 7GHz and 15GHz. |
| Skyworks R4-2520292 R4-2520545 | Proposal on Tx Requirements: - Default supported UL QAM: - UL 256QAM for band ≤5GHz - UL 64QAM for band >5GHz and <16GHz - FFS for >16GHz - For NTN bands it may not be needed to specify A-MPR for the highest QAM - UL 1024QAM is considered optionally - The Tx impairments should be further improved accordingly, and Tx EVM should be based on assuming better Rx EVM on the BS side. - ACLR for up to 29dBm nominal output power: - 31dB for band ≤5GHz - 27dB for band >5GHz and <16GHz - FFS for >16GHz - FFS if frequency thresholds should be chosen differently - Tx impairments up to 29dBm nominal output power: - 34dB image and carrier for band ≤5GHz - 31dB image and carrier for band >5GHz and <16GHz - FFS for >16GHz - FFS if frequency thresholds should be chosen differently - Measurement BW for the first SEM segment (first OOB MHz): - 1%CBW ≤50MHz CBW - 500kHz for >50MHz CBW - FFS if frequency threshold should be chosen differently (within 40MHz to 50MHz) - Spurious emissions should be based on the worst case worldwide regulation per relevant frequency ranges - If a local regulation is significantly more restrictive/permissive, it may be useful to consider this as a band specific requirement under NS. - This may result into different MPR/A-MPR in different frequency ranges. In any case, the number of MPR/A-MPR requirements should be reduced by covering single and multiple Tx by the same requirements. - To simplify MPR specification across #Tx, antenna coupling assumptions could be varied across frequency ranges and #Tx to have a single requirement at least up to 4Tx. - To simplify A-MPR, it could be defined only for the CBW that are relevant to the deployment in the region where the additional emissions apply and only tested for those. With wider channel bandwidths, the -50dBm/MHz band protection may become more difficult and be subject to A-MPR. - Since 6G is striving to offer flexibility in UL/DL BW support, the Tx requirements should target flexible BW and verification done only for a limited set of CBW. For A-MPR it may still be necessary to test for specific CBW. |
| Huawei R4-2520325 | Proposal 3-1: Consider different the ACLR requirements for sub-6GHz and above 6 GHz spectrum sub-ranges. The exact value of the ACLR requires further studies. Proposal 3-2: Consider reevaluation of SEM requirements, especially for the 1st MHz of ΔfOOB, if possible. Moreover consider different SEM requirements for sub-6 and above 6GHz (other ranges could be included). Proposal 3-3: As the 256QAM is being discussed to become mandatory in UL, adopt image imbalance and carrier feedthrough of 35dB. |
| Mediatek R4-2520342 | MPR/A-MPR Proposal 1: Consider whether it would be possible to study the feasibility and benefits of lo retuning to improve A-MPR, and potential MPR. Observation 2: Rel-19 MPR improvement feature can enable to de-restrict maximum output power by up to 1dB by converting “outer RB allocations” to “inner RB allocations”. Proposal 2: Consider re-application of the Rel-19 single carrier MPR improvement concept for 6GR. Observation 3: Extending the concept of relaxed OOB emissions to de-restrict Tx power can be extended to broader scenarios such as adjacent MNO site sharing within an area. Proposal 3: Consider extension of the Rel-19 single carrier MPR improvement general concept to other scenarios including adjacent channel MNO site sharing for 6GR. Proposal 4: To simplify MPR specification, RAN4 can consider whether single or scalable requirements to consider the multiple Tx antenna configuration, simple antenna isolation level. Proposal 5: To simplify MPR specification, RAN4 can consider whether scalable requirements to consider the multiple power classes. ACLR Observation 1: In BS RF session, RAN4 already agreed to evaluation the coexistence study to consider the different points compared to 5G coexistence parameters. The expected different points are supporting a wider CBW, enhanced gNB antenna elements and defaults power class as PC2 in around 7GHz. Proposal 1: Based on the coexistence evaluation results, RAN4 can update ACLR and ACS requirements according to frequency ranges (e.g, less 1GHz frequency, 2GHz, 3.5GHz, 4.5GHz and around 7GHz). It means different ACLR requirements can be considered and supported in 6G. Proposal 2: Consider specifying an operator-controlled relaxation in UE ACLR/SEM for 6GR from first Release, and further study the extended benefits of this for multi-operator site sharing scenarios. OOB boundary Proposal 1: RAN4 can consider the SM.1539-2 and SM.1541 recommendation to determine the boundary between the out of band emission and spurious domains for 6G study. |
| Samsung R4-2520358 | Proposal #2.4-1: For frequencies up to around 7GHz, adopt -35 dB for carrier leakage and -35 dB for image rejection. Proposal #2.4-2: For MPR evaluation, reuse the 5G assumption, i.e., - The LO is placed in the middle of the CBW - UE DPD is not considered in the transmission modelling Proposal #2.4-3: Strive to streamline MPR requirements when the performance differences are not significant across various RF architectures, Tx chain counts, and other relevant factors. - For instance, for one device type, with same power class and mode (i.e., single carrier, or C/NC CA), strive to specify only one set of MPR requirements. This doesnot preclude other architecture implementations (i.e., those not selected as reference architecture for deriving requirements), provided the specified requirements are met. |
| CMCC R4-2520432 | Proposal 3: further check whether the IQ suppression and carrier leakage can be enhanced from 28dBc to 31dBc. |
| Xiaomi R4-2520512 | Proposal 8: The following Tx impairments assumptions can be used for MPR/AMPR simulation. Carrier leakage: -34 dBc Image rejection: -34 dBc CIM3: -60 dBc CIM5: -70 dBc Proposal 9: The regulation and the impact to UE RF requirements for OOB boundary change should be studied. Proposal 10: The regulation for SEM requirement should be surveyed. Proposal 11: SEM requirement study uses 5G SU as the starting point. It can be revisited after there’s conclusion for 6G SU. |
| Nokia R4-2520554 | Observation 1: All analogue component performance improvements in mains supply operated UE compared to battery operated UE needs to be evaluated Proposal 4: Dedicated MPRs are defined for battery-operated UEs and mains supply operated UEs. Proposal 5: Dedicated A-MPRs are defined for battery-operated UEs and mains supply operated UEs. Proposal 6: UE type indication is needed to be able to separate battery-operated UEs and mains supply operated UEs. Observation 2: Best UE performance would be achieved if Foob is determined by BS channel bandwidth in case UE CH BW is smaller Proposal 8: PA dimensioning for battery operated eMBB UEs is MPR = 0 for CP-OFDM QPSK Inner region. Proposal 9: PA dimensioning for mains power operated eMBB will produce more UE Tx output power than PA dimensioning for battery operated eMBB Ues Proposal 10: Modulator impairments should be based on what is actually needed for most demanding modulation order to meet EVM requirement. |
| Apple R4-2520601 | Goals: - To address UE RF technology advancement since early 5G times - Model to take into account the Tx power management for power saving in real implementation such as APT or ET - Improve the accuracy of MPR/A-MPR requirements from 5G. Proposal 1: RAN4 to consider the following approach for 6G MPR/A-MPR simulation assumptions: - RF parameters relative to 5G - IQ image: Improved from -28 dB to -31 dB - Carrier leakage: Improved from -28 dBc to -31 dBc - CIM3: Kept as -60 dB - CIM5: Kept as -70 dB - To include DFE impairments and noise as necessary per company’s implementation - Incorporate APT or ET into PA model which is per company’s own modeling and may also include memory effect. - MPR calibration waveform – To consider 0dB MPR for the 6G calibration waveform which is jointly defined with default maximum output power requirement |
| vivo R4-2520724 | Impairment levels Proposal 6：Take the Tx impairment assumption from 256QAM in NR as the starting point for 6G, further study whether need to relax the impairment for PA considering the support of larger channel bandwidth. OOB boundary Observation 10: The spurious emission requirement in NR follows the category B limit in ITU-R SM.329 while the OOB boundary is more stringent than ITU suggested Proposal 7: The OOB boundary in 6G can consider follow the definition in SM.329 at least for 6G new bands, i.e., 250% from carrier center. SEM Observation 11: A relaxed SEM was discussed in WP5D and also already used in some regulation study. Proposal 8: RAN4 to study the feasibility to relax SEM in new 6G band, and the following table can be the starting point. Spectrum emission limit (dBm) / Channel bandwidth. ΔfOOB (MHz); 20 MHz; 25 MHz; 30 MHz; 40 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz; Measurement bandwidth. ± 0-1; −10; −10; −10; −10; 1% channel bandwidth. ± 0-1; −21; −21; −21; −21; −21; −21; 30 kHz. ± 1-5; −7; −7; −7; −7; −7; −7; −7; −7; −7; −7; 1 MHz. ± 5-105; Same as NR. |
| Spreadtrum, Unisoc R4-2520766 | Proposal 7: It is necessary to use EVM link budget to decompose and evaluate the effect of tx impairments. For high order modulation (e.g., 1024 QAM), we can wait for the EVM conclusion. Proposal 8: For 5G existing bands, there is no need to define new OOB boundary in 6GR. For new spectrum, it can be considered to be applied in new OOB boundary which is aligned with SM.1539 in 6GR, especially for large bandwidth. For spurious emissions, it still can refer to the SM.329. |
| Qualcomm R4-2520825 | Proposal: Define only LO for the center of the RF tuning BW with the level of 28 dBc and IQ image level same 28 dBc |
| ZTE R4-2521281 | Observation 6: RAN1 agreed to take 5G NR CP-OFDM and DFT-s-OFDM as 6GR waveform basis and consider enhancements/modifications on CP-OFDM and DFT-s-OFDM to further reduce the PAPR of the uplink signal, which provides possibility to reduce MPR/A-MPR. Observation 7: PA model/calibration point, reference waveform, IQ image and carrier leakage in 5G NR are reused from LTE, which may not be feasible for 6GR. Proposal 8: To use -36dB/dBc for both IQ image and carrier leakage requirements for 6GR MPR/A-MPR evaluation. Proposal 9: To discuss how to redivide the overall RB allocation range for 6GR MPR on the top of the 5G NR MPR RB regions. (FL: redivide means inner/outer region definition updates) Observation 8: If we continue to follow the MPR requirement framework in 5G NR, it would be a huge and dispersed work in RAN4 to reevaluate the MPR in 6GR. Observation 9: Current A-MPR is extremely complicated in terms of region division for different CBW and too many candidate groups of A-MPR values. Proposal 10: An efficient and appropriate approach to define MPR/A-MPR requirements in 6GR is needed. OOB boundary Observation 13: OOB boundary defined for channel bandwidth more than 5MHz in 3GPP is stricter than 250% of necessary channel bandwidth rule in ITU-R SM.329. Proposal 15: It is proposed to discuss the boundary between out-of-band emissions and spurious emissions domain for 6GR. Proposal 16: It is proposed to take SM.1539 and SM.1541 into consideration when discussing OOB boundary for 6GR. Proposal 17: It is proposed to define OOB boundary in 6GR as in the following table. Channel bandwidth; OOB boundary FOOB (MHz). 5; BWChannel + 5. 5 < BWChannel ≤ 50; BWChannel + 10. 50 < BWChannel ≤ 200; BWChannel + 50. [200 < BWChannel ≤ 400]; BWChannel + 100. |
| Oppo R4-2521450 | Observation 7: In MRP requirements definitions, waveform, modulation, RB allocation, power class, UE types (antenna isolation), MIMO configuration, and PA/RFIC models need to be considered. Among them some of the factors have some dependent on RAN1. Proposal 9: RAN4 can focus on below aspects to study the 6G MPR n Waveform: use DFT-S-OFDM and CP-OFDM as starting point n Modulation: use uniform modulations as starting point n Power class: take PC2 as starting point n UE types: use handheld UE as starting point, and FFS on how much dB antenna isolation need to be considered for other UE types n MIMO: single port as starting point n PA model: discuss which kind of PA models to be used in 6G, e.g., fixed biased model, APT model, etc. and also consider the new high bands n RFIC impairment: use 5G carrier leakage, IQ image, CIM3 assumptions as starting point and further discuss whether some updates can be made n Study how to better align simulation and measurements results. Observation 8: The CBW/RB allocation used in 5G MPR calibration is not the worst case in terms of spectrum utilization which makes the calibrated PA has worse linearity and finally large MPR requirements. Observation 9: The PA calibration results and MPR simulation results don’t show obvious difference between the calibration conditions of 20MHz/100RB@0 and 40MHz/full RB. Proposal 11: Continue to use 20MHz/100RB@0 as the MPR PA calibration condition in 6G. Observation 10: To pass the EVM requirements of 256QAM, -34dBc for carrier leakage and IQ image has been used in 5G MPR simulation/measurements. There is no difference in RFIC carrier leakage and IQ image in terms of modulation orders. Observation 11: Carrier leakage/IQ image tighten to -34dBc can improve the MPR for 64QAM around 0.6dB in some allocations, and is necessary for 256QAM. Proposal 12: The carrier leakage and IQ image condition for MPR calibration is tightened from -28dBc to -34dBc in 6G. Observation 12: Due to various PAs on the market and implemented in UE, no single PA model can be universally used in the simulations. This trend will continue in 6G. Proposal 13: The PA models used in 6G MPR simulation is still based on each company’s own choice. Observation 13: APT PA is widely used in UE implementation, and the APT PA will change its working modes when the output power is different which could reduce the power consumption but with the sacrifice of PA linearity, i.e., larger MPR. Observation 14: From simulation results it can be seen that the MPR required for APT PA could be 5dB higher than fixed bias PA in some configurations. Proposal 14: The APT PA models is encouraged to be used in 6G MPR simulation to better align with the real products and measurements. Proposal 15: The 6G MPR simulation calibration conditions is as below table. 6G. CBW, scs, RB allocation; 20MHz, 15khz scs, 100RB@0. Modulation; QPSK. Waveform; DFT-S-OFDM. Targe power level to meet ACLR; 1dB MPR. carrier leakage; -34dBc. IQ image; -34dBc. CIM3; -60dB. Observation 31: Device types discussed in RAN1 and RAN plenary mainly focus on the mandatory features for each device type which is different from RAN4 requirement definition. RAN1 may define many device types but in RAN4 they may share same RF requirements. Observation 32: RAN4 has defined many UE types but not be well organized in the 5G spec. This has caused many confusions in the industry in terms of which requirement to be applied to which UE types. Observation 33: For most of UE types defined in RAN4, the antenna numbers and antenna isolations were the main concern in 5G. Antenna numbers mainly about the relaxation from mandatory 4Rx to 2Rx for some specific UE type, and antenna isolation is mainly for the MRP requirement definition. Proposal 32: Consider using antenna isolations as the discriminator to differentiate applicable MPR requirements only and no bounding of UE types. Any UE that meets the antenna isolation conditions can apply the corresponding requirements. OOB boundary Observation 26: For wideband case, 1.5BN+BU boundary can be applied only when BN>BU in related frequency range. Observation 27: NR FR1 OOB boundary in current Spec does not align with the definition of ITU-R SM.1539. Proposal 25: In 6G, RAN4 need study whether FR1 OOB boundary need to define according to different CBWs in different frequency ranges, separately, as shown in Table 9. Table 9 Modified NR OOB boundary for FR1 bands |
| LG Electronixs R4-2521515 | Proposal 3: Consider Tx impairments for around 7 GHz - Post-PA Loss: 3.8~5.3 dB (including CA impact) - Carrier leakage: 34 dBc or 40 dBc - IQ Image: 34 dBc or 40 dBc - CIM3: 60 dBc |
| Sony R4-2522049 | Observation 8: algin the 3GPP requirements with ITU-R increase the credibility of 3GPP requirements globally. Observation 6: with a limited uplink bandwidth or RB length, e.g., 3MHz, the 6G massive IoT devices can be implemented as SAW-less natively, while power backoff or restriction in UL transmission may be needed if larger uplink bandwidth needs to be supported (see Appendix A for detail discussion). Proposal 8: define the general Tx/emission requirements based on ITU-R recommendations, while the local regulators can be taken into account case by case. Proposal 9: Defining the OOB boundary to be line with ITU-R SM.1541 with BW+ 5MHz, while the spurious emission limits in line with SM.329 Cat. B. |

## Open issues summary

### Sub-topic 3-1 Unwanted emissions framework

- Agreements

  - PA modelling is discussed in system parameter thread

  - In initial 6GR evaluations, use 5G NR unwanted emission requirements as starting point until requirement framework for 6GR is clear

  - Spurious emissions requirements, SEM and ACLR will be defined for 6GR

    - FFS for

      - OOB boundary

      - potential accommodations for different frequency sub-ranges

  - Encourage inputs on regulatory survey being conducted in spectrum agenda

  - Study the UE Tx impairment requirements/assumptions (e.g. carrier leakage, image rejection, CIM3 and CIM5)

  - In RAN4#117, prioritize discussion for UE Tx impairments and unwanted emission framework

Agreements from previous meeting

Issue 3-1-1: OOB boundary

Key observations:

- ITU-R SM.1539-2 allows wider OOB region as defined for LTE and NR.

- Proposals

  - Proposal 1: Retain current approach of CBW+ 5MHz

  - Proposal 2: Retain current approach of CBW+ 5MHz for bands defined also for 5G

  - Proposal 3: Study pros/cons of adopting OOB boundary up to 250% from carrier center.

    - One example

- Recommended WF

  - TBD

Issue 3-1-2: SEM

- Proposals

  - Proposal 1: Study the feasibility to relax SEM in new 6G band

  - Proposal 2: Study possible relaxation and applicable measurement bandwidth for 1st MHz of SEM mask, consider different CBWs

For all proposal, possible regulation on SEM needs to be considered.

- Recommended WF

  - Encourage further input on SEM regulation.

  - In SEM evaluations, consider 5G SU as starting point

Issue 3-1-3: ACLR

- Proposals

  - Proposal 1: Consider different ACLR requirements for sub-6GHz and above

  - Proposal 2: Based on coex results, consider different ACLR and ACS for different frequency sub-ranges (e.g, less 1GHz frequency, 2GHz, 3.5GHz, 4.5GHz and around 7GHz)

  - Proposal 3: Consider operator-controlled relaxation in UE ACLR/SEM e.g. when UE CBW is within wider NW CBW and other possible scenarios

- Recommended WF

  - Postpone discussion until further conclusions from coex evaluations and understanding on baseline ACLR requirements

### Sub-topic 3-2 MPR study assumptions

Issue 3-2-1: Tx impairments

- Proposals

  - Proposal 1: Study the feasibility and benefits of lo retuning to the center of carrier in contiguous CA operation to improve A-MPR, and potential MPR.

  - Proposal 2: Define only LO for the center of the RF tuning BW

  - Proposal 3: Consider UL scheduling range in Tx impairments

    - UL scheduling range is configured to UE, semi-statically.

    - UL scheduling range bandwidth is smaller or at maximum the same as the UE and/or BS channel bandwidth

    - UE places the local oscillator (LO) in the middle of the UL scheduling range (bandwidth)

    - UE can set UL baseband processing, LO and RF filters according to UL scheduling range bandwidth and location in frequency.

- Recommended WF

  - TBD

  - UL scheduling range needs further input from RAN1/2 on if there is some BWP equivalent solution defined for 6GR

Issue 3-2-2: Tx impairment levels

- Proposals for LO and IQ-image

  - Proposal 1: Retain -28 dBc

  - Proposal 2: Use improved values [-31 .. -40] dBc

- Proposals for CIM3 and CIM5

  - Proposal 1: Retain -60 dBc assumption for CIM3 and -70 dBc for CIM5

- Recommended WF

  - As a starting point, consider re-using CIM3 of -60 dBc and CIM5 of -70 dBc

Issue 3-2-3: PA model

Key observations:

- From simulation results it can be seen that the MPR required for APT PA could be 5dB higher than fixed bias PA in some configurations.

- If APT and ET including CFR/pre-distortion aspects must be considered, it is unlikely that a common model and assumptions can be agreed in RAN4.

  - Also, if these techniques can improve PA linearity in-band and within the ACLR1 region at low backoff levels, it comes at the expense of lower linearity improvement at higher backoff and potentially worse performance at larger frequency offsets

- Proposals

  - Proposal 1: Take into account Tx power management used in implementation

    - APT

    - ET

  - Proposal 2: PA model is up to companies’ own choice and may include memory effects

- Recommended WF

  - TBD

Issue 3-2-4: PA model calibration

- Proposals

  - Proposal 1: Calibration waveform should result in 0 dB MPR

    - Proposal 1a: MPR = 0 for CP-OFDM QPSK Inner region.

  - Proposal 2: Continue to use 20MHz/100RB@0 as the MPR PA calibration condition in 6G

- Recommended WF

  - TBD

Issue 3-2-5: MPR requirement framework

Key observations:

- If we continue to follow the MPR requirement framework in 5G NR, it would be a huge and dispersed work in RAN4 to reevaluate the MPR in 6GR.

- An efficient and appropriate approach to define MPR/A-MPR requirements in 6GR is needed.

- Proposals

  - Proposal 1: Number of MPR/A-MPR requirements should be reduced by covering single and multiple Tx by the same requirements

  - Proposal 2: RAN4 can consider whether scalable requirements to consider the multiple power classes.

  - Proposal 3: Consider using antenna isolations as the discriminator to differentiate applicable MPR requirements only and no bounding of UE types.

- Recommended WF

  - TBD

# Topic #4: Rx requirements

## Companies’ contributions summary

| T-doc number Company | Proposals / Observations |
| --- | --- |
| Murata R4-2520225 | For example, same linearity performance (IIPn) and RF front end architecture can be used to derive MSD due to IMD for same band group combination as in Table 2 with band group definition in Table 1 used in 5G. The band group definition might be different in 6G if new band group concept is introduced for 6G. Table 1 Band group definition in 5G FR1 FR1 band group range. Name; FR1-a (LB); FR1-b (MB); FR1-c (HB); FR1-d (VHB); FR1-e (UHB). Range (MHz); 600-1000; 1400-2200; 2300-2700; 3300-5000; 5250-7125. Duplex mode; Mostly FDD; Mostly FDD; FDD and TDD; TDD only; TDD only. Table 2 PC3 MSD due to 2 UL IMD (example) Aggressor 1; Aggressor 2; Victim; IMD order; MSD. LB; LB; MB; 2; A. MB; VHB; MB; 4; B. MB; HB; MB; 4; C. It should be noted that the MSD calculation mechanism is dependent on not only IMD order but also index. However, it can be simplified as one unified MSD number as HPUE MSD simplification rule with LUT approach. Similarly, harmonic and harmonic mixing MSD can be derived as in Table 3. Table 3 PC3 MSD due to harmonic and harmonic mixing (example) UL; DL; Harmonic Order; MSD. LB; MB; 3/1; D. MB; VHB; 2/1; E. VHB; HB; 2/3; F. For crossband isolation MSD, the filter attenuation and PA spectrum noise are dominant. In this case, the distance between UL-DL or fractional frequency information might be used such as LUT shown in Table 4. Table 4 PC3 MSD due to crossband isolation (example) (FreqDL-FreqUL)/ (FreqDL+FreqUL) (%); MSD. <1; G. 1≤ <2; H. 2≤ <4; I. 4≤ <6; J. 6≤ <8; K. 8≤ <10; L. ≥ 10; M. |
| Skyworks R4-2520292 R4-2520545 | Proposal on improved REFSENS parameters allowing REFSENS BW scaling for FDD and TDD bands: - Assuming 55dB T/R isolation for duplexers <2.7GHz, FFS if higher value can be agreed vs for lower frequency range. - Improved IMD2 in the receiver - Improved linearity of the PA and improved image and carrier rejection to 34dB for FDD bands - Assume systematically a 12dB UL interference imbalance between main and diversity - Use a limited number of UL RBs (using a maximum of 20-25RBs depending on the channel CBW) - Place the UL RB in the most favourable position where the IMDs related to its image and carrier leakage do not affect the DL. An indication may still be given on what the worst-case configuration is, FFS if a desense number is provided. - DeltaR is not specified and is absorbed within REFSENS while considering multi-band support and triplexing frequency sub-ranges onto one antenna. - Re-consider implementation margin and using MRC combining as the baseline. Proposal on Rx requirements: - 2Rx is still the baseline for REFSENS and scalable by RB including for FDD bands. - 1Rx to 8Rx scalability is granted by using: - 12dB UL interference imbalance between main and diversity to derive 1Rx REFSENS from 2Rx reference - scaling with 10*log(#Rx/2)-#Rx/4*0.5dB for #Rx>2 - FFS the exact values and equations - Default supported DL QAM: - DL 1024QAM for band ≤5GHz - UL 256QAM for band >5GHz and <16GHz - FFS for >16GHz - For NTN bands it may not be needed to specify the highest QAM - DL 4096QAM is considered optionally - The Rx impairments like IMD2, image and dynamic range should be further improved accordingly, and Rx EVM should be based on assuming better Tx EVM on the BS side. - Rx requirement should be scalable in BW for: - 3 to 100MHz for FDD band < 2.7GHz - 10 to 200MHz for TDD bands <5GHz - 20 to 400MHz for TDD bands >5GHz and <16GHz - 20 to 1600MHz for bands >16GHz - FFS if the frequency thresholds should be defined differently: - At least potentially for NTN bands - Depending on SCS and FFT size support per band. - Antenna scalability: - Up to 8Rx is supported up to 16GHz, FFS for >#Rx for larger form factors - 8x8 DL MIMO should be considered for smartphone form factor. - 2x1 to 8x8 antenna arrays are supported from >10GHz to cover all mobile/transportable UE form factors. FFS for larger arrays. 2x2 DL MIMO is supported - Support of circular polarisation should be studied for NTN - DL CA operation: - DeltaR is not specified and is absorbed within REFSENS while considering multi-band support and triplexing frequency sub-ranges onto one antenna. - Intra-band contiguous and non-contiguous is supported with up to 4CC but the #Rx may be scaled back from the single CC operation depending on the total BW and number of DL clusters to be supported - Intra-band contiguous DL is only supported if the BW cannot be supported with a single CC - For FDD bands no specific REFSENS requirement is specified - Inter-band DL CA with 1UL or 2UL bands is automatically supported without MSD specified if the bands are in separate band groups. FFS if the bands are part of the same band group. - In any case if MSD cases are specified, an automatic calculation based on band group assumption are studied. |
| Huawei R4-2520325 | Proposal 4-1: For single carrier REFSENS, the UE receiver noise figure is preferred to be assumed as 9dB while the diversity gain and implementation margin can be further considered in a case by case manner considering realistic implementation complexity. MSD Observation 4-1: Most parameters are consistent; variations mainly depend on band group/frequency range Observation 4-2: The feasibility of generic MSD upon types is demonstrated via Harmonic MSD & IMD MSD analysis Proposal 4-2: Simplify band combination MSD as much as possible and focus on the typical MSD types only Proposal 4-3: Study whether generic MSD requirements could be specified for the baseline default power class Proposal 4-4: Study whether testing burden could be reduced if generic MSD requirements are defined per order/band group and the test is done via declared/selected band combinations. |
| Mediatek R4-2520342 | Proposal 1: As for deriving 6G TDD band wider CBW (e.g., >100MHz) REFSENS, discuss whether the 5G TDD band REFSENS equation could be a starting point with modification (e.g., consider performance impact due to wider CBW). Proposal 2: The 5G ΔRIB,c usage shall be kept for 6G CA. For a 6G UE supporting CA, the minimum requirement for single-carrier reference sensitivity shall be increased by the amount given by ΔRIB,c specified in 6G. Proposal 3: Regarding HPUE self-interference REFSENS RSD issue in FDD bands, discuss whether it would be proper to study RSD mitigation. |
| Samsung R4-2520358 | Observation #2.5-1: It remains unclear whether the GSM network will be fully decommissioned prior to the finalization of the first 6G specification. Proposal #2.5-1: All existing Rx requirements remain relevant for respective bands with a CBW of up to 100MHz. Same requirements framework/concept can be applicable to frequencies up to around 7GHz with a CBW of up to 200MHz. Proposal #2.5-2: For the derivation of REFSENS in the frequency range up to around 7GHz with a CBP of up to 200MHz, the formula "REFSENS = -174 + 10lgCBW + NF + SNR + IM - Gdiv" shall still be adopted. Proposal #2.5-3: The 2Rx REFSENS shall remain the baseline, with delta values applied to it to derive REFSENS for other Rx count such as 4Rx, 6Rx, 8Rx and 1Rx. Proposal #2.5-4: For FDD REFSENS, PC3 is the basis, and delta values to higher power class can be added to derive HPUE REFSENS. - For the same power class, only one set of requirements is defined, regardless of the number of Tx chains. Proposal #2.5-5: For FDD UL configuration for REFSENS, discuss the feasibility to adopt full RB allocation. Proposal #2.5-6: Remove ΔRIB from CA requirement, and incorporate equivalent effect as ΔRIB into implementation margin (IM) of single carrier REFSENS. Proposal #2.5-7: Discuss and establish the 6G MSD framework, before starting MSD OTA testing discussion. - Alt1: The MSD calculation and specification manner remain unchanged, i.e., accounting for both the conductive part and OTA part, which are then combined - Alt2: Conductive-only MSD calculation and specification, i.e., the MSD calculation and specification only consider the conductive part. The terminology must be changed (e.g., MSDConductive) to differentiate with 5G. - Alt3: Separate conductive MSD and OTA MSD specification. E.g., Define MSDConductive and MSDOTA - Alt4: UE Dynamic MSD reporting [3][6] → Whether the proposal is UE to report the actual MSD for any configurations requested by NW needs further clarification - Alt5: UE Semi-static MSD reporting, i.e., UE can report the actual MSD for certain specific configurations for each combo if NW requests such configurations |
| Xiaomi R4-2520512 | Proposal 12: The cases for the scalable REFSENS framework study should be clarified. Proposal 13: The benefit to change the UL RB allocation for REFSENS should be clarified to continue the study. Proposal 14: If RSD is still defined, the RSD value is rounded to 0.5 dB. Proposal 15: RAN4 to discuss the possibility to remove CA MSD requirements from 3GPP spec. Proposal 16: Large CBW REFSENS study is started after the general REFSENS discussion is clear. |
| Nokia R4-2520554 | Refsens Observation 3: Current specification with only absolute numbers is unintuitive. Observation 4: Current way of specifying REFSENS is not future proof. Proposal 11: To make specifications future proof document the underlying assumptions, band specific impairment factors, and calculation methods explicitly in the normative specifications. Observation 5: UE NF of 9.0 dB was first proposed in 1999. Observation 6: Implementation margin of 2.5 dB as a catch all margin is ambiguous and contributes to the intuitiveness of the specification. Proposal 12: A unified equation covering various cases such as different duplexing, multiple receivers, etc., shall be utilized for defining the REFSENS requirements. Proposal 13: A better NF assumption shall be agreed for 6GR keeping the technology advancement of last two decades. Proposal 14: Investigate if different REFSENS needs to be defined for battery operated and mains supplu operated UEs Proposal 15: Study the need and possible values for implementation margin. ACS and blocking Observation 7: There would need to be very strong need and justifications to alter the current blocking requirements for 6G in terms of blocker level and interferer offsets. Observation 8: How the UE blocking requirements are defined/organized should be discussed and not just copied from NR. MSD Observation 9: Marking a CA combination for MSD is easily misinterpreted as a CA combination that should be avoided, but this is often not the case. Observation 10: Self-interference may be mitigated with Tx-Rx switching Proposal 16: The switching method designed through the low-band carrier aggregation switching study should be considered for 6GR as a standardized configuration for band combinations that has MSD. Proposal 17: RAN4 to discuss whether to define for 6GR a single table of supported band combinations together with their impairments requiring relaxation and pointer to MSD LUTs or as an alternative define rules for band combinations which point to MSD LUTs Observation 11: There is a need to study advances in UE self-interference determination how that information is provided for NW as a further assistance information. |
| Apple R4-2520601 | Goals: - Take into account UE RF technology advancement together with additional RF front-end insertion loss for supporting CA. - FDD band REFSENS requirements scalable with channel bandwidth to simplify the specifications - Avoid defining RSD (Reference Sensitivity Degradation) at maximum output power higher than default level for FDD bands. Proposal 3: RAN4 to consider the following approach for 6G single carrier REFSENS requirements framework: - Remove the ΔRIB relaxations and maintain the same single carrier REFSENS requirements from 5G as baseline. - For FDD bands with potential Tx self-interference, UL RB number restriction is specified with RB allocated at the center of the UL carrier such that Tx noise to DL carrier is substantially lower than Rx noise floor at maximum gain. - REFSENS is scaled with channel bandwidth for both TDD bands and FDD bands. - No RSD specified for FDD bands at maximum output power higher than the default level. |
| vivo R4-2520724 | Observation 12: Based on low, mid, high group definition for FR1, for UL harmonic the MSD of inter-group combs could be classified according to the harmonic order and allocation. Some exception cases also exist. Observation 13: Based on low, mid, high group definition for FR1, for harmonic mixing the MSD of inter-group combs exhibits greater dispersion compared to UL harmonic, but to some extent, it can still be classified based on harmonic order and allocation. Observation 14: Based on low, mid, high group definition for FR1, for MSD due to crossband isolation the inter-group combs are finite and MSD values could be effectively classified, fewer exception cases are expected. Observation 15: Based on low, mid, high group definition for FR1, the MSD of inter-group combs due to IMD exhibits the greatest dispersion but there is still some room for simplification. Proposal 9: Further study MSD simplification based on the band group concept discussed under 6G spectrum SI. Proposal 10: For each group combination, select several typical band combinations as the starting point for requirements simplification and the related study could also be used to guide the definition of band group. |
| Spreadtrum, Unisoc R4-2520766 | Proposal 9: Define a scalable way to simplify the definition of REFSENS, such as defining the same value within a band group for different bands. Proposal 10: It is necessary to study whether the value of margin implementation needs to be updated. Proposal 11: To simplify the MSD framework for CA, such as introduce band group concept. Proposal 12: Narrow band blocking can be removed if co-existence between NB-IoT and 6G is not considered based on the RAN guidance. Proposal 13: RX requirements used in 5G with CBW is up to 100MHz can be baseline in 6GR. Proposal 14: For Rx requirements that CBW is larger than 100MHz, we can wait for the conclusion about maximum CBW in system parameter agenda. |
| T-Mobile USA R4-2520796 | Observation 1: Companies have invested a tremendous amount of engineering hours developing the MSD requirements for 5G NR in RAN4. Observation 2: It would be a bad customer experience to go from 6 or 7 CC CA in 5G NR to 1 or two CCs in 6GR. Proposal 1: RAN4 should study if it will be possible to re-use the 5G NR MSD as a starting point for 6GR MSD, rather than starting over from scratch. Proposal 2: RAN4 should study re-using the 5G NR band combination database, along with the associated 5G NR MSD as a starting point for 6GR. |
| Qualcomm R4-2520825 | Observation 2.4.1-1: The Single Carrier RX requirements defined for current 5G band seem to be relevant also for respective 6G bands up to 100MHz BW, pending further studies. Observation 2.4.1-2: In case REFSENS is to be modified for 6G, careful evaluation for the wanted signal and/or blocker levels in all RX requirements is required. Observation 2.4.1-3: There could be room improve (tighten) NF/IM at least for some bands, but detailed studies are needed which probably take relatively long time until conclusions can be made. Proposal 2.4.1-1: Companies to provide their views on appropriate NF/IM level for REFSENS in 3GPP for different bands and frequencies. Assessments should include both evaluation on the said technology development, and also complexity since 3G times from where the current assumptions are taken from. Proposal 2.4.1-2: Companies to consider both pros and cons with potentially tighter REFSENS values, given that the default REFSENS is the basis for many other requirements in RAN4 Observation 2.4.1-4: UL RB allocation in single carrier case is relevant only for FDD band REFSENS Observation 2.4.1-5: There are no formal agreements how to determine UL RB allocation size and position for FDD bands, but despite this the REFSENS has been specified at rather constant way across the different bands and bandwidths Observation 2.4.1-6: Ideally, as result of the study RAN4 could have a set of agreements on how to determine the UL RB allocation size and position as a function of key parameters like e.g. Frequency, TX/RX-distance, and Bandwidth. Proposal 2.4.1-3: Companies to study different alternatives how to determine UL RB allocation size and position for FDD bands, keeping in mind that REFSENS is supposed is to characterise UE’s minimum performance. MSD Observation 2.4.2-1: Current MSD framework is not fully optimal due to various practical issues, but on the other hand it serves as good design reference and helps to compare different band combinations and their potential issues Observation 2.4.2-2: One of the challenges, infinite antenna isolation in conductive testing was investigated (e.g. using couplers between antenna connectors) in detail during LTE era and it was concluded that there is no feasible solution to address that. Observation 2.4.2-3: The MSD numbers may in some cases be pessimistic which is mostly due to usage of a set of bit pessimistic assumptions Observation 2.4.2-4: LowerMSD feature would allow indicating better than minimum performance, but it would not address the main challenges related to current MSD framework Proposal 2.4.2-1: RAN4 to solicit views especially from Infra vendors side to see what kind of improvement to current MSD framework would be beneficial in real deployments Proposal 2.4.2-2: Potential desense reporting should not cause adverse impacts to UE’s reporting good (low) MSD numbers, i.e. these UE’s should not be put into worse position compared with UE’s reporting worse (higher) MSD numbers. Proposal 2.4.2-3: Study both dynamic and static desense reporting and conclude if they are beneficial/if one of them is beneficial Proposal 2.4.2-4: When the baseline studies on the desense reporting and benefits are in good shape, study what is the allowed maximum desense to be reported vs the MSD requirement Proposal 2.4.2-5: Consider potential desense reporting as an amendment to current MSD framework with minimum requirements. Potential desense reporting shall not replace MSD requirements. |
| Ericsson R4-2521135 | Proposal 4.1: 6GR feature requirements should be based on realistic UE architecture assumptions, rather than relying on basic UE profiles that may not support the intended feature; Rx reference architectures should be based on the capability required for support of a feature, e.g. support of large power differences between carriers for spectrum aggregation schemes should not be based on worst-case assumptions of any UE implementation. Observation 4.1: selectivity/blocking are also important for user performance/DL-coverage, not only in-channel/link-budget performance. Proposal 4.2: minimum requirements for MSD for band combinations are also specified OTA to complement the existing noise-factor (MSD) requirement since also relevant for support of simultaneous Rx-Tx for band combinations. Observation 4.2: It is important that margins to actual REFSENS performance including any MSD are not too excessive as simultaneous Rx-Tx is crucial for performance regardless of any scheduler coordination. |
| ZTE R4-2521281 | Proposal 18: For 6GR handheld UE, it is still reasonable to define the REFSENS requirements based on the 2Rx. Proposal 19: On top of the basic receiver antenna numbers, it is proposed to use the reference sensitivity allowance values to scale the REFSENS requirements for other receiver antenna numbers for the same device types. Observation 14: There has not been uniform principle in RAN4 on how to define the FDD band REFSENS with impact of Tx self-interference as well as the UL RB configurations. Proposal 20: The existing equation-based approach can also be reused for TDD band supporting >100MHz channel bandwidths. Proposal 21: It is proposed to discuss a general principle for FDD REFSENS requirements definition considering the impact of Tx self-interference as well as the UL RB configurations. Proposal 22: It is proposed to check whether NF=7dB is feasible . MSD Proposal 23: To improve/simplify the MSD requirements, it is proposed use practical RF parameters to derive the MSD value based on PRD guidance. FFS on LUT-like or band group concept approaches. Proposal 24: For ACS/IBB/OOBB requirements for the 6GR single channel bandwidth >100MHz, to adopt the same approach as NR intra-band contiguous CA. |
| Oppo R4-2521450 | Proposal 27: The REFSENS calculation equation below can be reused in 6G, and 3dB for diversity gain with 2Rx antennas, 2.5dB implementation margin. For the NF, RAN4 can start with the discussion of targeting bands and analyze its related NF. Ÿ REFSENS (dBm) = -174dBm/hz + 10lg(RxBW) + NF + 10lg(SU) – DivGain + SNR + IM Observation 29: The REFSENs relaxation for larger CBW of FDD bands is not very clearly in 5G. Proposal 28: Review REFSENS for larger CBW and clarify the impact factors. Observation 30: ACS and in-band blocking in 5G have different interferer bandwidth for below 2700MHz and above 3300MHz bands. Proposal 29: For ACS and in-band blocking, RAN4 could study whether can adopt the same the bandwidth of interferer between the bands below 2700MHz and above 3300MHz including 6G new bands Proposal 30: RAN4 should consider to unify the parameters for out of band blocking, spurious response and wide band intermodulation between the bands below 2700MHz and above 3300MHz including 6G new bands. Proposal 31: Discuss whether narrow band blocking can be removed from 6G. |
| LG Electronixs R4-2521515 | Proposal 4: The REFSENS equation used in 5G can also be used in 6G. REFSENS (dBm) = 10log10(KTB) + NF + SNR – GDIV + IM Proposal 5: The noise figure around 7 GHz for 6G can be considered in conjunction with ∆ Rib Noise Figure (NF) Calculation Considerations for the 6G around 7GHz TDD Band - RF front end block loss evaluation - Diplexer: 1.5 dB - Switch: 0.8 dB - Additional switch loss subject to CA considering ∆Rib: 1 dB - TDD switch: 1dB - Total RF front end loss (around 7 GHz): 3.3 dB (including CA impact) - LNA - Gain: 15 dB - NF:3~7 dB - Gain block - Gain:20 dB - NF: 7 dB - NF calculation - Total noise figure: 6.6~ 10.4 dB (Typical temp), 9.6~13.4 dB (extreme temp) Proposal 6: The criterion for the IM (implementation margin) value needs to be more clearly defined in 6G. |
| Sony R4-2522049 | Proposal 10: RAN4 shall study improving the MSD requirement framework with possibilities to support a dynamic MSD reporting mechanism, making the feature useful for the network. Proposal 11: RAN4 to study the OTA requirement for MSD in 6GR if feasible. |

## Open issues summary

### Sub-topic 4-1 Single carrier Rx requirements

- Agreements

  - Study if the RX requirements used in 5G are still relevant for respective 6G bands up to 100MHz CBW and how to define relevant RX requirements for >100MHz CBW’s

  - Study concept for RX requirements for new frequencies, especially those above 7.125GHz

  - Study how to specify 6G REFSENS in a scalable way considering that 6G will support devices with different implementations/capabilities (e.g. different number of Rx chains) potentially addressing different use cases

  - Study single carrier REFSENS requirements considering technology advancement as well as implementation constraints such as UEs may support CA as well

    - Study the 6G REFSENS level and respective UL RB allocations for FDD bands as well as other RX requirements using respective 5G requirements as reference

  - Study improvements to MSD framework for 6GR in parallel to progressing single carrier requirements

  - Study multi-carrier requirements (e.g. ACS, blocking) once single carrier work is more stable

Agreements from previous meeting:

Issue 4-1-1: General

- Proposals

  - Proposal 1: All existing Rx requirements remain relevant for respective bands with a CBW of up to 100MHz.

  - Proposal 2: Same sub-6GHz requirement framework can be applicable to frequencies up to and including around 7GHz with a CBW of up to 200MHz.

    - For Rx requirements that CBW is larger than 100MHz, we can wait for the conclusion about maximum CBW in system parameter agenda.

- Recommended WF

  - TBA

Issue 4-1-2: Reference sensitivity

- Proposals

  - Proposal 1: Improved baseline noise figure

  - Proposal 2: UE receiver noise figure is assumed as 9dB while the diversity gain and implementation margin can be further considered in a case by case manner considering realistic implementation complexity

  - Proposal 3: NF/IM assumption can be different for different bands and/or frequency subranges

  - Proposal 4: Impact to other Rx requirements needs to be considered since they are defined relative to Refsens

  - Proposal 5: To make specifications future proof document the underlying assumptions, band specific impairment factors, and calculation methods explicitly in the normative specifications.

  - Proposal 6: The 2Rx REFSENS shall remain the baseline, with delta values applied to it to derive REFSENS for other Rx count such as 4Rx, 6Rx, 8Rx and 1Rx.

- Recommended WF

  - TBA

Issue 4-1-3: Reference sensitivity for FDD bands including scalability for different CBW

Baseline idea is that reference sensitivity parameters are chosen in a manner which allows reference sensitivity power level to be directly scaled as a function of DL BW.

Key observations:

- Ideally, as result of the study RAN4 could have a set of agreements on how to determine the UL RB allocation size and position as a function of key parameters like e.g. Frequency, TX/RX-distance, and Bandwidth.

- Proposals

  - Proposal 1: Study UL RB allocation for FDD bands  and whether and how to enable refsens scalability

    - Option 1: Study different alternatives how to determine UL RB allocation size and position for FDD bands, keeping in mind that REFSENS is supposed is to characterise UE’s minimum performance. Consider e.g.

      - T/R isolation

      - IMD2

      - PA linearity

      - IQ and DC leakage levels

      - UL interference delta between main and diversity

    - Option 2: UL RB number restriction is specified with RB allocated at the center of the UL carrier such that Tx noise to DL carrier is substantially lower than Rx noise floor at maximum gain

    - For both options: Study whether RSD is needed with the scalable solution

- Recommended WF

  - TBA

Issue 4-1-4: Blocking

Key observations

- There would need to be very strong need and justifications to alter the current blocking requirements for 6G in terms of blocker level and interferer offsets.

- How the UE blocking requirements are defined/organized should be discussed and not just copied from NR.

- Proposals

  - Proposal 1: For ACS and in-band blocking, RAN4 could study whether can adopt the same the bandwidth of interferer between the bands below 2700MHz and above 3300MHz including 6G new bands

  - Proposal 2: RAN4 should consider to unify the parameters for out of band blocking, spurious response and wide band intermodulation between the bands below 2700MHz and above 3300MHz including 6G new bands.

  - Proposal 3: Discuss whether narrow band blocking can be removed from 6G.

- Recommended WF

  - TBA

Issue 4-1-5: MSD minimum requirements

Key observations:

- It is important that margins to actual REFSENS performance including any MSD are not too excessive as simultaneous Rx-Tx is crucial for performance regardless of any scheduler coordination.

- It would be a bad customer experience to go from 6 or 7 CC CA in 5G NR to 1 or two CCs in 6GR.

- Marking a CA combination for MSD is easily misinterpreted as a CA combination that should be avoided, but this is often not the case.

- Carrier switching can be used to avoid self-interference

- Proposals

  - Proposal 1: RAN4 to solicit views especially from Infra vendors side to see what kind of improvement to current MSD framework would be beneficial in real deployments

  - Proposal 2: Study if it will be possible to re-use the 5G NR MSD as a starting point for 6GR MSD, rather than starting over from scratch.

  - Proposal 3: Study simplifying MSD minimum requirements by defining requirements for band groups (frequency sub-ranges) and IMD-orders

    - E.g. same MSD always for 3rd harmonic between low and high band

  - Proposal 4: Study whether MSD requirements could be specified only for default power class

  - Proposal 5: Study testing OTA MSD

  - Proposal 6:  Do not specify MSD requirements while MSD mechanisms per band combination may be captured.

- Recommended WF

  - TBA

Issue 4-1-6: MSD reporting

Key observations:

- LowerMSD feature would allow indicating better than minimum performance, but it would not address the main challenges related to current MSD framework

- There is a need to study advances in UE self-interference determination how that information is provided for NW as a further assistance information.

- Proposals

  - Proposal 1: Study desense reporting as an amendment to MSD framework with minimum requirements

    - dynamic MSD reporting

      - Actual MSD for a given configuration

    - (Semi-)static MSD reporting

      - Actual MSD for larger set of specific configurations beyond MSD minimum requirements

Downselection to take place later based on study outcome.

- Recommended WF

  - TBA

# Topic #5: Frequency range between FR1 and FR2-1

## Companies’ contributions summary

| T-doc number Company | Proposals / Observations |
| --- | --- |
| Skyworks R4-2520292 R4-2520545 | Proposal on RFFE characteristics covering the entire 0.4GHz to 52GHz frequency range: - In the separate antenna range up to 16GHz - Requirements tend to be related to conducted measurements and the radiation pattern is mostly omnidirectional - Multiplexing multiple frequency sub-ranges is a must and - Many bands are multiplexed with a frequency sub-range - non-3GPP systems/bands are also sharing cellular antennas (for example: GNSS, WiFi and other connectivity systems) - TN FDD bands are <2.7GHz requiring duplexers with small UL/DL gaps and TDD bands > 1.4GHz which, outside exceptions where a band needs to be protected in close proximity in frequency, the filtering requirement is more relaxed and co-banding - NTN FDD bands go up to 16GHz with some tight duplex separation for n247/248 or n508/n509 bands - To optimize the multiplexing, switching and hardware, co-banding is a critical aspect of the RFFE performance/size and cost. - In the antenna array range from 10GHz and above - Requirements tend to be related to OTA measurements and the radiation pattern is mostly directional or with beamforming - Multiplexing multiple bands or frequency sub-ranges to the same array is limited as the array dimensioning is related to the lambda at the operating frequency - TN bands are TDD and when feasible within a small frequency range, co-banding is used. - NTN bands are FDD with large UL/DL frequency separation thus using separate arrays but the for the n247/248 or n508/n509 bands the array or antennas could be shared raising the need for duplexers. - At least up to 16GHz, the technology set used in today’s T/R modules today can be re-used with small enhancements. This covers switching, filtering, multiplexing, LNA and PAs. - Above 20GHz silicon level integration is preferred for antenna arrays with only limited filtering capability for UEs with size constraints. |
| Mediatek R4-2520342 | Observation 1: To achieve same coverage between 3.5GHz and new 6G candidate spectrum in 7GHz and 15GHz, RAN4 need to compensate channel loss and penetration loss about 12 dB in 7GHz frequency range and compensate 22dB in 15GHz frequency range. Proposal 1: At least new 7GHz candidate spectrum, FR1-panel architecture with directional antenna system (as refer TR38.901 v19.1.0) can be considered as baseline in 6GR. Proposal 2: On top of with UE/gNB enhanced antenna elements with enhanced system configuration, RAN4 shall consider additional enhancements skills such as coherent UL-MIMO/beam forming, power boosting with relaxed ACLR and Tx EVM, default power class with 26dBm and SBFD in both gNB/UE and other candidate schemes. |
| Samsung R4-2520358 | Additionally, in our view, the default power class, number of Tx/Rx Chains, and maximum CBW should be discussed in System parameter thread under the umbrella of device type discussion to get a full picture. Thermal issue is not supposed to be addressed by RAN4 spec. Noise figure is something can be further discussed based on the various values documented in different TRs. Overall, it would needed to figure out first what is the scope for this topic. Proposal #2.6-1: Discuss and clarify what else to study beyond that has already been studied and documented in 5G TR 38.922 and covered by other 6G threads (such as System parameter and Spectrum). |
| Nokia R4-2520554 | Observation 12: Discussion on general aspects of the frequency range between FR1 and FR2-1 takes place under the Spectrum agenda (8.5). Specific for UE requirements is included under this agenda. Observation 13: Adding additional hardware, as antennas, for the new frequency range between FR1 and FR2-1 may result in degradation in overall performance of the UE across all supported frequency bands. Proposal 18: RAN4 shall ensure that including the frequency range between FR1 and FR2-1 shall not negatively impact requirements for operation in other frequency bands. |
| Spreadtrum, Unisoc R4-2520766 | Proposal 15: The homodyne architecture can be assumed for around 7GHz, the heterodyne architecture can be assumed for around 15GHz. The breakpoint for UE architecture can be further studied.(e.g., 10GHz or 15GHz). Observation1: Pathloss in free space around 7GHz is lower 6~7dB than around 15GHz. Observation2: The antenna size of 12-15GHz array antenna is similar with sub-6GHz, array antenna for 12-15GHz is feasible in antenna size. Observation3: The antenna gain of array antenna can be higher than patch antenna with no array. Proposal 16: Array antenna can be studied for around 15GHz. For around 7GHz, the breakpoint for with or without array antenna can be FFS. Proposal 17: For around 7GHz (e.g., 7-10GHz), NF is 10dB can be a starting point; for around 15GHz, NF is 13.5dB can be a starting point. Implementation margin can be further discussed. Proposal 18: Regarding conducted or OTA requirements for around 7GHz and around 15GHz, which can be postponed until the conclusion about antenna technology selection. Proposal 19: It is necessary to improve coverage at least around 7GHz in 6GR (e.g., PC2 default power class, array antenna, DL and UL MIMO, relaxing RF requirements). |
| Qualcomm R4-2520825 | Proposal 2.5-1: Each spectrum band in FR3 warrants its own separate discussion on UE RF front end choice. For example, discussion of < 8.4 GHz is decoupled from discussion on 13-15 GHz band. |
| ZTE R4-2521281 | Proposal 1: All of the related Tx/Rx requirements for the new wider channel bandwidths should be studied based on practical RF parameters/PA model assumption. |
| Oppo R4-2521450 | Between FR1 and FR2-1 Observation 51: 6425-7125 MHz is totally overlapping with NR band n104. Observation 52: 10-10.5 GHz with very restrict limitation, no operator will support it in 6G clearly. Observation 53: Candidate 6G Spectrum in WRC-27 includes 6 425-7 125 MHz, 4400-4800 MHz, 7125-8400 MHz, and 14.8-15.35 GHz which also need to be studied in RAN4. Proposal 48: Around 7GHz and around 15G can adopt FR1-like antenna and conducted RF requirements. |
| Sony R4-2522049 | Observation 1: Frequencies around 7 GHz can be seen as an extension of FR1, where similar antenna reference architecture as in FR1 can be expected. Proposal 1: FR1-like antenna architecture with 4 antennas can be assumed for handheld devices around 7GHz. Observation 2: The space in a smartphone for placing additional antennas around 15 GHz is extremely limited; therefore, reusing the physical structure of the existing antenna around 15 GHz might be necessary. Observation 3: The achievable rank around 15 GHz would be in the range of 2-4 for an indoor environment, implying that a UE with 4 antennas can almost exploit the full rank of the channel, and the system performance with more than 4 UE antennas could be limited. Proposal 2: It is proposed to consider four antennas around 15 GHz with an FR1-like antenna architecture for a broadband device with a limited form factor, e.g., smartphones, as the starting point. |
| LG Electronixs R4-2521515 | Proposal 7: Consider UE RF front-end characteristics as shown below. - Around 7 GHz: closer to FR1 characteristics; a baseline assumption could be similar to the 5G FR1 default architecture (e.g., 1T2R) but with a higher power class. Around 15 GHz: closer to FR2 characteristics; a baseline assumption could follow the 5G FR2 PC3 default architecture (e.g., 4-array, 1-panel) with beamforming capability. |

## Open issues summary

### Sub-topic 5-1 RF characteristics

- Agreements

  - Study UE RF front end characteristics considering potential differences between e.g. frequencies around 7 GHz and around 15 GHz.

Agreement from previous meeting

Issue 5-1-1: Study scope

Key observations

- To achieve same coverage between 3.5GHz and new 6G candidate spectrum in 7GHz and 15GHz, RAN4 need to compensate channel loss and penetration loss about 12 dB in 7GHz frequency range and compensate 22dB in 15GHz frequency range.

- For about 7GHz, to achieve comparable coverage as C-band with same assumption of inter-site deployment, BS can implement more antenna elements

- Proposals

  - Study for around 7 GHz and around 15 GHz

    - Whether both frequencies can and should adopt FR1-like antenna and conducted RF requirements

    - Realistic antenna count considering also impact to other frequency ranges

    - NF assumption and implementation margin

- Recommended WF

  - TBA

# Topic #6: Spectrum aggregation

## Companies’ contributions summary

| T-doc number Company | Proposals / Observations |
| --- | --- |
| R4-2520093 CATT | Observation 1: Based on the RAN 6G SI and SID of RAN WGs 6G, the spectrum aggregation framework (i.e. multi carriers operation) become more and more important at 6G stage. Observation 2: at the 6G stage, most of the operators have the adequate spectrum resources to initiate one UL band below 2.7GHz and one “band” with large channel bandwidth above 3GHz for 6G usage. This strategy can guarantee a good UL capability and UL coverage especially for the demands of deep coverage at the 6G stage. Proposal 5: the following common UE capabilities should be mandatorily supported in 6GR from RF’s perspective: P5-1: To support concurrent operation between one UL band below 2.2GHz and one DL band above 3.3GHz. P5-2: To support concurrent operation between one DL band below 2.2GHz and one DL band above 3.3GHz. Switching Summary of 5G Scenarios; Values; The RF components which may affect the RF switch-time performance. UL and DL switching in the same LTE TDD carrier; 20us; - The switch component in main path between PA/LNA and bands filter. - PA on-off time.. UL and DL switching in the same NR TDD carrier; 10us; - The switch component in main path between PA/LNA and bands filter. - PA on-off time.. UL and UL switching between different NR bands/carriers.; 0us; Two Tx chains were considered as mandatory UE capability. One Tx chain can be switched to the other Tx chain which have been prepared.. 35us; One Tx chain is shared between two different UL bands/carriers. Power supply for PA need to be switched without much LO retuning time due to close carrier frequency between two carriers.. 140us; One Tx chain is shared between two different UL bands/carriers. Longer enough time to retune the LO between two different UL bands/carriers.. 210us; One Tx chain is shared between two different UL bands/carriers. Some BB parameters should be reconfigured between two different UL bands/carriers.. Sub1GHz switching between FDD and SDL.; 35us; ASM switching + Antenna retuning?? The LO is not shared.. 70us; There is no technical discussion on this value.. 140us; ASM switching + Antenna retuning + LO retuning Rx chain below LNA including LO is shared between FDD DL band and SDL band.. Proposal 6: For UL Tx switching scenarios between different bands/carriers, it’s proposed to merge PA on-off time into the UL Tx switching time. Otherwise, more symbols will be blanked in the real deployment which is unexpected. |
| Huawei R4-2520325 | Switching Observation 5-1: Just saying "study RF switch-time performance" is not clear and it is beneficial to discuss specific scenarios such as study switching-time for Tx switching, or Low band CA switching etc. Observation 5-2: It is not clear which switching features are supported from 6G day 1 and if something new is introduced or not. Observation 5-3: Band group concept may require switching mechanisms that the current 5G switching features are not able to cope with, given that if one band group may include more than two bands and/or multiple duplex modes. Observation 5-4: It is not practical to discuss RF switch-time performance without knowing what kind of switching features are supported by UE and side conditions, e.g., the number of bands to be switched, duplex modes, if the switching is RRC based or non-RRC based, etc. Proposal 5-1: Postpone the discussion on RF switch-time performance until which kind of switching features are ready in 6G day1. Benefits-drawbacks of using CA vs. using wider single carrier BW Observation 5-5: Following aspects should be further taken into account for future study - Common to NW and UE RF: Spectrum utilization - Network: Operation complexity and associated unnecessary overhead and latency. - UE RF: On top of power consumption specifically for around 7GHz, following performance aspects are - Tx side - PAPR benefit including MPR aspect, for DFT-s-OFDM depending on implementations, and PA model(s). Possible cases are summarized in below Table. Table 2-1 RF chain assumption for CA and single CC 2*200MHz; 1*400MHz. Case 1; 1 single chain; 1 single chain. Case 2; 2 separate chains; 1 single chain. Case 3; 2 separate chains; 2 separate chains. - Out of band emissions like ACLR, SEM specifically for around 7GHz - RX - SNR considering image rejection and lo position - Blocking with ADC design and Low-pass filter rejection |
| Mediatek R4-2520342 | Observation 1: The requirements of interruption including the RF switching time in previous mobile generation has not been optimised since NR Rel-15. Observation 2: In 5G NR, interruptions due to RF retuning were defined with slot-level granularity and without differentiating between control and data channels. Proposal 1: RAN4 (RRM/RF) to further study the possibility of reducing the interruption time including the RF retuning time for different UE procedures. |
| Samsung R4-2520358 | Proposal #2.7-1: For FR1, 210 us as the assumption for LO switching time for switching relevant requirements. Proposal #2.7-2: Check if non-contiguous ULCA still remains attractive in the context of 6G, taking into account the identified drawbacks in 5G and potential technology advancements in 6G. Proposal #2.7-3: For intra-band contiguous CA, the CA bandwidth class concept is kept. For TDD bands, - Further check if class B is still needed, by considering the operators’ demand and if there if benefit over single carrier - If 200MHz CBW is introduced for certain bands, further check if class C is still needed, by considering the operators’ demand and if there is benefit over single carrier UL-DL decoupling: Proposal #2.7-4: The UL-DL pairing between two different bands (e.g., UL in band X and DL in band Y) must be conditioned upon UE RF hardware already supporting CA between these two bands (i.e., CA_X-Y). This ensures that the necessary RF front-end components are in place to achieve such pairing. N-Carrier Single cell Observation #2.5-2: Potential scenarios for N-carrier single cell are identified as follows. BWP adaption RAN4 shall hold on and wait for RAN1 conclusion as RAN1 will discuss whether or not to still have BWP or similar concept in 6G. |
| CMCC R4-2520432 | Proposal 4: Elastic cell with multiple carriers is suggested to be studied for multi-carrier operation and RAN4 is suggested to be evolved based on RAN1 LS, with the following characteristics, - Multiple carriers are co-located, either intra-band or inter-band. - RAN4 input for the feasibility of DL carrier and UL carrier couple. Proposal 5: it’s suggested to use 5G-A Tx switching requirements as baseline for 6G. |
| Xiaomi R4-2520512 | Obseravtion on fragmented spectrum: - Operators’ spectrum maybe fragemented over same band or across adjacent bands which bring implemenation difficulty and contraints - It’s worth to study enhanced solutions on supported fragemented carriers with affordale cost and improved spectrum usage efficiency Observation on UL Tx switching: - NR developed UL Tx chain switching in several release to accommodate different cases/scenarios which is lack of harmonized scheme and solutions and make the feature fragmented over several release and difficulty for implementation. - It’s worth to study unified solutions/schemes in 6G day1 to accommodate different scenarios/cases to simplify implementation and improve the performance Observation on LB CA band combinations - Due to RF front-end design challenge (antenna, filter etc.) , it’s difficult to support simultaneously Tx/Rx across closed/overlapped bands which make LB CA band combination hard to be deployed and implemented. - It’s worth to study the restriction and optimized solution to support difficult band combinations in 6GR day e.g. LB and LB band combination. Proposal 17: RAN4 shall be deeply involved in day 1 for flexible spectrum usage/spectrum aggregation study with tight cooperation with other WGs i.e., DL/UL decoupling, Fragmented carrier (single cell multi carriers), UL Tx switching/CA via switching Proposal 18: RAN4 work shall focus on RF/RRM feasibility study incl. reference architecture, related scheduling conditions and capability design and the benefits compared to traditional CA framework Proposal 19: RAN4 initial study can be focused on following area: - Fragmented spectrum (single cell multi carriers/BWPs) - Reference RF and BB architecture - RF/RRM feasibility and operation schemes between different operation modes - Leverage Rel-19 RAN4 fragmented carrier SI with extension to inter-band case - CA via switching and Uplink Tx switching with unified framework and mechanism - Leverage NR design on LB CA via switching and uplink Tx switching with extension to more cases |
| Nokia R4-2520554 | Proposal 19: UL switching should be based on semi-static switching pattern Proposal 20: Re-evaluation of switching period lengths to match modern RF chip technology Proposal 21: Re-evaluation of the need for DL interruptions considering modern RF chip technology Observation 14: UL switching framework is ill-suited for DL. Observation 15: Transient periods and timing advance are adversely affecting the DL throughput. Proposal 22: RAN4 shall aim for unified framework covering all three switching methods Observation 17: Wide single carrier bandwidth brings clear benefits over CA, and there may be no need to support multiple solutions for the cases where aggregated BW is not larger than minimum requirement for maximum (DL/UL) UE CBW. Observation 18: For cell-edge UEs the network could configure UL carrier on a lower frequency band like the 700 MHz frequency band while DL carrier could be maintained on a higher frequency band like 3.5 GHz or upper 6GHz band with wider channel bandwidth. Observation 19: Flexible pairing of DL and UL will be also beneficial from the UE power saving perspective Observation 20: Flexible DL/UL pairing would benefit load balancing in practical deployments |
| Apple R4-2520601 | Goals: - Avoid requirements fragmentation with multiple BCS. - Avoid defining the legacy MSD requirements which do not reflect the real-field sensitivity degradation and cannot be faithfully verified in conductive tests. - Allow network to be aware of the UL self-interference level to the scheduled DL carriers in real-field operation. - Avoid defining ΔTIB and ΔRIB requirements to reduce the RAN4 workload and specifications complexity. - Avoid defining CA features and requirements which are subject to poor operation efficiency such as intra-band non-contiguous UL CA and intra-band contiguous UL CA with non-contiguous allocations. Proposal 4: RAN4 to consider the following approach for 6G CA requirements framework: - Do not define BCS for 6G CA while allow UE to indicate its maximum aggregated bandwidth capability. - Completely remove the ΔTIB and ΔRIB requirements where the potential insertion loss from supporting CA is merged into single carrier maximum output power capability and REFSENS requirements. - Do not specify MSD requirements while MSD mechanisms per band combination may be captured. - Introduce an optional mechanism to allow network to trigger UE to perform UL self-interference level measurement and reporting. - For intra-band non-contiguous UL CA, only single switched UL is supported if specified. - For intra-band contiguous UL CA, only contiguous allocations are supported. - Mandatory simultaneous Rx/Tx except for Rx and Tx bands with: - Frequency range overlap, frequency adjacency, or close frequency proximity where no feasible filter is available to provide sufficient isolation between Rx and Tx |
| vivo R4-2520724 | Observation 16: The intra-band non-contiguous UL CA and RB allocations suffer from high MPR which can only be used in limited condition. Proposal 11: RAN4 to enable Tx chain switching to utilizing intra-band non-contiguous resource (e.g., CCA with non-contiguous RB allocation, NCCA) in 6G. Observation 17: UL 400MHz single carrier may require large MPR which make the coverage target in 7GHz is hard to achieve. Proposal 12: RAN4 consider use intra-band CA to support UL 400MHz rather than single carrier in 6G, with the 2x200MHz as reference architecture. |
| Spreadtrum, Unisoc R4-2520766 | Single cell multi carrier Proposal 20: It is necessary to study the feasibility from RAN4 perspective in advance to provide essential information to RAN1. RAN4 can be discussed as follows as a starting point. - UE architecture - Intra-band or inter-band - Applicability of measuring Decoupling DL and UL Proposal 21: To support flexible utilization of spectrum resources for DL and UL over different carriers/bands, these aspects need to be considered. - Whether to redefine the definition of bands - Whether the UE architecture is changed(e.g., filter) - Interference between DL and UL spectrum resources. Proposal 22: RF switching-time performance can be further discussed in new spectrum based on UE architecture in 6GR. Proposal 23: Whether to remove intra-band non-contiguous UL CA or not is up to operators’ demand. From RAN4 perspective, a single RF chain for intra-band non-contiguous or some UL RB allocation can be studied in 6GR. Observation 4: CA and wider single carrier bandwidth have their own draw-backs and benefits. They are not mutually exclusive, but can be complementary. Proposal 24: Band combinations that cannot support simultaneous Tx/Rx can be addressed case by case and release independence. |
| Qualcomm R4-2520825 | Proposal 2.6-1: Strive to clarify the scope or “RF switch-time” to have more focused studies from the companies Proposal 2.6-2: Evaluate whether intra-band non-contiguous UL CA should or should not be specified in RAN4 accounting all the constraints observed during 5G Proposal 2.6-3: Strive to specify wider single CC BW’s instead of Intra-band contiguous CA, unless study shows uses cases where wider single CC cannot be used. Proposal 2.6-4: Study which combinations should be specified for simultaneous TX/RX, and study how to capture support of simultaneous TX/RX in the specifications in consistent manner |
| ZTE R4-2521281 | Observation 15. It seems there are no benefits to support class B intra-band contiguous CA in the case of the maximum aggregated bandwidth is smaller than or equals to the supported maximum channel bandwidth for that band. Proposal 25: RAN4 should begin studies from CA as the spectrum aggregation discussion, and involve into the discussion on other mechanisms such as UL-DL decoupling, cell related pending on the RAN1’s progress. Proposal 26: Not to introduce class B intra-band contiguous CA in the case of the maximum aggregated bandwidth is smaller than or equals to the supported maximum channel bandwidth for that band. Proposal 27: Not to define the MPR for intra-band configurations CA with non-contiguous RB allocation. Proposal 28: If RAN4 would like to optimize the switching period, then up to 140us switching period would be enough. |
| Oppo R4-2521450 | Observation 35: To resolve the deployment scenarios, Tx switching, SDL switching and intra-band NC CA sharing one RF chain were enhanced under NR CA framework, however, they were all configured all with one cell and one CC. Observation 36: SCMC concept is discussing in RAN1 mainly for the purpose of solving CA configuration complexity and redundant control configurations. Observation 37: From UE perspective, SCMC and CA can share one set of RF implementation, there is no significant difference between them. Proposal 36: Study whether it is possible to define only one set of requirements for CA and SCMC when introduced in RAN1 considering CA and SCMC can share the same RF implementation. Observation 38: The complexity of supporting SCMC + CA simultaneously is high. Proposal 37: In 6G day one, mixed combination of SCMC + CA is not supported. Observation 39: UL/DL decouple is discussed in RAN1 for the purpose of cell-edge performance and UL coverage. Observation 40: UL/DL decouple have not any impact on UE RF implementation when band A and Band B adopt separate antenna in UE RF implementation. Observation 41: New RF front-end component will increase the complexity of UE implementation and new band number cause more band combinations and bring more workload. Proposal 38: Avoid to define a new band for UL/DL decouple pair. Observation 42: A UE supports inter-band CA (Band A + Band B), will support the recouple pair of Band A UL and Band B DL by CA enhancement. Proposal 39: To enable more efficient Cell/UE management, UL/DL decouple based on non-CA mode can be studied. Observation 43: New RF front-end component will increase the complexity of UE implementation although can save one set of RF chain band BB. Observation 44: If both bands are within same band group (e.g., both are low bands), then for shared antenna UEs, this will need a new quadplexes which will increase much complexity in implementation. Proposal 40: The recouple pair of Band A UL and Band B DL can work simultaneously or by switching between Band A UL and Band B DL. Observation 45: Carrier switching is under discussion in RAN1 which is targeted to simplify from RF and baseband especially for the band combinations which cannot supported simultaneously like low band FDD + FDD. Proposal 41: Study the RF impacts for carrier/band switching in terms of considering the switched carriers as one cell or two cells. Observation 46: All these new candidates aggregated technologies have similar purpose with enhanced CA in NR, except the different of Cell/UE management. Proposal 42: For one band combination, which aggregated technologies will be adopt need further down selection, and UE implementation complexity need to be well considered. Observation 47: BCS4 is introduced in NR in Release 17. It can be supported in 6G day one. Proposal 43: Consider remove BCS definition for band combination. Observation 48: In NR, there are many intra-bands congruous CA with smaller aggregated bandwidth, which will not be a typical application. Proposal 44: In 6G, strive to avoid using multiple small contiguous CCs aggregation to derive the large CBW, and use one CC with large CBW as much as possible. Observation 49: LTE has treated the inter-band CA requirements based on CA classes. Proposal 45: RAN4 can simplify the MSD definition for BC based on look up table and “band group” concept. Proposal 46: Further study how to define the band groups. Observation 50: ΔTIB,C and ΔRIB,C were studied based on CA classes in the beginning of LTE. Proposal 47: RAN4 can simplify the definition of ΔTIB,C and ΔRIB,C by combination of the “band group” concept and look up table method. |
| LG Electronixs R4-2521515 | Proposal 8: Do not define the ∆Tib/Rib from 6G requirements and, instead, integrate its performance impact directly into the single carrier requirements. |

## Open issues summary

Agreements in RAN1#122

Agreement

Study and identify the lessons learned from NR spectrum utilization and aggregation framework.

- DC is subject to RANP decision in June 2026.

- Note: MRSS aspects are separate discussion.

Agreement

Study and identify the lessons learned from NR BWP framework.

Agreements in RAN#109

- No agreement on dual connectivity

Sub-topic 6-1 Spectrum aggregation

Spectrum aggregation discussion will be in RAN1 agenda in February meeting.

- Agreements

  - Below spectrum aggregation topics can be discussed as starting point.

      - RF switch-time performance

      - Need for intra-band support of non-contiguous UL CA and RB allocations

      - Study benefits-drawbacks of using CA vs. using wider single carrier BW

      - Study how to account for simultaneous TX/RX between the bands properly from the beginning

  - Discuss band combination simplification in spectrum agenda

Agreements in RAN4 in previous meeting:

Issue 6-1-1: Intra-band non-contiguous CA

Key observations:

- The intra-band non-contiguous UL CA and RB allocations suffer from high MPR which can only be used in limited condition.

- Proposals

  - Proposal 1: Check for operator demand for intra-band non-contiguous UL CA

  - Proposal 2: Evaluate whether intra-band non-contiguous UL CA should or should not be specified in RAN4 accounting all the constraints observed during 5G

  - Proposal 3: Consider switched UL to utilize non-contiguous carriers

- Recommended WF

  - TBA

Issue 6-1-2: Intra-band contiguous CA

- Proposals

  - Study

    - Need for BCS

    - Whether requirements for non-contiguous RBs are needed

    - Need for bandwidth class B

- Recommended WF

  - TBA

Issue 6-1-3: Single wide carrier vs. intraband CA

Key observations:

- Wide single carrier bandwidth brings clear benefits over CA, and there may be no need to support multiple solutions for the cases where aggregated BW is not larger than minimum requirement for maximum (DL/UL) UE CBW.

  - Single wideband carrier maximizes UL power, minimizes overhead e.g., in terms of common channels and avoidance of guard bands, and enables easier BW adaptation.

- UL 400MHz single carrier may require large MPR which make the coverage target in 7GHz is hard to achieve.

- Following aspects should be further taken into account for future study

  - Spectrum utilization

  - Operation complexity and associated unnecessary overhead and latency.

  - UE RF power consumption

  - PAPR benefit including MPR aspect, for DFT-s-OFDM depending on implementations, and PA model(s).

  - Out of band emissions like ACLR, SEM specifically for around 7GHz

  - RX SNR considering image rejection and lo position

  - Blocking with ADC design and Low-pass filter rejection

- Proposals

  - Proposal 1: Consider to use intra-band CA to support UL 400MHz rather than single carrier in 6G, with the 2x200MHz as reference architecture

  - Proposal 2: Strive to specify wider single CC BW’s instead of Intra-band contiguous CA, unless study shows uses cases where wider single CC cannot be used.

- Recommended WF

  - TBA

Issue 6-1-4: Switch-time performance

Summary of 5G from R4-2520093

| Scenarios | Values | The RF components which may affect the RF switch-time performance |
| --- | --- | --- |
| UL and DL switching in the same LTE TDD carrier | 20us | - The switch component in main path between PA/LNA and bands filter. - PA on-off time. |
| UL and DL switching in the same NR TDD carrier | 10us | - The switch component in main path between PA/LNA and bands filter. - PA on-off time. |
| UL and UL switching between different NR bands/carriers. | 0us | Two Tx chains were considered as mandatory UE capability. One Tx chain can be switched to the other Tx chain which have been prepared. |
|  | 35us | One Tx chain is shared between two different UL bands/carriers. Power supply for PA need to be switched without much LO retuning time due to close carrier frequency between two carriers. |
|  | 140us | One Tx chain is shared between two different UL bands/carriers. Longer enough time to retune the LO between two different UL bands/carriers. |
|  | 210us | One Tx chain is shared between two different UL bands/carriers. Some BB parameters should be reconfigured between two different UL bands/carriers. |
| Sub1GHz switching between FDD and SDL. | 35us | ASM switching + Antenna retuning?? The LO is not shared. |
|  | 70us | There is no technical discussion on this value. |
|  | 140us | ASM switching + Antenna retuning + LO retuning Rx chain below LNA including LO is shared between FDD DL band and SDL band. |

- Proposals

  - Proposal 1: Re-evaluation of switching period lengths to match modern RF chip technology

    - Proposal 1a: Merge PA on-off time into the UL Tx switching time

    - Proposal 1b: Use [140us, 210us] for switching including LO re-tuning

  - Proposal 2: Postpone the discussion on RF switch-time performance until which kind of switching features are ready in 6G day1.

  - Proposal 3: Strive to clarify the scope or “RF switch-time” to have more focused studies from the companies

  - Proposal 4: Use 5G-A Tx switching requirements as baseline for 6G.

  - Proposal 5: RAN4 shall aim for unified framework covering all three switching methods (UL switching, DL switching, UL-DL switching

- Recommended WF

  - It is clear that UL-DL switching in TDD carrier will be needed. There are proposals in this meeting e.g. for single switched UL to replace non-contiguous UL CA and LB switching like scheme to avoid MSD.

  - Aim to identify relevant switching scenarios during the meeting, further study if switching time for those scenarios can be improved compared to 5G

Issue 6-1-5: Simultaneous Tx-Rx

- Proposals

  - Proposal 1: the following common UE capabilities should be mandatorily supported in 6GR from RF’s perspective:

    - To support concurrent operation between one UL band below 2.2GHz and one DL band above 3.3GHz.

    - To support concurrent operation between one DL band below 2.2GHz and one DL band above 3.3GHz.

  - Proposal 2: Mandatory simultaneous Rx/Tx except for Rx and Tx bands with:

    - Frequency range overlap, frequency adjacency, or close frequency proximity where no feasible filter is available to provide sufficient isolation between Rx and Tx

  - Proposal 3: Study which combinations should be specified for simultaneous TX/RX, and study how to capture support of simultaneous TX/RX in the specifications in consistent manner

- Recommended WF

  - TBA

Issue 6-1-6: Interruptions

- Proposals

  - Proposal 1: Re-evaluate the need for DL interruptions considering modern RF chip technology

  - Proposal 2: RAN4 (RRM/RF) to further study the possibility of reducing the interruption time including the RF retuning time for different UE procedures.

- Recommended WF

  - Further study need and duration for DL interruptions

  - Further discuss whether to cover the topic in UE RF or RRM thread

Issue 6-1-7: RAN1-led schemes: UL-DL decoupling, single cell multi carrier

- Proposals on RAN4 timeline

  - Proposal 1: It is necessary to study the feasibility from RAN4 perspective in advance to provide essential information to RAN1.

  - Proposal 2: RAN4 should begin studies from CA as the spectrum aggregation discussion, and involve into the discussion on other mechanisms such as UL-DL decoupling, cell related pending on the RAN1’s progress

  - Proposal 3: Elastic cell with multiple carriers is suggested to be studied for multi-carrier operation and RAN4 is suggested to be evolved based on RAN1 LS

- Proposals on UL-DL decoupling

  - Proposal 4: The UL-DL pairing between two different bands (e.g., UL in band X and DL in band Y) must be conditioned upon UE RF hardware already supporting CA between these two bands (i.e., CA_X-Y). This ensures that the necessary RF front-end components are in place to achieve such pairing.

  - Proposal 5: Following aspects need to be considered

    - Whether to redefine the definition of bands

    - Whether the UE architecture is changed(e.g., filter)

    - Interference between DL and UL spectrum resources

- Proposals on single carrier multi-cell

  - Proposal 7: Study whether it is possible to define only one set of requirements for CA and SCMC when introduced in RAN1 considering CA and SCMC can share the same RF implementation.

  - Proposal 8: RAN4 work shall focus on RF/RRM feasibility study incl. reference architecture, related scheduling conditions and capability design and the benefits compared to traditional CA framework

- Recommended WF

  - Deprioritize discussion in this meeting given no RAN1 work on these yet

  - Encourage companies to review all inputs to this meeting as well as encourage inputs on which aspects RAN4 should

# Topic #7: Joint UE and BS RF

## Companies’ contributions summary

| T-doc number Company | Proposals / Observations |  |
| --- | --- | --- |
| R4-2520093 CATT | Observation 3: For 5GA Tx EVM relaxation, it’s the truth that the existing 5G requirements and RF impairment assumptions should be took into consideration. For 6G Tx EVM relaxation, RAN4 has to wait for the discussion on 6G RF requirements and RF impairment assumptions. Proposal 7: For the SI scope on Tx EVM relaxation, the following points should be captured. - Study the impacts on BS receiver from both RF and demod perspectives. - Study the system gain for Tx EVM relaxation. - Study the potential impacts on the other WGs if network control is needed. Study NBC issues of BS receiver for 5GA if network control is not needed. |  |
| Huawei R4-2520325 | Proposal 6-1: Propose to add dedicated agenda item for the common part of 5GA and 6G Tx EVM relaxation study. Proposal 6-2: Adopt the following principles regarding the RF assumptions used for 5GA and 6G Tx EVM relaxation study. - 5GA Tx EVM study should be entirely independent from UE RF or system parameter discussion in 6G SI. - The existing RF requirements (other than EVM) and RF impairment assumptions shall be used as starting point for 5GA Tx EVM study. - The existing RF requirements (other than EVM) and RF impairment assumptions can be used as starting point for 6G Tx EVM study. Further update can be considered if there are related discussions concluded in 6G SI. “Common part” between 5GA non-AI solution and 6G AI solution Proposal 6-3: For simulation assumption alignment purpose, adopt the Tx non-linearity model that had been agreed for UL 256QAM MPR/AMPR evaluation as starting point. - The total EVM for 256QAM is contributed by four components, i.e. 1.85% from PA, 1.19% from Transmitter, 1.78% from Phase Noise and 2.06% from I/Q imbalance. Proposal 6-4: To get both of the MPR gain at transmitter and compensation gain at receiver, RAN4 study should determine PA as the major contributor for the relaxed EVM, while the EVM contribution of Transmitter, Phase Noise and I/Q imbalance maintains the level comparable to before EVM relaxation. Proposal 6-5: For simulation assumption alignment purpose, the polynomial model in R4-164542 can be used as a starting point. And RAN4 not to limit the PA model used for evaluation on Tx EVM relaxation, but all PA models shall follow the same calibration conditions. - DFT-s-OFDM waveform and QPSK modulation in 20MHz 100RB - 30 dBc ACLR (for PC3) or 31 dBc ACLR (for PC2), -28dBc IQ image, -28dBc carrier leakage and 60dBc CIM3 - 1dB MPR Work steps and evaluation methodology Proposal 6-6: Adopt the following work steps for both 5GA non-AI and 6G AI study. - Step-1: Clarify RF and Demod simulation assumptions for simulation - Tx non-linearity model, including total EVM contributors and PA model - Existing UE RF requirements except EVM remain the same (at least for 5GA study) - Reference DPoD algorithm(s) - PUSCH configurations including MCS, DMRS etc. - Step-2: Simulation result collection - At transmitter, get the minimum MPR for a given MCS that meets the target EVM and all the other UE RF requirements - At receiver, get the SNR corresponding to 10% BLER for the given MCS - Use the net gain i.e. MPR gain minus demodulation loss as the metric Proposal 6-7: Adopt the following link level simulation configurations for both 5GA and 6G Tx EVM relaxation study as starting point. Refinement can be considered if necessary. Parameter; Value. UE power class; PC3 and/or PC2. MCS; 17 for 64QAM and 23 for 256QAM (Table 5.1.3.1-2 in TS 38.214). Waveform; CP-OFDM. PA model; Polynomial model e.g. R4-164542. Tx number; Both 1 and 2 can be considered. Rank; Both 1 and 2 can be considered. DMRS type; Type 1. Carrier frequency; 3.5 GHz and/or 7 GHz. SCS; 30 kHz. Channel model; TDL-D. Channel estimation; Real channel estimation based on DMRS. Receiver; MMSE-IRC DPoD is enabled (e.g. algorithm in [5]). Proposal 6-8: RAN4 can study on BS demodulation performance to reflect DPoD implementation at BS receiver associated with relaxed UE Tx EVM, corresponding demodulation requirements and test setup can be left to WI stage. Proposal 6-9: RAN4 study prioritizes Case#1 and other cases e.g., Case #2 can be considered. - Case #1: evaluate the MPR gain under relaxed Tx EVM when single UE is scheduled. - E.g., a range of relaxed EVM can be considered for 64QAM and 256QAM. - Case #2: evaluate the MPR gain under relaxed Tx EVM when single UE is scheduled along with interference. - E.g., a range of relaxed EVM can be considered for 64QAM and 256QAM. - The PA model and/or EVM level of the interference can be different from the scheduled UE. - The interference can be FDMed or SDMed with the UL signal transmitted by the scheduled UE. |
| Mediatek R4-2520342 | Observation 1: For the target UL enhancement scenario i.e. Inner RB region with 64QAM and 256QAM in CP-OFDM waveform, Tx EVM is the bottleneck for meeting MPR requirements of high-order modulations. To evaluate Tx EVM analysis, RAN4 can reuse the agreed 5G-A WI scope as follow Proposal 1: RAN4 can consider the inner RB allocation first based on the 5G-A Tx EVM relaxation scope as starting points. - Study the feasibility to reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM, 256QAM - The study is performed based on non-AI-based approach at BS receiver under existing and relaxed UE TX EVM requirements - Non-linearity model(s) of transmission signals is studied to capture PA non-linearity and other RFIC impairment at least including I/Q imbalance. - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged. - The study is performed based for FR1 single CC operation. - Example bands as n41, n77/n78 - PC3 with 1Tx, PC2 with both 1Tx and 2Tx - Waveform: CP-OFDM Also, the evaluation metric is net gain can be aligned with RAN1 as following. Proposal 2: The net gain is calculated as achieved MPR gain – loss SNR level i.e., (original MPR – relaxed MPR) - (updated SNR – original SNR). |  |
| Samsung R4-2520358 | Observation #2.8-1: The goal of the work is to enable low-end UE to transmit higher power, by leveraging the capabilities of advanced BS receiver. Observation #2.8-2: The EVM of higher modulation is significantly affected by I/Q imbalance. Proposal #2.8-1: Adopt the following study scope of 5G-A solution. Reduction of UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders - Study the feasibility to reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM, 256QAM - The study is performed based on non-AI-based approach at BS receiver under existing and relaxed UE TX EVM requirements - Non-linearity model(s) of transmission signals is studied to capture PA non-linearity and other RFIC impairment at least including I/Q imbalance. - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged. - The study is performed based for FR1 single CC operation. - Example bands as n41, n77/n78 - PC3 with 1Tx, PC2 with both 1 TX and 2Tx - Waveform: CP-OFDM. General assumption for both AI and Non-AI solutions Proposal #2.8-2: Non-linearity model(s) of transmission signals (selected for the evaluation of BS compensation capabilities and UE MPR reduction limits) and the working steps for feasibility confirmation, are determined in UE RF session, which are common for BS compensation with AI and Non-AI solutions. Proposal #2.8-3：Non-linearity model(s) of transmission signals shall account for both PA non-linearity and other RFIC impairment including at least I/Q imbalance - For evaluation purpose, adopt generalized memory-based polynomial (GMP) model in TR 38.803 Proposal #2.8-4: For evaluation purpose, the above I/Q imbalance modelling can be considered. - Wherein amplitude imbalance (gI,gQ) ~ [0, 0.5dB] unified distributed, phase imbalance(θI,θQ) ~ [-5, 5 degree] unified distributed. Proposal #2.8-5: Uplink 1024 QAM can be evaluated as part of the 6G Tx EVM relaxation study once it is confirmed as one of the 6G modulation schemes. - Downlink 1024 QAM MCS table can be reused Working steps for feasibility study for both AI and Non-AI solutions Proposal #2.8-6: Adopt the following working steps for feasibility study for both AI and Non-AI solutions. - Study item phase (demod centric) - Step-1: Selection of Non-linearity model(s) of transmission signals - UE RF session firstly to determine the non-linearity model(s) - Note: UE DPD is not considered in modelling - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure - Note: For feasibility study, demodulation relevant evaluation is performed in UE RF session for non-AI based solution, demodulation relevant evaluation is performed in AI session for AI based solution - Including the non-linearity model(s) into the RAN4 PUSCH demodulation test case(s) for both 64QAM and 256QAM - For a given Tx EVM value derived based on the non-linearity models, evaluate the performance with following assumptions on selected RAN4 test cases to find the target SNR to meet RAN4 requirement or test metric, e.g., 70% of maximum throughput or 10% BLER - Performance evaluation - Case 1: Conventional receiver including non-linearity model: SNR-a - Case 2: Non-AI based non-linearity (NC) receiver with including non-linearity model: SNR-b - Case 3: Conventional receiver without including any non-linearity model: SNR-c - Note: Case 3 is considered as upper bound performance, Case 1 is expected to be low bound performance - Performance comparison - Compare SNR-b and SNR-c, the results should be comparable - FFS the criteria to be claimed as “comparable” (e.g., within certain x dB), and the target SNR-b(c) should be feasible for network deployment - Perform evaluation with different Tx EVM values, find/determine the maximum Tx EVM with the target SNR-b derived based on Case 2 to meet RAN4 requirement or test metric and comparable with the target SNR-c derived based on Case 3. - Perform different PA models, if needed - WI phase (UE RF centric) - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation and evaluate the Net Gain: - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged - For instance for 256QAM Tx EVM, 3.5% -> 6%, - For selected PA model(s), evaluate the allowed TX power boosting/gain (i.e., MPR reduction) for different power class - Net Gain [dB] = (Tx power boosting/gain relative to reference) − (SNR degradation relative to reference at 10% BLER) - FFS the necessity for new EVM model for RAN4 demodulation test cases. Note: Above approaches are applicable for both 5G waveforms and 6G waveforms Proposal #2.8-7: For performance evaluation, following simulation assumption with considering the typical PUSCH demodulation assumption in RAN4 can be considered as starting point. Parameters; Value. UE power classs; PC3 with 1Tx PC2 with both 1Tx and 2Tx. PA model; Generalized memory-based polynomial (GMP) model in TR 38.803. I/Q imbalance modelling; amplitude imbalance (g_I,g_Q) ~ [0, 0.5dB], phase imbalance(θ_I,θ_Q) ~ [-5, 5 degree]. Carrier frequency; 3.5 GHz. # of TX layers/users; Both 1/2 can be considered. # of RX antennas; 4/8/64. Waveform; CP-OFDM. # of PRB; 50. # of symbol per slot; 12. # of DMRS per slot; 2. Modulation; 64 / 256 QAM. MCS index; [28] for 64QAM and [28] for 256QAM (Table 5.1.3.1-2 in TS 38.214). Channel model; TDL-A 30ns. Subcarrier spacing; 30 kHz. Evaluation metric; BLER with 10%. Benchmark; Conventional receiver (without non-linearity compensation): e.g., MMSE-IRC Non-AI based non-linearity receiver: e.g., GMP based method. |
| CMCC R4-2520432 | Proposal 6: RAN4 is suggested to be evolved early for LP-WUR design study based on RAN1 signal design. |  |
| Nokia R4-2520554 | Observation 16: The 5G NR BWP design, despite aiming for UE power saving and diverse bandwidth support, is hampered by excessive signalling overhead, poor switching robustness and latency, a lack of RAN4 involvement leading to large MPR/A-MPR, and restrictive coupled UL/DL BWPs in TDD. Proposal 23: Study 6GR bandwidth scalability by early cooperation with other working groups with the following aspects: - Simplify Bandwidth Adaptation: o Reduced configuration overhead o Reduced number of bandwidth adaptation options compared to NR. o Support only a single Subcarrier Spacing (SCS) per carrier. o Minimize the number of BW adaptation types and BW-specific parameters. - Separate DL and UL BW Adaptation: o Consider DL and UL bandwidth adaptation independently to optimize each direction. o Assume separate LO for DL and UL BW adaptation in TDD - Improve Robustness, Reduce Latency and minimize interruptions in BW adaptation: o Enhance the reliability and speed of bandwidth adaptation – the procedure must support robust failure recovery. - Study to optimize RF/RRM switching delays and MPR/A-MPR with better assumptions related to LO placement in bandwidth adaptation - Streamline UE Capabilities: o Define concise set of UE capabilities for BW adaptation and identify the essential operation to ensure good baseline feature support for BW adaptation operation. Observation 21: Impact to MPR reduction is achieved by proper LO placement and reduction of UL scheduling range bandwidth into what is needed instead of what is UE channel bandwidth. Observation 22: UL scheduling range impact to A-MPR reduction will be significant as in some cases A-MPR can be fully avoided. Proposal 24: RAN4 needs to consider bandwidth adaptation (UL scheduling range) from the beginning, to minimize MPR and A-MPR. Observation 23: UL scheduling range is configured by the network semi-statically for each UE individually. Proposal 25: Network control would be needed in case 3GPP agrees to introduce UE Tx EVM feature Proposal 26: RAN4 to discuss the scope and assumptions for the relaxed UE Tx EVM feature Observation 24: There would be gain potential mainly for 256QAM where EVM is always the bottle neck, while for lower orders (e.g., 64QAM) it is mainly either IBE (in the inner region) or ACLR (in the outer region) Proposal 27: Gain potential for Tx EVM relaxation should be studied with RF-simulations taking into account all the relevant RF-requirements, such as EVM, IBE, ACLR, OBW, SEM, and spurious emission. Proposal 28: Aim to use the same PA model for RAN1 waveform evaluation and RAN4 TxEVM relaxation under advanced receiver assumption study. Observation 25: Having a memoryless PA model gives unrealistic advantage for Digital post distortion evaluation Proposal 29: Use a Hammerstein (N-L) model for modelling of the PA as an example Observation 26: IBE is already limiting the UL maximum power in 5G NR in many cases such as for all waveforms, power classes, most modulation orders and RB allocations, etc. Observation 27: Partial and full IBE relaxation can increase UL power for all waveforms and most modulation orders in all frequency ranges and can increase Tx EVM relaxation gain for high modulation orders such as 256 QAM. Proposal 30: RAN4 to consider NW controlled partial and full IBE relaxation without Tx EVM relaxation for modulation order up to 64QAM, and with Tx EVM relaxation for higher modulation order. |  |
| vivo R4-2520724 | Proposal 13: For net gain evaluation, the EVM distortion should be generated from real PA model rather than adding white noise. Proposal 14: RAN4 need to study how to validate that a stable performance gain can be achieved across all different PA models, which should be one of aspect for feasibility conclusion. |  |
| Qualcomm R4-2520825 | Observation 2.7.1-1 : Transmitter non-linearity is impacted not only by PA but also by other factors. Proposal 2.7.1-1 : Study the following factors which impact transmitter performance. - Tx DFE - Crest Factor Reduction (CFR) - Digital Pre-Distortion (DPD) - Others (thermal noise, quantization noise etc.) - Tx AFE - IQ imbalance - Phase noise - LO leakage - PA non-linearity (architecture, mode of operation, power class, technology, frequency range, bandwidth : memory effects, calibration point etc.) - Filter characteristics (amplitude flatness, group delay etc.) - Others (thermal noise, CIM3/5, etc.) Observation 2.7.1-2 : UE Tx EVM relaxation impacts in-band emission (IBE) metric. Proposal 2.7.1-2 : Linearity metrics other than EVM impacting MPR like ACLR, SEM, SE, and IBE (calculated with non-relaxed EVM) should remain the same as in 5G. |  |
| Ericsson R4-2521135 | LP-WUR Observation 5.1: RAN1 start to evaluate the OFDM based DL WUS. Proposal 5.1: Study WUR feature enhancement from RAN4 aspects. Observation 5.2: UE autonomously enables/disables wake up receiver makes the power saving gain of WUR unpredictable. Proposal 5.2: Not considering the 5G autonomous WUR/MR switching behaviour in 6G WUR study. Proposal 5.3: Study the WUR NF in 6G study. |  |
| ZTE R4-2521281 | Proposal 11: The evaluation for relaxing Tx EVM should priority high order modulation. Proposal 12: Non-linearity issues including PA model, IQ image/carrier leakage, CFR, DPD for UE transmitter side should be investigated for Tx EVM relaxing. Coverage enhancement Observation 16: There were many mechanisms introduced to increase UL coverage in 5G NR. Proposal 29: To consider the following aspects for 6GR coverage enhancement in RAN4: - Large array UL MIMO - Coherent UL MIMO - Low PAPR waveform (FDSS) for high order modulation and CP-OFDM - Relax EVM/MPR requirements |  |
| Oppo R4-2521450 | Proposal 10: Study whether the requirement (ACLR/SEM/EVM) relaxation scenarios could be used as default in 6G to make it more useful. DPOD Observation 19: Usually, RAN4 non-AI based solution will be done prior to AI based solution. Observation 20: To avoid duplicated discussion, the common part for AI and non-AI based solution will be studied. Proposal 19: Below RAN4 work arrangement to be followed: • The common part of non-AI and AI based solution is discussed and aligned. • Under the UE RF thread, the non-AI based solution can go prior to AI-based solution to provide a baseline performance. • The evaluation assumption including PA non-linearity, UE RF requirements for MPR simulation will be based on the available information on the table. Further change can be applied if new PA model and/or UE RF requirement is introduced. Observation 21: The specific part for AI based solution is the BS AI DPoD algorithm. Observation 22: The common part of AI and non-AI based solution is evaluation method, simulation assumption and the justification metric. Observation 23: One of the evaluations of relaxed EVM is the UE output power increase after EVM relaxation compared to the output power without EVM relaxation. Observation 24: The other evaluation of relaxed EVM is the potential SNR degradation for UE with higher output power and relaxed EVM compared to UE with lower output power and legacy EVM. Proposal 20: To evaluate the UE output power with EVM relaxation. Proposal 21: To evaluate the potential SNR degradation for UE with higher output power and relaxed EVM compared to UE with lower output power and legacy EVM. Observation 25: RAN4 MPR simulation only require to calibrate the PA at specific point. Proposal 22: Same PA calibration point as NR is proposed. Proposal 23: Same RF impairment of NR MPR simulation is proposed. Proposal 24: To further align the link level simulation assumption. |  |
| Sony R4-2522049 | Observation 9: EVM relaxation is effective in reducing MPR for high-order modulations. Proposal 11: it is proposed that RAN4 focus on 256QAM as starting point to carry out the study. Proposal 12: Based on the agreed PA model, RAN4 shall consider the following two aspects when determining the feasibility of EVM relaxation: 1) the EVM relaxation level needed to achieve a meaningful MPR reduction, e.g., larger than 1.5 dB based on 5G NR UE PA model as starting point. 2) The feasibility from BS side how to demod with the corresponding relaxed EVM level Proposal 13: If approved as feasible, RAN4 may consider relaxing the EVM requirement for higher-order modulation schemes in 6GR compared to 5G NR. LP-WUR Proposal 14: 6G to support LP-WUS and LR from day-1, with consideration of more usage scenario, e.g., RRM measurement being offloaded to 6G LR design, to increase the power saving gain in real life. |  |

## Open issues summary.

- Agreements

  - Encourage companies to contribute whether there are concrete study points for low power receiver.

    - Coexistence study including need for coex study will be discussed in agenda for BS RF and co-existence.

  - Prioritize work for the baseline single carrier output power requirement framework before detailed studies on ACLR and SEM relaxation.

  - Discuss PAPR reduction topics in system parameter agenda

  - For Tx EVM relaxation

    - Discuss non-AI based UL demod in UE RF thread

      - The impact on BS receiver could be considered.

    - Do not discuss AI based solution(s) in UE RF thread, except the potential common part.

    - The potential common part between AI and non-AI solutions will be discussed in this thread. AI solutions, which are discussed under AI agenda, should leverage the agreements in this thread as much as possible.

      - Strive to study TX non-linearity sources, including PA, in this thread

      - It is FFS whether and how, if agreed, to take into consideration of the existing 5G requirements and RF impairment assumptions for both 5GA and 6G.

    - Strive to avoid overlapping work between different threads

    - In RAN4#117, prioritize discussion on study scope and necessary details aligned with RAN guidance, e.g.

      - Which aspects need to be considered to conclude on feasibility

      - Need for network control

Agreements in previous meeting

### Sub-topic 7-1 Low power receiver

Issue 8-1-1: Low-power receiver

- Proposals

  - Proposal 1: RAN4 is suggested to be evolved early for LP-WUR design study based on RAN1 signal design.

  - Proposal 2: 6G to support LP-WUS and LR from day-1, with consideration of more usage scenario, e.g., RRM measurement being offloaded to 6G LR design, to increase the power saving gain in real life.

  - Proposal 3: Study WUR feature enhancement from RAN4 aspects

    - Not considering the 5G autonomous WUR/MR switching behaviour in 6G WUR study.

    - Study the WUR NF in 6G study

- Recommended WF

  - Aim to identify whether there is a need to study LP-WUR and what would be the study scope

### Sub-topic 7-2 UE coverage enhancements

Issue 7-2-1: UE requirement coverage enhancements

- Proposals

  - Proposal 1: RAN4 study new UE framework for the missing UE UL performance and the verification of UL features including UE RF related coverage from system perspective or verification of the whole UE RF&BB Tx chains.

- Recommended WF

  - Postpone discussion on this, encourage further inputs

  - FL to identify more appropriate place for this discussion within this thread

Issue 7-2-2: UE UL link coverage enhancements

- Proposals

  - Proposal 1: consider the following aspects for 6GR coverage enhancement in RAN4:

- Large array UL MIMO

- Coherent UL MIMO

- Low PAPR waveform (FDSS) for high order modulation and CP-OFDM

- Relax EVM/MPR requirements

- Recommended WF

  - Postpone discussion on this: Chair has postponed UL-MIMO discussions. EVM relexation is discussed in Sub-topic 7-3, Low-PAPR waveforms are part of system parameters.

### Sub-topic 7-3 Tx EVM relaxation

Power enhancement with relaxed UE Tx EVM and advanced BS receiver

- Note: Discuss both 5G-Adv and 6G reduced UE MPR values with the relaxed Tx EVM requirement as part of 6G study. RAN4 will further discuss and finalize (if agreed) the scope and other necessary details (e.g. NW control of this feature) in 2025.Q4 under 6G agenda items. It will be further decided in RAN#113 (Sep, 2026) whether or not there will be a follow up Rel-20 WI under 5G-Adv. For 5G-Adv, solution should not be AI based and there should not be any impact to RAN1.

EVM relaxation in 5G-Advanced is discussed within the 6G study based on the note included in the 5G-Advanced WI in RP-252952:

- For Tx EVM relaxation

  - Discuss non-AI based UL demod in UE RF thread

    - The impact on BS receiver could be considered.

  - Do not discuss AI based solution(s) in UE RF thread, except the potential common part.

  - The potential common part between AI and non-AI solutions will be discussed in this thread. AI solutions, which are discussed under AI agenda, should leverage the agreements in this thread as much as possible.

    - Strive to study TX non-linearity sources, including PA, in this thread

    - It is FFS whether and how, if agreed, to take into consideration of the existing 5G requirements and RF impairment assumptions for both 5GA and 6G.

  - Strive to avoid overlapping work between different threads

  - In RAN4#117, prioritize discussion on study scope and necessary details aligned with RAN guidance, e.g.

    - Which aspects need to be considered to conclude on feasibility

    - Need for network control

WF agreement in RAN4#116bis

Issue 7-3-1: High level study scope

- Proposal 1: Scope in proposal 1 was compiled by feature lead over multiple company inputs

  - Study the feasibility to reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM, 256QAM, FFS 1024QAM

    - Study the impacts on BS receiver from both RF and demod perspectives.

    - Study the system gain for Tx EVM relaxation

  - The study is performed based on non-AI-based approach at BS receiver under existing and relaxed UE TX EVM requirements

    - Non-linearity model(s) of transmission signals are studied to capture PA non-linearity and other RF impairment

    - The requirements for other gating factors that impact MPR remain unchanged.

      - Use 5G requirements as starting point

  - The study is performed based for FR1 single CC operation.

    - Example bands as n41, n77/n78

    - PC3 with 1Tx, PC2 with both 1Tx and 2Tx

  - Waveform: CP-OFDM

  - Proposal 2: RAN4 to consider NW controlled partial and full IBE relaxation without Tx EVM relaxation for modulation order up to 64QAM, and with Tx EVM relaxation for higher modulation order.

- Proposal 3: 5GA Tx EVM study should be entirely independent from UE RF or system parameter discussion in 6G SI.

  - The existing RF requirements (other than EVM) and RF impairment assumptions shall be used as starting point for 5GA Tx EVM study.

- Recommended WF

  - TBA

Issue 7-3-2: Network control

- Proposals

  - Proposal 1: Study the potential impacts on the other WGs if network control is needed. Study NBC issues of BS receiver for 5GA if network control is not needed.

  - Proposal 2: Network control would be needed in case 3GPP agrees to introduce UE Tx EVM feature

- Recommended WF

  - TBA

Issue 7-3-3: RF modelling

Key observations:

- Having a memoryless PA model gives unrealistic advantage for Digital post distortion evaluation

- Transmitter non-linearity is impacted not only by PA but also by other factors.

- Proposals

  - Proposal 1: RAN4 need to study how to validate that a stable performance gain can be achieved across all different PA models, which should be one of aspect for feasibility conclusion.

  - Proposal 2: Study following factors which impact transmitter performance

    - Tx DFE

      - Crest Factor Reduction (CFR)

      - Digital Pre-Distortion (DPD)

      - Others (thermal noise, quantization noise etc.)

    - Tx AFE

      - IQ imbalance

      - Phase noise

      - LO leakage

      - PA non-linearity (architecture, mode of operation, power class, technology, frequency range, bandwidth : memory effects, calibration point etc.)

      - Filter characteristics (amplitude flatness, group delay etc.)

      - Others (thermal noise, CIM3/5, etc.)

  - Proposal 3: For simulation assumption alignment purpose, adopt the Tx non-linearity model that had been agreed for UL 256QAM MPR/AMPR evaluation as starting point.

    - The total EVM for 256QAM is contributed by four components, i.e. 1.85% from PA, 1.19% from Transmitter, 1.78% from Phase Noise and 2.06% from I/Q imbalance.

  - Proposal 4: For simulation assumption alignment purpose, the polynomial model in R4-164542 can be used as a starting point. And RAN4 not to limit the PA model used for evaluation on Tx EVM relaxation, but all PA models shall follow the same calibration conditions.

    - DFT-s-OFDM waveform and QPSK modulation in 20MHz 100RB

    - 30 dBc ACLR (for PC3) or 31 dBc ACLR (for PC2), -28dBc IQ image, -28dBc carrier leakage and 60dBc CIM3

    - 1dB MPR

- Proposal 5: For evaluation purpose, adopt generalized memory-based polynomial (GMP) model in TR 38.803

- Recommended WF

  - TBA

Issue 7-3-4: Aspects to consider to conclude on feasibility

- Proposals

  - Proposal 1: Large enough net gain is achieved with large enough set of realistic UE Tx models

- Recommended WF

  - TBA

# Topic #8: Other

## Companies’ contributions summary

| T-doc number Company | Proposals / Observations |
| --- | --- |
| R4-2520093 CATT | RAN4 study the following metrics at least for UE related energy efficiency and model for UE related energy efficiency between energy consumption and Throughput. 1) Energy Efficiency (EE, unit bit/J) = Throughput (bps) / Power (Watt) or = d Throughput(Power)d (Power) 2) Bits Transmitted per Watt (bit/W) = Energy Efficiency (EE, unit bit/J) × time (s) or = EEtdt 3) Energy Efficiency Ratio: e.g. Tx Energy Efficiency Ratio (EER, unit %) = Tx Output Power (PA/antenna connector/OTA, unit Watt) / Total Input Power for Tx chain (unit Watt) 4) Standby Power Consumption VS Operating Power Consumption, but it’s targeted to achieve zero vision, i.e. Zero Bit Zero Watt. |
| Skyworks R4-2520292 R4-2520545 | Proposal on UE digital beamforming for multiple antennas: Investigate higher phase step granularity for single stream MIMO to improve radiation pattern directivity for above two discrete UL/DL antennas. |
| Samsung R4-2520358 | Proposal #2.1-1: Consider a TN-centric 6G baseline, with NTN supported only through minimal, scalable extensions. The focus should remain on preserving TN performance and simplicity, while limiting NTN features to essential aspects such as time/frequency compensation. Proposal #2.9-1: The introduction of such IEs, designed solely to indicate minor requirement differences without impacting network scheduling, should be avoided. FL: TxD and dualPA mentioned in the text |
| Nokia R4-2520554 | Observation 28: RAN4 should not define requirements that cannot be verified Observation 29: Detailed testing and verification discussions is done in testing/ota agenda. |
| Ericsson R4-2521135 | Observation 2.1. In FR1 conducted TS 38.101-1, “for UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s)”. It is worth discussing if the conducted requirement could be related to the OTA requirement by at least assuming a reasonable antenna efficiency, e.g. a typical FR1 antenna efficiency is -5.5 dB. Proposal 2.1: discuss a more realistic assumption on antenna efficiency for FR1 band tests, e.g. -5.5 dB. UE time mask Observation 3.2: There exist various configuration and inter RAT co-existence dependencies which motivate keeping N_(TA offset )the same as in NR FR1 and FR2, like MRSS. Observation 3.3: In Ericsson’s RRM contribution we propose to keep TDD Cell Phase Synchronization requirement the same as in NR. Proposal 3.5: Transmitter transient period can be the same in NR for FR1 and FR2. |
| ZTE R4-2521281 | Proposal 1: All of the related Tx/Rx requirements for the new wider channel bandwidths should be studied based on practical RF parameters/PA model assumption. Observation 5: Besides for increase of #. Tx in a UE, UL coherent transmission and corresponding high-resolution UL codebook become critical for boosting up UL transmission, especially for cell-edge UE (50~100% UPT gain). Proposal 6: Enable Coherent UL MIMO feature for 6GR. Proposal 7: To meet the relative phase and power requirement for coherent UL MIMO, UL gap for coherence calibration could be further studied in 6GR if necessary. |
| Oppo R4-2521450 | General Observation 1: RAN1 has decided to continue use OFDM in 6GR. From RAN4 perspective, most of the 5G NR UE RF requirement frameworks can be reused for 6GR. Below gives the initial view on the RF requirements for 6GR where some requirements may depend on RAN1/2 discussion while some requirements can reuse the 5G NR framework especially for the legacy bands. RAN4 discussion can start from the requirements which have less RAN1/2 dependants. Proposal 1: Consider take below table 1 as starting point for 6G UE RF discussion. Table 1 Initial view on UE RF requirements for 6GR Modulation Observation 18: Besides of the uniform modulations already defined in 5G, some enhancements including even higher modulation, and non-uniform modulations are under discussion in RAN1 which will have impact to RAN4 EVM requirements. Proposal 18: RAN4 needs to closely follow the RAN1 discussion on even higher modulations in UL like 1024QAM with AI/non-AI based schemes, and the non-uniform modulations like probabilistic shaping or geometric shaping. And study the impacts in terms of implementation complexity, requirements and performance. UL-MIMO Observation 28: Coherent UL MIMO was defined in 5G, but no be supported in UE/network. Proposal 26: Study how to enable coherent UL MIMO to improve UE performance. Capabilities Observation 34: In the testing/certification, UE declaration approach can be applied to make it clear which requirements should be tested. There is no need to define UE capabilities just for the purpose of differentiating the applicable requirements for a UE. Proposal 33: In 6G, the testing purpose singling should be avoided and when new signaling is introduced it shall be justified that it is useful for BS scheduling. Proposal 34: In 6G, avoid defining UE capabilities with small granularity when it is unnecessary. FR2 Proposal 35: In 6G, reuse 5G FR2 requirements as much as possible and focus on 1CC in the beginning. |
| Sony R4-2522049 | Device Types Observation 4: a true SAW-less HD-FDD designed massive IoT device can enable a true single stock-keeping unit (SKU) design for global operations, with improved efficiency in Tx/Rx and a reduced chip footprint/module size. Observation 5: The impact to enable a true SAW-less design is very limited and will not jeopardize the overall 6GR design. Observation 6: with a limited uplink bandwidth or RB length, e.g., 3MHz, the 6G massive IoT devices can be implemented as SAW-less natively, while power backoff or restriction in UL transmission may be needed if larger uplink bandwidth needs to be supported (see Appendix A for detail discussion). Proposal 4: Depending on the system parameters choice, RAN4 shall study the most proper approach to address the design of true SAW-less. Testability Proposal 12: For the core requirement defined in 6GR, RAN4 shall make it clear from the beginning that the core requirement itself is not coupled to certain test mode but rather shall be met under actual UE behavior in the field. |

## Open issues summary

### Sub-topic 8-1 MIMO

MIMO is expected to be discussed Q2/2026 based on Chairman’s guidance in R4-2511652. Still, proposals are gathered here for better visibility

Issue 8-1-1: MIMO

- Proposals

  - Proposal 1: Enable Coherent UL MIMO feature for 6GR.Proposal 2: Study on the necessity of ULFPTx capability in 6G.

  - Proposal 2: To meet the relative phase and power requirement for coherent UL MIMO, UL gap for coherence calibration could be further studied in 6GR if necessary.

- Recommended WF

  - Postpone discussion on MIMO operation

### Sub-topic 8-2 Massive IoT

Issue 8-2-1: Massive IoT

- Proposals

  - Proposal 1: 6G massive IoT design shall target a truly SAW-less design as baseline.

  - Proposal 2: Depending on the system parameters choice, RAN4 shall study the most proper approach to address the design of true SAW-less.

  - Proposal 3: Discuss filter assumptions for massive IoT

- Recommended WF

  - Devices types are discussed in system parameters

### Sub-topic 8-3 Energy efficiency

Issue 11-3-1: RAN4 UE RF work scope on energy efficiency

- Proposals

  - Proposal 1: Study the feasibility of introducing performance metric to quantify the energy saving gain in 6G. Consider e.g.

    - 1) Energy Efficiency (bit/J)

    - 2) Bits Transmitted per Watt (bit/W)

    - 3) Energy Efficiency Ratio (%) output power/input power

    - 4) Standby Power Consumption VS Operating Power Consumption

- Recommended WF

Postpone discussion, encourage other companies to contribute on energy efficiency and consider how to minimize overlap with RAN1 energy efficiency work.

### Sub-topic 8-4 Testability

- Proposals

  - Proposal 1: For the core requirement defined in 6GR, RAN4 shall make it clear from the beginning that the core requirement itself is not coupled to certain test mode but rather shall be met under actual UE behavior in the field.

  - Proposal 2: In 6G, the testing purpose singling should be avoided and when new signaling is introduced it shall be justified that it is useful for BS scheduling.

- Recommended WF

  - TBD

### Sub-topic 8-5 FR2

- Proposals

  - Proposal 1: For FR2, different power classes for handheld UE to address identified issue(s) together with scenarios such as hot spot in 5G should be considered.

  - Proposal 2: In 6G, reuse 5G FR2 requirements as much as possible and focus on 1CC in the beginning.

- Recommended WF

Postpone discussion

### Sub-topic 8-6 UE time mask

- Proposals

  - Proposal 1: Transmitter transient period can be the same in NR for FR1 and FR2.

- Recommended WF

Postpone discussion

### Sub-topic 8-7 TN and NTN integration

- Proposals

  - Proposal 1: Consider a TN-centric 6G baseline, with NTN supported only through minimal, scalable extensions. The focus should remain on preserving TN performance and simplicity, while limiting NTN features to essential aspects such as time/frequency compensation.

- Recommended WF

  - TBD