# 3GPP TSG-RAN WG4 Meeting #117	R4-2522630

# Dallas, USA, Nov 17 – 21, 2025

Agenda item:			6.3.1

Source:	Moderator (Apple)

Title:	WF on NR_LBCA_Sw_RRM

Document for:	Approval

# Topic #1: Core part for R19 LB CA via switching (4.8.2)

### Issue 1-1: SDL SCell activation/deactivation in LB CA

Issue 1-1-1: interruption or scheduling restriction during SDL SCell activation/deactivation

- Proposal 1 (HW):

  - Proposal 1: Not to define interruption requirements during SCell activation in LB-CA. Make clarification in spec: The current interruption requirements in SCell activation procedure are not applicable for LB-CA scenario.

- Recommended WF

  - FFS:

    - Not to define interruption requirements during SCell activation in LB-CA. Make clarification in spec: The current interruption requirements in SCell activation procedure are not applicable for LB-CA scenario.

Issue 1-1-2: switching pattern based carrier switching in SCell activation

Agreement:

For MAC CE based SCell activation procedure,

If the n+Tharq+6 is the slot of SCell on-duration as per switching pattern, UE shall switch to SDL SCell by end of n+Tharq+5; otherwise UE shall switch to the SDL SCell by end of the slot immediately prior to the 1st slot in the SCell on-duration based on the switching pattern.

For RRC based direct SCell activation procedure,

If the n+TRRC_Process +2 is the slot of SCell on-duration as per switching pattern, UE shall switch to SDL SCell by end of n+ TRRC_Process +1; otherwise UE shall switch to the SDL SCell by end of the slot immediately prior to the 1st slot in the SCell on-duration based on the switching pattern.

RAN4 can revisit this agreement if RAN1 spec has any conflicting with the above agreement. But if no any conclusion was made in RAN1 in Q1 2026, RAN4 will draft the CR based on the above agreement in Q2 2026 and the CR wording can be further discussed.

Issue 1-1-3a: Application time of disabling switching pattern in timer based SCell deactivation

FFS:

  - for timer based SDL SCell deactivation:

    - The switching pattern is NOT applied after UE completed the SCell deactivation (n+(3ms+ TLBCA)/slot-length).

      - TLBCA =1ms

  - FFS: if TLBCA = 2ms can be assumed considering the TA in N2.

Issue 1-1-3b: switching pattern based carrier switching in timer based SCell deactivation

FFS:

  - If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation,

    - Option 1: UE switches to PCell by end of the n+4.

    - Option 2: UE’s ready to start switching to PCell from the end of n+4.

Issue 1-1-4: Impact of C-DRX on LBCA measurement

- Option 1 (Nokia):

  - Switching pattern is not applied when UE is in DRX.

    - a.	Remove the impact of switching pattern for Kp factor for measurements during DRX.

  - If above bullet is agreeable, inform RAN1 about the agreement in an LS

- Option 2 (HW):

  - Prefer not to introduce the optimization of removing Kp factor for measurement during DRX.

Agreement:

This issue is closed.

Issue 1-1-5: Delay requirement in SDL SCell activation

- Proposal 1 (Ericsson):

  - RAN4 to agree on the following text in the SCell activation delay requirement for the  cases of none of the SSB overlapped with the pattern or some of the SSb/SMTC occasions are overlapped with the pattern:

    - For the UE supporting LowBandCA-via-Switching-r19 and configured with LBCA-SwitchingPattern in FR1, the reference time for evaluating Tactivation_time is from the switching pattern activation time. Based on the content of Tactivation_time, the following applies:

      - if it contains only TFirstSSB_MAX or TFirstSSB or TFirstSSB_MAX, enhanced, it corresponds to the time to the end of the first complete SSB burst that does not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern.

      - if it contains TFirstSSB_MAX + TSMTC_MAX, it represents the shortest combined time at which both a valid SMTC occasion and a complete SSB burst are available, without overlapping the slots indicated by '0' in the LBCA-SwitchingPattern. The order in which the first non-overlapping SMTC occasion and the first non-overlapping complete SSB burst occur is not constrained.

      - if it contains TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced, it refers to the time to the end of two complete SSB bursts, based on the SSB periodicity of the activated SDL SCell, where both bursts do not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern.

- Proposal 2 (ZTE):

  - The partial availability of L3/L1 RS occasions during the SCell activation procedure should be considered. It could be captured into the spec with the clarification on the available RS occasions based on the switching pattern.

- Recommended WF

  - FFS: Discuss P1/P2 or check if P1/P2 can be discussed directly in the CR.

### Issue 1-2: BFD/CBD/RLM requirement in LB-CA

Issue 1-2-1: Switching pattern application after BF(beam failure) or LF(link failure)

- Proposal 1 (Ofinno):

  - The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering radio link failure (RLF) (i.e., upon starting T310). (Ofinno, vivo)

  - The switching pattern (i.e., switchingPattern-r19) is activated upon stopping RLF timer (i.e., upon stopping T310). (Ofinno, vivo)

  - The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering beam failure detection (BFD).

  - The switching pattern (i.e., switchingPattern-r19) is activated upon detecting a new beam on the PCell during the beam failure recovery procedure. Exact timeline is FFS.

- Proposal 2 (LGE):

  - RAN4 to consider not to apply switching pattern, and stay on FDD carrier until link recovery is completed after LRR (link recovery request) is provided for SDL SCell

  - RAN4 to consider not to apply switching pattern, and stay on FDD carrier until radio link recovery is completed after all resources configured as RLM-RS falls below the threshold Qout

  - RAN4 to consider not to apply switching pattern, and stay on FDD carrier until link recovery is completed after PRACH is provided for FDD PCell

- Proposal 3 (vivo):

  - There is no need to enhance CBD measurement requirements for both FDD PCell and SDL SCell.

- Proposal 4 (Ericsson):

  - RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritise to receive RS for for faster link recovery.

- Proposal 5 (Apple, ZTE(PCell)):

  - No optimization for switching pattern applicability is needed related to BFD/CBD/RLM in Rel-19.

Agreement:

Conclude this issue in the next meeting.

Issue 1-2-2: BFR requirements for SDL SCell

- Proposal 1 (LGE):

  - RAN4 to consider BFR requirements for SCell in 8.5.9 as follows:

    - The UE shall be capable of transmit PUCCH with an LRR at the first available PUCCH occasion that is not overlapped with SDL ON duration corresponding to the LB CA switching pattern after T2+D

      - where, T2 is CBD-RS evaluation period, and D is 2ms UE processing time

- Recommended WF

  - FFS: Discuss if P1 is agreeable.

### Issue 1-3: BWP switching requirement for LB CA

Issue 1-3-1: BWP switching requirement for LB CA

- Proposal 1 (Apple, HW, vivo):

  - No requirements are defined for overlapping of BWP switching and carrier switching.

- Proposal 2 (Ofinno):

  - DCI or timer-based BWP switching:

    - Clarify the UE behavior for both timer-based and DCI-based BWP switch by adding the following text in clause 8.6.2:

      - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the timer-based or the DCI-based BWP switch delay requirements refers to the first DL or UL slot, which:

        - is available after the completion of the BWP switching, and

        - is assigned to the same serving cell on which the active BWP switching occurs.

  - RRC based BWP switching:

    - Clarify the UE behavior for the RRC-based BWP switch by adding the following text in clause 8.6.3:

      - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the RRC based BWP switching delay requirements refers to the first DL or UL slot, which:

        - is available after the completion of the BWP switching, and

        - is assigned to the FDD PCell.

- Proposal 3 (LGE):

  - Regarding active BWP switching for FDD PCell and SDL SCell, support the principle as follows:

    - For a UE supporting LB-CA via switching and configured with switch pattern, during the active BWP switching delay and the first DL or UL slot, the UE remains on the same carrier where the active BWP switching occurs

Agreement:

Conclude this issue in the next meeting.

### Issue 1-4: others for LB CA

Issue 1-4-1: CSSF inside gap for LB CA

- Proposal 1 (Apple, HW): the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern.

- Recommended WF

  - FFS: Discuss if P1 is agreeable.

Issue 1-4-2: UE capability for RRC based SDL SCell activation in LB CA

Agreement:

  - RAN4 introduces a new capability that allows UE to signal whether it supports RRC based direct SCell activation in Rel-19 NR LB-CA.

  - Potential details of the capability are shown in following Table .

| Features | Index |  | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55. NR_LBCA_Sw | 55-x |  | RRC based Direct SCell Activation at SCell addition in Rel-19 NR LB-CA via switching | UE supports RRC based direct SCell activation at SCell addition in Rel-19 NR LB-CA via switching | 55-1 | Yes | N/A | UE does not support RRC based direct SCell activation at SCell addition in Rel-19 NR LB-CA via switching | Per band pair per band combination |  | FR1 only | N/A |  | Optional with capability signaling |

  - RAN4 will have one extra TC for direct SCell activation in LB-CA for UE supporting above FG.

    - Nokia will draft this TC.

Issue 1-4-3: UL TCI state activation

- Proposal 1 (Nokia): Introduce UL TCI state switching delay requirements for PCell by taking the LBCA switching pattern impact into account in PL-RS estimation delay.

- Recommended WF

  - FFS: Discuss P1.

# Topic #2: Performance part for R19 LB CA via switching (6.3.2)

### Issue 2-1: General

Issue 2-1-1: TC list

- Whether define TC#4 BFD/CBD for SDL SCell or not?

  - Option 1 (HW): Yes, define TC on SSB based BFD/CBD for SDL SCell, where SSBs are partial overlapping with switching pattern.

  - Option 2 (vivo, MTK): No, no need to introduce test to verify BFD/CBD measurement requirements when switching pattern is configured for both FDD PCell and SDL SCell.

- Whether define TC#6 PL-RS switch delay for PCell or not?

  - Option 1 (HW, vivo, ZTE): Define TC on SSB based PL-RS switch delay for PCell, where SSBs are partial overlapping with switching pattern.

    - The timeline of disabling the switching pattern by MAC CE based SCell deactivation is defined in core part but not any already approved test case to verify it. (ZTE)

    - (ZTE)In TC#6, the following three test motivations could be verified:

      - Before the switching pattern being disable, UE follows the switching pattern to pick the available RS occasions;

      - During the PCell measurement(PL-RS based), verify the timeline of (THARQ+3ms+ 1ms)/slot-length to disable the switching pattern when receiving the SCell deactivation;

      - All RS occasions are available in PCell after the processing time of disabling the switching pattern.

  - Option 2 (MTK): No, no need to define test case to cover PL-RS switch delay for PCell.

- Recommended WF

  - FFS: Discuss following proposals:

    - Whether define TC#4 BFD/CBD for SDL SCell or not?

      - No need to introduce test to verify BFD/CBD measurement requirements when switching pattern is configured for both FDD PCell and SDL SCell in LB CA

    - Whether define TC#6 PL-RS switch delay for PCell or not?

      - Define TC on SSB based PL-RS switch delay for PCell, where SSBs are partial overlapping with switching pattern.

      - FFS: In TC#6, the following three test motivations could be verified:

        - Before the switching pattern being disable, UE follows the switching pattern to pick the available RS occasions;

        - During the PCell measurement(PL-RS based), verify the timeline of (THARQ+3ms+ 1ms)/slot-length to disable the switching pattern when receiving the SCell deactivation;

        - All RS occasions are available in PCell after the processing time of disabling the switching pattern.

### Issue 2-2: TC configuration

Issue 2-2-1: Switching pattern related design

- Proposal 1 (vivo):

  - The switching pattern can be configured as 10 slots for SDL SCell and 30 slots for FDD PCell.

  - Switching gap is 3 symbols from FDD PCell to SDL SCell and 14 symbols from SDL SCell to FDD PCell.

- Proposal 2 (Ericsson):

  - RAN4 to agree following parameters for the switching pattern.

| Parameter | Value |
| --- | --- |
| Switching pattern | 0000001111111110000000011100000111111000 |
| N1 | 35 µs RF switching delay UE, 1 140 µs RF switching delay UE, 3 |
| N2 | 35 µs RF switching delay UE, 2 140 µs RF switching delay UE, 4 |

  - RAN4 to agree on following SSB pattern 14 for the LB CA via switching test cases.

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 10 MHz |
| SSB SCS | 15 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 2 | 2-5 |
| Slot numbers containing SSB Note 2 | 5 |
| SFN containing SSB | SFN mod (max(TSSB,10 ms)/10 ms) = 0 |
| PRB numbers containing SSB within channel BW | (RBJ, PRBJ+1,.…, PRBJ+19)Note 1 |
| NOTE 1: PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

  - RAN4 to agree on following SMTC pattern 13 for the LB CA via switching test cases.

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 20 ms |
| SMTC offset | 5 ms |
| SMTC duration | 1 ms |

- Recommended WF

  - FFS: Discuss the P1/P2

Issue 2-2-2: Other detailed configurations

- Proposal 1 (HW):

  - In L1-RSRP test for UE supporting LB CA via switching, TReport is set to 40ms.

- Proposal 2 (OPPO):

  - Focus on non-DRX case when defining test cases.

  - Not configure gap in the test cases.

- Proposal 3 (Ericsson):

  - For Intra-frequency L3 measurement for activated SDL SCell or PCell test, use event A3 and A6 in the test configuration.

  - Switching gap duration is verified by counting the HARQ NACK percentage. The HARQ NACK percentage observed during the test shall not be lower than the HARQ NACK percentage measured for normal LB CA operation without the switching mechanism. Exact HARQ NACK percentage can be set by RAN5.

- Recommended WF

  - FFS: Discuss P1/2/3, discuss the followings:

    - In L1-RSRP test for UE supporting LB CA via switching, TReport is set to 40ms.

    - Focus on non-DRX case when defining test cases.

    - Not configure gap in the test cases.

    - For Intra-frequency L3 measurement for activated SDL SCell or PCell test, use event A3 and A6 in the test configuration.

    - Switching gap duration is verified by counting the HARQ NACK percentage. The HARQ NACK percentage observed during the test shall not be lower than the HARQ NACK percentage measured for normal LB CA operation without the switching mechanism. Exact HARQ NACK percentage can be set by RAN5.