3GPP TSG-RAN WG4 Meeting #117	R4-2522635

Dallas, USA, 17th November ‒ 21st November, 2025

Title: 	WF on NR_RRM_Ph5_Part2

Agenda Item:	6.10.1

Source: 	Moderator (CATT)

Document for:	Approval

- Introduction

This WF includes the agreements and open issues discussed in the topic summary for [117][207] RRM requirements for NR_RRM_Ph5_Part2.

Topic #1: RRM performance requirements for FR2-1 L3 measurement delay by optimizing Rx BSF

    - Sub-topic 1-1 RRM performance requirements for FR2-1 L3 measurement delay by optimizing Rx BSF

      - Issue 1-1-1: Whether to test FBS quitting

Agreement:

    - Do not test FBS quitting in the test.

      - Issue 1-1-2: Whether to test FBS re-evaluation

Agreement:

    - Do not test FBS re-evaluation.

      - Issue 1-1-3: Details of test cases

Agreement:

    - A short value that can cover the measurement with FBS and the first reporting is used, e.g., 1 s or 0.5 s.

Topic #2: RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

    - Sub-topic 2-1 RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

      - Issue 2-1-1: Applicability rules for test cases

Agreement:

    - Rule 1: for UE supporting eEMR-based Direct SCell activation at SCell addition or eEMR-based PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped.

    - Rule 2: for UE supporting both eEMR-based Direct SCell activation at SCell addition and eEMR-based PUCCH SCell activation, eEMR-based PUCCH SCell activation can be skipped.

    - Rule 3: for UE which can pass R19 test case for fast SCell activation based on R18 early measurement report, corresponding R18 test case with valid report can be skipped.

      - Issue 2-1-2: Report scheme for test cases

Agreement:

    - No need to verify the legacy report mechanism in the test case.

      - Issue 2-1-3: Time length of test cases

Checking test case CRs directly. Issue closed without agreement needed.

References

- R4-2520832, Topic summary for [117][207] NR_RRM_Ph5_Part2, Moderator (CATT), RAN4#117.

- R4-2522634, Ad-hoc minutes for NR_RRM_Ph5, Ericsson, RAN4#117.