3GPP TSG-RAN WG4 Meeting #117											                  R4-2522639

Dallas, USA, 17th ‒ 21st November, 2025

Agenda item:			6.13.1

Source:	Qualcomm

Title:	WF on NR_duplex_evo_RRM

Document for:	Approval

# Introduction

This WF captures all the agreements, open issues and work-split related to RRM core and performance requirements for R19 WI NR_duplex_evo, based on the documents submitted under agenda 4.19.3 and 6.13.3.

# Agreements

# Topic #1: Core Part

### Issue 1-1-1: Applicability of SBFD to NR-DC and CA

- Way forward

  - Discuss the following options for SBFD with NR-DC

    - Option 1: SBFD is supported with NR-DC (Qualcomm, ZTE)

        - Reply to RAN2 LS confirming feasibility of SBFD with NR-DC

    - Option 2: SBFD is not supported with NR-DC (E///, Nokia)

    - Option 3: Wait for the decision in other WGs (OPPO, CATT, E///, Samsung, Nokia)

    - Option 4: RAN4 can provide the analysis on the RRM impact for DC, and leave the decision in other WGs (ZTE, Samsung, QC, Apple)

  - Further discuss whether to send a reply LS to RAN2 based on the above

### Issue 1-1-2: RRM spec change needed

- Agreement

  - In order to support inter-band CA with SBFD operation for a UE supporting simultaneousRxTxInterBandCA, the scheduling restriction of L1 CLI measurement is to be captured in the specification reusing the legacy principle in L1 measurement.

# Topic #2: Performance part

### Issue 2-1-1: Side conditions on time offset

- Agreement

  - Specify the time offset conditions for L1-SRS-RSRP measurement accuracy requirements with cell phase error for 15kHz SCS and 30kHz SCS and without cell phase error for other SCS

  - Note: Test-cases will be defined with only 30 kHz SCS for FR1

### Issue 2-2-1: Measurement accuracy of CSI-RS based L1 measurements for case 3

- Way-forward:

  - Option 1: (CATT, CMCC, CTC, Huawei, Ericsson, Samsung, Nokia - compromise)

    - For UE indicating CSI processing time scaled by 2, existing accuracy requirements based on 48 RB for Case 3 apply.

    - For UE not indicating CSI processing time scaled by 2, accuracy requirements based on 24 RB for Case 3 apply

    - Under the condition of AWGN channel in the accuracy requirement and the test case.

  - Option 2: (Qualcomm, Oppo, MTK)

    - Maintain the agreement from RAN4#114bis and specify CSI-RS-based L1-RSRP and L1-SINR measurement accuracy requirements based on 24 RBs for Case 3, irrespective of CSI processing time indication

  - Option 3:

    - Case 3: X1 < 48 and X2 < 48, (X1 + X2) >= [72], existing accuracy requirements based on 48 RB for Case 3 apply.

    - Under the condition of AWGN channel in the accuracy requirement and the test case.

### Issue 2-2-2: Measurement accuracy for CSI-RS based L1 measurements based on 24 PRBs

- Agreement

  - Discuss the following options for CSI-RS based L1-RSRP measurement accuracy relaxation based on 24 PRBs as compared to 48 PRBs

    - Option 1: 1dB (OPPO)

    - Option 2: 0.5dB (HW, QC – if this can also be agreed for case 3)

    - Option 3: 0.8dB (OPPO)

    - Option 4: 0dB (E///, Nokia)

### Issue 2-3-1: Test configuration for FR2

- Agreement

  - DU: 100 MHz channel BW with 50MHz – 50MHz split

  - DUD: 200 MHz channel BW with 75 MHz – 50 MHz – 75 MHz split

### Issue 2-3-2: SSB configuration

- Agreement

  - Introduce a new SSB configuration for FR1 SBFD test cases, as follows:

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10 ms)/10 ms) = 0 |
| PRB numbers containing SSB within channel BW | (RBJ, PRBJ+1,.…, PRBJ+19)Note 1 |
| NOTE 1: PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. NOTE 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves |  |

  - For FR2, for CBW equal to 100 MHz, the configuration SSB.1 FR2 can be reused

  - For FR2, introduce a new SSB configuration for CBW of 200 MHz

### Issue 2-3-3: UL scheduling pattern

- Agreement

  - Single collision in the test-case and let TE choose the occasion randomly

  - FFS: Whether to also test the collisions for CSI-RS based L1-RSRP measurements as well.

### Issue 2-4-1: Test case for CSI-RS based L1-SINR measurements

- Agreement

  - Further discuss whether or not to define the following test cases for CSI-RS L1-SINR measurements

  - Interested companies can bring draft CRs on the test cases, and whether the CRs can be endorsed is up to the conclusion in the next meeting

| 19-20 | CSI-RS based L1-SINR measurement in SBFD FR1/FR2 | A.6.6.8.x A.7.6.6.x | L1-SINR delay, SSB as CMR and CSI-RS as IMR, distribution in two symbol types | DU, DRX CSI-RS has occasions on both SBFD and non-SBFD symbols with different RSRP level |
| --- | --- | --- | --- | --- |

### Issue 2-4-2: Testing collision with UL

- Agreement

  - Remove TC9-10 and verify “collision with UL” in TC13-14.

# Topic #4: Test-cases and Work-split

### Issue 4-1-1: Test case scenarios

- Agreements

  - Agreed test-cases (details in the work-split table)

    - L1-SRS-RSRP measurement delay and scheduling restrictions

    - L1-CLI-RSSI measurement delay and scheduling restrictions

    - L1-SRS-RSRP measurement accuracy

    - L1-CLI-RSSI measurement accuracy

    - Radio Link Monitoring In-sync Test for PCell configured with CSI-RS-based RLM in SBFD

    - Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in SBFD

    - CSI-RS based L1-RSRP measurement in SBFD

    - CSI-RS based L1-RSRP measurement accuracy in SBFD

  - Further discuss whether to define the following test-cases:

    - CSI-RS L1-SINR measurement (delay and accuracy)

### Issue 4-1-2: Work-split

| No. | Title | Clause | Test purpose | Test setup and procedure | Company |
| --- | --- | --- | --- | --- | --- |
| 1-2 | L1-SRS-RSRP measurement in FR1/FR2 | A.6.6.x.1 A.7.6.x.1 | Delay and scheduling restriction for L1-SRS-RSRP | DU P resource on SBFD symbols; AP report 2 time periods, RSRP changed at beginning of T2. time offset: 17.67 for FR1, 7.67 for FR2 Power level refer to existing L3 CLI test cases UE is continuously scheduled except for the restricted symbols | Qualcomm |
| 3-4 | L1-CLI-RSSI measurement in FR1/FR2 | A.6.6.x.2 A.7.6.x.2 | Delay and scheduling restriction for L1-CLI-RSSI | DUD RSSI resource in 2 DL subbands, i.e. method#1 resource type#2 P resource on SBFD symbols; AP report 2 time periods, RSSI/Noc changed at beginning of T2. Power level refer to existing L3 CLI test cases UE is continuously scheduled except for the restricted symbols | Samsung |
| 5-6 | L1-SRS-RSRP measurement accuracy in FR1/FR2 | A.6.7.x.1 A.7.7.x.1 | Accuracy for L1-SRS-RSRP | DU P resource on SBFD symbols; AP report time offset: 17.67 for FR1, 7.67 for FR2 Power level refer to existing L3 CLI test cases | CATT |
| 7-8 | L1-CLI-RSSI measurement accuracy in FR1/FR2 | A.6.7.x.2 A.7.7.x.2 | Accuracy for L1-CLI-RSSI | DUD RSSI resource in UL subband, i.e. method#3 P resource on SBFD symbols; AP report Power level refer to existing L3 CLI test cases | Huawei |
| 11-12 | Radio Link Monitoring In-sync Test for FR1/FR2 PCell configured with CSI-RS-based RLM in SBFD | A.6.5.1.y A.7.5.1.y | CSI-RS based RLM, collision with UL | DU DRX All CSI-RS on SBFD symbols UL scheduling hit some CSI-RS occasions | CMCC |
| 13-14 | Beam Failure Detection and Link Recovery Test for FR1/FR2 PCell configured with CSI-RS-based BFD and LR in SBFD | A.6.5.5.x A.7.5.5.x | CSI-RS based BFR, puncturing in frequency domain | DUD Non-DRX All CSI-RS on SBFD symbols, CSI-RS BW is same as CBW | LGE |
| 15-16 | CSI-RS based L1-RSRP measurement in SBFD FR1/FR2 | A.6.6.4.x A.7.6.3.x | CSI-RS based L1-RSRP delay, distribution in two symbol types | DU Non-DRX CSI-RS has occasions on both SBFD and non-SBFD symbols with differnet RSRP level | Ericsson |
| 17-18 | CSI-RS based L1-RSRP measurement accuracy in SBFD FR2 | A.7.7.4.x | CSI-RS based L1-RSRP accuracy, puncturing in frequency domain (Case 3) | DUD Non-DRX CSI-RS has occasions on SBFD symbol, CSI-RS BW is 24 on each DL subband plus UL subband | vivo |
| Acc1 | Performance requirements for L1 CLI measurement | 10.1.X | Accuracy and report mapping for L1-SRS-RSRP and L1-CLI-RSSI | N/A | Nokia |
| Acc2 | Performance requirements for CSI-RS based L1-RSRP and L1-SINR measurement | 10.1.19.2 10.1.20.2 10.1.27 10.1.28 | Relaxed accuracy based on 24RB for L1-RSRP and L1-SINR | N/A | Oppo |
| TestConf | Test configuration for SBFD | A.3.x A.3.10 A.3.1.4 | Time and frequency domain configuration for SBFD SSB configuration TDD UL/DL configuration | N/A | ZTE |