3GPP TSG-RAN WG4 Meeting #117                                                                R4-2522649

Dallas, US, 17th - 21st Oct 2025

Agenda item:			6.17.1

Source:	China Telecom

Title:	WF on NR_Mob_Ph4_RRM_Part2

Document for:	Approval

Notes

In this document,

- < Agreement > represents the decisions made in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other proposals can be proposed.

Topic #1: RRM discussion

CLTM

Issue 1-1: Applicability rule for CLTM TCs

< Agreement >

- For the test cases for CLTM, the applicability rules are,

    - For UE supporting “cltm-EarlyTA-Indication-r19” and not supporting “intraFreqL1-MeasConfig-r18”, UE is required to pass the test case:

      - RACH-less intra-frequency PCell L3 triggered CLTM cell switch from FR1 to FR1

    - For UE supporting “intraFreqL1-MeasConfig-r18” and not supporting “cltm-EarlyTA-Indication-r19” & “ltm-InterFreqMeasGap-r18”, UE is required to pass the test case:

      - RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

      - RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2

    - For UE supporting “cltm-EarlyTA-Indication-r19” & “intraFreqL1-MeasConfig-r18”, UE is required to pass the test case:

      - RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

      - RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2.

    - For UE not supporting “cltm-EarlyTA-Indication-r19” and supporting “ltm-InterFreqMeasGap-r18”, UE is required to pass the test case:

      - RACH-based inter-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

Issue 1-2: Early DL sync

< Agreement >

- With and without early DL sync.

    - Depending on UE capability, early TCI state activation is included or not in the same test case.

Issue 1-3: CSI-RS-based triggering CLTM

< Agreement >

- This issue is closed.

Subsequent CLTM

Issue 2-1: Test case list for Subsequent CLTM

< Agreement >

- Additionally, define one test case for subsequent CLTM.

    - #4: RACH-less intra-frequency PCell L3 triggered CLTM cell switch from FR1 to FR1