3GPP TSG-RAN WG4 Meeting # 117	R4-2522709

# Dallas, USA, November 17th – 21st, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for Conditional Intra-CU LTM Test Case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Perf |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of NR_Mob_Ph4 test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of the following CLTM test case: - RACH-less intra-frequency L1-triggered CLTM PCell switch from FR1 to FR1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements are not tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### A.6.3.X.X	RACH-less intra-frequency L1-triggered CLTM PCell switch from FR1 to FR1

##### A.6.3.X.X.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 RACH-less intra-frequency L1-triggered CLTM PCell switch specified in clause 6.3.2 for both with and without early TCI state activation.

##### A.6.3.X.X.2	Test Parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. Supported test configurations are shown in table A.6.3.4.3.2-1. Both cell switch delay and interruption length are tested by using the parameters in tables A.6.3.X.X.2-1 and A.6.3.X.X.2-2.

The test consists of 2 tests, and UE is required to pass one among Test 1, Test 2.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18

No gap patterns are configured in the test case.

The test consists of five successive time periods, with time durations of T1, T2, T3, T4, and T5 respectively.

During T1, for Test 1, 2:

-	A measurement object is configured for the frequency of the Cell 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	T1 ends with UE reporting L3 measurement results of Cell 2 to Cell 1.

During T2, for Test 1, 2:

-	At the start of T2, UE is provided with LTM-Candidate-r18 for Cell 2.

-	In Test 1 and Test 2, joint TCI state configurations as defined in table A.6.3.X.X.2-1 are provided.

-	LTM-Candidate-r18 includes the L1 condition implying cell switch to Cell 2.

-	Event LTM3 is used in the CLTM execution condition as defined in table A.6.3.X.X.2-1.

-	UE is configured with SSB-based L1-RSRP measurements of Cell 2.

During T3, for Test 1:

-	At the start of T3, UE receives candidate cell TCI state activation MAC CE for Cell 2.

-	In Test 1, CandidateTCI-State#1 is activated.

-	T4 ends 50 ms after the candidate cell TCI state activation MAC CE transmission.

-	In Test 2, T3 is skipped.

During T4, for Test 1, 2:

-	At the start of T4, UE receives PDCCH order to trigger PRACH transmission on Cell 2.

-	T4 ends 5 ms after the UE transmits the PRACH to Cell 2.

During T5, for Test 1, 2:

-	At the start of T5, Cell 2 meets the CLTM execution condition.

-	Cell 2 continuously schedules PUSCH for the UE.

-	T5 ends at the reception of PUSCH at Cell 2.

Table A.6.3.X.X.2-1: General test parameters for intra-frequency CLTM cell switch from FR1 to FR1

| Parameter |  |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 | Void | Test 2 | Void |  |
| Initial L3 conditions | Active cell |  |  | Cell 1 |  |  |  |  |
|  | Neighbouring cell |  |  | Cell 2 |  |  |  | Cell 2 is the candidate cell |
| Final L3 condition | Active cell |  |  | Cell 2 |  |  |  |  |
| a3-Offset |  |  | dB | -6 |  |  |  |  |
| hysteresis |  |  | dB | 0 |  |  |  |  |
| timeToTrigger |  |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  |  | 0 |  |  |  | L3 filtering is not used |
| Initial L1 conditions |  | Active cell |  | Cell 1 |  |  |  |  |
|  |  | Neighbouring cell |  | Cell 2 |  |  |  | Cell 2 is the target cell |
| Final L1 condition |  | Active cell |  | Cell 2 |  |  |  |  |
| ltm3-Offset-r19 |  |  | dB | [-6] |  |  |  |  |
| hysteresis-r19 |  |  | dB | 0 |  |  |  |  |
| timeToTrigger-r19 |  |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  |  | 0 |  |  |  | L1 filtering is not used |
| includeBeamMeasurements |  |  |  | True |  |  |  |  |
| DRX |  |  |  | OFF |  |  |  | DRX is not used |
| Access Barring Information |  |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between cells |  |  |  | 2 s |  |  |  | RTD between cells is less than CP |
| deriveSSB-IndexFromCell |  |  |  | Enabled |  |  |  |  |
| EarlyUL-SyncConfig | frequencyInfoUL |  |  | NR RF Channel Number 1 |  |  |  |  |
|  | PRACH configuration |  |  | FR1 PRACH configuration 5 |  |  |  | RACH bandwidth is within active UL BWP of PCell |
|  | bwp-GenericParameters |  |  | ULBWP.0.1 |  |  |  |  |
|  | n-TimingAdvanceOffset |  | Tc | 25600 |  |  |  |  |
| ltm-CSI-SSB-ResourceList |  |  |  | SSB 0 of candidate cell |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  |  | DLorJoint TCI.State.0 |  | DLorJoint TCI.State.1 |  | As specified in clause A.3.16B. In Test 1, CandidateTCI-State#1 is configured for early TCI state activation. CandidateTCI-State#2 is selected by the UE. In Test 2, CandidateTCI-State#1 is selected by the UE. |
|  | CandidateTCI-State#2 |  |  | DLorJoint TCI.State.1 |  | N/A |  |  |
| ltm-ConfigComplete |  |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  |  | s | <3 |  |  |  |  |
| T2 |  |  | s | 0.2 |  |  |  |  |
| T3 |  |  | s | 0.1 |  |  |  |  |
| T4 |  |  | s | 0.2 |  |  |  |  |
| T5 |  |  | s | [0.1] |  |  |  |  |

Table A.6.3.X.X.2-2: Cell specific test parameters for intra-frequency CLTM cell switch from FR1 to FR1

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1~T4 | T5 | T1~T4 | T5 |
| NR RF Channel Number |  |  |  | 1 |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |
| DRX Cycle |  |  | Ms | Not Applicable |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 6 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |
|  |  |  | dB | [8] | [-3.3] | [-Infinity] | [2.36] |
|  |  |  | dB | [8] | [8] | [-Infinity] | [11] |
| SSB_RP | Config 1,2 |  | dBm/SCS | [-90] | [-90] | [-Infinity] | [-75] |
|  | Config 3 |  | dBm/SCS | [-87] | [-87] | [-Infinity] | [-72] |
| IoNote3 | Config 1,2 |  | dBm/ 9.36 MHz | [-61.41] | [-57.06] | [-61.41] | [-57.06] |
|  | Config 3 |  | dBm/ 38.16 MHz | [-55.31] | [-50.96] | [-55.31] | [-50.96] |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.6.3.X.X.3	Test Requirements

The UE shall start to transmit PUSCH to Cell 2 in no later than Tmeasure + TCLTM-RRC-processing + TCLTM-interrupt from the start of T5 and the interruption during T5 shall not exceeed TCLTM-interrupt= TLTM-processing + Tfirst-RS + TRS-proc + TCLTM-IU, where:

-	Tmeasure = [820] ms.

-	TCLTM-RRC-processing = 10 ms if UE does not support ltm-FastProcessingConfig-r18, otherwise TCLTM-RRC-processing =0 ms.

-	TLTM-processing = 10 ms if the UE supports ltm-FastUE-Processing-r18 capability and UE reports 10 ms for FR1-to-FR1 cell switch in the capability.

-	TLTM-processing = 15 ms if the UE supports ltm-FastUE-Processing-r18 capability and UE reports 15 ms for FR1-to-FR1 cell switch in the capability.

-	TLTM-processing = 20 ms if the UE does not support ltm-FastUE-Processing-r18 capability.

-	Tfirst-RS + TRS-proc = 0 ms for Test 1, Tfirst-RS + TRS-proc = 22 ms for Test 2.

-	TCLTM-IU is the uncertainty on transmitting the new uplink transmission on Cell 2.

The rate of correct cell switches observed during repeated tests shall be at least 90 %.

==============End of change==============