3GPP TSG-RAN4 Meeting #117	R4-2522749

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on correction of ATG CA requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - From the RAN4 agreement and related RAN2 signalling design, neighbour cell skipping is supported in following scenarios - Intra-band CA, Antenna array for both PCC and SCC - Inter-band CA, Antenna array for both PCC and SCC, One common Rx beam - Inter-band CA, Antenna array for both PCC and SCC, two simultaneous separate Rx beams - Inter-band CA, Omni-directional antenna for PCC and Antenna array for SCC - However, the Klayer1_measurement and N1 for L3 measurement defined in current spec only applicable for the last two cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Revise the Klayer1_measurement and N1 for L3 measurement to reflect the impact from the following cases: - Intra-band CA, Antenna array for both PCC and SCC - Inter-band CA, Antenna array for both PCC and SCC, One common Rx beam |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The R19 ATG spec will be incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2D.5, 9.10D.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The revision is based on the agreed draft BigCR R4-2514872 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2520975 |  |  |  |  |  |  |  |  |

# ==============First change==============

9.2D.5	Intra-frequency measurements without measurement gaps

9.2D.5.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra-frequency SS block of an already detected cell within Tidentify_intra_without_index.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TSSB_time_index_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2D.5.1-1 or table 9.2D.5.1-2 (CC with deactivated SCell).

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2D.5.1-3 or table 9.2D.5.1-4 (CC with deactivated SCell).

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2D.5.2-1 or table 9.2D.5.2-2 (CC with deactivated SCell).

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in clause 9.1D.5.1 for measurement conducted outside measurement gaps, i.e. when intra-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps,  or according to CSSFwithin_gap,i in clause 9.1D.5.2 for measurement conducted within measurement gaps, i.e. when intra-frequency SMTC is fully overlapping with measurement gaps.

For a UE that supports Pre-MG, an SMTC occasion is only considered to be overlapped by Pre-MG if the Pre-MG is activated.

If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps,

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in clause 9.1.8.3.

Kp = 1 when Navailable = 0.

-	Otherwise, when UE is not configured with or UE does not support concurrent measurement gaps:

When intra-frequency SMTC is fully non overlapping with measurement gaps or NCSG, or intra-frequency SMTC is fully overlapping with MGs or NCSG, Kp=1

When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When intra-frequency SMTC is partially overlapping with NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP. For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.	If the higher layer signaling in TS 38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index

For UE capable of antennaArrayType-r18 on the measured carrier,

Klayer1_measurement=1,

If inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of antennaArrayType-r18 on the other serving carrier and UE support two simultaneous separate Rx beams

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on one serving cell outside measurement gap are not fully overlapped by intra-frequency SMTC occasions of the same serving cell, or

-	if the measured carrier is the SCC with servingcellMO configured, and the network indication neighSCellMeasSkipping is set to ‘enable’ to UE

Otherwise,

If the measured carrier is the SCC with servingcellMO configured, and the network indication neighSCellMeasSkipping is set to ‘enable’ to UE

-    if SMTC occasion for PCC and SCC are not overlapped in the time domain, or

-    if SMTC occasion for PCC and SCC are overlapped in the time domain, and all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency outside measurement gap are not fully overlapped by overlapped SMTC occasions of serving cells

Otherwise,

-     if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency outside measurement gap are not fully overlapped by intra-frequency SMTC occasions

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

Otherwise, Klayer1_measurement=1

Table 9.2D.5.1-1: Time period for PSS/SSS detection, (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600 ms, ceil(5 x Kp x N1Note 3,4 x Klayer1_measurement) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(1.5 x 5 x Kp x N1Note 3,4 x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(5 x Kp x N1 Note 3,4 x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: void. NOTE 3: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance information on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 4: For ATG UE capable of antennaArrayType-r18, for the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and inter-band carrier aggregation within FR1 is configured and UE not capable of antennaArrayType-r18 on the other serving carrier or UE support two simultaneous separate Rx beams, or when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and SMTC occasion for PCC and SCC are not overlapped in the time domain and intra-band carrier aggregation within FR1 is configured or inter-band carrier aggregation within FR1 is configured and UE support one common Rx beam. For other cases,N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

Table 9.2D.5.1-2: Time period for PSS/SSS detection, CC with deactivated SCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp x N1Note 1,2) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(5 x Kp x N1Note 1,2) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp x N1Note 1,2) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance information on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 2: For ATG UE capable of antennaArrayType-r18, for the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and inter-band carrier aggregation within FR1 is configured and UE not capable of antennaArrayType-r18 on the other serving carrier or UE support two simultaneous separate Rx beams, or when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and SMTC occasion for PCC and SCC are not overlapped in the time domain and intra-band carrier aggregation within FR1 is configured or inter-band carrier aggregation within FR1 is configured and UE support one common Rx beam. For other cases, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

Table 9.2D.5.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, ceil(3 x Kp x N1 Note 3,4 x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil (1.5 x 3 x Kp x N1 Note 3,4 x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(3 x Kp x N1 Note 3,4 x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: void NOTE 3: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 4: For ATG UE capable of antennaArrayType-r18, for the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and inter-band carrier aggregation within FR1 is configured and UE not capable of antennaArrayType-r18 on the other serving carrier or UE support two simultaneous separate Rx beams, or when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and SMTC occasion for PCC and SCC are not overlapped in the time domain and intra-band carrier aggregation within FR1 is configured or inter-band carrier aggregation within FR1 is configured and UE support one common Rx beam. For other cases, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

Table 9.2D.5.1-4: Time period for time index detection, CC with deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp x N1Note 1,2) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(3 x Kp x N1Note 1,2) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(3 x Kp x N1Note 1,2) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance information on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 2: For ATG UE capable of antennaArrayType-r18, for the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and inter-band carrier aggregation within FR1 is configured and UE not capable of antennaArrayType-r18 on the other serving carrier or UE support two simultaneous separate Rx beams, or when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and SMTC occasion for PCC and SCC are not overlapped in the time domain and intra-band carrier aggregation within FR1 is configured or inter-band carrier aggregation within FR1 is configured and UE support one common Rx beam. For other cases, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

9.2D.5.2	Measurement period

The measurement period for intra-frequency measurements without gaps is as shown in table 9.2D.5.2-1, 9.2D.5.2-2(CC with deactivated SCell).

If the higher layer signaling in TS 38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for TSSB_measurement_period_intra

For a UE that supports Pre-MG, an SMTC occasion is only considered to be overlapped by Pre-MG if the Pre-MG is activated.

Table 9.2D.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil( 5 x Kp x N1 Note 2,3 x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x 5 x Kp x N1 Note 2,3 x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil( 5 x Kp x N1 Note 2,3 x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 3: For ATG UE capable of antennaArrayType-r18, for the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and inter-band carrier aggregation within FR1 is configured and UE not capable of antennaArrayType-r18 on the other serving carrier or UE support two simultaneous separate Rx beams, or when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and SMTC occasion for PCC and SCC are not overlapped in the time domain and intra-band carrier aggregation within FR1 is configured or inter-band carrier aggregation within FR1 is configured and UE support one common Rx beam. For other cases, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

Table 9.2D.5.2-2: Measurement period for intra-frequency measurements without gaps (CC with deactivated SCell) (FR1)

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp x N1 Note 2,3) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(1.5x 5 x Kp x N1 Note 2,3) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp x N1 Note 2,3) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 3: For ATG UE capable of antennaArrayType-r18, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and inter-band carrier aggregation within FR1 is configured and UE not capable of antennaArrayType-r18 on the other serving carrier or UE support two simultaneous separate Rx beams, or when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and SMTC occasion for PCC and SCC are not overlapped in the time domain and intra-band carrier aggregation within FR1 is configured or inter-band carrier aggregation within FR1 is configured and UE support one common Rx beam. For other cases, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

# ==============Next change==============

9.10D.2.5	Intra-frequency measurements without measurement gaps

If a UE is configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the CSI-RS based measurement shall include PSS/SSS detection time of associatedSSB, the time period used to acquire the SFN information and CSI-RS based measurement period without gap.

-	PSS/SSS detection time of associatedSSB is the intra-frequency TPSS/SSS_sync_intra in clause 9.2D.5.1.

-	The time period used to acquire the SFN information is equal to 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, the time period used to acquire the SFN information is TCSI-RS_SFN_intra as shown in table 9.10D.2.5-3 for FR1.

-	If the associatedSSB, which has been detectable at least for the time period Tidentify_intra_with_index defined in clause 9.2D.5.1, becomes undetectable for a period ≤ 5 seconds and then the associatedSSB becomes detectable again with the same spatial reception parameter provided the timing to that cell has not changed more than  3200/2µ Tc, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3], PSS/SSS detection time and time period used to acquire the SFN information are equal to 0.

The measurement period for CSI-RS based intra-frequency measurements without gaps is as shown in table 9.10D.2.5-1.

Additionally, for a given CSI-RS resource, if the associated SS/PBCH block is configured but not detected by the UE, or if CSI-RS is configured with associated SSB but not QCL-ed to the associated SSB, the UE is not required to monitor the corresponding CSI-RS resource.

Table 9.10D.2.5-1: Measurement period for intra-frequency CSI-RS based measurements without gaps(FR1)

| DRX cycle | T CSI-RS_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil( 5 x Kp_CSI-RS x N1Note 3 x Klayer1_measurement ) x CSI-RS period) x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x 5 x Kp_CSI-RS x N1Note 3 x Klayer1_measurement ) x max(CSI-RS period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil( 5 x Kp_CSI-RS x N1Note 3 x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: The requirements apply assuming CSI-RS configuration with {D=3 with PRBs ≥ 48}. D is frequency domain density for the 1-port CSI-RS for L3 mobility defined in clause 7.4.1 of TS 38.211 [6]. NOTE 2: Kp_CSI-RS is applicable for a UE supporting concurrent gaps NOTE 3: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 4: For ATG UE capable of antennaArrayType-r18, for the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and inter-band carrier aggregation within FR1 is configured and UE not capable of antennaArrayType-r18 on the other serving carrier or UE support two simultaneous separate Rx beams, or when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and SMTC occasion for PCC and SCC are not overlapped in the time domain and intra-band carrier aggregation within FR1 is configured or inter-band carrier aggregation within FR1 is configured and UE support one common Rx beam. For other cases, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in clause 9.1D.5.

For a UE not supporting concurrentMeasGap-r17 or for a UE is supporting concurrentMeasGap-r17 but not configured with concurrent measurement gaps,

-	if the intra-frequency CSI-RS resource does not overlap with any measurement gaps, Kp_CSI-RS=1;

-	if some occasions of the intra-frequency CSI-RS resource is overlapped with measurement gaps, Kp_CSI-RS = 1/(1- (CSI-RS resource period /MGRP)) , where CSI-RS resource period < MGRP, and the MGRP is the periodicity of the measurement gap.

-	Otherwise, when UE supports concurrent measurement gaps, and concurrent measurement gaps are configured, Kp_CSI-RS is the scaling factor for a CSI-RS frequency layer to be measured outside gap which is defined as Kp_CSI-RS = Ntotal / Navailable

For a window W of duration max(CSI-RS period,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG, and starting at the beginning of any gap occasions covering the CSI-RS resources:

Ntotal is the total number of CSI-RS resources within the window, including those overlapped with other MG occasions within the window, and

Navailable is the number of CSI-RS resources that are not overlapped with any other non-dropped MG occasion within the window W, after accounting for MG collisions by applying the selected gap collision rule.

Kp_CSI-RS = 1 when Navailable = 0

For UE supporting antennaArrayType-r18 on the measured carrier,

Klayer1_measurement=1,

If inter-band carrier aggregation within FR1 is configured and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE support two simultaneous separate Rx beams

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on one serving cell outside measurement gap are not fully overlapped by intra-frequency SMTC occasions and CSI-RSs occasions of same serving cell, or

-	if the measured carrier is the SCC with servingcellMO configured, and the network indication neighSCellMeasSkipping is set to ‘enable’ to UE

Otherwise,

If the measured carrier is the SCC with servingcellMO configured, and the network indication neighSCellMeasSkipping is set to ‘enable’ to UE

-    If SMTC occasion and CSI-RSs occasions for PCC and SCC are not overlapped in the time domain, or

-    If SMTC occasion and CSI-RSs occasions for PCC and SCC are overlapped in the time domain, and all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency outside measurement gap are not fully overlapped by overlapped SMTC occasions and CSI-RSs occasions of serving cells

Otherwise

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency outside measurement gap are not fully overlapped by intra-frequency SMTC occasions and CSI-RSs occasions

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

For UE not supporting antennaArrayType-r18 on the measured carrier, Klayer1_measurement=1.

Table 9.10D.2.5-2: Void

Table 9.10D.2.5-3: Time period for SFN acquisition for intra-frequency CSI-RS based measurements without gaps(FR1)

| DRX cycle | TCSI-RS_SFN_intra |
| --- | --- |
| No DRX | max(200 ms, ceil(5 x Kp_CSI-RS x N1Note 2 x Klayer1_measurement ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(2000 ms, ceil (1.5 x 5 x Kp_CSI-RS x N1Note 2 x Klayer1_measurement ) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(5 x Kp_CSI-RS x N1Note 2 x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 3: For ATG UE capable of antennaArrayType-r18, for the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and inter-band carrier aggregation within FR1 is configured and UE not capable of antennaArrayType-r18 on the other serving carrier or UE support two simultaneous separate Rx beams, or when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and SMTC occasion for PCC and SCC are not overlapped in the time domain and intra-band carrier aggregation within FR1 is configured or inter-band carrier aggregation within FR1 is configured and UE support one common Rx beam. For other cases, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

# ==============End of change==============