# 3GPP TSG-RAN WG4 Meeting #117	R4-2522753

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on 38.101-4 for ATG CA demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-10-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel -19, CA demodulation requirement for ATG scenario was introduced. Therefore, CA demodulation requirements shall be captured in the 38.101-4 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce CA demodulation requirements for Rel-19 ATG feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-19 ATG CA requirements will be missing from the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A, 5.2A.2.7, 5.2A.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2521178 |  |  |  |  |  |  |  |  |

< START OF CHANGE >

#### 5.2A.2.7	Minimum requirements for ATG CA

The performance requirements are specified in Table 5.2A.2.7-8 and Table 5.2A.2.7-9 based on the single carrier requirements for different bandwidth specified in Table 5.2A.2.7-5, Table 5.2A.2.7-6, and Table 5.2A.2.7-7, with the addition of test parameters in Table 5.2A.2.7-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2A.2.7-1.

Table 5.2A.2.7-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 2 receive antenna conditions in the ATG scenario defined in B.1.1 with CA | 1-1, 1-2, 2-1, 2-2 |

Table 5.2A.2.7-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| TDD pattern |  |  | FR1.30-1 or FR1.30-8 |
| Propagation condition |  |  | Static propagation condition |
| Antenna configuration |  |  | 2x2 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | As defined in Table 5.2A-2 for FR1.30-1 TDD pattern, Table 5.2A.2.7-3 for FR1.30-8 TDD pattern |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3 for FR1.30-1 TDD pattern, Table 5.2A.2.7-4 for FR1.30-8 TDD pattern |

Table 5.2A.2.7-3: Test parameters for number of HARQ processes for ATG CA requirements

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 13 |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 13 | N/A |
| Note: The test parameters only applicable for FR1.30-8 TDD pattern. |  |  |  |

Table 5.2A.2.7-4: Test parameters for K1 values for ATG CA requirements

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | 2 if mod(i,40) = {1,4,5,9,10,14,15,19,20,24,25,28,29} |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 33 if mod(i,40) = 1 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 10 if mod(i,40) = 29 | N/A |
| Note: The test parameters only applicable for FR1.30-8 TDD pattern. |  |  |  |

Table 5.2A.2.7-5: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-26.1 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | 12.57 |
| 10 | R.PDSCH.1-3.6 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | 12.53 |
| 15 | R.PDSCH.1-26.2 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | 12.53 |
| 20 | R.PDSCH.1-26.3 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | 12.50 |
| 25 | R.PDSCH.1-26.4 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | 12.53 |
| 30 | R.PDSCH.1-27.1 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | 12.57 |
| 35 | R.PDSCH.1-27.2 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | 12.50 |
| 40 | R.PDSCH.1-27.3 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | 12.50 |
| 45 | R.PDSCH.1-27.4 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | 12.53 |
| 50 | R.PDSCH.1-27.5 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | 12.60 |

Table 5.2A.2.7-6: Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-43.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.60 |
| 10 | R.PDSCH.2-43.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.67 |
| 15 | R.PDSCH.2-43.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.73 |
| 20 | R.PDSCH.2-43.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.70 |
| 25 | R.PDSCH.2-43.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.60 |
| 30 | R.PDSCH.2-44.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.73 |
| 40 | R.PDSCH.2-3.6 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.50 |
| 50 | R.PDSCH.2-44.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.70 |
| 60 | R.PDSCH.2-44.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.67 |
| 80 | R.PDSCH.2-44.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.73 |
| 90 | R.PDSCH.2-44.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.57 |
| 100 | R.PDSCH.2-44.6 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.57 |

Table 5.2A.2.7-7: Single carrier performance for TDD (UL-DL pattern FR1.30-8) 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-45.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.60 |
| 10 | R.PDSCH.2-45.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.67 |
| 15 | R.PDSCH.2-45.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.73 |
| 20 | R.PDSCH.2-45.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.70 |
| 25 | R.PDSCH.2-45.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.60 |
| 30 | R.PDSCH.2-46.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.73 |
| 40 | R.PDSCH.2-32.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.50 |
| 50 | R.PDSCH.2-46.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.70 |
| 60 | R.PDSCH.2-46.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.67 |
| 80 | R.PDSCH.2-46.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.73 |
| 90 | R.PDSCH.2-46.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.57 |
| 100 | R.PDSCH.2-46.6 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x2 | 70 | 12.57 |

Table 5.2A.2.7-8: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1-1 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.2.7-5 and Table 5.2A.2.7-6 per CC |
| 1-2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.2.7-6 |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7.8. Note 2: The minimum requirements only applicable for FR1.30-1 TDD pattern. |  |  |

Table 5.2A.2.7-9: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 2-1 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.2.7-5 and Table 5.2A.2.7-7 per CC |
| 2-2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.2.7-7 |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7.8. Note 2: The minimum requirements only applicable for FR1.30-8 TDD pattern. |  |  |

< NEXT OF CHANGE >

5.2A.3.6	Minimum requirements for ATG CA

The performance requirements are specified in Table 5.2A.3.6-6 and Table 5.2A.3.6-7 based on the single carrier requirements for different bandwidth specified in Table 5.2A.3.6-3, 5.2A.3.6-4, and Table 5.2A.3.6-5, with the addition of test parameters in Table 5.2A.3.6-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2A.3.6-1.

Table 5.2A.3.6-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions in the ATG scenario defined in B.1.1 with CA | 1-1, 1-2, 2-1, 2-2 |

Table 5.2A.3.6-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| TDD pattern |  |  | FR1.30-1 or FR1.30-8 |
| Propagation condition |  |  | Static propagation condition |
| Antenna configuration |  |  | 2x4 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | As defined in Table 5.2A-2 for FR1.30-1 TDD pattern, Table 5.2A.2.7-3 for FR1.30-8 TDD pattern |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3 for FR1.30-1 TDD pattern, Table 5.2A.2.7-4 for FR1.30-8 TDD pattern |

Table 5.2A.3.6-3: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-26.1 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | 9.8 |
| 10 | R.PDSCH.1-3.6 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | 9.73 |
| 15 | R.PDSCH.1-26.2 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | 9.73 |
| 20 | R.PDSCH.1-26.3 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | 9.63 |
| 25 | R.PDSCH.1-26.4 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | 9.67 |
| 30 | R.PDSCH.1-27.1 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | 9.73 |
| 35 | R.PDSCH.1-27.2 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | 9.60 |
| 40 | R.PDSCH.1-27.3 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | 9.67 |
| 45 | R.PDSCH.1-27.4 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | 9.83 |
| 50 | R.PDSCH.1-27.5 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | 9.8 |

Table 5.2A.3.6-4 Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-43.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.63 |
| 10 | R.PDSCH.2-43.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.67 |
| 15 | R.PDSCH.2-43.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.70 |
| 20 | R.PDSCH.2-43.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.67 |
| 25 | R.PDSCH.2-43.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.77 |
| 30 | R.PDSCH.2-44.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.70 |
| 40 | R.PDSCH.2-3.6 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.60 |
| 50 | R.PDSCH.2-44.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.67 |
| 60 | R.PDSCH.2-44.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.63 |
| 80 | R.PDSCH.2-44.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.73 |
| 90 | R.PDSCH.2-44.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.63 |
| 100 | R.PDSCH.2-44.6 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.63 |

Table 5.2A.3.6-5 Single carrier performance for TDD (UL-DL pattern FR1.30-8) 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-45.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.63 |
| 10 | R.PDSCH.2-45.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.67 |
| 15 | R.PDSCH.2-45.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.70 |
| 20 | R.PDSCH.2-45.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.67 |
| 25 | R.PDSCH.2-45.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.77 |
| 30 | R.PDSCH.2-46.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.70 |
| 40 | R.PDSCH.2-32.1 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.60 |
| 50 | R.PDSCH.2-46.2 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.67 |
| 60 | R.PDSCH.2-46.3 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.63 |
| 80 | R.PDSCH.2-46.4 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.73 |
| 90 | R.PDSCH.2-46.5 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.63 |
| 100 | R.PDSCH.2-46.6 TDD | 64QAM, 0.65 | AWGN | 500Hz | 2x4 | 70 | 9.63 |

Table 5.2A.3.6-6: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1-1 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.6-3 and Table 5.2A.3.6-4 per CC |
| 1-2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.6-4 |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7.8. Note 2: The minimum requirements only applicable for FR1.30-1 TDD pattern. |  |  |

Table 5.2A.3.6-7: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 2-1 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.6-3 and Table 5.2A.3.6-5 per CC |
| 2-2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.6-5 |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7.8. Note 2: The minimum requirements only applicable for FR1.30-8 TDD pattern. |  |  |

< END OF CHANGE >