3GPP TSG-RAN WG4 Meeting #117	R4-2522947

Dallas, US, November 17th – 21st 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | draft | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.141-1 for introduction of SBFD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce the manufacturer declaration, test configuration, test model, etc., for SBFD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Manufacturer declaration, test signal, test configuration, test model, measurement uncertainty, test tolerance are introduced for SBFD conformance testing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conformance spec is incomplete for SBFD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------Start of change-----------------------------------------------------

## 4.6	Manufacturer declarations

The following BS declarations listed in table 4.6-1, when applicable to the BS under test, are required to be provided by the manufacturer for the conducted requirements testing of the BS type 1-C and BS type 1-H.

For SBFD-capable BS, declarations for both non-SBFD operation and SBFD operation are required to be provided by the manufacturer for the separate conducted requirements testing.

For the BS type 1-H declarations required for the radiated requirements testing, refer to TS 38.141-2 [3].

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.1 | BS requirements set | Declaration of one of the NR base station requirement's set as defined for BS type 1-C, or BS type 1-H. | x | x |
| D.2 | BS class | BS class of the BS, declared as Wide Area BS, Medium Range BS, or Local Area BS. | x | x |
| D.3 | Operating bands and frequency ranges | List of NR operating band(s) supported by single-band connector(s) and/or multi-band connector(s) of the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in. Declarations shall be made per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.4 | Spurious emission category | Declare the BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. | x | x |
| D.5 | Additional operating band unwanted emissions | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.6.4.5.6 apply. (Note 3, Note 6). | x | x |
| D.6 | Co-existence with other systems | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another band are deployed. | x | x |
| D.7 | Co-location with other base stations | The manufacturer shall declare whether the BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another band. | x | x |
| D.8 | Single band connector or multi-band connector | Declaration of the single band or multi-band capability of single band connector(s) or multi-band connector(s), declared for every connector. | x | x |
| D.9 | Contiguous or non-contiguous spectrum operation support | Ability to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier. Declared per single band connector or multi-band connector, per operating band. | x | x |
| D.10 | void | void | x | x |
| D.11 | Maximum Base Station RF Bandwidth | Maximum Base Station RF Bandwidth in the operating band for single-band operation. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. (Note 2) | x | x |
| D.12 | Maximum Base Station RF Bandwidth for multi-band operation | Maximum Base Station RF Bandwidth for multi-band operation. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.13 | Total RF bandwidth (BWtot) | Total RF bandwidth BWtot of transmitter and receiver, declared per the band combinations (D.27). | x | x |
| D.14 | NR supported channel bandwidths and SCS | NR supported SCS and channel bandwidths per supported SCS. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.15 | CA only operation | Declaration of CA-only operation (with equal power spectral density among carriers) but not multiple carriers, declared per operating band per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.16 | Single or multiple carrier | Capable of operating with a single carrier (only) or multiple carriers. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.17 | Maximum number of supported carriers per operating band in single band operation | Maximum number of supported carriers per supported operation band in single band operation. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. (Note 2) | x | x |
| D.18 | Maximum number of supported carriers per operating band in multi-band operation | Maximum number of supported carriers per supported operation band in multi-band operation. (Note 2) | x | x |
| D.19 | Total maximum number of supported carriers in multi-band operation | Maximum number of supported carriers for all supported operating bands in multi-band operation. Declared for all connectors (D.18). | x | x |
| D.20 | Other band combination multi-band restrictions | Declare any other limitations under simultaneous operation in the declared band combinations (D.27) for each multi-band connector which have any impact on the test configuration generation. Declared for every multi-band connector. | x | x |
| D.21 | Rated carrier output power (Prated,c,AC, or Prated,c,TABC) | Conducted rated carrier output power, per single band connector or multi-band connector. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. (Note 1, 2, 7) | x | x |
| D.22 | Rated total output power (Prated,t,AC, or Prated,t,TABC) | Conducted total rated output power. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. For multi-band connectors declared for each supported operating band in each supported band combination. (Note 1, 2, 7) | x | x |
| D.23 | Rated multi-band total output power, Prated,MB,TABC | Conducted multi-band rated total output power. Declared per supported operating band combinations, per multi-band connector. (Note 1, 7) | x | x |
| D.24 | Ncells | Number corresponding to the minimum number of cells that can be transmitted by a BS in a particular operating band with transmission on all TAB connectors supporting the operating band. |  | x |
| D.25 | Maximum supported power difference between carriers | Maximum supported power difference between carriers. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.26 | Maximum supported power difference between carriers is different operating bands | Supported power difference between any two carriers in any two different supported operating bands. Declared per supported operating band combination, per multi-band connector. | x | x |
| D.27 | Operating band combination support | List of operating bands combinations supported by single-band connector(s) and/or multi-band connector(s) of the BS. Declared per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.28 | void | void | x | x |
| D.29 | Intra-system interfering signal declaration list | List of single band connector(s) or multi-band connector(s) for which an intra-system interfering signal level is required to be declared. Declaration is required if the intra-system interfering signal level is larger than the co-location interfering signal level. |  | x |
| D.30 | Intra-system interfering signal level | The interfering signal level in dBm. Declared per supported operating band, per TAB connector for BS type 1-H covered by D.29. |  | x |
| D.31 | TAE groups | Set of declared TAB connector beam forming groups on which the TAE requirements apply. All TAB connectors belong to at least one TAB connector beam forming group (even if it's a TAB connector beam forming group consisting of one connector). The smallest possible number of TAB connector beam forming groups need to be declared such that there is no TAB connector not contained in at least one of the declared TAB connector beam forming groups. Declared per supported operating band. |  | x |
| D.32 | Equivalent connectors | List of antenna connectors of BS type 1-C, or TAB connector of BS type 1-H, which have been declared equivalent. Equivalent connectors imply that the antenna connector of BS type 1-C, or TAB connector of BS type 1-H, are expected to behave in the same way when presented with identical signals under the same operating conditions. All declarations made for the antenna connector of BS type 1-C, or TAB connector of BS type 1-H are identical and the transmitter unit and/or receiver unit driving the antenna connector of BS type 1-C or TAB connector of BS type 1-H are of identical design. | x | x |
| D.33 | TAB connector RX min cell group | Declared as a group of TAB connectors to which RX requirements are applied. This declaration corresponds to group of TAB connectors which are responsible for receiving a cell when the BS type 1-H setting corresponding to the declared minimum number of cells (Ncells) with transmission on all TAB connectors supporting an operating band. |  | x |
| D.34 | TAB connector TX min cell group | Declared group of TAB connectors to which TX requirements are applied. This declaration corresponds to group of TAB connectors which are responsible for transmitting a cell when the BS type 1-H setting corresponding to the declared minimum number of cells (Ncells) with transmission on all TAB connectors supporting an operating band. |  | x |
| D.35 | Connecting network loss range for BS testing with ancillary RF amplifiers | Declaration of the range of connecting network losses (in dB) for BS type 1-C testing with ancillary Tx RF amplifier only, or with Rx RF amplifier only, or with combined Tx/Rx RF amplifiers. (Note 4) | x |  |
| D.36 | Relation between supported maximum RF bandwidth, number of carriers and Rated total output power | If the rated total output power and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters: - The reduced number of supported carriers at the rated total output power; - The reduced total output power at the maximum number of supported carriers. | x | x |
| D.37 | TAB connectors used for performance requirement testing | To reduce test complexity, declaration of a representative (sub)set of TAB connectors to be used for performance requirement test purposes. At least one TAB connector mapped to each demodulation branch is declared. |  | x |
| D.38 | Inter-band CA | Band combinations declared to support inter-band CA (per CA capable multi-band connector(s), as in D.15). Declared for every multi-band connector which support CA. | x | x |
| D.39 | Intra-band contiguous CA | Bands declared to support intra-band contiguous CA (per CA capable single band connector(s) or multi-band connector(s), as in D.15). Declared per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.40 | Intra-band non-contiguous CA | Bands declared to support intra-band non-contiguous CA (per CA capable single band connector(s) or multi-band connector(s), as in D.15). Declared per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.41 | NB-IoT operation | Manufacturer shall declare the support of NB-IoT operation in NR in-band and the number of supported NB-IoT carriers in total and for each supported band, frequency range and channel bandwidth. | x |  |
| D.42 | NB-IoT sub-carrier spacing | If the BS supports NB-IoT operation in NR in-band, manufacturer shall declare if it supports 15 kHz sub-carrier spacing, 3.75 kHz sub-carrier spacing, or both for NPUSCH. | x |  |
| D.43 | NB-IoT power dynamic range | If the BS supports NB-IoT operation in NR in-band, manufacturer shall declare the maximum power dynamic range it could support with a minimum of +6dB or +3dB as specified in clause 6.3.4 of TS 38.104 [2] (Note 5). | x |  |
| D.100 | PUSCH mapping type | Declaration of the supported PUSCH mapping type as specified in TS 38.211 [17], i.e., type A, type B or both. | x | x |
| D.101 | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s), i.e., pos0, pos1 or both. |  |  |
| D.102 | PUCCH format | Declaration of the supported PUCCH format(s) as specified in TS 38.211 [17], i.e., format 0, format 1, format 2, format 3, format 4. | x | x |
| D.103 | PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: 0, A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both. | x | x |
| D.104 | Additional DM-RS for PUCCH format 3 | Declaration of the supported additional DM-RS for PUCCH format 3: without additional DM-RS, with additional DM-RS or both. | x | x |
| D.105 | Additional DM-RS for PUCCH format 4 | Declaration of the supported additional DM-RS for PUCCH format 4: without additional DM-RS, with additional DM-RS or both. | x | x |
| D.106 | PUCCH multi-slot | Declaration of multi-slot PUCCH support. | x | x |
| D.107 | UL CA | For the highest supported SCS, declaration of the carrier combination with the largest aggregated bandwidth. If there is more than one combination, the carrier combination with the largest number of carriers shall be declared. | x | x |
| D.108 | High speed train | Declaration of high speed train scenario support, i.e. HST support or no HST support | x | x |
| D.109 | Maximum speed of high speed train for PUSCH | Declaration of supported maximum speed for high speed train scenario, i.e. 350 km/h or 500 km/h. This declaration is applicable to PUSCH for high speed train and UL timing adjustment only if BS declares to support high speed train in D.108. | x | X |
| D.110 | PRACH format for high speed train | Declaration of supported PRACH format(s) for high speed train scenario, i.e. format 0 restricted set type A, format 0 restricted set type B, format A2, format B4, format C2. This declaration is applicable to PRACH for high speed train only if BS declares to support high speed train in D.108. | x | x |
| D.111 | Interlaced formats | Declaration of support of interlaced PUSCH and PUCCH formats. | x | x |
| D.112 | PRACH format with LRA = 1151 for 15 kHz SCS and LRA = 571 for 30 kHz SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both. | x | x |
| D.113 | CG-UCI | Declaration of support of GC-UCI multiplexed on PUSCH as specified in TS 38.211 [17]. | x | x |
| D.114 | 2-step RA | Declaration of support of 2-step RA type. | x | x |
| D.115 | PUSCH 256QAM | Declaration of PUSCH 256QAM support | x | x |
| D.116 | PUCCH sub-slot based repetition formats | Declaration of PUCCH sub-slot based repetition formats | x | x |
| D.117 | PUSCH TB over Multi-slots | BS support TBoMS over physical consecutive UL slots | x | x |
| D.118 | PUSCH TB over Multi-slots | BS support TBoMS over physical non-consecutive UL slots | x | x |
| D.119 | Supported SCS for TDD PUSCH DM-RS bundling and PUCCH DM-RS bundling | Declaration of supported SCS for TDD PUSCH DM-RS bundling and and PUCCH DM-RS bundling and, i.e. {15kHz, 30kHz, 60kHz 120kHz} | x | x |
| D.120 | Supported FDD PUSCH DM-RS bundling and and PUCCH DM-RS bundling and | Declaration of supporting FDD PUSCH DM-RS bundling and PUCCH DM-RS bundling | x | x |
| D.121 | MCS index table 3 | Declaration of support MCS index table 3 as specified in TS 38.214 [18]. | x | x |
| D.122 | PUSCH repetition type A | Declaration of support PUSCH repetition type A | x | x |
| D.123 | Air-to-ground scenario | Declaration of air-to-ground scenario support, i.e., ATG support or no ATG support | x | x |
| D.124 | PUSCH with enhanced DM-RS | Declaration of support for enhanced-dmrs-Type r18 as specified in TS 38.211 [17]. | x | x |
| D.125 | PRACH format for HAPS scenario | Declaration of supported PRACH format(s) for HAPS scenario, i.e., format 1. | x | x |
| D.126 | Enhanced performance requirements | Declaration of BS supporting enhanced performance requirements. | x | x |
| D.127 | LP-WUS operation | Declaration of support for LP-WUS. | x | x |
| D.128 | LP-WUS power boosting | Declaration of support for power boosting for LP-WUS with a minimum of +3dB which is applicable to CBWs equal to or larger than 50 MHz, different boosting levels may be declared for smaller CBWs. | x | x |
| D.129 | Subband full duplex | Declaration of subband full duplex support, i.e., SBFD support or no SBFD support |  | x |
| D.130 | Operating bands and frequency ranges for SBFD operation of SBFD-capable BS | List of NR operating band(s) supported by single-band connector(s) of the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in for SBFD operation. Declarations shall be made per TAB connector for BS type 1-H. |  | x |
| D.131 | NR supported channel bandwidth and subband configurations for SBFD operation of SBFD-capable BS | List of supported channel bandwidths for SBFD operation. List of supported subband configurations per supported channel bandwidth for SBFD operation, which includes: - The SBFD pattern (e.g., DUD, DU/UD). - The transmission bandwidth of the uplink subband (UL). - The transmission bandwidth of the downlink subbands (DL). - The bandwidth of the guard bands between subbands. - All RF channels that implement analogue - filters. Declare how these configurations correspond to the supported channel bandwidth(s). Declared per supported operating band, per TAB connector for BS type 1-H. |  | x |
| D.132 | Single SBFD carrier operation or multi-carrier operation for SBFD-capable BS | Capable of operating with a single SBFD carrier (only), or multiple carriers, i.e., a SBFD carrier is operated in conjunction with one or more TDD carriers. For multi-carrier operation, declare the association between the SBFD carrier and each TDD carrier, providing configuration details and operational conditions. Declared per supported operating band, per TAB connector for BS type 1-H. |  | x |
| D.133 | Maximum number of supported carriers per operating band in single band SBFD operation of SBFD-capable BS | For multi-carrier operation, maximum number of supported carriers per supported operation band in single band operation. Declared per supported operating band, per TAB connector for BS type 1-H. (Note 2) |  | x |
| D.134 | Rated SBFD carrier output power (PSBFD,rated,c,TABC) for SBFD operation of SBFD-capable BS | For single carrier operation, conducted rated SBFD carrier output power per single band connector for SBFD operation. Declared per supported operating band, per TAB connector for BS type 1-H. (Note 1, 2, 7) |  | x |
| D.135 | Rated carrier output power for SBFD operation of SBFD-capable BS | For multi-carrier operation: - Conducted rated carrier output power (PSBFD,rated,c,TABC) of SBFD carrier per single band connector for SBFD operation. - Conducted rated carrier output power (Prated,c,TABC) of each TDD carrier per single band connector. |  | x |
| D.136 | Rated total output power (PSBFD,rated,t,TABC) for SBFD operation of SBFD-capable BS | For multi-carrier operation, conducted total rated output power for SBFD operation. Declared per supported operating band, per TAB connector for BS type 1-H. (Note 1, 2, 7) |  | x |
| D.137 | Maximum supported power difference between carriers for SBFD operation of SBFD-capable BS | Maximum supported power difference between carriers including SBFD carrier and TDD carrier(s). Declared per supported operating band, per TAB connector for BS type 1-H. |  | x |
| D.138 | Relation between supported maximum RF bandwidth, number of carriers and rated total output power for SBFD operation of SBFD-capable BS | If the rated total output power and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters: - The reduced number of supported carriers at the rated total output power; - The reduced total output power at the maximum number of supported carriers. |  | x |
| NOTE 1: If a BS is capable of 1024QAM DL operation then up to three rated output power declarations may be made. One declaration is applicable when configured for 1024QAM transmissions, a different declaration is applicable when configured 256QAM transmissions and the other declaration is applicable when configured neither for 256QAM nor 1024QAM transmissions. NOTE 2: Parameters for contiguous or non-contiguous spectrum operation in the operating band are assumed to be the same unless they are separately declared. When separately declared, they shall still use the same declaration identifier. NOTE 3: If BS is declared to support Band n20 (D.3), the manufacturer shall declare if the BS may operate in geographical areas allocated to broadcasting (DTT). Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 4: This manufacturer declaration is optional. NOTE 5: This manufacturer may declare two values, one with a minimum of +6dB and the other with a minimum of +3dB. NOTE 6: If BS is declared to support Band n24 (D.3), the manufacturer shall declare if the BS may operate in geographical areas where FCC regulations apply. Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 7: If a BS is capable of 256QAM DL operation but not capable of 1024QAM DL operation then up to two rated output power declarations may be made. One declaration is applicable when configured for 256QAM transmissions, and the other declaration is applicable when not configured for 256QAM transmissions |  |  |  |  |

## 4.7	Test configurations

### 4.7.1	General

The test configurations shall be constructed using the methods defined below, subject to the parameters declared by the manufacturer for the supported RF configurations as listed in clause 4.6. The test configurations to use for conformance testing are defined for each supported RF configuration in clauses 4.8.3 and 4.8.4.

The applicable test models for generation of the carrier transmit test signal are defined in clause 4.9.

NOTE:	If required, carriers are shifted to align with the channel raster.

### 4.7.2	Test signal used to build Test Configurations

The signal's channel bandwidth and subcarrier spacing used to build NR Test Configurations shall be selected according to table 4.7.2-1.

Table 4.7.2-1: Signal to be used to build NR TCs

| Operating Band characteristics |  | FDL_high – FDL_low <100 MHz | FDL_high – FDL_low ≥ 100 MHz |
| --- | --- | --- | --- |
| TC signal | BWchannel | 5 MHz (Note 1, 2) | 20 MHz (Note 1) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |  |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. NOTE 2: For NRTC1, if NB-IoT is not supported, the signal's channel bandwidth shall be the narrowest supported one. |  |  |  |

For configuration-limited SBFD-capable BS during SBFD operation, the signal’s channel bandwidth and subcarrier spacing used to build NR Test Configurations shall be selected according to table 4.7.2-2.

Table 4.7.2-2: Signal to be used for configuration-limited SBFD-capable BS to build NR-SPFD TCs

| SBFD-capable BS type |  | Configuration-limited |
| --- | --- | --- |
| TC signal characteristics | BWchannel | All declared channel bandwidths |
|  | Subcarrier spacing | 30 kHz for FR1 120 kHz for FR2-1 |
|  | Subband configuration | All declared subband configurations |

For configuration-flexible SBFD-capable BS during SBFD operation, the signal’s channel bandwidth and subcarrier spacing used to build NR Test Configurations shall be selected according to table 4.7.2-3.

Table 4.7.2-3: Signal to be used for configuration-flexible SBFD-capable BS to build NR TCs

| SBFD-capable BS type |  |  | Configuration-flexible |  |
| --- | --- | --- | --- | --- |
| TC signal characteristics | BWchannel | All declared channel bandwidths |  |  |
|  | Subcarrier spacing | 30 kHz for FR1 120 kHz for FR2-1 |  |  |
|  | Subband configuration | DUD |  | DU/UD |
|  |  | - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL,1, NRB,SBFD,DL,2. - The configuration with the widest UL subband size, and the largest NRB,SBFD,DL,1. NRB,SBFD,DL,2. |  | - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL. - The configuration with the widest UL subband size, and the largest NRB,SBFD,DL. |

### 4.7.3	NRTC1: Contiguous spectrum operation

------------------------------------------End of change------------------------------------------------------

------------------------------------------Start of change-----------------------------------------------------

#### 4.7.8.2	NRTC6 power allocation

Set the power spectral density of each carrier to the same level so that the sum of the carrier powers equals the rated total output power (Prated,t,AC, or Prated,t,TABC, D.22) according to the manufacturer's declaration in clause 4.6.

### 4.7.9	NRTC-SBFD: SBFD Contiguous spectrum operation

The purpose of test configuration NRTC-SBFD is to verify SBFD-capable BS requirements under contiguous spectrum conditions, for multi-carrier operation.

#### 4.7.9.1 	NRTC-SBFD generation

NRTC-SBFD shall be constructed on a per-band, per-declared-channel-bandwidth and per-subband-configuration basis using the following method.

For SBFD operation of SBFD-capable BS, for each supported operating band, for each declared channel bandwidth, and for each declared subband configuration for configuration-limited SBFD BS/each worst-case scenario of declared subband configurations for configuration-flexible SBFD BS according to clause 4.6 (D.129-D.138) and clause 4.7.2, perform the configuration steps below.

- Declared maximum Base Station RF bandwidth supported for contiguous spectrum operation (D.11) shall be used.

b) Place the SBFD carrier within the declared Base Station RF bandwidth according to the manufacturer declaration of association between the SBFD carrier and each TDD carrier (D.132 and D.133). Configure TDD carriers adjacently, occupying the remaining declared Base Station RF bandwidth. The nominal channel spacing between carriers shall comply with TS 38.104 [2], clause 5.4.1.

The test configuration shall be constructed per band for all carriers (SBFD and non-SBFD) transmitted through the same antenna connector.

#### 4.7.9.2 	NRTC-SBFD power allocation

For single-carrier operation: The rated SBFD carrier output power (PSBFD,rated,c,TABC, D.134) shall be distributed across the declared SBFD DL subband(s) according to the manufacturer’s declaration (D.131) in clause 4.6.

For multi-carrier operation: Set the PSD of the DL subband(s) of the SBFD carrier and TDD carrier(s) to the same level, such that the sum of all carrier powers (including SBFD) equals the declared total rated output power (PSBFD,rated,t,TABC, D.136). For SBFD carrier, the output power over the DL subband(s) shall be aligned with the rated output power declared for SBFD operation (PSBFD,rated,c,TABC, D.134).

Any guardband between SBFD DL/UL subbands shall be left unmodulated (zero power).

## 4.8	Applicability of requirements

### 4.8.1	General

### 4.8.2	Requirement set applicability

In table 4.8.2-1, the requirement applicability for each requirement set is defined. For each requirement, the applicable requirement clause in the specification is identified.

Table 4.8.2-1: Requirement set applicability

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | 1-C | 1-H |
| BS output power | 6.2 | 6.2 |
| Output power dynamics | 6.3 | 6.3 |
| Transmit ON/OFF power | 6.4 | 6.4 |
| Transmitted signal quality | 6.5 | 6.5 |
| Occupied bandwidth | 6.6.2 | 6.6.2 |
| ACLR | 6.6.3.5.3 | 6.6.3.5.4 |
| Operating band unwanted emissions | 6.6.4.5.7 | 6.6.4.5.8 |
| Transmitter spurious emissions | 6.6.5.5.3 | 6.6.5.5.4 |
| Transmitter intermodulation | 6.7.5.1 | 6.7.5.2 |
| Reference sensitivity level | 7.2 | 7.2 |
| Dynamic range | 7.3 | 7.3 |
| In-band selectivity and blocking | 7.4 | 7.4 |
| Out-of-band blocking | 7.5 | 7.5 |
| Receiver spurious emissions | 7.6.5.2 | 7.6.5.3 |
| Receiver intermodulation | 7.7 | 7.7 |
| In-channel selectivity | 7.8 | 7.8 |
| Performance requirements | 8 | 8 |

In table 4.8.2-2, the requirement applicability for each requirement set is defined for SBFD-capable BS. For each requirement, the applicable requirement clause in the specification is identified. Requirements not included in a requirement set is marked not applicable (NA).

Table 12.1.1-1: Requirement set applicability for SBFD-capable BS

| Requirement | Requirement Set for non-SBFD symbols/slots | Requirement Set for SBFD symbols/slots |
| --- | --- | --- |
|  | BS type 1-H | BS type 1-H |
| BS output power | 6.2 | TBD |
| Output power dynamics | 6.3 | TBD |
| Transmit ON/OFF power | 6.4 | TBD |
| Transmitted signal quality | 6.5 | TBD |
| Occupied bandwidth | 6.6.2 | TBD |
| ACLR | 6.6.3.5.4 | TBD |
| Operating band unwanted emissions | 6.6.4.5.8 | TBD |
| Transmitter spurious emissions | 6.6.5.5.4 | TBD |
| Transmitter intermodulation | 6.7.5.2 | TBD |
| Transient period between SBFD and non-SBFD | NA | TBD |
| In-channel adjacent subband leakage | NA | TBD |
| Reference sensitivity level | 7.2 | TBD |
| Dynamic range | 7.3 | TBD |
| In-band selectivity and blocking | 7.4 | TBD |
| Out-of-band blocking | 7.5 | TBD |
| Receiver spurious emissions | 7.6.5.3 | TBD |
| Receiver intermodulation | 7.7 | TBD |
| In-channel selectivity | 7.8 | TBD |
| In-channel adjacent subband blocking | NA | TBD |
| Performance requirements | 8 | TBD |

### 4.8.3	Applicability of test configurations for single-band operation

------------------------------------------End of change------------------------------------------------------

------------------------------------------Start of change-----------------------------------------------------

## 4.9	RF channels and test models

### 4.9.1	RF channels

For the single carrier testing many tests in this TS are performed with appropriate frequencies in the bottom, middle and top channels of the supported frequency range of the BS. These are denoted as RF channels B (bottom), M (middle) and T (top).

Unless otherwise stated, the test shall be performed with a single carrier at each of the RF channels B, M and T.

Many tests in this TS are performed with the maximum Base Station RF Bandwidth located at the bottom, middle and top of the supported frequency range in the operating band. These are denoted as BRFBW (bottom), MRFBW (middle) and TRFBW (top).

Unless otherwise stated, the test shall be performed at BRFBW, MRFBW and TRFBW defined as following:

-	BRFBW: maximum Base Station RF Bandwidth located at the bottom of the supported frequency range in the operating band.

-	MRFBW: maximum Base Station RF Bandwidth located in the middle of the supported frequency range in the operating band.

-	TRFBW: maximum Base Station RF Bandwidth located at the top of the supported frequency range in the operating band.

Occupied bandwidth test in this TS is performed with the aggregated BS channel bandwidth and sub-block bandwidths located at the bottom, middle and top of the supported frequency range in the operating band. These are denoted as BBW Channel CA(bottom), MBW Channel CA (middle) and TBW Channel CA (top) for contiguous spectrum operation.

Unless otherwise stated, the test for contiguous spectrum operation shall be performed at BBW Channel CA, MBW Channel CA and TBW Channel CA defined as following:

-	BBW Channel CA: aggregated BS channel bandwidth located at the bottom of the supported frequency range in each operating band;

-	MBW Channel CA: aggregated BS channel bandwidth located close in the middle of the supported frequency range in each operating band;

-	TBW Channel CA: aggregated BS channel bandwidth located at the top of the supported frequency range in each operating band.

For BS capable of multi-band operation, unless otherwise stated, the test shall be performed at BRFBW_T'RFBW and B'RFBW_TRFBW defined as following:

-	BRFBW_T'RFBW: the Base Station RF Bandwidths located at the bottom of the supported frequency range in the lowest operating band and at the highest possible simultaneous frequency position, within the Maximum Radio Bandwidth, in the highest operating band.

-	B'RFBW_TRFBW: the Base Station RF Bandwidths located at the top of the supported frequency range in the highest operating band and at the lowest possible simultaneous frequency position, within the Maximum Radio Bandwidth, in the lowest operating band.

NOTE:	BRFBW_T'RFBW = B'RFBW_TRFBW = BRFBW_TRFBW when the declared Maximum Radio Bandwidth spans all operating bands. BRFBW_TRFBW means the Base Station RF Bandwidths are located at the bottom of the supported frequency range in the lower operating band and at the top of the supported frequency range in the upper operating band.

When a test is performed by a test laboratory, the position of B, M and T for single carrier, BRFBW, MRFBW and TRFBW for single band operation, BBW Channel CA, MBW Channel CA and TBW Channel CA for contiguous spectrum operation in each supported operating band, the position of BRFBW_T'RFBW and B'RFBW_TRFBW in the supported operating band combinations shall be specified by the laboratory. The laboratory may consult with operators, the manufacturer or other bodies.

For single-carrier SBFD operation:

- For a configuration-limited SBFD-capable BS, all declared RF channels (D.131) shall be tested.

- For a configuration-flexible SBFD-capable BS, all tests shall be performed with a single SBFD carrier at each of the RF channels B, M and T.

For multi-carrier SBFD operation:

- For a configuration-limited SBFD-capable BS, all declared RF channels (D.131) shall be tested, and for each declared RF channel, the SBFD carrier in conjunction with other TDD carriers configured according to manufacturer declaration (D.132, D.133) shall be tested.

- For a configuration-flexible SBFD-capable BS, all tests shall be performed with the maximum Base Station RF Bandwidth located at BRFBW (bottom), MRFBW (middle) and TRFBW (top) of the supported frequency range in the operating band. The placement of the SBFD carrier and other TDD carriers shall follow the manufacturer declaration (D.132, D.133).

### 4.9.2	Test models

------------------------------------------End of change------------------------------------------------------

------------------------------------------Start of change-----------------------------------------------------

## 9	Conducted characteristics for SBFD-capable BS

## 9.1	General

The conducted characteristics specified in clause 9 shall apply to SBFD-capable BS during SBFD operation, for BS type 1-H single-band operation. BS type 1-C, NB-IoT, and multi-band operation are not supported in the context of SBFD operation.

------------------------------------------End of change------------------------------------------------------