3GPP TSG-RAN WG4 Meeting #117	R4-2522990

# Dallas, USA, 17th – 25th

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test case on timing advance with SAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case for UE timing advance requirements shall be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the test case for UE timing advance requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No any test case for timing advance requirements for RedCap UEs in NTN scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) A.20.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

A.20.3.2	Timing advance for satellite access for RedCap UE

A.20.3.2.1	SA FR1 timing advance adjustment accuracy for RedCap UE

A.20.3.2.1.1	Test Purpose and Environment

The test purpose and environment in clause A.14.3.2.1.1 shall apply for RedCap UE.

A.20.3.2.1.2	Test Parameters

The test parameters in clause A.14.3.2.1.2 shall apply for RedCap UE except that:

- Table A.14.3.2.1.2-1 is replaced with A.20.3.2.1.2-1, and,

- Table A.14.3.2.1.2-3 is replaced with A.20.3.2.1.2-3, and,

- Table A.14.3.2.1.2-4 is replaced with A.20.3.2.1.2-4.

- Table A.20.3.2.1.2-1: Timing advance supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

- Table A.20.3.2.1.2-3: Cell specific test parameters for timing advance

| Parameter |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| Duplex mode |  | Config 1,2,3,4 |  | FDD |  |
| Satellite information |  | Config 1,3 |  | SSC.1 |  |
|  |  | Config 2,4 |  | SSC.2 |  |
| BWchannel |  | Config 1,2,3,4 | MHz | 10: NPRB,c = 52 |  |
| BWP BW |  | Config 1,2,3,4 | MHz | 10: NPRB,c = 52 |  |
| DRX Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  | Config 1,2,3,4 |  | SR.1.1 FDD |  |
| RMSI CORESET Reference Channel |  | Config 1,2,3,4 |  | CR.1.1 FDD |  |
| Dedicated CORESET Reference Channel |  | Config 1,2,3,4 |  | CCR.1.1 FDD |  |
| TRS configuration |  | Config 1,2,3,4 |  | TRS.1.1 FDD |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |
| SMTC configuration |  | Config 1,2,3,4 |  | SMTC.1 FR1 |  |
| SSB configuration |  | Config 1,2,3,4 |  | SSB.1 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,3,4 | kHz | 15 kHz |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2,3,4 | kHz | 15 kHz |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15 kHz | -98 |  |
| Note2 | Config 1,2,3,4 |  | dBm/SCS | -98 |  |
|  |  |  | dB | 3 |  |
|  |  |  | dB | 3 |  |
| IoNote3 | Config 1,2,3,4 |  | dBm/ 9.36 MHz | -67.57 |  |
| Propagation condition |  |  | - | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

- Table A.20.3.2.1.2-4: Sounding Reference Symbol Configuration for timing advance

| Field |  | Value | Comment |
| --- | --- | --- | --- |
| c-SRS | Config 1,2,3,4 | 12 | Frequency hopping is disabled |
| b-SRS |  | 0 |  |
| b-hop |  | 0 |  |
| freqDomainPosition |  | 0 | Frequency domain position of SRS |
| freqDomainShift |  | 0 |  |
| groupOrSequenceHopping |  | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset |  | sl5=2 for SCS 15 kHz | Once every 5 slots |
| pathlossReferenceRS |  | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage |  | Codebook | Codebook based UL transmission |
| startPosition |  | 0 | resourceMapping setting. SRS on last symbol of slot, and 1 symbols for SRS without repetition. |
| nrofSymbols |  | n1 |  |
| repetitionFactor |  | n1 |  |
| combOffset-n2 |  | 0 | transmissionComb setting |
| cyclicShift-n2 |  | 0 |  |
| nrofSRS-Ports |  | port1 | Number of antenna ports used for SRS transmission |
| NOTE: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |  |

A.20.3.2.1.3	Test Requirements

Test requirements in clause A.14.3.2.1.3 apply for RedCap UEs.

==============End of change==============