3GPP TSG-RAN WG4 Meeting # 118                                       R4-2600168

Gothenburg, Sweden, 9th – 13th Feb., 2026

Agenda Item: 8.8.2

Source: MediaTek Inc.

Title: Discussion on AI-based non-linearity compensation

Document for: Discussion

- Introduction

RAN4#117 concluded to study AI-based non-linearity compensation, including AI DPD at UE side and AI DPoD at NW side. As guided, in this paper, we would like to provide our views on AI DPD at UE side.

- Discussion on AI DPD at UE side

RAN4#117 reached the following agreement on the evaluation methodology. As highlighted, there are still several points for further discussion. We would like to discussion some of the points at first.

Issue 2-6: AI-DPD in UE – evaluation related proposals Agreement: - Evaluation methodology: - For a given non-linearity models, evaluate the performance with following assumptions to find the target output power (after applying AI-based DPD) to meet RAN4 Tx requirements, including ACLR/SEM/IBE/EVM - FFS how to perform the evaluation on the impact of remaining non-linearity after AI-DPD. - Case 2: No DPoD + non-linearity TX model (AI-based DPD) - Benchmark - Benchmark 1): No DPoD + non-linearity TX model (non-AI DPD) - Additional benchmark: No DPoD + non-linearity TX model (no DPD) - FFS how to proceed the work before non-linearity model is not agreed in other session. - Strive to have aligned model as DPoD evaluation. - FFS how to proceed if there are multiple non-linearity models from 6G system parameter and UE RF sessions. - FFS Tx requirements values - Detailed RF transmission scheme and configuration: - Use 5G NR existing supported scheme and configuration as starting point, and will be finally determined based on the conclusion from 6G system parameter and UE RF sessions

PA model

In 6G System parameters, RAN4 reached the following agreement on PA model. According to the agreement, at least GMP PA model is one of the options. As memory effects become more and more important as the bandwidth increases. One of the key functions of DPD is to counteract memory effects. We think the PA model adopted in AI DPD evaluation should reflect the memory effects and GMP PA model can be used as one option.

PA model - Agreement - Staged development of the PA model used for waveform evaluation. - For RAN1: Provide a model for timely waveform comparison. - Targeted for RAN4#118 meeting - For RAN4: Continue internal development of PA models, if needed, with more realistic considerations for RF requirements evaluation. - Prioritize PA model(s) for 7 GHz, PC2/PC3 for waveform evaluation. - Consider models like the Generalized Memory Polynomial (GMP) - Other options not precluded - Develop PA models covering different frequency ranges, power classes if single PA model is not accurate enough for all evaluation scenarios. - Agree on calibration conditions and applicable requirements for the PA model (e.g., achieved ACLR for a reference waveform at a specific MPR) to ensure fair comparisons.

Proposal 1: PA model adopted in AI DPD evaluation should reflect the memory effects and GMP PA model can be used as an option.

Now it is not sure when RAN4 can reach consensus on the PA model in 6G System parameters. Before that, we think companies can use any proprietary (e.g., obtained by fitting measured PA data) or public PA models (e.g., models in TR38.803) with memory effects. According to the above agreement in 6G System parameters, it is possible there will be several PA models agreed finally. For AI DPD evaluation, generalization is also one of the aspects that should be studied. If several PA models with memory effects are agreed finally in 6G System parameters, those PA models can be used for generalization study.

Proposal 2: Before PA model is agreed in 6G System parameters, companies can choose PA model(s) with memory effects at their own discretion.

Proposal 3: If there are multiple PA models from 6G system parameter, the models with memory effects can be used for generalization study during AI DPD evaluation.

How to perform the evaluation for a given PA model

We think it is important to carry out mutual cross-validation. The issue to discuss is how to perform the evaluation for a given PA model. In our understanding, for a given PA model, the diagram shown in Fig.1 (use CP-OFDM as an example) can be an option to get the dataset to derive non-AI DPD or AI DPD as well as the dataset to check the performance.

Fig. 1

As far as we known, a PA model is usually derived from measurements with input and output data under certain conditions, e.g., at a certain sample rate, for a certain signal bandwidth, at a certain average power, modulation, waveform and so on. For a particular PA, the AM-AM and AM-PM characteristic vary with different BW, power, modulation and waveform. For matching a PA model, it will be useful for cross-validation if companies can provide not only the analytical expression of the PA model but also the related information on getting the measurement data to derive the PA model.

Proposal 4: To facilitate cross-validation, it is better that companies can provide not only the analytical expression of the PA model but also the related information of getting the measurement data to derive the PA model, e.g, sampling rate, BW, average power, modulation, waveform and so on.

Spec impact?

Although it is concluded to study AI DPD in 6G SI, we are still not clear about the potential spec impact. RAN4 only defines minimum requirement, UE can always do better, no matter adopting AI solution or non-AI solution. Without specification, UE can still use AI based DPD. If the intention is only to tighten RAN4 requirement, we prefer not to keep studying. 6G Day-1 should focus on the features that require specification. If there is other spec impact, the potential spec impact should be justified and discussed.

Proposal 5: Potential spec impact of AI DPD should be justified and discussed.

Views on the three potential directions

In R4-2521440, there are three potential directions proposed. If our understanding is correct, it seems the method in other proponents’ contributions (R4-2520711 and R4-25212430) is direction 1 in R4-2521440. For information, we copied the direction 1 here:

Direction 1: AI-based DPD

As shown in Fig.1, up-sampling is usually performed before DPD. Then there are totally K_up_sampling*SCS*FFT size/s inputs to DPD module, where K_up_sampling is the up-sampling ratio. Take 30kHz SCS and 2048 FFT size as an example. The value is K_up_sampling*61.44M/s. If to use AI/ML model to generate the output of DPD in the field, the model should inference K_up_sampling*61.44M times every second. In other words, the required inference delay should be less than 16.3us/ K_up_sampling (1/61.44M/K_up_sampling). For 6G, it is very likely 8192 FFT size will be supported as baseline, which requires even shorter inference delay and DPD module needs to run even frequently. Take R19 AI/ML BM as an example, the AI model inference delay is several ms, which would make AI DPD as direction 1 not workable. So we propose to discuss whether it is workable to use AI/ML model to generate the real-time output of DPD in the field, as AI DPD supposed to execute quite frequently which is different from other use cases, e.g., BM, CSI compression, CSI prediction.

Proposal 6: As DPD is supposed to execute quite frequently, discuss whether it is workable to use AI/ML model to generate the real-time output of DPD in the field considering AI/ML model inference delay, model switch delay, etc.

For direction 2 and 3 as copied below, it seems AI is used as a tool to generate the LUT. Then the complexity of generating the real-time output of DPD depends on the form of LUT. If the form of the LUT is the same as non-AI DPD, then the computational complexity of generating the real-time output of DPD will be the same as non-AI solutions. It is not clear to us which aspect of complexity (training complexity or inference complexity) is being compared between AI DPD with non-AI DPD. It would be helpful if proponent(s) can make it clear.

Direction 2: AI-based LUT DPD update

Direction 3: AI based PA modelling to generate and update LUT

- Summary

In this paper, we provide some views on AI-based non-linearity compensation. We have the following proposals:

Proposal 1: PA model adopted in AI DPD evaluation should reflect the memory effects and GMP PA model can be used as an option.

Proposal 2: Before PA model is agreed in 6G System parameters, companies can choose PA model(s) with memory effects at their own discretion.

Proposal 3: If there are multiple PA models from 6G system parameter, the models with memory effects can be used for generalization study during AI DPD evaluation.

Proposal 4: To facilitate cross-validation, it is better that companies can provide not only the analytical expression of the PA model but also the related information of getting the measurement data to derive the PA model, e.g., sampling rate, BW, average power, modulation, waveform and so on.

Proposal 5: Potential spec impact of AI DPD should be justified and discussed.

Proposal 6: As DPD is supposed to execute quite frequently, discuss whether it is workable to use AI/ML model to generate the real-time output of DPD in the field considering AI/ML model inference delay, model switch delay, etc.