3GPP TSG-RAN WG4 Meeting #118	R4-2600755

Gothenburg, Sweden, Feb 9-13, 2026

Agenda Item:	7.12.2

Source:	Huawei, HiSilicon

Title:	Discussion on general aspects in AI/ML mobility

Document for:	Discussion and Decision

# Introduction

According to new WI on AI/ML for mobility in NR [1], the core part work of interoperability and RRM requirement in RAN4 includes:

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any

In this contribution, we discuss general aspects for AI mobility.

# Discussion on general aspects

- UE prediction behavior in temporal domain A and B

It is observed that in real field deployments, the UE does not always transmit periodic L3 measurement reports. Whether continuous L3 measurement prediction is supported for future scenarios depends on UE implementation. If the UE indicates to NW that the AI model is activated via the applicable functionality reporting mechanism, the predicted L3 measurements will be reported. Measured L3 measurements may also be reported for monitoring purposes. In addition, for the temporal prediction in Case B, since some UEs do not support AI prediction, the NW shall transmit SSBs periodically in the legacy manner.

Proposal 1: For temporal prediction Case A and Case B, how to perform measurement and prediction is up to UE.

- Frequencies used in frequency domain prediction simulation

According to the WF from last meeting, many options for frequency domain prediction simulation are identified.

- Issue 1-2-3: Frequencies used in frequency domain prediction simulation Way forward: - Further discuss the following options to be used as starting point for frequency domain prediction simulation: - Option 1: UE use the measurement on 2GHz to predict 4GHz. (Reuse RAN2 SI evaluation) - Option 2: UE use the measurement on 800MHz to predict 3.5GHz. (Low-band to predict mid-band) - Option 3: UE use the measurement on 1.8GHz to predict 2.1GHz. (Small gap between two frequencies) - Option 4: UE use the measurement on 2.6GHz to predict 4.9GHz. (Real deployment)

Considering that RAN2 has completed evaluations for the 2 GHz and 4 GHz bands, and feasibility has been validated for collocated scenarios, RAN4 may first focus on the 2 GHz and 4 GHz bands to fully leverage RAN2’s evaluation results. For the frequency domain prediction direction, evaluations for the 2 GHz → 4 GHz band combination should be sufficient.

Proposal 2: For frequency prediction, RAN4 focus on the prediction of 4GHz using measurements from 2GHz.

- Generalization aspects for AI mobility

It can be observed from generalization evaluations from RAN2, within a limited set of defined scenarios, an AI model can achieve a certain level of prediction accuracy during inference if it has been exposed to sufficiently rich dataset features in the training phase. If RAN4 plans to consider generalization testing, test metrics can be defined separately for different scenarios. Whether the UE adopts a generalization-compliant model and passes the relevant tests may be left to UE implementation. At current stage, we can first focus on scenario specific test cases.

Proposal 3: Postpone generalization testing before scenario specific testing is completed.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: For temporal prediction Case A and Case B, how to perform measurement and prediction is up to UE.

Proposal 2: For frequency prediction, RAN4 focus on the prediction of 4GHz using measurements from 2GHz.

Proposal 3: Postpone generalization testing before scenario specific testing is completed.

# References

- RP-252899, “Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”, 3GPP RAN #109, 2025.