3GPP TSG-RAN WG4 Meeting #118				                                　 R4-2600886

Gothenburg, Sweden, 9th – 13th Feb 2026

Source: 	Huawei, HiSilicon

Title: 	On system parameters for 6G —— Modulation

Agenda Item:	8.2.2

Document for:	Approval

Introduction

This contribution provides our further consideration on system parameters for 6G on modulation.

Discussion

Modulation

Evaluation cases

Agreement on modulation in RAN4#116bis is excerpted as below.

Agreement - RAN4 evaluation work focus on the feasibility study and RF requirements impact - To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1 - Identify the main affected requirements for modulation evaluations - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions. - Align on the evaluation assumptions. - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation. - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed. - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1. - Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies. - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

There was no agreement on modulation in last meeting, but FL recommendation can be found in the WF [1] as a reference, as it is based on the inputs from companies in last meeting.

Evaluation of modulation could be considered for the following cases:

Case 1: Existing modulations with new Tx assumptions including new PA model

- UL: BPSK, QPSK, 16QAM, 256QAM

- Uniform constellation as NR

- Both existing bands and new spectrum should be considered

Case 2: Existing modulations with new Tx assumptions including new PA model

- UL: BPSK, QPSK, 16QAM, 256QAM

- Potential non-uniform constellation (pending on RAN1 decision)

- Both existing bands and new spectrum should be considered

Case 3: New modulations with new Tx assumptions including new PA model

- UL: 1024QAM

- Uniform constellation as NR

- Potential non-uniform constellation (pending on RAN1 decision)

- Both existing bands and new spectrum should be considered

There could be another case for DL 4096QAM, but there was not much discussion in RAN4 for this aspect in last meeting. Our preference is to defer the discussion unless there is an RAN1 agreement for such modulation.

Proposal 1: Identify clear evaluation cases for modulation in RAN4. The initial evaluation is based on uniform constellation as NR, whether to extend the evaluation to non-uniform constellation and/or 4096QAM pending on RAN1 conclusion.

Evaluation assumptions

For all listed cases in 2.2.1, further study depends on the progress on RF impairments discussed under UE RF as well as the progress on PA model under system parameter. Non-uniform constellation also depends on the progress and decision of RAN1.

Regarding 1024QAM, the following EVM requirements are excerpted from NR BS specification.

TS 38.104 Table 6.5.2.2-1: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |
| 1024QAM | 2.5 %1 2.8 %2 |
| Note1: This requirement is applicable for frequencies equal to or below 4.2 GHz. Note 2: This requirement is applicable for frequencies above 4.2 GHz. |  |

It is noted that BS EVM requirements are defined with different values for different frequency bands. In practice, BS transmitter EVM is primarily attributed to three key factors: local oscillator (LO) phase noise, clipping noise induced by the crest factor reduction (CFR) algorithm, and PA non-linearity. Provided that the individual EVM contribution of each factor can be quantified, the overall EVM can be derived under the assumption that the impairments introduced by each source are uncorrelated.

It is well known that digital pre-distortion (DPD) is widely employed in BS implementations to compensate for PA nonlinearities. In contrast, DPD is not commonly assumed in UE implementations. If linearity must be achieved solely through power back-off, the feasibility of such an approach, considering the trade-off between performance gain and implementation constraints, remains an open question.

Furthermore, as applied in UE maximum power reduction evaluation, the total EVM budget should be appropriately allocated among various transmitter impairment factors, such as PA non-linearity, LO phase noise, and CFR noise. Whether such a budget partitioning is achievable for each impairment to meet EVM targets comparable to those on the BS side requires thorough investigation.

Proposal 2: For the identified evaluation cases, defer the evaluation until concrete conclusions are reached regarding RF impairments and the PA modeling.

Proposal 3: For UL 1024QAM, the EVM budget and implementation feasibility should be assessed and concluded in advance for all relevant Tx impairment factors, including at least PA non-linearity, I/Q imbalance, LO phase noise, and CFR-induced noise.

Regarding the assumptions listed in the FL recommendations as below,

- Scenarios and frequencies: Focus evaluations on agreed scenarios (TBD, like Urban Macro and indoor hotspot), across agreed frequencies (TBD, like ~700 MHz, 2 GHz, and 7 GHz).

- Bandwidth: Consider wider channel bandwidths (TBD, e.g., 200 MHz)

For scenarios, our preference is ~7GHz band.

Evaluation method

As agreed in last RAN4, both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1. This is mainly for higher order modulations.

For such evaluation, e.g., UL 1024QAM, the performance gain should take potential larger power back off into consideration.

Proposal 4: For existing NR modulations with uniform constellations, the evaluation should focus on assessing the MPR reduction under new transmitter impairment assumptions and with the new PA model.

Proposal 5: For existing NR modulations that may adopt non-uniform constellations, the evaluation should focus on determining whether new EVM requirements should be considered. When to start the evaluation pending on RAN1 conclusion.

Proposal 6: For uplink 1024QAM, the evaluation should focus on the implementation feasibility and performance gain, evaluated through both link-level and system-level simulations.

Conclusion

This contribution provides our consideration on system parameters for 6G.

Evaluation cases

Proposal 1: Identify clear evaluation cases for modulation in RAN4. The initial evaluation is based on uniform constellation as NR, whether to extend the evaluation to non-uniform constellation and/or 4096QAM pending on RAN1 final conclusion.

Case 1: Existing modulations with new Tx assumptions including new PA model

- UL: BPSK, QPSK, 16QAM, 256QAM

- Uniform constellation as NR

- Both existing bands and new spectrum should be considered

Case 2: Existing modulations with new Tx assumptions including new PA model

- UL: BPSK, QPSK, 16QAM, 256QAM

- Potential non-uniform constellation (pending on RAN1 decision)

- Both existing bands and new spectrum should be considered

Case 3: New modulations with new Tx assumptions including new PA model

- UL: 1024QAM

- Uniform constellation as NR

- Potential non-uniform constellation (pending on RAN1 decision)

- Both existing bands and new spectrum should be considered

Evaluation assumptions

Proposal 2: For the identified evaluation cases, defer the evaluation until concrete conclusions are reached regarding RF impairments and the PA modeling.

Proposal 3: For UL 1024QAM, the EVM budget and implementation feasibility should be assessed and concluded in advance for all relevant Tx impairment factors, including at least PA non-linearity, I/Q imbalance, LO phase noise, and CFR-induced noise.

Evaluation Method

Proposal 4: For existing NR modulations with uniform constellations, the evaluation should focus on assessing the MPR reduction under new transmitter impairment assumptions and with the new PA model.

Proposal 5: For existing NR modulations that may adopt non-uniform constellations, the evaluation should focus on determining whether new EVM requirements should be considered. When to start the evaluation pending on RAN1 conclusion.

Proposal 6: For uplink 1024QAM, the evaluation should focus on the implementation feasibility and performance gain, evaluated through both link-level and system-level simulations.

- References

[1] R4-2522450, WF on 6GR system parameter