3GPP TSG-RAN WG4 Meeting #43 	Tdoc R4-070797

Kobe, Japan, 11th – 17th May 2007

| CR-Form-v7.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.133 | CR | 0908 |  rev |  |  Current version: | 6.17.0 |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Correction of Test Case for Inter-system Handover from UTRAN FDD to GSM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source:  | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI6 |  |  |  |  |  |  | Date:  |  | 07-05-2007 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-6 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Ph2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) |  |
| Reason for change:  |  | The test requirement specified in the test case is shorter than what is given in the core spec. In 25.133, section 5.4.2.1 the requirement is stated that the UE shall be ready to transmit (as specified in GSM 45.010) within 90 ms in this case. The ready to transmit is defined in 45.010, where the phrase "ready to transmit within x ms" means that the MS shall transmit no later than the first allowed transmission opportunity that occurs after the x ms”. The test requirement does not include the GSM timing uncertainty. Thus in some cases the UE may not be able to transmit in the next GSM frame either. Furthermore, the UE may not be able to transmit even in the next frame e.g. if it is an idle frame or slow associated control channel (SACCH) frame. Therefore total uncertainty of 2 GSM frames is needed. An extra delay of two GSM frames (10 ms) is therefore required in the testcase. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Change test requirement from 90 ms to 100 ms, i.e. 10 ms is added which is approximately a delay corresponding to 2 GSM frames is added. Note: If necessary RAN5 can update this test case in the lower releases (including R99) of the relevant RAN5 specification (TS 34.121). |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The test requirement will be harder than the core specification. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | A.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | 34.121 |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at http://www.3gpp.org/specs/CR.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

## A.5.4	Inter-system Handover from UTRAN FDD to GSM

#### A.5.4.1	Test Purpose and Environment

This test is to verify the requirement for the UTRAN to GSM cell handover delay reported in section 5.4.2.1.

The test parameters are given in Table A.5.0D, A.5.0E and A.5.0F below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event 3C shall be used.. The test consists of three successive time periods, with a time duration of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

The UTRAN shall send a Handover from UTRAN command with activation time "now" with a new active cell, cell 2. In the GSM Handover command contained in that message, the IE starting time shall not be included. The RRC HANDOVER FROM UTRAN COMMAND message shall be sent to the UE. The start of T3 is defined as the end of last TTI containing the HO command.

The requirements are also applicable for a UE not requiring compressed mode, in which case no compressed mode pattern should be sent for the parameters specified in table A5.0D

Table A.5.0D: General test parameters for Correct reporting of GSM neighbours in AWGN propagation condition

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| DCH parameters |  | DL Reference Measurement Channel 12.2 kbps | As specified in TS 25.101 section A.3.1 |
| Power Control |  | On |  |
| Target quality value on DTCH | BLER | 0.001 |  |
| Compressed mode patterns - GSM carrier RSSI measurement - GSM Initial BSIC identification - GSM BSIC re-confirmation |  | DL Compressed mode reference pattern 2 in Set 2 Pattern 2 Pattern 2 | Only applicable for UE requiring compressed mode patterns As specified in table A.22 TS 25.101 section A.5 As specified in section 8.1.2.5.2.1 table 8.7. As specified in section 8.1.2.5.2.2 table 8.8. |
| Active cell |  | Cell 1 |  |
| Inter-RAT measurement quantity |  | GSM Carrier RSSI |  |
| BSIC verification required |  | Required |  |
| Threshold other system | dBm | -80 | Absolute GSM carrier RSSI threshold for event 3B and 3C. |
| Hysteresis | dB | 0 |  |
| Time to Trigger | ms | 0 |  |
| Filter coefficient |  | 0 |  |
| Monitored cell list size |  | 24 FDD neighbours on Channel 1 6 GSM neighbours including ARFCN 1 | Measurement control information is sent before the compressed mode patterns starts. |
| N Identify abort |  | 66 | Taken from table 8.7. |
| T Reconfirm abort |  | 5.5 | Based on table 8.8 and requirement specified in section 10.3.6.33 of TS 25.331. |
| T1 | s | 20 |  |
| T2 | s | 5 |  |
| T3 | s | 1 |  |

- Table A.5.0E: Cell Specific Parameters for Handover UTRAN to GSM cell case (cell 1)

| Parameter | Unit |  | Cell 1 (UTRA) |
| --- | --- | --- | --- |
|  |  |  | T1, T2, T3 |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DCH_Ec/Ior | dB | Note 1 |  |
| OCNS_Ec/Ior | dB | Note 2 |  |
| Ior/Ioc | dB | 0 |  |
| Ioc | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -13 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: The DPCH level is controlled by the power control loop Note 2 : The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior . |  |  |  |

- Table A.5.0F: Cell Specific Parameters for Handover UTRAN to GSM cell case (cell 2)

| Parameter | Unit | Cell 2 (GSM) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2, T3 |
| Absolute RF Channel Number |  | ARFCN 1 |  |
| RXLEV | dBm | -85 | -75 |

#### A.5.4.2	Test Requirements

The UE shall begin to send access bursts on the new DCCH of the target cell less than 100 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The test requirement in this test case is expressed as:

THandover delay = 90 ms (Table 5.2) + Toffset + TUL

Toffset :	Equal to 4.65 ms, GSM timing uncertainty between the time from when the UE is ready to transmit until the start of the next timeslot in GSM 26 multiframe structure

TUL:	Equal to 4.65 ms, the time the UE has to wait in case the next timeslot is an idle frame or a SACCH frame.

This gives a total of 99.3 ms, allow 100 ms in the test case.