3GPP TSG-RAN4 Meeting #43 	Tdoc R4-070805

Kobe, Japan, 7 - 11 May 2007

| CR-Form-v8 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.101 | CR | 0554 |  rev | - |  Current version: | 7.7.0 |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | UE perforamnce requirements for high speed train |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source:  | NTT DoCoMo, Panasonic, Ericsson, Fujitsu, Motorola |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI7 |  |  |  |  |  |  | Date:  |  | 07/05/2007 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Ph2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | UE performance requiement for high speed train is added to release 7. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Requirement for demodulation of DCH and propagation condition for high speed train are added. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | There will be no requirements for high speed train. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 8.5A, B.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | 34.121 |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at http://www.3gpp.org/specs/CR.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

8.5A	Demodulation of DCH in high speed train condition

### 8.5A.1	General

The receiver performance of the DCH in high speed train condition is determined by the BLER values. BLER is measured for the individual data rate specified for the DPCH. DCH is mapped into DPCH.

### 8.5A.2	Minimum requirement

For the parameters specified in Table 8.18A the average downlink  power ratio shall be below the specified value for the BLER shown in Table 8.18B.

Table 8.18A: DCH parameters in high speed train condition

| Parameter | Unit |  | Test 1 |  |
| --- | --- | --- | --- | --- |
| Phase reference |  |  |  | P-CPICH |
|  |  | dB |  | 5 |
|  |  | dBm/3.84 MHz |  | -60 |
| Information Data Rate | kbps |  | 12.2 |  |

Table 8.18B: DCH requirements in high speed train condition

| Test Number |  | BLER |
| --- | --- | --- |
| 1 | -21.8 | 10-2 |

### {Unchanged sections}

## B.2.5	High speed train condition

The high speed train condition for the test of the baseband performance is a non fading propagation channel with one tap. Doppler shift is given by

fst=fdcosθt	(B.2)

where fst is the Doppler shift and fd is the maximum Doppler frequency. The cosine of angle θtis given by

cosθt=Ds/2−vtDmin2+Ds/2−vt2, 0≤t≤Ds/v	(B.3)

where Ds/2 is the initial distance of the train from BS, and Dmin is BS-Railway track distance, both in meters; v is the velocity of the train in m/s, t is time in seconds. The parameters in the equation are shown in table B.3. Accordingly, Doppler shift changes to the following figure B.3.

Table B.3

| Parameter | Value |
| --- | --- |
| Ds | 300 m |
| Dmin | 2 m |
| v | 300 km/h |
| fd | 600 Hz |

- Figure B.3: Doppler shift trajectory