3GPP TSG-RAN4 Meeting #43 	R4-070808

- Kobe, Japan, 7-11 May 2007

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.101 | CR | 0555 |  rev | - |  Current version: | 7.7.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | HS-SCCH-less demodulation reuirements for CPC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | R4 |  |  |  |  |  |  |  |  |  |
| Work item code:  | RANImp-CPC |  |  |  |  |  |  | Date:  |  | 09/05/2007 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Introducing HS-SCCH-less operating mode requirement |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Introduce requirement and reference test configuration for HS-SCCH-less mode. The UE is required to be capable of decoding the HS-PDSCH on the first transmission without the HS-SCCH, and be capable of combining the first transmission and second transmissions. Corresponding throughput requirements are specified. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Unpredictable UE behaviour in HS-SCCH-less operating mode |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 9.5, 9.5.1, A.7.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| Affected: |  | X |  | Test specifications |  |  | 34.121-1 |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

## HS-SCCH-less demodulation of HS-DSCH (Fixed Reference Channel)

The receiver performance of the High Speed Physical Downlink Shared Channel (HS-DSCH) with HS-SCCH-less operation in multi-path fading environment is determined by the information bit throughput R.

The propagation conditions for this subclause are defined in table B.1C.

During the Fixed Reference Channel tests the behaviour of the Node-B emulator in response to the ACK/NACK signalling field of the HS-DPCCH is specified in Table 9.1A.

Performance requirements in this section assume sufficient power allocation to HS-SCCH_1, so that the probability of detection failure, when the HS-SCCH-1 uses the identity of the UE under test, is very low.

### 9.5.1	Requirement QPSK, Fixed Reference Channel (FRC) H-Set 7

The requirements are specified in terms of a minimum information bit throughput R for the DL reference channels H-set 7 specified in Annex A.7.1.7, with the addition of the parameters in Table 9.54A and the downlink physical channel setup according to table C.8.

Using this configuration the throughput shall meet or exceed the minimum requirements specified in table 9.54B. Enhanced performance requirements type 1 specified in Table 9.54C are based on receiver diversity.

- Table 9.54A: Test Parameters for Testing QPSK FRCs H-Set 7

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Phase reference | - |  |  | P-CPICH |
|  | dBm/3.84 MHz |  |  | -60 |
| Redundancy and constellation version coding sequence | - |  |  | {0,3} |
| Maximum number of HARQ transmission | - |  |  | 2 |
| NOTE: The HS-SCCH-1 and HS-PDSCH shall be transmitted continuously with constant power. HS-SCCH-1 shall only use the identity of the UE under test for redundancy version 2 transmissions intended for the UE. |  |  |  |  |

- Table 9.54B: Minimum requirement, Fixed Reference Channel (FRC) H-Set 7

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | (dB) | T-put (kbps) |
| 1 | Case 8 | -6 | 0 | [168] |

- Table 9.54C: Enhanced requirement type 1, Fixed Reference Channel (FRC) H-Set 7

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | HS-PDSCH (dB) | (dB) | T-put (kbps) |
| 1 | Case 8 | -9 | 0 | [193] |

>>> End of changes to this section

## A.7.1	Fixed Reference Channel (FRC)

### A.7.1.7	Fixed Reference Channel Definition H-Set 7

Table A.30A: Fixed Reference Channel H-Set 7

| Parameter | Unit | Value |
| --- | --- | --- |
| Nominal Avg. Inf. Bit Rate | kbps | 302.5 |
| Inter-TTI Distance | TTI’s | 8 |
| Number of HARQ Processes | Processes | 8 |
| Information Bit Payload () | Bits | 605 |
| Number Code Blocks | Blocks | 1 |
| Binary Channel Bits Per TTI | Bits | 960 |
| Coding Rate |  | 0.66 |
| Number of Physical Channel Codes | Codes | 1 |
| Modulation |  | QPSK |
| Note: This FRC is used to verify CPC operation. The HS-DSCH shall be transmitted continuously with constant power but only every 8th TTI shall be allocated to the UE under test. |  |  |

Figure A.22: Coding rate for Fixed Reference Channel H-Set 7 (QPSK)

>>> End of changes to this section