3GPP TSG-RAN WG4 Meeting #42																			R4-070xxx

Kobe, Japan, xth – xth of May, 2007

Source: 	Nokia, [Qualcomm]

Title: 	Requirement scenario for discontinuous UL DPCCH transmission operation

Agenda Item:	x.x

Document for:	Discussion

# Introduction

In RAN4 meeting #42 different proposal requirement scenarios related to the Continuous Packet Connectivity (CPC) were presented. Two proposals were presented for verifying discontinuous UL DPCCH transmission operation [2][3]. As these were very similar it was agreed to combine these proposals. In this contribution we present the merged proposal

# Proposal for requirement scenario for discontinuous UL DPCCH transmission operation

The aim of the described scenario is to verify that UE uses only the proper TPC commands carried on F-DPCH to derive it’s transmit power. During UL transmission gaps when no UL DPCCH is transmitted, NodeB may transmit random TPC commands or no TPC commands at all. Thus in order to ensure proper system behaviour UE should neglect the TPC commands on the F-DPCH slots corresponding to UL DPCCH slots which were not transmitted. The requirement scenario is divided in two phases; initialization phase and actual verification phase. During the initialisation phase the preparatory actions to enable the performance verification are carried out. First actual active test phase tries to verify that UE accounts only the proper TPC commands for setting the transmit power by using different patterns for the UL TPC commands. In Figure 1 example of such set up is given.

Initialization:

-This phase is presented just for information and the actual test functionality should be specified by RAN5.

- The RL link is configured with selected DTX and DRX parameters (cycle lengths and burst lengths and the timing offset). The DTX is disabled for duration of initialization phase via HS-SCCH type2 orders. Parameter values for DTX and DRX are given in the Annex.

- Prior the start of the actual test, UL power control is active. The UE transmit power is adjusted (by the NodeB emulator) to a predefined level. A target level of -15dBm is proposed here, (similarly as in some earlier test configurations) to allow sufficient variation of the UE output power. The actual level of the of the UE output power depend naturally on the performance of the NodeB emulators UL SIR estimator and on the accuracy of the UE power control steps.

- The F-DPCH_Ec/Ior is set high enough level to ensure near error free detection of TPC commands.

Verification:

- During a proper time when UE DL reception is active the DTX is enabled via HS-SCCH type2 order. Note that by setting DTX pattern to be longer than DRX pattern this could be done during the UL transmission gap when reception should be active.

- During this phase the UE transmit power is monitored to verify the correct behaviour.

- On those F-DPCH slots corresponding to the UL DPCCH slots which are transmitted, the TPC commands on F-DPCH as set to maintain the UE transmit power at the target level by transmitting a varying pattern of UP and DOWN. On the F-DPCH slots which correspond to UL transmission gap, NodeB emulator shall transmit ‘UP’ commands.

- If UE follows only the proper TPC commands and neglects those TCP commands which correspond to UL transmission gap the UE transmit power should remain in proximity of the target level. In practise the UE transmit power will vary slightly (due non-zero TPC error rate, accuracy of the UL SIR estimator and power control step) but should stay within the acceptable limits (determined by the requirement) from the target level. If UE erroneously follows the commands that are send during UL transmission gaps it power shall increase up to maximum level.

Figure 1. Example of configuration with DTX cycle of 10 subframes and DRX pattern of 5 subrames

# Requirement scenario

### X.5 UE UL power control operation with discontinuous UL DPCCH transmission operation

#### x.5.1 Power control in uplink

This test verifies that the UE follows only those TPC commands that correspond to the UL DPCCH slots which are transmitted.

#### x.5.1.1	Minimum requirement

Test parameters are specified in Table z1. The parameters for discontinuous UL DPCCH transmission operation are as specified in Table A.zz. Before the start of the tests, the UE transmit power shall be initialised to -15 dBm. An actual UE transmit power may vary from the target level of -15 dBm due to inaccurate UE output power step.

During test  the UE transmit power samples, which are defined as the mean power over one timeslot, shall stay within the range defined in Table z2.

Table z: Parameters for UE UL power control operation with discontinuous UL DPCCH transmission

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Phase reference | - |  | P-CPICH |
| HS-SCCH_1 | dB |  | -10 |
| F-DPCH | dB |  | Note 1 |
| Îor1/Ioc | dB |  | 10 |
|  | dBm/3.84 MHz |  | -60 |
| Power-Control-Algorithm | - |  | Algorithm 1 |
| UL Power Control step size, TPC | dB |  | 1 |
| TPC commands corresponding to the UL DPCCH slots which are transmitted | - |  | {0,1,0,1,0,1 } Note 2 |
| Fixed Reference Channel | - |  | H-Set 7 |
| Propagation condition | - |  | Static |
| Note 1: The F-DPCH Ec/Ior is set at a level which ensures very low TPC error rate. Note 2: The sequence of TPC commands corresponds to the UL DPCCH slots that are transmitted. During those slots which correspond to UL DPCCH slots that are not transmitted, the UP-commands shall be transmitted. |  |  |  |

Table z2: Test requirements for UE UL power control operation with discontinuous UL DPCCH transmission

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| UE output power | dBm | -15 ± [3] dB |

# Conclusion

In this contribution we presented a proposal for a requirement scenario aiming to verify the UL PC behaviour with discontinuous UL DPCCH transmission. The proposed test aims to verify that UE follows only the proper DL TPC commands corresponding to UL DPCCH slots which are transmitted.

# References

- R4-070315, Summary of CPC offline discussions, Nokia

- R4-070134, Requirement scenario for discontinuous UL DPCCH transmission operation, Discussion, Nokia

- R4-070214, CPC UL PC Test, Approval, Qualcomm Europe

# Annex A. Some parameters for discontinuous UL DPCCH transmission operation verification

In the Table 1 parameters for DTX and DRX operation are given. Values are selected so that they can be used for both, 2ms and 10m E-DCH TTI’s.

It is desirable to have long enough UE_DTX_cycle so that UE’s not disregarding the TPC on the F-DPCH slots corresponding to UL DPCCH slots which were not transmitted can be more easily identified. Thus have sufficiently long DTX patterns with corresponding periods when UP-commands are transmitted on the F-DCPH. Shorter patterns could also be used but then it could be required to identify smaller the changes in UL transmission power. It also could be beneficial to select the length of the DPCCH burst so that even number of TPC commands could be transmitted, minimizing the UL transmit power variation. By selecting the values for the DRX cycle so that that the DTX cycle (gap) is longer than DRX cycle, it would allow functional verification of DRX operation.

Table 1. Discontinuous uplink DPCCH transmission and downlink reception parameters

| Parameter |  | Value set |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2ms |  | 10ms |  |  |  |
| UE_DTX_cycle_1 | 10 subframes |  | 10 subframes |  |  |  |  |
| UE_DTX_cycle_2 | - |  | - |  | Set to same as cycle_1 |  |  |
| UE_DRX_DRX_offset | 0 |  | 0 |  |  |  |  |
| Inactivity_threshold_for_UE_DTX_cycle2 | 0 |  | 0 |  | To enable the fast start of test. |  |  |
| UE_DPCCH_burst_1 | 1 subframe |  | 1 subframe |  | Set so that paired number of slots will be transmitted |  |  |
| UE_DPCCH_burst_2 | - |  | - |  | Set to same as burst_1 |  |  |
| UE_DTX_long_preamble | False |  | False |  | True/false |  |  |
| UE_Inactivity_Threshold | 0 subframes |  | 0 subframes |  |  |  |  |
| UE_DRX_cycle | 5 subframes |  | 5 subframes |  | Set to be half of UE_DTX_cycle |  |  |
| CQI_DTX_TIMER | 0 subrames |  | 0 subframes |  |  |  |  |
| Inactivity_threshold_for_UE_DRX | 0 subframes |  | 0 subframes |  |  |  |  |