3GPP TSG-RAN WG4 (Radio) Meeting #43                                              		R4-07xxx

Kobe, Japan, May 7-11, 2007

Agenda Item:	6.4.4.1

Source: 	Ericsson

Title: 	A Text Proposal on Physical Layer Measurement Period for TR 36.801

Document for:	Approval

# Discussion

During LTE RRM adhoc session in RAN4#43 it was agreed that for intra-frequency RSRP and E-UTRA carrier RSSI measurements in RRC_CONNECTED state without UE DRX a physical layer measurement period of 200 ms will be used as the working assumption [1]. In order to capture this decision and current status of the work in the technical report TR 36.801 [2] a text proposal is provided in Annex A.

# Reference

  - R4-07xxx, “Summary of E-UTRA RRM Ad Hoc”, RAN4#43, Nokia.

  - TR 36.801, “Evolved Universal Terrestrial Radio Access (E-UTRA); Measurement Requirements”.

# Annex A

-------------------------------------------- Start text proposal -------------------------------------------------------------------

## 12	Measurement Performance Requirements

### 12.1	UE physical layer measurement period

The UE physical layer shall be capable of providing the measurement result to its higher layers after every a  physical layer measurement period.

In RRC_CONNECTED state if UE DRX is not active the physical layer measurement period for intra frequency RSRP measurement is 200 ms.

In RRC_CONNECTED state if UE DRX is not active the physical layer measurement period for intra frequency E-UTRA carrier RRSI measurement is 200 ms.

In RRC_CONNECTED state if UE DRX is active the physical layer measurement periods of intra frequency RSRP and intra frequency E-UTRA carrier RSSI measurements are FFS.

In RRC_CONNECTED state the physical layer measurement periods of inter frequency RSRP and inter frequency E-UTRA carrier RSSI measurements are FFS.

-------------------------------------------- End text proposal -------------------------------------------------------------------