3GPP TSG-RAN WG4 Meeting #43 	DocNumber

- Kobe, Japan, 7 – 11 May

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.101 | CR | CRNum |  rev | - |  Current version: | 7.7.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Introduction of requirements for UE UL power control operation with discontinuous UL DPCCH transmission operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia,Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG:  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  | RANimp-CPC |  |  |  |  |  |  | Date:  |  | 08/05/2007 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Introduction of requirements for for UE UL power control operation with discontinuous UL DPCCH transmission operation |  |  |  |  |  |  |  |  |
| Summary of change:  |  | To section 8.14 text describing the minimum requirement and tables including parameters (Table 8.45A) and and requirements (Table 8.45B) are added. Note that Section 8.14 was existing section (Void). Furthermore, to Annex A.4A the reference parameters for discontinous UL DPCCH tranmission are introduced. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | No requirements would exist for operation of UE UL power control with discontinuous UL DPCCH transmission operation |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 8.14, A.4A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

### 8.14	 UE UL power control operation with discontinuous UL DPCCH transmission operation

#### 8.14.1	Minimum requirement

This test verifies that the UE follows only those TPC commands that correspond to the UL DPCCH slots which are transmitted.

Test parameters are specified in Table 8.45A. The discontinuous UL DPCCH transmission is enabled during the test. The parameters for discontinuous UL DPCCH transmission operation are as specified in Table A.20A. Before the start of the tests, the UE transmit power shall be initialised to -15 dBm. An actual UE transmit power may vary from the target level of -15 dBm due to inaccurate UE output power step.

During test the UE transmit power samples, which are defined as the mean power over one timeslot, shall stay within the range defined in Table 8.45B.

Table 8.45A: Parameters for UE UL power control operation with discontinuous UL DPCCH transmission

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Phase reference | - |  | P-CPICH |
| HS-SCCH_1 | dB |  | -10 |
| F-DPCH | dB |  | [TBD] |
| F-DPCH slot format | - |  | 0 |
| Îor1 | dBm/3.84 MHz |  | -60 |
| Power-Control-Algorithm | - |  | Algorithm 1 |
| UL Power Control step size, TPC | dB |  | 1 |
| TPC commands corresponding to the UL DPCCH slots which are transmitted | - |  | {0,1,0,1,0,1 } Note 1 |
| Propagation condition | - |  | Static without AWGN source |
| Note 1: The sequence of TPC commands corresponds to the UL DPCCH slots that are transmitted. During those slots which correspond to UL DPCCH slots that are not transmitted, UP-commands shall be transmitted. |  |  |  |

Table 8.45B: Test requirements for UE UL power control operation with discontinuous UL DPCCH transmission

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| UE output power | dBm | -15 ± [TBD] |

<Change of section>

# A.4A	 Reference parameters for discontinuous UL DPCCH transmission

The parameters for the UE UL power control operation with discontinuous UL DPCCH transmission test is specified in Table z2. Same parameter values are used for 2ms and 10ms E-DCH TTI.

Table A.20A:  Parameters for the discontinuous UL DPCCH transmission

| Parameter | Unit | Level |
| --- | --- | --- |
| Enabling_Delay | Radio frames | 0 |
| UE_DTX_cycle_1 | Subframes | 10 |
| UE_DTX_cycle_2 | Subframes | 10 |
| UE_DTX_DRX_offset | Subframes | 0 |
| Inactivity_threshold_for_UE_DTX_cycle2 | E-DCH TTI | 1 |
| UE_DPCCH_burst_1 | Subframes | 1 |
| UE_DPCCH_burst_2 | Subframes | 1 |
| UE_DTX_long_preamble_length | Slots | 2 |
| CQI Feedback cycle, k | Milliseconds | 0 |
| CQI_DTX_TIMER | Subframes | 0 |

Table A.20B: (void)

Figure A.11A (void)