3GPP TSG-RAN WG4 #68bis	R1-135326

Riga, Latvia, 7th – 11th, Oct. 2013

Source: 	Ericsson

Title:	TR 36.863 V0.3.1: Draft TP on link and system level summary for CRS-IM performance in homogeneous network

Agenda Item:	10.5

Document for:	Approval

# Discussion

In RAN4#68 meeting, link level simulation results are submitted by several interested companies [2]~[7]. These results are summarized in [1]. Additionally, system level results are also proposed in [9] by some interested companies. In this paper, the conclusion based on the available results is proposed.

# Reference

R4-134481, Summary of link level simulation and side condition for CRS-IM, Ericsson, ST Ericsson, Noted

R4-133997, Link level results for CRS-IM, Ericsson, ST Ericsson, Noted

R4-133144, CRS IC Performance in Homogeneous Network, NEC, Noted

R4-133263, Link level simulation results for CRS-IM in homogeneous network, Huawei, HiSilicon, Noted

R4-133306, Preliminary link level simulaion results, ZTE, Noted

R4-133468, Link-level simulation results of CRS-IM, NSN, Nokia Corporation, Noted

R4-133853, Performance evaluation of CRS-IC and considerations on interference modelling, Renesas Mobile Europe Ltd, Noted

R4-133935, Link level performance  of CRS-IC for Homogeneous network, LG Electronics, Noted

R4-133998, Summary of system level simulation and side condition for CRS-IM, Ericsson, ST Ericsson, Noted

# Text Proposal

< Start of draft text proposal for TR36.863 (TR for CRS-IM)>

<Start of the first change>

## 7.4	Summary

From the link simulation results based on weighted IorjNoc , approximately 8%–102% gain for CRS based transmission (serving cell: TM2) when the average resource utilization of aggressor cells is 10% for cell-edge UEs. For the conditional median IorjNoc , the gain is about 40% with the same resource utilization.  When the average resource utilization of aggressor cells is 50%, approximately 4% - 59% gain can be achieved with the same transmission configuration.  For the conditional median IorjNoc , the gain is about 16% with the same resource utilization.

<End of the first change>

<Start of the second change>

## 8.4	Summary

The study concluded that there is indeed an increase in UPT (user perceived throughput) to be seen with low load (resource utilization is about 10%~50%). For the 5% UPT, the gain is in the order of 10-38% when resource utilization is 30%, additionally approximately 8 – 19 % gain for 50% UPT would be obtained.

<End of the second change>