3GPP TSG-RAN WG4 #68bis	R1-135327

Riga, Latvia, 7th – 11th, Oct. 2013

Source: 	Ericsson

Title:	TR 36.863 V0.3.1: Draft TP on receiver structure for CRS-IM performance in homogeneous network

Agenda Item:	10.5

Document for:	Approval

# Discussion

In RAN4#66bis, the reference receiver is discussed and the agreement is captured in [1]. But the agreement was not captured in the TP. In this TP, the agreement about receiver structure is proposed.

# Reference

[1] R4-132020, WF on CRS-IM performance evaluation, Ericsson, April, 2013.

# Text Proposal

# <Start of the first change>

# Receiver structure

The baseline detector structure is that of an MMSE-IRC detector as defined in [2] combined with a CRS-IM receiver. The CRS IM receiver assumed for Release 11 FeICIC [1] is reused. The CRS-IM receiver is with the following assumption:

- Single FFT

- Mitigates CRS interference of up to 2 cells

- No restriction on whether mitigated CRS interference is intra-/inter-site, under the assumption of synchronous networks

The MMSE-IRC detector is MMSE-IRC based receiver with interference covariance matrix estimation as defined in TR36.829 as the baseline receiver [2], and additionally with the following assumptions:

- MMSE-IRC does not differentiate CRS or data interference when suppressing them.

- No optimization of the MMSE-IRC receiver is considered in this SI.

- Use of MMSE-IRC as the baseline for the SI does not preclude options for the WI

<End of the third change>