3GPP TSG-RAN WG4 (Radio) Meeting #72	R4-144118

Dresden, Germany, 18 – 22 August 2014

Agenda Item:	8.1.1

Source: 	Alcatel-Lucent

Title: 	Recommendation on band plan selection for 2 GHz LTE Band for Region 1

Document for:	Discussion

# 1.	Introduction

The WI proposal [1] for 2 GHz LTE Band for Region 1 was approved in RAN #64. The objective of this WI is to standardize a new FDD E-UTRA band in the 2 GHz MSS band in Region 1. It is stated in [1] that:

•	Initially until RAN #65, two candidate band plan options will be considered in the work item:

o	30x30 MHz (1980-2010 MHz uplink and 2170-2200 MHz downlink)

o	90x90 MHz (1920-2010 MHz uplink and 2110-2200 MHz downlink)

•	RAN4 will work towards selecting one of the candidate band plan options by RAN4 #72.  In the absence of RAN4 consensus after RAN4 #72, RAN will decide on a band plan in RAN #65

•	The work item will progress with the selected band plan per defined scope below after RAN #65.

In this paper, we provide our recommendation on the band plan selection, according to the thorough discussions from the UE perspective during the SI phase.

# 2.	Discussions

Since the beginning of the 2 SIs for 2 GHz Band for Region 1 [2] in December 2012 and Region 3 [3] in September 2012, different band plan options have been studied from the UE perspective with respect to deployment scenario, implementation feasibility and harmonization between the two regions. After a year of study, the way forward proposal [4] was approved in RAN4 #69 (November 2013), where Way Forward 1 was that:

  - The following channel arrangement is not a candidate band.

    - UL: 1920 – 2010 MHz, DL: 2110 – 2200 MHz

Hence the 90x90 MHz (1920-2010 MHz uplink and 2110-2200 MHz downlink) option was agreed by RAN4 not to be a candidate band plan. Other band plan options have since been studied but none of the options can be agreed on.

Therefore, in order to honour the agreement in RAN4 #69, we propose to select the 30x30 MHz (1980-2010 MHz uplink and 2170-2200 MHz downlink) band plan to progress the work item, unless agreement can be made in RAN4 #72 on the 90x90 MHz (1920-2010 MHz uplink and 2110-2200 MHz downlink) option.

# 3.	Conclusions

In this paper, we have provided our view on the 2GHz LTE band plan selection for Region 1 according to the thorough discussions from the UE perspective during the SI phase, and proposed to select the 30x30 MHz (1980-2010 MHz uplink and 2170-2200 MHz downlink) band plan to progress the work item, unless agreement can be made in RAN #72 on the 90x90 MHz (1920-2010 MHz uplink and 2110-2200 MHz downlink) option.

# References

[1]	RP-141009, “New WID:  2 GHz LTE Band for Region 1”, DISH Network, Solaris Mobile Ltd.

[2]	RP-122031, “New SI Study on 2GHz FDD for UTRA and LTE in Region 1 (1980-2010 MHz and 2170-2200 MHz Bands)”, Solaris Mobile Ltd.

[3]	RP-121420, “New SI proposal: Study on LTE FDD in the bands 1980-2010 MHz and 2170-2200 MHz”, TTA.

[4]	RP-137089, “Way forward on how to define a new band for MSS 2GHz”, NTT DOCOMO, Qualcomm, Nokia corporation, Ericsson, KT.