3GPP TSG-RAN4 Meeting #72 	R4-144121

# Dresden, Germany, 18 – 22 August 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.141 | CR | 602 | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of inter-band Carrier Aggregation of Band 2 and Band 30 to TS 36.141 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_B2_B5_B30-Perf |  |  |  |  |  |  | Date: |  | 2014-08-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 30 is introduced to TS36.141. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Carrier aggregation of Band 2 and Band 30 is added to the Table 5.5-3 Inter-band carrier aggregation bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 30 cannot be operated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR 36.104 CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

## 5.5	Operating bands

E-UTRA is designed to operate in the operating bands defined in Table 5.5-1.

Table 5.5-1 E-UTRA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
| 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | FDD |
| 6 (NOTE 1) | 830 MHz | – | 840 MHz | 875 MHz | – | 885 MHz | FDD |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 9 | 1749.9 MHz | – | 1784.9 MHz | 1844.9 MHz | – | 1879.9 MHz | FDD |
| 10 | 1710 MHz | – | 1770 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
| 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
| 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
| 14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
| 15 | Reserved |  |  | Reserved |  |  | FDD |
| 16 | Reserved |  |  | Reserved |  |  | FDD |
| 17 | 704 MHz |  | 716 MHz | 734 MHz |  | 746 MHz | FDD |
| 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
| 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
| 22 | 3410 MHz | – | 3490 MHz | 3510 MHz | – | 3590 MHz | FDD |
| 23 | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz | FDD |
| 24 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| 27 | 807 MHz | – | 824 MHz | 852 MHz | – | 869 MHz | FDD |
| 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD |
| 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD (Note 2) |
| 31 | 452.5 MHz | – | 457.5 MHz | 462.5 MHz | – | 467.5 MHz | FDD |
| 32 | N/A |  |  | 1452 MHz | – | 1496 MHz | FDD (NOTE 2) |
| 33 | 1900 MHz | – | 1920 MHz | 1900 MHz | – | 1920 MHz | TDD |
| 34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
| 35 | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz | TDD |
| 36 | 1930 MHz | – | 1990 MHz | 1930 MHz | – | 1990 MHz | TDD |
| 37 | 1910 MHz | – | 1930 MHz | 1910 MHz | – | 1930 MHz | TDD |
| 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
| 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| 43 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |
| 44 | 703 MHz | – | 803 MHz | 703 MHz | – | 803 MHz | TDD |
| NOTE 1: Band 6 is not applicable. NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |

E-UTRA is designed to operate for the carrier aggregation bands defined in Tables 5.5-2 to 5.5-4.

Table 5.5-2. Intra-band contiguous carrier aggregation bands

| CA Band | E-UTRA operating band |
| --- | --- |
| CA_1 | 1 |
| CA_3 | 3 |
| CA_7 | 7 |
| CA_23 | 23 |
| CA_27 | 27 |
| CA_38 | 38 |
| CA_39 | 39 |
| CA_40 | 40 |
| CA_41 | 41 |

Table 5.5-3. Inter-band carrier aggregation bands

| CA Band | E-UTRA operating bands |
| --- | --- |
| CA_1-5 | 1 |
|  | 5 |
| CA_1-8 | 1 |
|  | 8 |
| CA_1-11 | 1 |
|  | 11 |
| CA_1-18 | 1 |
|  | 18 |
| CA_1-19 | 1 |
|  | 19 |
| CA_1-20 | 1 |
|  | 20 |
| CA_1-21 | 1 |
|  | 21 |
| CA_1-26 | 1 |
|  | 26 |
| CA_2-4 | 2 |
|  | 4 |
| CA_2-5 | 2 |
|  | 5 |
| CA_2-12 | 2 |
|  | 12 |
| CA_2-13 | 2 |
|  | 13 |
| CA_2-17 | 2 |
|  | 17 |
| CA_2-29 | 2 |
|  | 29 |
| CA_2-30 | 2 |
|  | 30 |
| CA_3-5 | 3 |
|  | 5 |
| CA_3-7 | 3 |
|  | 7 |
| CA_3-8 | 3 |
|  | 8 |
| CA_3-19 | 3 |
|  | 19 |
| CA_3-20 | 3 |
|  | 20 |
| CA_3-26 | 3 |
|  | 26 |
| CA_3-27 | 3 |
|  | 27 |
| CA_3-28 | 3 |
|  | 28 |
| CA_4-5 | 4 |
|  | 5 |
| CA_4-7 | 4 |
|  | 7 |
| CA_4-12 | 4 |
|  | 12 |
| CA_4-13 | 4 |
|  | 13 |
| CA_4-17 | 4 |
|  | 17 |
| CA_4-27 | 4 |
|  | 27 |
| CA_4-29 | 4 |
|  | 29 |
| CA_5-7 | 5 |
|  | 7 |
| CA_5-12 | 5 |
|  | 12 |
| CA_5-17 | 5 |
|  | 17 |
| CA_5-25 | 5 |
|  | 25 |
| CA_7-20 | 7 |
|  | 20 |
| CA_7-28 | 7 |
|  | 28 |
| CA_8-20 | 8 |
|  | 20 |
| CA_8-40 | 8 |
|  | 40 |
| CA_11-18 | 11 |
|  | 18 |
| CA_12-25 | 12 |
|  | 25 |
| CA_19-21 | 19 |
|  | 21 |
| CA_20-32 | 20 |
|  | 32 |
| CA_23-29 | 23 |
|  | 29 |
| CA_39-41 | 39 |
|  | 41 |

Table 5.5-4. Intra-band non-contiguous carrier aggregation bands (with two sub-blocks)

| CA Band | E-UTRA operating band |
| --- | --- |
| CA_2-2 | 2 |
| CA_3-3 | 3 |
| CA_4-4 | 4 |
| CA_7-7 | 7 |
| CA_23-23 | 23 |
| CA_25-25 | 25 |
| CA_41-41 | 41 |
| CA_42-42 | 42 |

<End of change>