3GPP TSG RAN WG4 Meeting #72	        R4-144560

# Dresden, Germany, 18th  to 22th, August 2014

- Agenda Item:	7.28.1

- Source:	LG Electronics

- Title:	Measurements results of additional ILs and desense levels for CA_1A-3A UE

Document for:	Approval

- Introduction

In RAN4 #71 meetings, RAN4 did not have consensus for additional ILs and REFSENS test configurations for inter-band CA UE [1][2]. There are some issues as follows

- ∆TIB = FFS (0.2~0.7dB) and ∆RIB = FFS (0.0~0.7dB)

- Cross isolation level to reduce self desense impact

- Band 1 UL configuration for Band 3 DL REFSENS test

In this contribution, we we provide measured values for ILs of Quadplexer and Duplexer and analysis of desense level for Band 3 DL frequency of CA_1A-3A UE.

- Measurement on the Front End Filters for CA_1A-3A

In the Extreme Temperature Conditions (ETC), insertion losses of the Quadplexer are measured by filter vendors which are given in Table 1.

Table 1: IL values for band 1 + 3 Quadplexers (under ETC)

|  | Band 1 UL | Band 1 DL | Band 3 UL | Band 3 DL |
| --- | --- | --- | --- | --- |
| Vendor A (Quadplexer) | 2.7 | 3.0 | 3.5 | 4.2 |
| Vendor B (Quadplexer) | 2.3 | 2.7 | 4.0 | 4.0 |
| Vendor C (Quadplexer) | 2.2 | 2.5 | 3.5 | 3.5 |

The worst case duplexer performance from the same vendors for each band is shown in Table 2.

Table 2: IL values of Duplexers (under ETC)

|  | Band 1 UL | Band 1 DL | Band 3 UL | Band 3 DL |
| --- | --- | --- | --- | --- |
| Vendor A (Duplexer) | 1.8 | 2.4 | 3.0 | 3.6 |
| Vendor B (Duplexer) | 2.0 | 2.4 | 3.5 | 3.5 |
| Vendor C (Duplexer) | 2.0 | 2.3 | 3.1 | 3.5 |

Therefore, the additional insertion loss for combining these two bands when comparing worst case can be summarized in Table 3.

Table 3: Additional IL of Quadplexers (under ETC)

|  | Band 1 UL | Band 1 DL | Band 3 UL | Band 3 DL |
| --- | --- | --- | --- | --- |
| Vendor A | 0.9 | 0.6 | 0.5 | 0.6 |
| Vendor B | 0.3 | 0.3 | 0.5 | 0.5 |
| Vendor C | 0.2 | 0.2 | 0.4 | 0.0 |
| Average ILs | 0.47 | 0.36 | 0.47 | 0.36 |

From the data, we propose the following TIB,c and RIB,c for CA_1A-3A UE in Table 4 and Table 5.

Table 4: ΔTIB,c for CA_1A-3A

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A | 1 | 0.3 |
|  | 3 | 0.3 |

Table 5: ΔRIB,c for CA_1A-3A

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_1A-3A | 1 | 0.1 |
|  | 3 | 0.1 |

- Analysis of desense level on Band 3 DL frequency for CA_1A-3A

For the desense analysis of CA_1A-3A UE, we assume two cases as shown in Figure1, one is UL-DL gap of 40MHz and the other is UL-DL gap of 60MHz. And also, we assume

- RB Allocation :

  - 25RBs for 20MHz, full RBs for 10MHz in Band 1 UL channel

(RBs allocated in Lower edge of B1 UL frequency for worst case condition)

  - 25RBs for 5MHz in Band 3 DL channel

- Minimum gap between B1 Tx and B3 DL: 40, 60 MHz

- Quadplexer Characteristics: B1 Tx to B3 Rx cross- isolation: 46~66dB

In Case 1 of Figure 1, the # of UL RB are considered with 25RBs in 20MHz CBW and 50RBs in 10MHz CBW.

- Case1: minimum gap with 40MHz

- Case2: minimum gap with 60MHz

Figure 1 Desense level analysis test cases for CA_1A-3A UE

To analyze the desense level in B3 DL, we measured thermal noise levels in B3 DL frequency which is shown in Figure 2 according to the various UL configurations in some channel bandwidths.

- Case1: Min. gap = 40MHz                         (b) Case 2: Min. gap = 60MHz

Figure 2 Thermal noise levels in B3 DL frequency by Band 1 Tx leakage

In figure 2-(a), the TX thermal noise level is -120dBm/Hz when 25RBs restriction in 20MHz CBW and 40MHz minimum gap are assumed. And also the TX thermal noise level is -121dBm/Hz in figure 2-(b) when 25RBs restriction in 20MHz CBW and 60MHz minimum gap are assumed.

Figure 3 shows the simulation results for desense levels of Case 1. From Figure 3-(a), we can see the desense level is 3.8 dB when we assume the cross-isolation of Quadplexer is 50dB and # of RB is restricted with 25RBs in 20MHz channel bandwidth.

And also form the Figure 3-(b), we can also see can the required desense level is 4.3dB when we assume the cross-isolation of Quadplexer is 50dB and full RBs are allocated in 10MHz channel bandwidth.

- 25RBs in 20MHz CBW                              (b) 50RBs in 10MHz CBW

Figure 3 Desense levels in B3 DL frequency for Case1

- 25RBs in 20MHz CBW                               (b) 50RBs in 10MHz CBW

Figure 4 Desense levels in B3 DL frequency for Case2

Figure 4 shows the simulation results for desense levels of Case 2.  From  Figure 4-(a), we can see the desense level is 2.9 dB when we assume the cross-isolation of Quadplexer is 50dB and # of RB is restricted with 25RBs in 20MHz channel bandwidth.

And also form the Figure 4-(b), we can see the required desense level is 0.8dB when we assume the cross-isolation of Quadplexer is 50dB and full RBs are allocated in 10MHz channel bandwidth

These results are summarized in Table 6.

Table 6: Self-desensitization in Band 1 by coupling loss for 2ULs CA_1A-5A UE

| B1 Tx to B3 Rx Cross isolation level of Quadplexer (dB) | Case1 (Min. gap =40MHz) |  | Case2 (Min. gap =60MHz) |  |
| --- | --- | --- | --- | --- |
|  | Desensitization level (dB) (25RBs in 20MHz) | Desensitization level (dB) (50RBs for 10MHz) | Desensitization level (dB) (25RBs in 20MHz) | Desensitization level (dB) (50RBs for 10MHz) |
| 48 | 5.2 | 6.0 | 4.1 | 1.8 |
| 50 | 3.8 | 4.3 | 2.9 | 0.8 |
| 55 | 0.7 | 1.1 | 0.1 | 0.1 |
| 60 | 0.0 | 0.0 | 0.0 | 0.0 |

From the analysis, we propose as follow

Proposal 1: For additional ILs, RAN4 can use the additional ILs in Table 4 and Table 5 for CA_1A-3A UE.

Proposal 2: For the REFSENS test configuration, RAN4 should consider worst case scenario which mean the minimum gap is 40MHz and # of RB is restricted with 25RBs in 20MHz CBW.

Proposal 3: To determine the MSD levels for REFSENS requirements, 50dB cross-isolation level could be considered.

- Conclusions

In this contribution we provided measured values of the ILs for Quadplexer and Duplexer for CA_1A-3A UE, and analyzed the desense level in B3 DL frequency. From the results, we propose as follow

Proposal 1: For additional ILs, RAN4 can use the additional ILs in table 4 and table 5 for CA_1A-3A UE.

Proposal 2: For the REFSENS test configuration, RAN4 should consider worst case scenario which mean the minimum gap is 40MHz and # of RB is restricted with 25RBs in 20MHz CBW.

Proposal 3: To determine the MSD levels for REFSENS requirements, 50dB cross-isolation level could be considered.

- Reference

- R4-142947, “TP for 36.851:  B1 + B3 reference sensitivity,” Qualcomm

- R4-143959, “Way Forward on B1+B3 CA,” Qualcomm.

- R4-143006, “Consideration and way forward on B1+B3 CA” Intel

- TS 36.101 v12.4.0