3GPP TSG RAN WG4 Meeting #72BIS	         R4-145791

# Singapore, Singapore, 6th  to 10th, October 2014

- Agenda Item:	7.33

- Source:	LG Electronics

- Title:	Self-desense analysis for 2ULs inter-band CA

Document for:	Approval

- Introduction

In RAN4 #72 meetings, UE vendors had provided MSD levels for self-desensitization analysis in 2ULs inter-band CA Class A4. However the variance of MSD levels for each 2UL inter-band CA combination are quite large up to 18dB among UE vendors in each band combinations. The MSD level should be determined for REFSENS requirements to close 2ULs inter-band CA WI until RAN WG4 #73 meeting in San Francisco.

In this contribution, we analyzed IMD levels for IMD2 to IMD5 level for 2ULs CA band combination such as B5+B7, B3+B5 and B3+B8 CA UE based on the agreed WF [3].

- Simulation assumptions for 2ULs CA UE

Table 1 show the UE RF FE components parameters to determine MSD levels for each CA band combinations.

Table 1 UE RF Front-end component parameters for B5+B7 & B3+B5

| Component | B5+B7 & B3+B5 |  |  |  | B3+B8 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 112 | 68 | 55 | 55 | 118 | 70.6 | 55.2 | 54.9 |
| Diplexer | 115 | 87 | 55 | 55 | 118 | 89 | 55 | 55 |
| Duplexer | 98 | 75 | 55 | 53 | 100 | 75 | 55 | 53 |
| PA Forward | 28.5 | 32 | 30 | 28 | 28 | 32 | 30 | 28 |
| PA Reversed | 40.5 | 30.6 | 30 | 30 | 37 | 34 | 33 | 32 |
| LNA | 10 | 0 | 0 | -10 | 10 | 0 | 0 | -10 |

Table 2 UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Diplexer | 15 | High/low band isolation |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band (diplexer + duplexer) |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band (diplexer + duplexer) |
| Duplexer | 50 | Tx band rejection at Rx band |

Table 2 show the isolation levels according to the RF component to RF component.

- Analysis IMD levels for 2ULs CA UE

From the simulation assumption, we analyzed the IMD products levels of each CA band combination as shown in Table3.

Table 3 Analyzed MSD levels for 2ULs inter-band CA band combinations

| CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3 | IMD4 | 3*fB8 - fB3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
| B8 |  |  | 900 | 5 | 25 | 945 | 3.1 | 9.2 |
| B3 | IMD5 | 2*fB3 - 3*fB8 | 1780 | 10 | 50 | 1875 | N/A | N/A |
| B8 |  |  | 882.5 | 5 | 25 | 927.5 | 31.2 | 0 |
| B3 | IMD2 | fB3 - fB5 | 1721 | 10 | 50 | 1816 | N/A | N/A |
| B5 |  |  | 838 | 5 | 25 | 883 | 2.4 | 23.2 |
| B3 | IMD4 | 2*fB3 - 2*fB5 | 1771 | 10 | 50 | 1866 | 1.5 | 4.0 |
| B5 |  |  | 838 | 5 | 25 | 883 | N/A | N/A |
| B5 | IMD3 | fB7 - 2*fB5 | 834 | 5 | 25 | 879 | 2.8 | 10.6 |
| B7 |  |  | 2547 | 10 | 50 | 2667 | N/A | N/A |
| B5 | IMD5 | 4*fB5 - fB7 | 846.5 | 5 | 25 | 891.5 | 3.4 | 8.4 |
| B7 |  |  | 2505 | 10 | 50 | 2625 | N/A | N/A |
| Note 1: RBstart = 0 |  |  |  |  |  |  |  |  |

From the analyzed results, we propose the MSD levels for B5+B7, B3+B5 and B3+B8 2ULs inter-band CA combinations.

Table 4 Proposed MSD levels for 2ULs inter-band CA band combinations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |
| CA_3A-5A | 3 | 1771 | 10 | 50 | 1866 | 4.0 | FDD |
|  | 5 | 838 | 5 | 25 | 883 | N/A |  |
| CA_3A-5A | 3 | 1721 | 10 | 50 | 1816 | N/A | FDD |
|  | 5 | 838 | 5 | 25 | 883 | 23.2 |  |
| CA_3A-8A | 3 | 1755 | 10 | 50 | 1850 | N/A | FDD |
|  | 8 | 900 | 5 | 25 | 945 | 9.2 |  |
| CA_5A-7A | 5 | 834 | 5 | 25 | 879 | 10.6 | FDD |
|  | 7 | 2547 | 10 | 50 | 2667 | N/A |  |
| Note 1: RBSTART = 0 |  |  |  |  |  |  |  |

- Conclusions

In this contribution we analyzed the IMD level based on RF component characteristics according to the each CA band combinations. We propose the MSD levels for B5+B7, B3+B5 and B3+B8 2ULs inter-band CA combinations in Table 4.

- Reference

- R4-141259, “Higher order IMD and harmonic table for UE self-desensitization analysis,” Intel

- R4-140605, “More on IMD measurements for 2UL inter-band CA,” Media Tek Inc.

- R4-144031, “WF on 2UL inter-band CA MSD test configurations,” MediaTek

- R4-144718, “MSD for band combinations with IMD2  IMD5 issues,” Nokia

- R4-145130, “Inter-band 2UL class A4 self interference,” Qualcomm

- R4-143227, “TP for self-desense analysis for CA_5A-7A UE,” LG Electronics

- R4-141544, “Spectrum study on intermodulation analysis for 2 UL CA,” Intel

- TR 36.860 v0.10.0

- TS 36.101 v12.5.0