3GPP TSG-RAN Working Group 4 Meeting #72bis                   R4-146085

Singapore, 6-10 Oct, 2014

Source:	Samsung

Title:	TP for Rel-13 3DL TR 36.8xx: TDD-FDD CA for CA_26A-41A-41A

Agenda item:	7.6.4.3

Document for:	Approval

- 1	Introduction

In RAN#64, the WI was approved on LTE Advanced 3 Band Carrier Aggregation(3DL/1UL) of Band 26, Band 41 and Band 41[1]. This belongs to low-high CA band combinations, and it exit intermodulation issue. As discussed in last RAN4 meeting, Harmonic Trap Filter(HTF) was agreed to be working assumption to handle the problem that the 3rd order harmonic of Band 26 UL may fall into the Band 41 receive band. In this contribution, a relevant text proposal for Rel-13 3 DL TR TR36.8xx was provided, which include channel bandwidth combination, harmonic analysis results and transmitter/receiver insertion losses, etc. It should be noted that co-existence issue of CA_26A-41A-41A is similar as CA_26A-41A, which have been described in [2].

- 2	Text proposal

Text Proposal for TR 36.8xx (Rel-13 3DL)

<Start of change section>

6 3 Band Carrier Aggregation with single UL: band combination specific part

---Unchanged section omitted---

6.X LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 26, Band 41 and Band 41

6.X.1 Operating bands for Band 26, Band 41 and Band 41

Table 6.X.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_26-41 | 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.X.2 Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_26A-41A-41A | 26 |  |  | Yes | Yes | Yes |  | 55 | 0 |
|  | 41 | See Table 5.6A.1-3 CA_41A-41A Set 1 |  |  |  |  |  |  |  |

NOTE 1: For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [4] when operating in single carrier mode.

6.X.3 Co-existence studies for Band 26, Band 41 and Band 41

6.X.3.1 For the BS

Table 6.X.3.1-1provides the intermodulation products for Band 26 + Band 41 +Band 41 CA. For the IMD analysis the maximum transmission as defined in Table 6.X.3.1-1 is considered. It can be seen from the table that the 2nd harmonics of BS transmitting in Band 26 may fall into the BS receive band of Bands 3, 4, 9 and 10 and the 3rd harmonics of BS transmitting in Band 26 may fall into the BS receive band of Bands 38 and 41, while the 2nd IMD products caused by BS supporting CA of Band 26 and Band 41 may fall into the BS receive band of Band 3, 4, 9, 10, 22, 24 and 42, and the 3rd IMD products may fall into the BS receive band of Bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 38, 41 and 44. Therefore, sharing antenna for CA operation between Band 26 and Band 41 is not recommended.  In addition, when one deploys CA_26-41 operation in certain region, these co-existence studies should be taken into account because harmonics and IMDs could fall into twenty-two bands in total.

Table 6.X.3.1-1: B26 + B41 + B41 harmonics and IMD products frequency limits

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 859 | 894 | 2496 | 2690 |
| 2nd order harmonics frequency range (MHz) | 1718 | 1788 | 4992 | 5380 |
| 3rd order harmonics frequency range (MHz) | 2577 | 2682 | 7488 | 8070 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 1602 | 1831 | 3355 | 3584 |
| 3rd order IMD products | ¦f2_high – 2*f1_low¦ | ¦f2_low – 2*f1_high¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 708 | 972 | 4098 | 4521 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 4214 | 4478 | 5851 | 6274 |
| 3rd order IMD products | ¦f1_low – f2_high + f2_low¦ | ¦f1_high + f2_high – f2_low¦ | ¦f2_low – f1_high + f1_low¦ | ¦f2_high + f1_high – f1_low¦ |
| IMD frequency limits (MHz) | 665 | 1088 | 2461 | 2725 |
| 3rd order IMD products (with maximum channel bandwidth) | ¦f1_low – max BW f2¦ | ¦f1_high + max BW f2¦ | ¦f2_low – max BW f1¦ | ¦f2_high + max BW f1¦ |
| IMD frequency limits (MHz) | 839 | 914 | 2486 | 2700 |

6.X.3.2 For the UE

Table 6.X.3.2-1 provides the harmonic products for CA_26A-41A-41A with 1 UL. 3rd harmonic products of Band 26 UL fall into the Band 41 receiver band. Then harmonic Trap Filter(HTF) should be used in transmission link of Band 26 to avoid harmonic product interference to Band 41. Only if in future spectrum deployment requirement from operators lead to limitation usage of interference frequency range for B41 receive band due to Band 26 3rd harmonic products, then HTF is not necessary.

Table 6.X.3.2-1: 1UL B26 + B41 + B41 harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 2496 | 2690 |
| 2nd order harmonics frequency range (MHz) | 1628 to 1698 |  | 4992 to 5380 |  |
| 3rd order harmonics frequency range (MHz) | 2442 to 2547 |  | 7488 to 8070 |  |

6.X.4 ∆TIB,C and ∆RIB,C

For three simultaneous DL and one UL, the TIB,c and RIB values are shown in Table 6.X.4-1 and Table 6.X.4-2 respectively.

Table 6.X.4-1 : ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_26A-41A-41A | 26 | FFS |
|  | 41 | FFS |

Table 6.X.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_26A-41A-41A | 26 | FFS |
|  | 41 | FFS |

<End of change section>

---Unchanged section omitted---

Reference

- RP-140964, " New Work Item proposal: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 26, Band 41 and Band 41 " KDDI

- TR 36.851 V 0.13.0