3GPP TSG-RAN WG4 #72	R4-146429

Singapore, October 06 – October 10, 2014

Source: 	Ericsson

Title:	Further discussion on eIMTA demodulation/CSI test setup

Agenda Item:	7.5.3

Document for:	Discussion

# Introduction

In RAN4 #72 meeting, eIMTA CSI/demodulation test setup is further discussed. One high level WF about eIMTA CSI/demodulation was agreed in [1]. In one companion paper [2], we further address our view on the test case list for eIMTA, as shown in Table 1. In this paper, we discuss details on the test setup for different test cases.

Table 1: Test cases list for eIMTA

| Test case | Test configuration |  |  |  | Test purpose |
| --- | --- | --- | --- | --- | --- |
|  | L1 Signaling | Rel-12 subframe set | Two ZP-CSI-RS | TM |  |
| PDSCH Perf. | Yes | No | No | 2 | - UL-DL re-configuration via L1 signaling ( Corresponding to 7-1) |
| ePDCCH Perf. | No | Yes | Yes | 10 | - Configuration of two ZP CSI-RS for TM 10 and EPDCCH RE mapping by rate matching around the configured ZP-CSI-RS configurations, if the UE supports TM10 and EPDCCH (Sub-item 6 of Feature group 7-3) |
| Periodic CSI test for TM2 | Yes | Yes | Yes | 2 | - To handle different interference level - Support of explicit L1 signalling of UL/DL reconfiguration (Corresponding to 7-1) - CSI measurement and reporting with Rel-12 DL subframe sets for TM 1 – 9 (sub-item 1&2 of Feature group 7-3) - Configuration of two ZP CSI-RS for TM 1 -- 9 and PDSCH RE mapping by rate matching around the configured ZP-CSI-RS configurations (sub-item 4 of Feature group 7-3) |
| Aperiodic CSI test for TM 9 | Yes | Yes | No | 9 | - New definition of aperiodic CSI report - New CSI reference resource definition |
| CSI test with two CSI-IMs for each CSI process | No | Yes | Yes | 10 | - CSI measurement and reporting with Rel-12 DL subframe sets for TM 10, and configuration of additional CSI-IM resource for a CSI processe for TM 10, if the UE supports TM10 (sub-item 3 of Feature group 7-3) |

# Common setup for demodulation/CSI

In this section, common setup for demodulation/CSI is discussed. In the WF [3], we have the following agreements:

- TDD UL-DL configurations:

  - SIB1 TDD UL-DL configuration #0

  - DL HARQ reference TDD UL-DL configuration is FFS (#2 or #5)

  - Dynamic TDD UL-DL configurations  are FFS

    - Option 1: Possible subset is {0, 1, 2, 6} for DL HARQ reference TDD UL-DL configuration #2

    - Option 2: Possible subset is {0, 1, 2, 3, 4, 5, 6} for DL HARQ reference TDD UL-DL configuration #5

    - Option 3: Possible subset is {0, 5} for DL HARQ reference TDD UL-DL configuration #5

  - Dynamic TDD UL-DL configurations are randomly selected from a possible subset on a per-DCI basis.

For option 2, for subframe 6, in some configuration, it is a special subframe, and in other configuration, it is a normal subframe. It leads to some difficulties to collect the throughput. For Option 3, in most of the 36.101 defined test cases, subframe 5 will not use for PDSCH transmission in order to avoid the problem of SIB-1 transmission. In other words, for option 3, to some extent, only subset {0} is used for PDSCH transmission. Compared with the option 1, less benefit can be observed for option 2 and option 3. Hence, we prefer to use option 1.

Proposal 1: Using option 1 for dynamic TDD UL-DL configuration

For L1 reconfiguration DCI transmission parameters, in the WF [3], we have the following agreements:

- L1 reconfiguration DCI transmission parameters

  - Periodicity of monitoring the L1 reconfiguration DCI is 10ms

  - Set of subframes to monitor the L1 reconfiguration DCI is FFS

    - Option 1: Set of subframes includes all SIB-1 DL/S subframes. Subframes for reconfiguration DCI transmission are chosen in a random way on a per-DCI basis.

    - Option 2: The L1 reconfiguration DCI is transmitted in SF #0

    - Other options are not precluded

For option 2, it is easy for UE cheating, since UE can only detect the SF #0 to pass the test. Further, according to the RAN1 agreements, the valid duration is dependent on the transmitted subframe for the L1

Table 2:  Valid duration for the L1 explicit signalling

| Periodicity (ms) | Offset (ms) (0≤＜) | Valid duration |
| --- | --- | --- |
| 10 | A X-bit bitmap to indicate a set of SIB-1 DL/S subframe. Starting from the MSB to LSB, the bitmap corresponds to subframe #{[Xa, Xb, Xc, …]}. The bit “1” indicates that UE shall monitor the reconfiguration DCI in the corresponding subframe, and the bit “0” indicates otherwise. | “Current” for the reconfiguration DCI in subframe #0 “Next” for the reconfiguration DCI in the SIB1 DL/S subframes other than subframe #0 |

reconfiguration DCI, as shown in Table 1 [2].  If it is transmitted in SF #0, the valid duration is “current”. Otherwise, the valid duration is “Next”. If option 2 is adopted, only the “current” valid duration is verified, and the “next” valid duration is not verified. From the test coverage point of view, option 1 is much better than option 2. Hence, we prefer to have option 1 for the set of subframes to monitor.

In RAN4#72 online meeting, some companies have different understanding to the definition of valid duration when the reconfiguration DCI is not in subframe #0. If the company cannot align with the definition of valid duration, it will put network at risk in case network wants to have other configuration in which reconfiguration DCI is transmitted other than subframe #0. It makes option 1 is more desirable. Hence, we propose:

Proposal 2:  Option 1 is selected for the set of subframes to monitor the L1 reconfiguration DCI.

# Test setup for different test cases

## PDSCH performance test

For PDSCH test, the traditional test  (section 8.2.2.2.1 of 36.101 ) for transmit diversity can be reused except some eIMTA related parameters are added. In Table 3, PDSCH test parameters for eIMTA are listed. The parameters which are specific to eIMTA are marked as yellow background.

Table 3: Test Parameters for Transmit diversity Performance (FRC)

| Parameter |  | Unit |  |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 2 |
| tdd-Config in SIB-1 |  |  | 0 |
| eimta-HarqReferenceConfig |  |  | sa2 |
| eimta-CommandPeriodicity |  |  | sf10 |
| eimta-CommandSubframeSet-r12 |  |  | 1100011000 |
| Transmission of eIMTA Command |  |  | Random selected with equal probability from eIMTA command subframe set |
| UL-DL reconfiguration in explicit L1 signalling |  |  | Random select with equal probability from {0,1,2,6} |
| Reference channel |  |  | TBD |
| Test Metric |  |  | Fraction of maximum throughput (@70%) |
| Antenna Configuration |  |  | 2x2 |
| Channel model |  |  | TBD |
| Bandwidth |  | MHz | 10 |
| UE category |  |  | 2-8 |
| Note 1: . |  |  |  |

## ePDCCH performance test

For ePDCCH performance test, the localized transmission with TM10 Type B quasi co-location type (section 8.8.3.2 of 36.101) can be reused with minor modification. The detail parameters are shown in Table 4. The text with yellow background is the modification part compared with Rel-11 ePDCCH performance test. The main modification is to add two ZP-CSI-RS configurations.

## Periodic CSI test for TM2

For Periodic CSI test, PUCCH 1-0 static test (TDD) (Section 9.2.1.2 of 36.101) can be reused in addition to the eIMTA related parameters, as shown in Table 5. The main difference compared with traditional PUCCH 1-0 static test (TDD) is different noise level may be used for fixed subframes and flexible subframes.

## Aperiodic CSI test for TM9

For aperiodic CSI test, test case for PUSCH 3-1 (CSI reference symbol)  (Section 9.3.1.2.2 of 36.101) could be re-used in addition to the eIMTA related parameters, as shown in Table 6. Besides eIMTA related parameters, two Rel-12 subframe sets are configured.

## CSI test with two CSI-IMs for each CSI process

For CSI test with two CSI-IMs for each CSI process, the test case designed for Minimum requirement PUCCH 1-1 (With Single CSI Process) (Section 9.2.4.2 of 36.101) can be reused with minor modification. The modification is as shown in Table 7 marked as yellow background. The main modification is to add the Rel-12 subframe set, and configure two CSI-IMs. The BLER metric can be reused for each subframe set. Other metric can be further discussed.

Table 4: Test Parameters for Localized Transmission TM10 Type B quasi co-location type

| Parameter |  |  | Unit | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TP 1 | TP 2 |
| PHICH duration |  |  |  | Normal |  |
| Downlink power allocation |  |  | dB | 0 |  |
|  |  |  | dB | 0 |  |
|  |  |  | dB | -3 |  |
|  |  |  | dB | 0 |  |
|  |  |  | dB | Reference value in Table 8.8.3.2-2 | Reference value in Table 8.8.3.2-2 |
| at antenna port |  |  | dBm/15kHz | -98 |  |
| Bandwidth |  |  | MHz | 10 | 10 |
| Number of configured EPDCCH Sets |  |  |  | 2 (Note1) |  |
| EPDCCH-PRB-Set ID (setConfigId) |  |  |  | 0 | 1 |
| Transmission type of EPDCCH-PRB-set |  |  |  | Localized | Localized |
| Number of PRB pair per EPDCCH-PRB-set |  |  | PRB | 8 | 8 |
| EPDCCH beamforming model |  |  |  | Annex B.4.5 | Annex B.4.5 |
| PDSCH transmission mode |  |  |  | TM10 | TM10 |
| PDSCH transmission scheduling |  |  |  | Probability of occurrence of PDSCH transmission is 30% (Note 3) | Probability of occurrence of PDSCH transmission is 70% (Note 3) |
| CSI reference signal configurations |  |  |  | Antenna ports 15,16 | Antenna ports 15,16 |
| Non-zero power CSI reference signal (NZPId=1) | CSI reference signal configuration |  |  | N/A | 0 |
|  | CSI reference signal subframe configuration ICSI-RS |  |  | N/A | 0 |
| Non-zero power CSI reference signal (NZPId=2) | CSI reference signal configuration |  |  | 10 | N/A |
|  | CSI reference signal subframe configuration ICSI-RS |  |  | 0 | N/A |
| Zero power CSI reference signal (ZPId=1) | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) |  | Bitmap | N/A | 1000010000000000 |
|  | CSI-RS subframe configuration ICSI-RS |  |  | N/A | 0 |
| Zero power CSI reference signal (ZPId=2) | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) |  | Bitmap | N/A | 1000010000000000 |
|  | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) |  |  | N/A | 3 |
| Zero power CSI reference signal (ZPId=3) | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) |  | Bitmap | 1000010000000000 | N/A |
|  | CSI-RS subframe configuration ICSI-RS |  |  | 0 | N/A |
| Zero power CSI reference signal (ZPId=4) | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) |  |  | 0100001000000000 | N/A |
|  | CSI-RS subframe configuration ICSI-RS |  |  | 3 | N/A |
| PQI set 0 (Note 4) | Non-Zero power CSI RS Identity (NZPId) |  |  | N/A | 1 |
|  | Zero power CSI RS Identity (ZPId) |  |  | N/A | 1 & 2 |
| PQI set 1 (Note 4) | Non-Zero power CSI RS Identity (NZPId) |  |  | 2 | N/A |
|  | Zero power CSI RS Identity (ZPId) |  |  | 3&4 | N/A |
| Number of PDCCH symbols |  |  | Symbols | 1 (Note 2) |  |
| EPDCCH starting position |  |  |  | pdsch-Start-r11=2 (Note 2) | pdsch-Start-r11=2 (Note 2) |
| Subframe configuration |  |  |  | Non-MBSFN | Non-MBSFN |
| Time offset between TPs |  |  | s | N/A | 2 |
| Frequency shift between TPs |  |  | Hz | N/A | 200 |
| Cell ID |  |  |  | 0 | 126 |
| TDD UL/DL configuration |  |  |  | 2 |  |
| TDD special subframe |  |  |  | 1 |  |
| Note 1: Resource blocks nPRB = 0, 7, 14, 21, 28, 35, 42, 49 are allocated for both the first set and the second set. Note 2: The starting OFDM symbol for EPDCCH is determined from the higher layer signalling pdsch-Start-r11. And CFI is set to 1. Note 3: The TP from which PDSCH is transmitted shall be randomly determined independently for each subframe. Probabilities of occurrence of PDSCH transmission from TP 1 and TP 2 are specified. Note 4: For PQI set 0, PDSCH and EPDCCH are transmitted from TP 2. For PQI set 1, PDSCH and EPDCCH are transmitted from TP1. EPDCCH and PDSCH are transmitted from same TP. |  |  |  |  |  |

Minimum Performance

| Test number | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 2 ECCE | R.59 FDD | OP.7 FDD | EVA5 | 2 x 2 Low | 1 | TBD |

Table 5: PUCCH 1-0 static test (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 2 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (2 x 2) |  |  |  |
| tdd-Config in SIB-1 |  |  | 0 |  |  |  |
| eimta-HarqReferenceConfig |  |  | sa2 |  |  |  |
| eimta-CommandPeriodicity |  |  | sf10 |  |  |  |
| eimta-CommandSubframeSet-r12 |  |  | 1100011000 |  |  |  |
| Transmission of eIMTA Command |  |  | Random selected with equal probability from eIMTA command subframe set |  |  |  |
| UL-DL reconfiguration in explicit L1 signalling |  |  | Random select with equal probability from {0,1,2,6} |  |  |  |
| csi-MeasSubframeSet-r12 |  |  | 0001100011 |  |  |  |
| Interference model |  |  | Use different interference level for two CSI subframe set |  |  |  |
| Zero power CSI reference signal (ZPId=1) |  | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) | 1000010000000000 |  |  |  |
|  |  | CSI-RS subframe configuration ICSI-RS | 0 |  |  |  |
| Zero power CSI reference signal (ZPId=2) |  | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) | 1000010000000000 |  |  |  |
|  |  | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) | 3 |  |  |  |
| SNR (Note 2) |  | dB | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 10 |  |  |  |
| cqi-pmi-ConfigIndex |  |  | 9 |  |  |  |
| cqi-pmi-ConfigIndex2 |  |  | 14 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.1 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. |  |  |  |  |  |  |

Table 6: Sub-band test for TDD

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |
| Transmission mode |  |  | 9 |  |
| Uplink downlink configuration |  |  | 2 |  |
| Special subframe configuration |  |  | 4 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
| SNR (Note 3) |  | dB | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |
| Antenna configuration |  |  | 2x2 |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |
| CRS reference signals |  |  | Antenna port 0 |  |
| CSI reference signals |  |  | Antenna port 15,16 |  |
| tdd-Config in SIB-1 |  |  | 0 |  |
| eimta-HarqReferenceConfig |  |  | sa2 |  |
| eimta-CommandPeriodicity |  |  | sf10 |  |
| eimta-CommandSubframeSet-r12 |  |  | 1100011000 |  |
| Transmission of eIMTA Command |  |  | Random selected with equal probability from eIMTA command subframe set |  |
| UL-DL reconfiguration in explicit L1 signalling |  |  | Random select with equal probability from {0,1,2,6} |  |
| csi-MeasSubframeSet-r12 |  |  | 0001100011 |  |
| Interference model |  |  | Use different interference level for two CSI subframe set |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 3 |  |
| CSI-RS reference signal configuration |  |  | 4 |  |
| CodeBookSubsetRestriction bitmap |  |  | 000001 |  |
| Reporting interval (Note 4) |  | ms | 5 |  |
| CQI delay |  | ms | TBD |  |
| Reporting mode |  |  | PUSCH 3-1 |  |
| Sub-band size |  | RB | 6 (full size) |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.8 TDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#3 and #8 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#2 and #7. |  |  |  |  |

Table 7: PUCCH 1-1 static test (TDD)

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | TP1 | TP2 |  |
| Bandwidth |  | MHz | 10 |  |  |
| PDSCH transmission mode |  |  | 10 |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |
| Special subframe configuration |  |  | 4 |  |  |
| Downlink power allocation (Note 1) |  | dB | 0 | 0 |  |
|  |  | dB | 0 | 0 |  |
|  | Pc | dB | -6 | -6 |  |
|  |  | dB | -3 | N/A |  |
| Cell ID |  |  | 0 |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0, 1 | (Note 2) |  |
| csi-MeasSubframeSet-r12 |  |  | 0001100011 | N/A |  |
| Interference model |  |  | Use different interference level for two CSI subframe set |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,22 | N/A |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/3 | N/A |  |
| CSI-RS configuration |  |  | 0 | N/A |  |
| Zero-Power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 3 / 0010000000000000 | 3 / 1000010000000000 |  |
| The second Zero-Power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 0 / 0010000000000000 | 3 / 1000010000000000 |  |
| CSI-IM configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 3 / 0010000000000000 | N/A |  |
| The second CSI-IM configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 0 / 0010000000000000 | N/A |  |
| CSI process configuration Signal/Interference/Reporting mode |  |  | CSI-RS/CSI-IM/PUCCH 1-1 |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (8 x 2) | Clause B.1 (2 x 2) |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0020 0000 0000 0001 0000 | 100000 |  |
| SNR (Note 3) |  | dB | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 |  |  |
| Modulation / Information bit payload |  |  | (Note4) | QPSK / 4392 (TBD) |  |
| Max number of HARQ transmissions |  |  | 1 | N/A |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note5) | N/A |  |
| PUCCH Report Type for CQI/second PMI |  |  | 2b | N/A |  |
| Physical channel for RI reporting |  |  | PUSCH | N/A |  |
| PUCCH Report Type for RI/ first PMI |  |  | TBD | N/A |  |
| Reporting periodicity |  | ms | Npd = TBD | N/A |  |
| CQI Delay |  | ms | TBD | N/A |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 | N/A |  |
| ri-ConfigIndex |  |  | 805 (Note 6) | N/A |  |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A |  |
| PDSCH scheduled sub-frames |  |  | Aligned with csi-MeasSubframeSet-r12 |  |  |
| Timing offset between TPs |  | us | 0 |  |  |
| Frequency offset between TPs |  | Hz | 0 |  |  |
| Note1: Reference measurement channel RC.10 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: REs for antenna ports 0 and 1 CRS have zero transmission power. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: N/A. Note 5: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#7 and #2. Note 6: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |  |

# Conclusion

In this contribution, we provide details on how to setup the demodulation and CSI test for eIMTA. The parameters are listed in Table 3~Table 7. We hope the group can consider these parameters in the eIMTA demodulation/CSI discussion.

# Reference

R4-145371, WF for demodulation and CSI test coverage of TDD eIMTA, CATT, Intel, Ericsson, Huawei, HiSilicon, Aug. 2014

R4-146430, Further discussion on eIMTA demodulation/CSI test cases, Ericsson, Oct. 2014

R4-143877, Way Forward on eIMTA UE demodulation and CSI tests, Intel Corporation, CATT, Huawei, HiSilicon, Qualcomm, May 2014.