3GPP TSG-RAN WG4 (Radio) Meeting #74	R4-150474

Athens, Greece, 9 – 13 February 2015

Agenda Item:	3.1

Source: 	Alcatel-Lucent, US Cellular

Title: 	TP for Rel-13 3DL TR 36.8xx: LTE Advanced Carrier Aggregation of Band Combination (2 + 2 + 12)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 2 and Band 12 was approved in RAN#64 [1]. In consequence of this WI approval, RAN4 should define the relevant requirements of the maximum output power relaxation and the reference sensitivity relaxation to support inter-band CA of Band 2, Band 2 and Band 12. In this contribution, both ∆TIB and ∆RIB values are proposed and the relaxation requirements are based on the RAN4 approved UE RF requirements for the inter-band Carrier Aggregation (2DL/1UL) of Band 2 and Band 12, and a text proposal is provided to include the values into the Rel-13 3DL CA TR [2].

# 2.	Text Proposal

<Start of change>

# 1	Scope

The present document is a technical report for 3DL Inter-band Carrier Aggregation under Rel-13 time frame. The purpose is to gather the relevant background information and studies in order to address 3DL Inter-band Carrier Aggregation requirements.

This TR covers relevant background information and studies in order to address 3DL Inter-band Carrier Aggregation requirements for the Rel-13 band combinations in Table 1-1.

Table 1-1: Release 13 3DL inter-band carrier aggregation combinations

| WI code | WI title |
| --- | --- |
| LTE_CA_B2_B2_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 2 and Band 12 |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

<New section>

## 6.X	LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 2, and Band 12

Table 6.X-1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-12 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |

Table 6.X-2: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Band or E-UTRA CA configuration | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_2A-2A-12A | CA_2A-2A |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  | 12 |  |  | Yes | Yes |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [z] when operating in single carrier mode

##### 6.X.2	ΔTIB,c and ΔRIB,c

For the UE which supports CA_2A-2A-12A the ΔTIB,c and ΔRIB,c  is defined for applicable bands in Table 6.X.2-1 and 6.X.2-2 respectively.

Table 6.X.2-1: ΔTIB,c for 3DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-2A-12A | 2 | 0.3 |
|  | 12 | 0.3 |

Table 6.X.2-2: ΔRIB,c for 3DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2A-2A-12A | 2 | 0 |
|  | 12 | 0 |

<End of change>

# References

[1]	RP-140647, “New WID: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 2, and Band 12”, U.S. Cellular.

[2]	R4-73AH-0074, “Rel-13 3DL CA TR 36.8xx version 0.2.0”, ZTE.