3GPP TSG-RAN WG4 (Radio) Meeting #74	R4-150475

Athens, Greece, 9 – 13 February 2015

Agenda Item:	7.21.2.3

Source: 	Alcatel-Lucent, US Cellular

Title: 	TP for Rel-13 3DL TR 36.8xx: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 2 + 12)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 2 and Band 12 was approved in RAN#64 [1]. The text proposal on the Coexistence Studies of Harmonics and Intermodulation Products for LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 2 and Band 12 [2] was approved into the Rel-12 3DL CA TR [3], while it should instead be put into the Rel-13 3DL CA TR [4]. In this contribution, we provide a text proposal to put the corresponding materials into the Rel-13 TR to avoid unnecessary confusion.

# 2.	Text Proposal

<Start of change>

##### 6.X.1	Co-existence studies for 1 UL/3 DL

The 3rd order IMD products caused in the BS by transmitting of Band 2, Band 2 and Band 12 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.X.1-1 below:

Table 6.X.1-1: Band 2, Band 2 and Band 12 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 1930 | 1990 | 1930 | 1990 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 669 |  |  | 806 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 669 |  |  | 806 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 3114 |  |  | 3251 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 4589 |  |  | 4726 |  |  |

It can be seen from Table 6.X.1-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (2 + 2 + 12) may fall into the BS receive band of Bands 12, 13, 14, 17, 28 and 44. However, Bands 28 and 44 are not intended for use in the same geographical area as Bands 2 and 12. Therefore, the focus here is on the 3rd order IMD products falling into Bands 12, 13, 14 and 17.

Note that the 3rd order IMD products may fall into the BS own (10 MHz) receive block within Band 12. Assume that the 3 carriers transmitted by the Band (2 + 2 + 12) BS are a, b and c MHz in channel bandwidth and d, e and f MHz from the lower edges of Bands 2 and 12 DL frequency bands as shown in Figure 6.X.1-1 below:

a MHz

b MHz

3rd CC

2nd CC

c MHz

1st CC

1930

729

d MHz

MHz

e MHz

f MHz

f1-low

f1-high

f2-low

f2-high

f3-low

f3-high

Figure 6.X.1-1: 3 carriers transmitted by the Band (2 + 2 + 12) BS

And the corresponding BS receive blocks are as shown in Figure 6.X.1-2 below:

a MHz

b MHz

3rd CC

2nd CC

c MHz

1st CC

1850

699

d MHz

MHz

e MHz

f MHz

f1-low

f1-high

f2-low

f2-high

f3-low

f3-high

Figure 6.X.1-2: 3 carriers received by the Band (2 + 2 + 12) BS

Then the 3rd order IMD products caused in the BS by transmitting of the 3 CC can be calculated as shown in Table 6.X.1-2 below:

Table 6.X.1-2: Band (2 + 2 + 12) BS DL 3rd order IMD products

| 3rd order IMD products | IMD frequency limits (MHz) |
| --- | --- |
| (f1-low + f2-low – f3-high) | 729 + d + e – f – c |
| (f1-high + f2-high – f3-low) | 729 + d + a + e + b – f |
| (f1-low + f3-low – f2-high) | 729 + d + f – e – b |
| (f1-high + f3-high – f2-low) | 729 + d + a + f + c – e |
| (f2-low + f3-low – f1-high) | 3131 + e + f – d – a |
| (f2-high + f3-high – f1-low) | 3131 + e + b + f + c – d |
| (f1-low + f2-low + f3-low) | 4589 + d + e + f |
| (f1-high + f2-high + f3-high) | 4589 + d + a + e + b + f + c |

Comparing the IMD frequency limits in Table 6.X.1-2 with the BS receive blocks in Figure 6.X.1-2, it can be deduced that the 3rd order IMD products caused in the BS by transmitting of the 3 CC will only fall into the BS own receive block of Band 12 under one of the transmit configurations shown in Table 6.X.1-3 below:

Table 6.X.1-3: Band (2 + 2 + 12) BS transmit configurations with 3rd IMD within Band 12 BS own receive blocks

| 1st CC channel bandwidth (a MHz) | 2nd CC channel bandwidth (b MHz) | 3rd CC channel bandwidth (c MHz) | CC position (d, e and f MHz) |
| --- | --- | --- | --- |
| 5 | 5 | 5 | 20 < f – e < 40 |
| 5 | 5 | 10 | 15 < f – e < 40 |
| 5 | 10 | 5 | 20 < f – e < 45 |
| 10 | 5 | 5 | 15 < f – e < 45 |
| 5 | 5 | 15 | 10 < f – e < 40 |
| 5 | 10 | 10 | 15 < f – e < 45 |
| 5 | 15 | 5 | 20 < f – e < 50 |
| 10 | 5 | 10 | 10 < f – e < 45 |
| 10 | 10 | 5 | 15 < f – e < 50 |
| 5 | 5 | 20 | 5 < f – e < 40 |
| 5 | 10 | 15 | 10 < f – e < 45 |
| 5 | 15 | 10 | 15 < f – e < 50 |
| 5 | 20 | 5 | 20 < f – e < 55 |
| 10 | 5 | 15 | 5 < f – e < 45 |
| 10 | 10 | 10 | 10 < f – e < 50 |
| 10 | 15 | 5 | 15 < f – e < 55 |
| 5 | 10 | 20 | 10 < f – e <= 40 |
| 5 | 15 | 15 | 15 < f – e <= 45 |
| 5 | 20 | 10 | 20 < f – e <= 50 |
| 10 | 5 | 20 | 5 < f – e <= 40 |
| 10 | 10 | 15 | 10 < f – e <= 45 |
| 10 | 15 | 10 | 15 < f – e <= 50 |
| 10 | 20 | 5 | 20 < f – e <= 55 |
| 5 | 15 | 20 | 15 < f – e <= 40 |
| 5 | 20 | 15 | 20 < f – e <= 45 |
| 10 | 10 | 20 | 10 < f – e <= 40 |
| 10 | 15 | 15 | 15 < f – e <= 45 |
| 10 | 20 | 10 | 20 < f – e <= 50 |
| 5 | 20 | 20 | 20 < f – e <= 40 |
| 10 | 15 | 20 | 15 < f – e <= 40 |
| 10 | 20 | 15 | 20 < f – e <= 45 |
| 10 | 20 | 20 | 20 < f – e <= 40 |

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 12, 13 or 14 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (2 + 2 + 12) BS transmitter does not share the same antenna with Band 12, 13 or 14 BS receiver. But there is not a large frequency gap between Band 12 DL and Band 17 UL, and hence Band 17 BS receiver desensitization may still be an issue.

Therefore, it is recommended that Band (2 + 2 + 12) BS transmitter should not share the same antenna with Band 17 BS receiver to prevent BS receiver desensitization, or with Band 12, 13 or 14 BS receiver or the own Band 12 BS receiver if the aforementioned BS transmit configurations are used unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 12, 13 or 14 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of change>

# References

[1]	RP-140647, “New WID: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 2, and Band 12”, U.S. Cellular.

[2]	R4-146844, “TP for TR 36.853: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 2 + 12)”, Alcatel-Lucent, US Cellular.

[3]	3GPP TS 36.853 v12.0.0, “LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) (Release 12)”.

[4]	R4-73AH-0074, “Rel-13 3DL CA TR 36.8xx version 0.2.0”, ZTE.