3GPP TSG-RAN WG4 #74	R4-150682

Athens, Greece, 9th Feb – 13th Feb 2015

Source: 	Ericsson

Title:	Link level simulation results for eIMTA PDSCH

Agenda Item:	6.2.3

Document for:	Discussion

# Introduction

In RAN4#73 meeting, the link level simulation assumption is updated a bit and the corresponding reference channel is updated. The updated is agreed in [1]. In this paper, we provide our link level results based on the updated.

# Simulation results

The simulation assumption is agreed in [1]. Based on agreements, there are two candidates for the reference channel, one is 16QAM CR 0.4 and the other one is QPSK CR 0.33. The results are given in Figure 1 and Figure 2, respectively.

Figure 1: Link level simulation results for 16QAM

Figure 2: Link level simulation results for QPSK

Based on the results, the target SNR@70 throughput for these two reference channel is tabulated in Table 1.

Table 1: Target SNR setup for different reference channel

|  | Target SNR@70% (ideal results) (dB) | Target SNR@70% (with implementation margin) (dB) |
| --- | --- | --- |
| 16QAM CR 0.4 | 2.3 | 3.8 |
| QPSK CR 0.33 | -3.5 | -2 |

# Conclusion

In this paper, link level simulation results are provided for eIMTA PDSCH. The target SNR with/without implementation margin is given in Table 1. We hope the group can consider these results in the final eIMTA PDSCH performance requirements.

# Appendix A

Table 1: eIMTA PDSCH test parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Transmission mode and antenna configuration |  |  | TM2, 2x2 medium |
| Propagation condition |  |  | EVA5 |
| UL/DL configuration in SIB 1 |  |  | 0 |
| Special subframe configuration |  |  | 4 |
| Downlink HARQ reference configuration |  |  | 5 |
| Dynamic UL/DL configuration in L1 signalling |  |  | For test case definition: Randomly selected with equal probability from {0, 1, 2, 3, 4, 5, 6}. In simulations, the random selection may be modelled as cyclical selection from {0, 1, 2, 3, 4, 5, 6}. |
| Subframe of dynamic UL/DL reconfiguration L1 transmission |  |  | The set of subframes to monitor the L1 reconfiguration DCI with UL/DL configuration indication for frame n includes subframes {1,5,6} in frame n-1 and subframe 0 in frame n. Subframes for reconfiguration DCI transmission are chosen in a random way on a per-DCI basis. |
| Periodicity of dynamic UL/DL reconfiguration L1 transmission |  | ms | 10 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Bandwidth |  | MHz | 10 |
| Cell ID |  |  | 0 |
| Inter-TTI distance |  |  | 1 |
| Number of HARQ processes per component carrier |  |  | 15 |
| Redundancy version coding sequence |  |  | {0, 1, 2, 3} |
| Max number of HARQ transmissions |  |  | 4 |
| Number of OFDM symbols for PDCCH |  |  | 2 |
| Test metric |  |  | SNR point @70% maximum throughput |
| Reference channel |  |  | Option 1: FRC #1 16QAM CR 0.4 (MCS12) (baseline) Option 2: FRC #2 QPSK CR 0.33 (MCS5) |

# Reference

- R4-147896, Way forward for eIMTA PDSCH demodulation test, Intel Corporation, CATT, Ericsson, Huawei, HiSilicon, Nov. 2014.