3GPP TSG-RAN WG4 (Radio) Meeting #75	R4-153749

Fukuoka, Japan, 25 – 29 May, 2015

Agenda Item:	7.15.2

Source: 	Alcatel-Lucent, Etisalat

Title: 	TP for TR 36.833-6-42: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced intra-band non-contiguous Carrier Aggregation in Band 42 for 3DL

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced intra-band non-contiguous Carrier Aggregation in Band 42 for 3DL was approved in RAN#67 [1]. One objective of the WI is to specify the specific RF requirements for Band 42 intra-band non-contiguous 3DL Carrier Aggregation (CA).

In this paper, we investigate the impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting intra-band non-contiguous CA in this band to the receiver of own or different BS, and provide a text proposal to record the findings in the Technical Report [2].

# 2.	Discussion

For LTE Advanced BS supporting intra-band contiguous or non-contiguous CA, the transceivers of the two or more contiguous or non-contiguous Component Carriers (CC) within the CA band would typically share a common radio front-end and antenna system. When BS for two or more bands share the same antenna, operators should ensure the following:

- One band BS transmitter Out-Of-Band Emissions (OOBE) including harmonics and IMD falling into the other BS receive band do not cause BS receiver desensitization; and one band BS transmit carrier powers do not result in the other band BS receiver blocking.

- Third order Passive Intermodulations (PIM) generated by BS antenna path (including antennas, cables and connectors) and caused by multiple transmit band carriers do not fall into one of multiple BS receive bands and result in BS receiver desensitization.

## 2.1	Harmonics and IMD Products

Harmonics are produced by the BS oscillator or generated in non-linear circuit elements; while IMD products are caused by the mixing of multiple frequencies in non-linear circuit elements. Wireless systems producing transmit signals within multiple frequency blocks must consider harmonics and IMD generation as BS receiver desensitization may occur when harmonics or IMD frequencies fall within the receive band of own or different BS. Usually the non-linear element is an amplifier or mixer, but RF connectors and BS antenna systems can also generate harmonics and IMDs due to imperfect physical contact or dissimilar metals used in their construction. Wireless BS are particularly sensitive to harmonics and IMD interference which fall upon uplink receiver channels that are in use by the BS. To limit the receiver desensitization to 0.4 dB, in-band harmonics and IMD interference must be limited to less than 10 dB below the receiver noise floor.

Harmonics and IMD are measured as the ratio in power levels between the distortion signal and the input signal in units of dBc. The magnitudes of the harmonics and IMD depend on the characteristics of the non-linear element, and the magnitude normally decreases as the order of the harmonics and IMD increases. For this reason, lower order harmonics and IMD products are considered the most damaging to wireless systems, while the higher order harmonics and IMD products are generally less damaging. Therefore, we will focus our investigation on the 2nd and 3rd order harmonics and IMD products caused by transmitting of non-contiguous DL carriers for Band 42 in the BS.

## 2.2	BS Harmonics and IMD for CA

The LTE operating bands currently defined in the 3GPP standards are provided in Table 1 below [3]:

Table 1: LTE frequency bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |  |  |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  |  |  | FDL_low – FDL_high |  |  |  |  |  |  |  |
| 1 | 1920 MHz | – |  | 1980 MHz |  | 2110 MHz |  | – |  |  | 2170 MHz |  | FDD |
| 2 | 1850 MHz | – |  | 1910 MHz |  | 1930 MHz |  | – |  |  | 1990 MHz |  | FDD |
| 3 | 1710 MHz | – |  | 1785 MHz |  | 1805 MHz |  | – |  |  | 1880 MHz |  | FDD |
| 4 | 1710 MHz | – |  | 1755 MHz |  | 2110 MHz |  | – |  |  | 2155 MHz |  | FDD |
| 5 | 824 MHz | – |  | 849 MHz |  | 869 MHz |  | – |  |  | 894MHz |  | FDD |
| 61 | 830 MHz | – |  | 840 MHz |  | 875 MHz |  | – |  |  | 885 MHz |  | FDD |
| 7 | 2500 MHz | – |  | 2570 MHz |  | 2620 MHz |  | – |  |  | 2690 MHz |  | FDD |
| 8 | 880 MHz | – |  | 915 MHz |  | 925 MHz |  | – |  |  | 960 MHz |  | FDD |
| 9 | 1749.9 MHz | – |  | 1784.9 MHz |  | 1844.9 MHz |  | – |  |  | 1879.9 MHz |  | FDD |
| 10 | 1710 MHz | – |  | 1770 MHz |  | 2110 MHz |  | – |  |  | 2170 MHz |  | FDD |
| 11 | 1427.9 MHz | – |  | 1447.9 MHz |  | 1475.9 MHz |  | – |  |  | 1495.9 MHz |  | FDD |
| 12 | 699 MHz | – |  | 716 MHz |  | 729 MHz |  | – |  |  | 746 MHz |  | FDD |
| 13 | 777 MHz | – |  | 787 MHz |  | 746 MHz |  | – |  |  | 756 MHz |  | FDD |
| 14 | 788 MHz | – |  | 798 MHz |  | 758 MHz |  | – |  |  | 768 MHz |  | FDD |
| 15 | Reserved |  |  |  |  | Reserved |  |  |  |  |  |  | FDD |
| 16 | Reserved |  |  |  |  | Reserved |  |  |  |  |  |  | FDD |
| 17 | 704 MHz | – |  | 716 MHz |  | 734 MHz |  | – |  |  | 746 MHz |  | FDD |
| 18 | 815 MHz | – |  | 830 MHz |  | 860 MHz |  | – |  |  | 875 MHz |  | FDD |
| 19 | 830 MHz | – |  | 845 MHz |  | 875 MHz |  | – |  |  | 890 MHz |  | FDD |
| 20 | 832 MHz | – |  | 862 MHz |  | 791 MHz |  | – |  |  | 821 MHz |  |  |
| 21 | 1447.9 MHz | – |  | 1462.9 MHz |  | 1495.9 MHz |  | – |  |  | 1510.9 MHz |  | FDD |
| 22 | 3410 MHz | – |  | 3490 MHz |  | 3510 MHz |  | – |  |  | 3590 MHz |  | FDD |
| 23 | 2000 MHz | – |  | 2020 MHz |  | 2180 MHz |  | – |  |  | 2200 MHz |  | FDD |
| 24 | 1626.5 MHz | – |  | 1660.5 MHz |  | 1525 MHz |  | – |  |  | 1559 MHz |  | FDD |
| 25 | 1850 MHz | – |  | 1915 MHz |  | 1930 MHz |  | – |  |  | 1995 MHz |  | FDD |
| 26 | 814 MHz | – |  | 849 MHz |  | 859 MHz |  | – |  |  | 894 MHz |  | FDD |
| 27 | 807 MHz | – |  | 824 MHz |  | 852 MHz |  | – |  |  | 869 MHz |  | FDD |
| 28 | 703 MHz | – |  | 748 MHz |  | 758 MHz |  | – |  |  | 803 MHz |  | FDD |
| 29 | N/A |  |  |  |  | 717 MHz | – |  |  | 728 MHz |  |  | FDD2 |
| 30 | 2305 MHz | – |  | 2315 MHz |  | 2350 MHz |  | – |  |  | 2360 MHz |  | FDD |
| 31 | 452.5 MHz |  | – |  | 457.5 MHz | 462.5 MHz |  |  | – |  |  | 467.5 MHz | FDD |
| ... |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 | 1900 MHz | – |  | 1920 MHz |  | 1900 MHz |  | – |  |  | 1920 MHz |  | TDD |
| 34 | 2010 MHz | – |  | 2025 MHz |  | 2010 MHz |  | – |  |  | 2025 MHz |  | TDD |
| 35 | 1850 MHz | – |  | 1910 MHz |  | 1850 MHz |  | – |  |  | 1910 MHz |  | TDD |
| 36 | 1930 MHz | – |  | 1990 MHz |  | 1930 MHz |  | – |  |  | 1990 MHz |  | TDD |
| 37 | 1910 MHz | – |  | 1930 MHz |  | 1910 MHz |  | – |  |  | 1930 MHz |  | TDD |
| 38 | 2570 MHz | – |  | 2620 MHz |  | 2570 MHz |  | – |  |  | 2620 MHz |  | TDD |
| 39 | 1880 MHz | – |  | 1920 MHz |  | 1880 MHz |  | – |  |  | 1920 MHz |  | TDD |
| 40 | 2300 MHz | – |  | 2400 MHz |  | 2300 MHz |  | – |  |  | 2400 MHz |  | TDD |
| 41 | 2496 MHz | – |  | 2690 MHz |  | 2496 MHz |  | – |  |  | 2690 MHz |  | TDD |
| 42 | 3400 MHz | – |  | 3600 MHz |  | 3400 MHz |  | – |  |  | 3600 MHz |  | TDD |
| 43 | 3600 MHz | – |  | 3800 MHz |  | 3600 MHz |  | – |  |  | 3800 MHz |  | TDD |
| 44 | 703 MHz | – |  | 803 MHz |  | 703 MHz |  | – |  |  | 803 MHz |  | TDD |
| Note 1: Band 6 is not applicable. Note 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |  |  |  |  |  |  |

It can be seen from Table 1 that Band 42 is operating at 3400 – 3600 MHz for both UL and DL. Therefore, the 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of non-contiguous Band 42 DL carriers can be calculated as shown in Table 2 below:

Table 2: Band 42 DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 3400 | 3600 |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -200 | 200 |
| 3rd order IMD products | (2*f-low – f-high) | (2*f-high – f-low) |
| IMD frequency limits (MHz) | 3200 | 3800 |
| 3rd order IMD products (with maximum channel bandwidth for contiguous CA) | (f-low – BWmax) | (f-high + BWmax) |
| IMD frequency limits (MHz) | 3360 | 3640 |

It can be seen from Table 2 that the 3rd order IMD products of BS transmitting non-contiguous carriers in Band 42 may fall into the BS receive band of Bands 22, 42 and 43.

## 2.3	Possible Remedies for BS Harmonic and IMD

Note that Band 22 is not intended for use in the same geographical area as Band 42, and the 3rd IMD products caused by BS supporting CA of Band 42 will not cause BS receiver desensitization with synchronized Band 42 or 43 TDD operations. Therefore, the focus here will be on the IMD falling into unsynchronized Band 42 or 43 BS receiver.

The possible remedies for harmonics and IMD caused by BS supporting non-contiguous CA in Band 42 into the BS receive band of unsynchronized Band 42 or 43 TDD operations are described below:

- Transmit Path Generated Harmonics and IMD: This type of harmonics and IMD is generated in the oscillator or as signals pass through the amplifier stages of the BS transmit path. The BS transmit path will generate harmonics and IMD which will appear at the BS equipment antenna connector (EAC) as spurious emissions. Spurious emissions appearing at the EAC and falling within the receive band of own or other BS will not be attenuated by the receive filter, and will pass into the receive path and desensitize the receiver. The 3rd order IMD products of non-contiguous Band 42 DL carriers falling within the receive band of unsynchronized Band 42 or 43 TDD operations can be mitigated by transmit filtering with not very sharp roll-off if there is sufficient frequency gap between Band 42 transmit frequency and unsynchronized Band 42 or 43 receive frequency.

- Receiver Path Generated Harmonics and IMD: This type of harmonics and IMD is generated when the BS transmit signals appearing at the EAC pass through the receive filter and are applied to the front end of the receiver chain. This type of interference can be mitigated through a combination of receive filter attenuation within the transmit band and high 2nd and 3rd order Input Intercept Point (IIP2 and IIP3) of the receive path. If there is a sufficient frequency gap between Band 42 transmit frequency and unsynchronized Band 42 or 43 receive frequency, the Band 42 or 43 BS receive filter attenuation in the Band 42 transmit frequency should be substantial enough to limit the level of the transmit signals applied to the receiver front end. And the high IIP3 of the BS receive path would further reduce the potential interference to well below the Band 42 or 43 BS receiver noise floor.

- Antenna Passive Intermodulation: Even passive systems such as antennas, duplexers or filter combiners can generate harmonics and IMD products via the non-linearities of dissimilar metals used in their construction, discontinuities in connectors or imperfections in the antenna system. This type of interference impacts all BS systems, but can be particularly troublesome when multiple BS operating in different frequency bands share a common antenna system. In the Band 42 case, the non-contiguous DL signals would be combined via a combiner and applied to a common antenna system, with the 3rd order IMD products fall within unsynchronized Band 42 or 43 receive band, and desensitize the BS receiver; in this case Band 42 BS transmitters should not share the same antenna with unsynchronized Band 42 or 43 BS receiver, unless the antenna path meets very stringent 3rd order PIM specification so that the 3rd order PIM will not cause Band 42 or 43 BS receiver desensitization. The following alternatives can be used to avoid antenna sharing of Band 42 BS transmitters with unsynchronized Band 42 or 43 BS receiver:

- Use separate antenna elements for the transmitter and receiver. This could be done with the higher frequency bands using state-of-the-art Adaptive Active Antenna (AAA). With the performance target of current antenna system and the extra coupling loss between the transmitter and receiver antenna elements, the harmonics and PIM products should be low enough in level that they would not cause BS receiver desensitization.

- Use separate transmitter chains and antenna elements for transmitting the Band 42 non-contiguous carriers. This technique could also be used for load balancing between the multiple transmit antennas of a BS.

# 3.	Conclusions

In this paper, we have investigated the impact of harmonics and IMD products caused by LTE Advanced BS supporting non-contiguous CA in Band 42 to the receiver of own or different BS. We have shown that the 3rd order IMD products caused by BS supporting non-contiguous CA in Band 42 may fall into the BS receive band of unsynchronized Band 42 or 43 TDD operations, hence BS receiver desensitization may be an issue.

However, with the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 42 or 43 BS receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that there is a sufficient frequency gap between Band 42 transmit frequency and unsynchronized Band 42 or 43 receive frequency, and Band 42 BS transmitters do not share the same antenna with unsynchronized Band 42 or 43BS receiver.

On the other hand, it is recommended that Band 42 BS transmitters should not share the same antenna with unsynchronized Band 42 or 43 BS receiver in order to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 42 or 43 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

# 4.	Text Proposal

<New section>

## 5.3	Specific BS RF requirements

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 42 non-contiguous DL carriers can be calculated as shown in Table 5.3-1 below:

Table 5.3-1: Band 42 non-contiguous DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 3400 | 3600 |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -200 | 200 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 3200 | 3800 |
| 3rd order IMD products (with maximum channel bandwidth for contiguous CA) | (f-low – BWmax) | (f-high + BWmax) |
| IMD frequency limits (MHz) | 3360 | 3640 |

It can be seen from Table 5.3-1 that the 2nd and 3rd harmonics as well as the 2nd IMD products of BS transmitting non-contiguous carriers in Band 42 do not fall into the BS receive band of any operating band, while the 3rd IMD products of BS supporting non-contiguous carrier aggregation of Band 42 may fall into the BS receive band of Bands 42 and 43. As stated in 36- and 37-series BS specifications, TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations, special co-existence requirements may apply that are not covered by the 3GPP specifications. Hence no specific coexistence issue is foreseen for Band 42 non-contiguous transmission.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 42 or 43 BS receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that there is a sufficient frequency gap between Band 42 transmit frequency and unsynchronized Band 42 or 43 receive frequency, and Band 42 BS transmitters do not share the same antenna with unsynchronized Band 42 or 43BS receiver.

On the other hand, it is recommended that Band 42 BS transmitters should not share the same antenna with unsynchronized Band 42 or 43 BS receiver in order to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 42 or 43 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of change>

# References

[1]	RP-150246, “New WI Proposal: LTE Advanced intra-band non-contiguous Carrier Aggregation in Band 42 for 3DL”, Huawei, Etisalat.

[2]	R4-153314, “TR 36.833-6-42 V0.1.0”, Huawei, Hisilicon.

[3]	3GPP TS 36.104 v12.7.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.