3GPP TSG-RAN WG4 (Radio) Meeting #76	R4-153988

Beijing, China, 24 – 28 August 2015

Agenda Item:	7.15.2

Source: 	Alcatel-Lucent, Verizon

Title: 	TP for TR 36.833-1-5: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced intra-band contiguous Carrier Aggregation in Band 5

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced intra-band contiguous Carrier Aggregation in Band 5 was approved in RAN#68 [1]. One objective of the WI is to specify the specific RF requirements for Band 5 intra-band contiguous 2DL Carrier Aggregation (CA).

In this paper, we investigate the impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting intra-band contiguous CA in this band to the receiver of own or different BS, and provide a text proposal to record the findings in the Technical Report [2].

# 2.	Discussion

For LTE Advanced BS supporting intra-band contiguous CA, the transceivers of the two or more contiguous Component Carriers (CC) within the CA band would typically share a common radio front-end and antenna system. When BS for two or more bands share the same antenna, operators should ensure the following:

- One band BS transmitter Out-Of-Band Emissions (OOBE) including harmonics and IMD falling into the other BS receive band do not cause BS receiver desensitization; and one band BS transmit carrier powers do not result in the other band BS receiver blocking.

- Third order Passive Intermodulations (PIM) generated by BS antenna path (including antennas, cables and connectors) and caused by multiple transmit band carriers do not fall into one of multiple BS receive bands and result in BS receiver desensitization.

## 2.1	Harmonics and IMD Products

Harmonics are produced by the BS oscillator or generated in non-linear circuit elements; while IMD products are caused by the mixing of multiple frequencies in non-linear circuit elements. Wireless systems producing transmit signals within multiple frequency blocks must consider harmonics and IMD generation as BS receiver desensitization may occur when harmonics or IMD frequencies fall within the receive band of own or different BS. Usually the non-linear element is an amplifier or mixer, but RF connectors and BS antenna systems can also generate harmonics and IMDs due to imperfect physical contact or dissimilar metals used in their construction. Wireless BS are particularly sensitive to harmonics and IMD interference which fall upon uplink receiver channels that are in use by the BS. To limit the receiver desensitization to 0.4 dB, in-band harmonics and IMD interference must be limited to less than 10 dB below the receiver noise floor.

Harmonics and IMD are measured as the ratio in power levels between the distortion signal and the input signal in units of dBc. The magnitudes of the harmonics and IMD depend on the characteristics of the non-linear element, and the magnitude normally decreases as the order of the harmonics and IMD increases. For this reason, lower order harmonics and IMD products are considered the most damaging to wireless systems, while the higher order harmonics and IMD products are generally less damaging. Therefore, we will focus our investigation on the 2nd and 3rd order harmonics and IMD products caused by transmitting of contiguous DL carriers for Band 5 in the BS.

## 2.2	BS Harmonics and IMD for CA

The LTE operating bands currently defined in the 3GPP standards are provided in Table 1 below [3]:

Table 1: LTE frequency bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |  |  |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  |  |  | FDL_low – FDL_high |  |  |  |  |  |  |  |
| 1 | 1920 MHz | – |  | 1980 MHz |  | 2110 MHz |  | – |  |  | 2170 MHz |  | FDD |
| 2 | 1850 MHz | – |  | 1910 MHz |  | 1930 MHz |  | – |  |  | 1990 MHz |  | FDD |
| 3 | 1710 MHz | – |  | 1785 MHz |  | 1805 MHz |  | – |  |  | 1880 MHz |  | FDD |
| 4 | 1710 MHz | – |  | 1755 MHz |  | 2110 MHz |  | – |  |  | 2155 MHz |  | FDD |
| 5 | 824 MHz | – |  | 849 MHz |  | 869 MHz |  | – |  |  | 894MHz |  | FDD |
| 6 (NOTE 1) | 830 MHz | – |  | 840 MHz |  | 875 MHz |  | – |  |  | 885 MHz |  | FDD |
| 7 | 2500 MHz | – |  | 2570 MHz |  | 2620 MHz |  | – |  |  | 2690 MHz |  | FDD |
| 8 | 880 MHz | – |  | 915 MHz |  | 925 MHz |  | – |  |  | 960 MHz |  | FDD |
| 9 | 1749.9 MHz | – |  | 1784.9 MHz |  | 1844.9 MHz |  | – |  |  | 1879.9 MHz |  | FDD |
| 10 | 1710 MHz | – |  | 1770 MHz |  | 2110 MHz |  | – |  |  | 2170 MHz |  | FDD |
| 11 | 1427.9 MHz | – |  | 1447.9 MHz |  | 1475.9 MHz |  | – |  |  | 1495.9 MHz |  | FDD |
| 12 | 699 MHz | – |  | 716 MHz |  | 729 MHz |  | – |  |  | 746 MHz |  | FDD |
| 13 | 777 MHz | – |  | 787 MHz |  | 746 MHz |  | – |  |  | 756 MHz |  | FDD |
| 14 | 788 MHz | – |  | 798 MHz |  | 758 MHz |  | – |  |  | 768 MHz |  | FDD |
| 15 | Reserved |  |  |  |  | Reserved |  |  |  |  |  |  | FDD |
| 16 | Reserved |  |  |  |  | Reserved |  |  |  |  |  |  | FDD |
| 17 | 704 MHz | – |  | 716 MHz |  | 734 MHz |  | – |  |  | 746 MHz |  | FDD |
| 18 | 815 MHz | – |  | 830 MHz |  | 860 MHz |  | – |  |  | 875 MHz |  | FDD |
| 19 | 830 MHz | – |  | 845 MHz |  | 875 MHz |  | – |  |  | 890 MHz |  | FDD |
| 20 | 832 MHz | – |  | 862 MHz |  | 791 MHz |  | – |  |  | 821 MHz |  |  |
| 21 | 1447.9 MHz | – |  | 1462.9 MHz |  | 1495.9 MHz |  | – |  |  | 1510.9 MHz |  | FDD |
| 22 | 3410 MHz | – |  | 3490 MHz |  | 3510 MHz |  | – |  |  | 3590 MHz |  | FDD |
| 23 | 2000 MHz | – |  | 2020 MHz |  | 2180 MHz |  | – |  |  | 2200 MHz |  | FDD |
| 24 | 1626.5 MHz | – |  | 1660.5 MHz |  | 1525 MHz |  | – |  |  | 1559 MHz |  | FDD |
| 25 | 1850 MHz | – |  | 1915 MHz |  | 1930 MHz |  | – |  |  | 1995 MHz |  | FDD |
| 26 | 814 MHz | – |  | 849 MHz |  | 859 MHz |  | – |  |  | 894 MHz |  | FDD |
| 27 | 807 MHz | – |  | 824 MHz |  | 852 MHz |  | – |  |  | 869 MHz |  | FDD |
| 28 | 703 MHz | – |  | 748 MHz |  | 758 MHz |  | – |  |  | 803 MHz |  | FDD |
| 29 | N/A |  |  |  |  | 717 MHz | – |  |  | 728 MHz |  |  | FDD (NOTE 2) |
| 30 | 2305 MHz | – |  | 2315 MHz |  | 2350 MHz |  | – |  |  | 2360 MHz |  | FDD |
| 31 | 452.5 MHz |  | – |  | 457.5 MHz | 462.5 MHz |  |  | – |  |  | 467.5 MHz | FDD |
| 32 | N/A |  |  |  |  | 1452 MHz |  | – |  |  | 1496 MHz |  | FDD (NOTE 2) |
| 33 | 1900 MHz | – |  | 1920 MHz |  | 1900 MHz |  | – |  |  | 1920 MHz |  | TDD |
| 34 | 2010 MHz | – |  | 2025 MHz |  | 2010 MHz |  | – |  |  | 2025 MHz |  | TDD |
| 35 | 1850 MHz | – |  | 1910 MHz |  | 1850 MHz |  | – |  |  | 1910 MHz |  | TDD |
| 36 | 1930 MHz | – |  | 1990 MHz |  | 1930 MHz |  | – |  |  | 1990 MHz |  | TDD |
| 37 | 1910 MHz | – |  | 1930 MHz |  | 1910 MHz |  | – |  |  | 1930 MHz |  | TDD |
| 38 | 2570 MHz | – |  | 2620 MHz |  | 2570 MHz |  | – |  |  | 2620 MHz |  | TDD |
| 39 | 1880 MHz | – |  | 1920 MHz |  | 1880 MHz |  | – |  |  | 1920 MHz |  | TDD |
| 40 | 2300 MHz | – |  | 2400 MHz |  | 2300 MHz |  | – |  |  | 2400 MHz |  | TDD |
| 41 | 2496 MHz | – |  | 2690 MHz |  | 2496 MHz |  | – |  |  | 2690 MHz |  | TDD |
| 42 | 3400 MHz | – |  | 3600 MHz |  | 3400 MHz |  | – |  |  | 3600 MHz |  | TDD |
| 43 | 3600 MHz | – |  | 3800 MHz |  | 3600 MHz |  | – |  |  | 3800 MHz |  | TDD |
| 44 | 703 MHz | – |  | 803 MHz |  | 703 MHz |  | – |  |  | 803 MHz |  | TDD |
| NOTE 1: Band 6 is not applicable. NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |  |  |  |  |  |  |

It can be seen from Table 1 that Band 5 is operating at 824 – 849 MHz for UL and 869 – 894 MHz DL. Therefore, the 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of contiguous Band 5 DL carriers can be calculated as shown in Table 2 below:

Table 2: Band 5 DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 869 | 894 |
| 2nd harmonics frequency limits (MHz) | 1738 | 1788 |
| 3rd harmonics frequency limits (MHz) | 2607 | 2682 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -25 | 25 |
| 3rd order IMD products | (2*f-low – f-high) | (2*f-high – f-low) |
| IMD frequency limits (MHz) | 844 | 919 |
| 3rd order IMD products (with maximum channel bandwidth for contiguous CA) | (f-low – BWmax) | (f-high + BWmax) |
| IMD frequency limits (MHz) | 854 | 909 |

It can be seen from Table 2 that the 2nd harmonics of BS transmitting in Band 5 may fall into the BS receive band of Bands 3, 4, 9 and 10, and the 3rd harmonics may fall into the BS receive band of Bands 38 and 41, while the 3rd IMD products caused by BS supporting contiguous CA in Band 5 may fall into the BS receive band of Bands 5, 8, 19, 20 and 26. Note that the calculation in Table 2 (except the last row) assumes the BS is transmitting with the whole 25 MHz DL frequency of Band 5. If the BS is only transmitting an up to 15 MHz DL in Band 5 as stated in the WIDS, then the 3rd IMD products will not fall into the BS receive band of Band 5, 19 or 26 as shown in the last row in Table 2.

## 2.3	Possible Remedies for BS Harmonic and IMD

Note that Bands 9, 10 and 20 are not intended for use in the same geographical area as Band 5. Therefore, the focus here is on the harmonics and IMD falling into the BS receive band of Bands 3, 4, 8, 38 and 41.

The possible remedies for harmonics and IMD caused by BS supporting contiguous CA in Band 5 into the BS receive band of Bands 3, 4, 8, 38 and 41 are described below:

- Transmit Path Generated Harmonics and IMD: This type of harmonics and IMD is generated in the oscillator or as signals pass through the amplifier stages of the BS transmit path. The BS transmit path will generate harmonics and IMD which will appear at the BS equipment antenna connector (EAC) as spurious emissions. Spurious emissions appearing at the EAC and falling within the receive band of own or other BS will not be attenuated by the receive filter, and will pass into the receive path and desensitize the receiver. The harmonics and  IMD of contiguous Band 5 DL carriers falling within the receive band of Bands 3, 4, 38 and 41 can be mitigated by transmit filtering with not very sharp roll-off if there is sufficient frequency gap between Band 5 DL and Bands 3, 4, 38 and 41 UL. However, the 3rd IMD products may still fall into the Band 8 BS receive band, which is overlapping with the Band 5 BS transmit band.

- Receiver Path Generated Harmonics and IMD: This type of harmonics and IMD is generated when the BS transmit signals appearing at the EAC pass through the receive filter and are applied to the front end of the receiver chain. This type of interference can be mitigated through a combination of receive filter attenuation within the transmit band and high 2nd and 3rd order Input Intercept Point (IIP2 and IIP3) of the receive path. Since there is a large frequency gap between Band 5 DL and Bands 3, 4, 38 and 41 UL, the Band 3, 4, 38 or 41 BS receive filter attenuation in the Band 5 BS transmit band should be substantial enough to limit the level of the transmit signals applied to the receiver front end. And the high IIP2 and IIP3 of the BS receive path would further reduce the potential interference to well below the Band 3, 4, 38 or 41 BS receiver noise floor. But again the 3rd IMD products may still fall into the Band 8 BS receive band, which is overlapping with the Band 5 BS transmit band.

- Antenna Passive Intermodulation: Even passive systems such as antennas, duplexers or filter combiners can generate harmonics and IMD products via the non-linearities of dissimilar metals used in their construction, discontinuities in connectors or imperfections in the antenna system. This type of interference impacts all BS systems, but can be particularly troublesome when multiple BS operating in different frequency bands share a common antenna system. In the Band 5 case, the contiguous DL signals would be combined via a combiner and applied to a common antenna system, with the harmonics and IMD fall within the BS receiver band of Bands 3, 4, 8, 38 and 41, and desensitize the BS receiver; in this case Band 5 BS transmitters should not share the same antenna with Band 3, 4, 8, 38 or 41 BS receiver, unless the antenna path meets very stringent 2nd and 3rd order PIM specification so that the PIM will not cause Band 3, 4, 8, 38 or 41 BS receiver desensitization. The following alternatives can be used to avoid antenna sharing of Band 5 BS transmitters with Band 3, 4, 8, 38 or 41 BS receiver:

- Use separate antenna elements for the transmitter and receiver. This could be done with the higher frequency bands using state-of-the-art Adaptive Active Antenna (AAA). With the performance target of current antenna system and the extra coupling loss between the transmitter and receiver antenna elements, the harmonics and PIM products should be low enough in level that they would not cause BS receiver desensitization.

- Use separate transmitter chains and antenna elements for transmitting the Band 5 contiguous carriers. This technique could also be used for load balancing between the multiple transmit antennas of a BS.

# 3.	Conclusions

In this paper, we have investigated the impact of harmonics and IMD products caused by LTE Advanced BS supporting contiguous CA in Band 5 to the receiver of own or different BS. We have shown that the harmonics and IMD products caused by BS supporting contiguous CA in Band 5 may fall into the BS receive band of Bands 3, 4, 8, 38 and 41, hence BS receiver desensitization may be an issue.

However, with the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 3, 4, 38 or 41 BS receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 5 BS transmitters do not share the same antenna with Band 3, 4, 38 or 41 BS receiver. However, the 3rd IMD products may still fall into the Band 8 BS receive band, which is overlapping with the Band 5 transmit band, and desensitize the receiver.

Therefore, in order to prevent BS receiver desensitization, it is recommended that Band 5 BS transmitters should not share the same antenna with Band 8 BS receiver, or with Band 3, 4, 38 or 41 BS receiver unless the antenna path meets very stringent 2nd and 3rd order PIM specification so that the PIM will not cause Band 3, 4, 38 or 41 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

# 4.	Text Proposal

<New section>

##### 6.X.1	Co-existence studies for 1 UL/2 DL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of contiguous Band 5 DL carriers can be calculated as shown in Table 6.X.1-1 below:

Table 6.X.1-1: Band 5 DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 869 | 894 |
| 2nd harmonics frequency limits (MHz) | 1738 | 1788 |
| 3rd harmonics frequency limits (MHz) | 2607 | 2682 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -25 | 25 |
| 3rd order IMD products | (2*f-low – f-high) | (2*f-high – f-low) |
| IMD frequency limits (MHz) | 844 | 919 |
| 3rd order IMD products (with maximum channel bandwidth for contiguous CA) | (f-low – BWmax) | (f-high + BWmax) |
| IMD frequency limits (MHz) | 854 | 909 |

It can be seen from Table 6.X.1-1 that the 2nd harmonics of BS transmitting in Band 5 may fall into the BS receive band of Bands 3, 4, 9 and 10, and the 3rd harmonics may fall into the BS receive band of Bands 38 and 41, while the 3rd IMD products caused by BS supporting contiguous CA in Band 5 may fall into the BS receive band of Bands 5, 8, 19, 20 and 26. Note that the calculation in Table 6.X.1-1 (except the last row) assumes the BS is transmitting with the whole 25 MHz DL frequency of Band 5. If the BS is only transmitting an up to 15 MHz DL in Band 5 as stated in the WIDS, then the 3rd IMD products will not fall into the BS receive band of Band 5, 19 or 26 as shown in the last row in Table 6.X.1-1.

Note that Bands 9, 10 and 20 are not intended for use in the same geographical area as Band 5. Therefore, the focus here is on the harmonics and IMD falling into the BS receive band of Bands 3, 4, 8, 38 and 41.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 3, 4, 38 or 41 BS receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 5 BS transmitters do not share the same antenna with Band 3, 4, 38 or 41 BS receiver. However, the 3rd IMD products may still fall into the Band 8 BS receive band, which is overlapping with the Band 5 transmit band, and desensitize the receiver.

Therefore, in order to prevent BS receiver desensitization, it is recommended that Band 5 BS transmitters should not share the same antenna with Band 8 BS receiver, or with Band 3, 4, 38 or 41 BS receiver unless the antenna path meets very stringent 2nd and 3rd order PIM specification so that the PIM will not cause Band 3, 4, 38 or 41 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of change>

# References

[1]	RP-151035, “New WID: LTE Advanced Carrier Aggregation of Intra-band Contiguous of Band 5”, Intel, Verizon.

[2]	R4-154116, “TR skeleton 36.833-1-05 LTE Advanced Intra-band contiguous Carrier Aggregation (CA) in Band 5”, Intel Corporation.

[3]	3GPP TS 36.104 v12.8.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.