3GPP TSG-RAN Working Group 4 Meeting #76	R4-154370

Beijing, China, 24th August – 28th August 2015

Source:	DISH Network

Title:	Band 66 UE Reference Sensitivity

Agenda item:	8.2.2

Document for:	Approval

## 1	Introduction

In this contribution, Band 66 UE filter data, which have been previously submitted [1-6], is further analyzed and some key observations are made.  Based on the analysis/observations, two proposals are presented on the UE REFSENS requirement for Band 66.

## 2	On Filter Data

There have been earlier contributions [5,6] on the UE reference sensitivity for Band 66.  One key discussion point from the contributions is whether a single REFSENS value for the band or a different, higher REFSENS value should be defined for the unpaired DL portion (2180-2200 MHz) compared to the paired DL portion (2110-2180 MHz).  It is noted that RAN4 has collected many Band 66 UE filter data points, as well as other SI band plan options, during the SI and current WI phases [1-4] and the following observations are made from the data:

Observation #1:  The average RX insertion loss increase by extending the DL from 70 MHz to 90 MHz is 0.5 dB

During the SI phase, filter data for the band plan candidates was collected and analyzed [1-3].  It has been shown that extending the DL band from 70 MHz to 90 MHz roughly incurs a 0.5 dB additional RX insertion loss.  This observation has also been confirmed by individual filter data submissions [2,3], as well as a recent public statement from a filter vendor [7].  Some vendor implementations even show no RX insertion loss impact extending from 70 MHz to 90 MHz; however, it is recognized that UE filter performance variations exist due to vendor-specific technology and implementation choices.

Observation #2:  Newer filter data indicates improving filter performance as designs for Band 66 are being optimized

The newer Band 66 filter data submitted recently [4] shows a noticeable improvement over the data collected during the SI phase and is evidence that filter UE vendors are optimizing their designs for Band 66.  Several vendors have also indicated further insertion loss and isolation improvements as they continue to optimize their Band 66 filter design.

Observation #3:  The data indicates virtually no impact on the UE TX insertion loss impact due to incorporating the unpaired DL (2180-2200 MHz) in the band

The filter data indicates virtually no impact on the UE TX insertion loss by extending the DL from 70 MHz to 90 MHz, thereby preserving the uplink performance, which is usually the limiting link.

## 3 				On Reference Sensitivity

On the need for a REFSENS relaxation on the unpaired DL portion of Band 66, it has been suggested [5,6] that such relaxation is required in order to preserve the Band 4 REFSENS requirement within the paired portion of Band 66.  The following observations on the UE reference sensitivity requirement for Band 66 are made:

Observation #4:  Some recent contributions on reference sensitivity are not based on the filter data on record

The Band 66 UE reference sensitivity must be, in part, based on the duplexer filter data presented to RAN4.  However, some recent contributions on the topic have cited data which neither aligns with the presented filter data nor is based on duplexer implementations.  In [5], it is claimed that, while the 70 MHz DL can meet the Band 4 UE REFSENS requirement, a REFSENS relaxation in the order of 1.5-2 dB is required for the 90 MHz DL.  However, the suggested relaxation clearly does not align with the duplexer filter data presented in RAN 4, which shows, on average, a 0.5 dB IL difference between the 70 MHz and 90 MHz DL.  In [6], two different Band 66 FBAR UE filter designs were presented; one optimized for the paired portion of the band and the other optimized for even response throughout the 90 MHz DL band.  It is questionable whether the data is indeed based on a duplexer design since FBAR duplexers typically yield much lower insertion losses than the one presented [4].  Multiplexer filter data incorporating Band 66 should be considered when appropriate inter-band carrier aggregation scenario(s) are introduced in order to determine if additional relaxations over the standalone band REFSENS would be required.

Observation #5:  There have been observations that 70+70 MHz filter can preserve Band 4 UE reference sensitivity

A 70+70 MHz paired band was one of the candidate band plans studied during the SI and some have observed that its filter implementations, which extend Band 10 by 10 MHz, can maintain the same reference sensitivity as Band 4 [8].  A similar assertion has been made in a recent contribution [5].  In addition, contributions on the ongoing 2 GHz MSS band plan work have indicated that 70 MHz DL band can meet the Band 1 REFSENS requirement [10,11], which is the same as Band 4.  In determining the UE REFSENS for Band 66, it needs to be factored that the filter data shows a 0.5 dB additional RX insertion loss for the 90 MHz DL over 70 MHz DL.

Observation #6:  The UE RX performance impact is minimal

In proposing a separate, higher REFSENS value for the unpaired portion of Band 66, some have suggested that the paired spectrum may be optimized at the expense of the unpaired spectrum which is used for SCell [5,6].  However, this suggestion of “intrinsic” importance of the paired spectrum over the unpaired within in a single band plan is unsubstantiated.  Band 66 consists of both paired and unpaired spectrum blocks involving multiple stakeholders and a fair, balanced approach would be to apply a uniform set of requirements for the entire band.

As for the potential impact on the paired spectrum, as noted earlier, incorporating the 20 MHz of unpaired DL into the band incurs a 0.5 dB additional insertion loss while not impacting the uplink insertion loss performance.  This 0.5 dB insertion loss is the averaged worst case (i.e. ETC) number and it is not clear how this loss is distributed between the paired and unpaired spectrum in the band.  In addition, some data presented on the issue actually shows there is no need to relax UE REFSEN due to extending the DL to 90 MHz, effectively negating the need for a separate REFSENS value.  Table 1 summarizes the data presented in [6] showing the RX IL performance of the aforementioned two Band 66 designs, along with the existing Band 4 performance from a filter vendor.  While the data seems to be based on multiplexers, rather than duplexers, it nonetheless shows that the Band 66 filter optimized for the entire 90 MHz DL band can still produce a compared performance to that of Band 4.  The data clearly does not justify the need to apply additional relaxation to the unpaired portion.

Table 1.  Different Band 66 filter design and Band 4 RX insertion loss comparison

|  | Max. RX IL, 2110-2180 MHz | Max. RX IL, 2180-2200 MHz |
| --- | --- | --- |
| Band 66, Optimized for 2110-2180 MHz | 2.0 dB | 2.6 dB |
| Band 66, Optimized for 2110-2200 MHz | 2.2 dB |  |
| Band 4, Shipping co-banded with Band 1 | 2.2 dB | N. A. |

Observation #7:  Having a single REFSENS value for the entire band does not limit implementation flexibility

The UE REFSENS requirement sets the minimum reference sensitivity performance baseline.  Having a single UE REFSENS value for Band 66 does not prevent operators and vendors from optimizing filter implementations.  UE designs and implementations which exhibit performance margins above the required minimum performance requirement can afford implementation flexibility and optimization as desired, without penalizing the minimum performance floor for the unpaired DL band.

Observation #8:  Defining two UE REFSENS values for the same band would be a departure from RAN4 methodology

Lastly, defining two UE REFSENS values for the same band would set a precedent in the future where specific motivations can lead to different requirements in a single band plan.  Multiple REFSENS values can impact other specifications such as ACS and blocking which are also based on REFSENS, and require additional core/test requirements.  It is noted that Band 4 is still an option for operators whose goal is to protect Band 4 performance.

## 4 				Conclusions and Proposals

One of the justifications for the AWS-Extension SI [9] was to “ensure common standard requirements across AWS-1/-3/-4 spectra unless there is a technical impediment.”  After studying various technical data, including the filter data discussed in this contribution, RAN4 agreed and RAN approved the 70+90 MHz band plan for the AWS extension band [1].  The 0.5 dB additional RX insertion loss incurred by adding 2180-2200 MHz to the band is not a new revelation; it was known, studied and debated during the SI.  By agreeing to the 70+90 MHz band plan, RAN4 has effectively endorsed that the benefits of incorporating the 2180-2200 MHz block into the band outweigh the potential cost.  Therefore, a common set of 3GPP standard requirements must be applied to the entire spectra as the original SI called for.  It is proposed that:

Proposal #1:  A single UE REFSENS value be defined for Band 66 covering the entire 90 MHz downlink band

In recommending a specific value for Band 66 UE REFSENS, it is important to understand Band 4 is a legacy band and strive to minimize Band 4 REFSENS impact in the new band if possible.  While extending Band 4 could have maximum performance implications, it is also recognized that the UE REFSENS requirement sets the minimum baseline.  The filter data indicates only a 0.5 dB additional RX insertion loss extending the downlink from 70 MHz to 90 MHz.  At the same time, the newly collected filter data [4] shows improving filter performance for Band 66.  Therefore, it is proposed that:

Proposal #2:  Band 66 UE REFSENS be the same as that of Band 4

## References

TR 36.849, “Study on Advanced Wireless Services (AWS) extension band for LTE v12.0.0”, Qualcomm, 3GPP RAN#66, Maui, U.S.A, December 2014

R4-145727, “Duplexer data for AWS extended band options”, DISH Network, 3GPP RAN4#72bis, Singapore, October 2014

R4-147747, “AWS-3 band plan considerations”, Qualcomm, 3GPP RAN4#73, San Francisco, U. S. A., November 2014

R4-153590, “TP for TR 36.869: Section 8.1 UE specific”, DISH Network, 3GPP RAN4#75, Fukuoka, Japan, May 2015

R4-152116, “Text proposal to TR 36.869: UE RX requirements”, Ericsson, 3GPP RAN4#74bis, Rio, Brazil, April 2015

R4-152969, “UE REFSENS for AWS extension”, Ericsson, 3GPP RAN4#75, Fukuoka, Japan, May 2015

Qorvo AWS extension public statement, April 2015

R4-145258, ““AWS3 Usage”, Intel, 3GPP RAN4#72, Dresden, Germany, August 2014

RP-141030, “New Study Item Proposal: AWS-Extension Band for LTE”, Verizon, US Cellular, AT&T, Qualcomm, Alcatel-Lucent, LGE, Motorola Mobility, Samsung, SouthernLINC Wireless, Intel, T-Mobile USA, NSN, Nokia, DISH Network, Broadcom, 3GPP RAN#64, Sophia Antipolis, France, June 2014

R4-140249, “Consideration of passband width for 1980-2010 MHz & 2170-2200 MHz Bands”, NTT DOCOMO, Inc., 3GPP RAN4#70, Prague, Czech Republic, February 2014

R4-140850, “Channel Arrangement for 2GHz MSS band”, NSN, Nokia Corporation, 3GPP RAN4#70, Prague, Czech Republic, February 2014